3GPP TSG-SA3 Meeting #95-LI	s3i240677

# Las Vegas, United States, 29th Oct 2024 - 1st Nov 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.127 | CR | 0252 | rev | 1 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | * |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Web RTC Access to IMS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | SA3LI (Ministère Economie et Finances) |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA3 |  |  |  |  |  |  |  |  |  |
| Work item code: | LI19 |  |  |  |  |  | Date: |  |  | 2024-10-03 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Lack of Web RTC Access to IMS architecture for LI (It has been already defined in TS 33.107) |  |  |  |  |  |  |  |  |
| Summary of change: |  | Define as IRI-POI the WWSF and eP-CSCF WWSF and eP-CSCF shall provide the IRI-POI |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Regulatory issue |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.3; 7.X; |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | * | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | * | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | * | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | s3i240645 |  |  |  |  |  |  |  |  |

*** Start of First Change ***

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

5GC	5G Core Network

5GMS	5G Media Streaming

5GS	5G System

AAnF	AKMA Anchor Function

AC	Application Client

ACR	Application Context Relocation

ADMF	LI Administration Function

AF	Application Function

AF_ID	Application Function Identity

AKA	Authentication and Key Agreement

A-KID	AKMA Key IDentifier

AKMA	Authentication and Key Management for Applications

AMF	Access and Mobility Management Function

AS	Application Server

AUSF	Authentication Server Function

BBIFF	Bearer Binding Intercept and Forward Function

BSS	Business Support System

CAG	Closed Access Group

CC	Content of Communication

CP	Control Plane

CPIM	Common Presence and Instant Messaging

CPS	Call Placement Service

CSI	Cell Supplemental Information

CSP	Communication Service Provider

CSR	Cell Site Report

CUPS	Control and User Plane Separation

DN	Data Network

DNAI	Data Network Access Identifier

DoNAS	Data over NAS

EAP	Extensible Authentication Protocol

EAS	Edge Application Server

ECGI	E-UTRAN Cell Global Identifier

eCNAM	Enhanced Calling Name

ECSP	Edge Computing Service Provider

E-CSCF	Emergency – Call Session Control Function

EDN	Edge Data Network

EEC	Edge Enabler Client

EECID	Edge Enabler Client IDentifier

EES	Edge Enabler Server

eIMS-AGW	enhanced IMS-AGW

eP-CSCF	enhanced P-CSCF

GPSI	Generic Public Subscription Identifier

HMEE	Hardware Mediated Execution Enclave

HR	Home Routed

IBCF	Interconnection Border Control Functions

ICF	Identity Caching Function

IEF	Identity Event Function

IMS-AGW	IMS Access Gateway

IM-MGW	IM Media Gateway

IP	Interception Product

IQF	Identity Query Function

IRI	Intercept Related Information

KAF	AKMA Application Key

KAKMA	AKMA Anchor Key

KID	Key IDentifier

KLI	Decryption key(s) for services encrypted by CSP-provided keys

KSF	Key Server Function

LAF	Location Acquisition Function

LALS	Lawful Access Location Services

LARF	Location Acquisition Requesting Function

LBO	Local Break Out

LEA	Law Enforcement Agency

LEMF	Law Enforcement Monitoring Facility

LI	Lawful Interception

LI CA	Lawful Interception Certificate Authority

LICF	Lawful Interception Control Function

LI_HI1	Lawful Interception Handover Interface 1

LI_HI2	Lawful Interception Handover Interface 2

LI_HI3	Lawful Interception Handover Interface 3

LI_HI4	Lawful Interception Handover Interface 4

LI_HILA	Lawful Interception Handover Interface Location Acquisition

LI_HIQR	Lawful Interception Handover Interface Query Response

LIID	Lawful Interception Identifier

LIPF	Lawful Interception Provisioning Function

LIR	Location Immediate Request

LI_SI	Lawful Interception System Information Interface

LISSF	Lawful Interception State Storage Function

LI_ST	Lawful Interception State Transfer Interface

LI_T1	Lawful Interception Internal Triggering Interface 1

LI_T2	Lawful Interception Internal Triggering Interface 2

LI_T3	Lawful Interception Internal Triggering Interface 3

LI_X0	Lawful Interception Internal Interface 0

LI_X1	Lawful Interception Internal Interface 1

LI_X2	Lawful Interception Internal Interface 2

LI_X2_LA	Lawful Interception Internal Interface 2 Location Acquisition

LI_X3	Lawful Interception Internal Interface 3

LI_X3A	Lawful Interception Internal Interface 3 Aggregator

LI_XEM1	Lawful Interception Internal Interface Event Management Interface 1

LI_XER	Lawful Interception Internal Interface Event Record

LI_XLA	Lawful Interception Internal Interface Location Acquisition

LI_XQR	Lawful Interception Internal Interface Query Response

LMF	Location Management Function

LMISF	LI Mirror IMS State Function

LMISF-CC	LMISF for the handling of CC

LMISF-IRI	LMISF for the handling of IRI

LTF	Location Triggering Function

MA	Multi-Access

MANO	Management and Orchestration

MDF	Mediation and Delivery Function

MDF2	Mediation and Delivery Function 2

MDF3	Mediation and Delivery Function 3

MRFP	Multimedia Resource Function Processor

MSRP	Message Session Relay Protocol

N3A	Non-3GPP Access

N3IWF	Non 3GPP Inter Working Function

N9HR	N9 Home Routed

NAS	Non-Access Stratum

NCGI	NR Cell Global Identity

NEF	Network Exposure Function

NFV	Network Function Virtualisation

NFVI	Network Function Virtualisation Infrastructure

NFVO	Network Function Virtualisation Orchestrator

NIDD	Non-IP Data Delivery

NPLI	Network Provided Location Information

NR	New Radio

NRF	Network Repository Function

NSSF	Network Slice Selection Function

NWDAF	Network Data Analytics Function

OSS	Operations Support System

PAG	POI Aggregator

PCF	Policy Control Function

P-CSCF	Proxy - Call Session Control Function

PEI	Permanent Equipment Identifier

PGW	PDN Gateway

PGW-C	PDN Gateway Control Plane

PGW-U	PDN Gateway User Plane

POI	Point Of Interception

PLMN	Public Land Mobile Network

PTC	Push to Talk over Cellular

RCD	Rich Call Data

RCS	Rich Communication Suite

S8HR	S8 Home Routed

SCEF	Service Capability Exposure Function

SCS	Service Capability Server

SGW	Serving Gateway

SGW-C	Serving Gateway Control Plane

SGW-U	Serving Gateway User Plane

SHAKEN	Signature-based Handling of Asserted information using toKENs

SIRF	System Information Retrieval Function

S-CSCF	Serving - Call Session Control Function

SIP	Session Initiation Protocol

SMF	Session Management Function

SMSF	SMS-Function

STF	Security Terminating Function

STIR 	Secure Telephony Identity Revisited

SUCI	Subscriber Concealed Identifier

SUPI	Subscriber Permanent Identifier

TAI	Tracking Area Identity

TF	Triggering Function

TLS	Transport Layer Security

TNGF	Trusted Non-3GPP Gateway Function

TrGW	Transit Gateway

TWIF	Trusted WLAN Interworking Function

UDM	Unified Data Management

UDR	Unified Data Repository

UDSF	Unstructured Data Storage Function

UPF	User Plane Function

VNF	Virtual Network Function

VNFC	Virtual Network Function Component

WAF	WebRTC Authorisation Function

W-AFG	Wireline Access Gateway Function

WIC	WebRTC IMS Client

WWSF	WebRTC Web Server Function

xCC	LI_X3 Content of Communication

xIRI	LI_X2 Intercept Related Information

*** End of First Change ***

*** Start of Second Change ***

## 7.X	LI at eP-CSCF for WebRTC access to the IMS

### 7.X.1	Background

Web Real-Time Communication (WebRTC) adds real-time communication to the browser with a focus on peer-to-peer communication. It enables multimedia interaction capabilities in web browsers on any platform without installing a specific plug-in or using a third-party application. WebRTC leaves the signalling plan to service developers, who are free to implement any call control protocol, e.g., SIP over secure WebSocket, XMPP (Extensible Messaging and Presence Protocol) over WebSocket, etc. The only requirement is that the session negotiation data be represented by the SDP (Session Description Protocol). WebRTC clients may have access to IMS through mediation functions for signalling and media. The enhanced P-CSCF (eP-CSCF) is the endpoint for the signalling connection from the WebRTC IMS Client (WIC) and is located in the operator network. The enhanced IMS-AGW (eIMS-AGW) is the endpoint for the media connection from the WIC. eP-CSCF shall adhere to all LI requirements pertaining to the P-CSCF in addition to the LI requirements pertaining to the support of WebRTC access to IMS.

Figure U.1.2-1 in clause U.1.2 of TS 23.228 [13] gives an overview of WebRTC IMS architecture.

The WebRTC Web Server Function (WWSF) is the initial point of contact in the Web that controls access to the IMS communications services for the user. It provides a web page, which is the user interface (UI) for the user. The WebRTC Authorisation Function (WAF) creates an authentication token for the user and deliver it to the WWSF. The WAF is used in the case of IMS registration scenario using Web Authentication and is not used in the case of IMS registration scenario using IMS Authentication.

NOTE:	The LI aspects when the WWSF is provided by third party, is not addressed in the present document.

### 7.X.2	Architecture

The WWSF and eP-CSCF shall provide the IRI-POI function. Figure 7.X.2-1 gives a reference point representation of the LI architecture with WWSF and eP-CSCF as CP NFs providing the IRI-POI function for WebRTC access to the IMS. This LI architecture is valid in non-roaming and roaming situations.

Figure 7.X.2-1: LI architecture for WebRTC access to the IMS

### 7.X.3	Target identities

The LIPF present in the ADMF provisions the intercept information associated with the following target identities to the IRI-POI present in the WWSF:

-	IMPU.

-	IMPI.

-	Web Identity.

The LIPF present in the ADMF provisions the intercept information associated with the following target identities to the IRI-POI present in the eP-CSCF:

-	IMPU.

-	IMPI.

### 7.X.4	IRI events

The IRI-POI in the WWSF shall generate xIRI when it detects the following specific events or information:

-	WIC authorization.

-	Start of interception with already authorized WIC

The IRI-POI in the eP-CSCF shall generate xIRI when it detects the following specific events or information:

-	Encapsulated signalling message

The WIC authorization xIRI is generated when the IRI-POI present in the WWSF detects that the WIC uses a Web identity to authenticate with the WWSF and obtains an authorization token.

The start of interception with already authorized WIC xIRI is generated when the IRI-POI present in the WWSF detects that interception is activated on the target WIC that has already been authenticated using its Web identity.

The encapsulated signalling message xIRI is generated when the IRI-POI present in the eP-CSCF detects that a signalling message (e.g., SIP over WebSocket, XMPP over WebSocket, etc) is received from, or sent to, a target WIC.

*** End of Second Change ***

*** End of Last Change ***