3GPP TSG-SA3 Meeting #95-LI	s3i240689

# Las Vegas, United States, 29th Oct 2024 - 1st Nov 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.127 | CR | 0261 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | LIHI1 and LIID usage |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | SA3-LI (OTD_US) |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA3 |  |  |  |  |  |  |  |  |  |
| Work item code: | LI19 |  |  |  |  |  | Date: |  |  | 2024-10-21 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Clause 5.4.3 presumes that the LEA provides the LIID to the CSP which is then used in the network and used in the HI2/3 interfaces when delivering intercept product to the LEA. In some regions, the LEA does not provide a unique interception identifier to the CSP, at least not one that is unique within the LEA’s country or even within the CSP’s network. In such, cases, the CSP converts any LEA provided interception identifier into a CSP unique LIID which is then used in the network and delivered over HI2/3. This value is also provided to the LEA at intercept request time (e.g., via manual/non-standard HI1 like interface). This last case appears to be excluded by the text of last bullet of the second paragraph of 5.4.3. This should be corrected to allow the alternate operation. This approach also aligns with the description in H.2.3.2 of ETSI TS 103 120. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In addition, the last paragraph needs to be updated to indicate that an LIHI1 “electronic” interface needs to support use of TS 103 120. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The standard would not be useable in certain regions where the LIID allocation is performed by the CSP rather than by the LEA. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## **** START OF FIRST CHANGE ****

### 5.4.3	Interface LI_HI1

LI_HI1 is used to send warrant and other interception request information from the LEA to the CSP. This interface may be electronic or may be an offline manual process depending on national warranty processes.

The following are some of the information elements sent over this interface:

-	Target identifier: used to identify the communications to be intercepted.

-	Type of intercept: used to indicate whether IRI only, CC only, or both IRI and CC, is to be delivered to the LEMF.

-	Location acquisition: used to indicate whether location acquisition is required (in addition to lawful interception for other services, only location acquisition, or no location acquisition) and the type of delivery (via LI_HILA, via LI_HI2, or both).

-	Service scoping: used to identify the service (e.g. voice, packet data, messaging, target positioning) to be intercepted.

-	Filtering criteria: used to provide additional specificity for the interception (e.g. for bandwidth optimization).

-	LEMF address: used to deliver the Interception Product.

-	An LEA provided interception identifier may be used unchanged as the Lawful Interception Identifier (LIID) or converted to an LIID by the CSP. The LIID is then used to associate the issued warrant with the Interception Product. When converted to an LIID, the CSP provides the LIID to the LEA.

LI_HI1 electionic interfaces shall support the use of ETSI TS 103 120 [7] for communication of warrant information between the LEA and CSP. However, default configurations, information element formats and other parameters as defined in the present document shall apply regardless of generic default options specified in ETSI TS 103 120 [7].

## **** END OF ALL CHANGES ****