3GPP SA3LI#96	s3i250029

# 28-31 January 2025, Sophia Antipolis (France)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.128 | CR | 0712 | rev | - | Current version: | 17.15.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on AMFIdentifierAssociation and MMEIdentifierAssociation (x)IRI |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | SA3-LI (Ericsson) |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA3 |  |  |  |  |  |  |  |  |  |
| Work item code: | LI17 |  |  |  |  |  | Date: |  |  | 2025-01-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | TS 33.127 states in clauses 6.2.2.6 and 6.3.2.5.1 that the identifier association xIRI shall include an association change type indication. This is not included in the related (x)IRIs specified in TS 33.128. A clarification is needed on a specific traffic case triggering the Identifier association (x)IRIs. A wrong internal reference needs correction. |  |  |  |  |  |  |  |  |
| Summary of change: |  | A new field is specified in a backward compatible way in the payload for AMFIdentifierAssociation and MMEIdentifierAssociation records. It is clarified that the (x)IRI shall be triggered by the AMF regardless of whether the CONFIGURATION UPDATE (5G-GUTI) or REGISTRATION ACCEPT procedure is subsequently successfully completed or not. Similarly, it is clarified that the (x)IRI shall be triggered by the MME regardless of whether the ATTACH ACCEPT, TRACKING AREA UPDATE ACCEPT or GUTI REALLOCATION COMMAND procedure is subsequently successfully completed or not. The internal reference is corrected. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misalignment between LI stage 2 and stage 3 specification; possible inconsistency in the information reported to the LEMF in case of identifier association xIRI. A reference to a wrong normative annex will remain. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.2.7; 6.3.2.2.2; 6.3.2.3.3; Annex A |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR is associated with the following changes in the Forge: Merge request: !303 Comit Hash: 7e77cc152aa1dd45910102640d14bf21bf20e2b4 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*** FIRST MAIN DOCUMENT CHANGE ***

##### 6.2.2.2.7	AMF identifier association

The IRI-POI present in the AMF shall generate an xIRI containing an AMFIdentifierAssociation record when the IRI-POI present in the AMF detects a new identifier association for a UE matching one of the target identifiers provided via LI_X1.

In the case where the IRI-POI present in the AMF detects that a REGISTRATION ACCEPT message or a CONFIGURATION UPDATE (5G-GUTI) message as defined in TS 24.501 [13] has been sent by the AMF towards a target UE, the IRI-POI present at the AMF shall immediately generate an AMFIdentifierAssociation xIRI. This xIRI shall be generated regardless of whether the CONFIGURATION UPDATE (5G-GUTI) or REGISTRATION ACCEPT procedure is subsequently successfully completed or not.

The IRI-POI present in the AMF shall include the deassociationIndicator field when the IRI-POI present in the AMF detects an identifier de-association for a UE matching one of the target identifiers provided via LI_X1.

NOTE 1:	The xIRI indicating a de-association of 5G-GUTI from a SUPI is only generated if a new 5G-GUTI is not allocated to a SUPI to update a previous association (e.g. at inter-AMF handover).

Generation of this record is subject to this record type being enabled for a specific target (see clause 6.2.2.2.1).

Table 6.2.2-6: Payload for AMFIdentifierAssociation record

| Field name | Description | M/C/O |
| --- | --- | --- |
| sUPI | SUPI associated with the procedure (see NOTE 1). | M |
| sUCI | SUCI used in the procedure, if applicable and if available. | C |
| pEI | PEI used in the procedure, if available (see NOTE 1). | C |
| gPSI | GPSI used in the procedure, if available (see NOTE 1). | C |
| gUTI | 5G-GUTI used in the procedure, see TS 24.501 [13] clause 9.11.3.4. | M |
| location | Location information available when identifier association occurs. Encoded as a userLocation parameter (location>locationInfo>userLocation) and, when Dual Connectivity is activated, as an additionalCellIDs parameter (location>locationInfo>additionalCellIDs), see Annex A. | M |
| fiveGSTAIList | List of tracking areas associated with the registration area within which the UE is current registered, see TS 24.501 [13], clause 9.11.3.9. (see NOTE 2) | C |
| deassociationIndicator | Boolean which indicates if the xIRI is generated due to a de-association. This shall be encoded either as "deassociation" = True, or “association” = False. Shall be provided. This parameter is conditional only for backwards compatibility. | C |
| NOTE 1: SUPI shall always be provided, in addition to the warrant target identifier if different to SUPI. Other identifiers shall be provided if available. NOTE 2: List shall be included each time there is a change to the registration area. |  |  |

The IRI-POI present in the AMF generating an xIRI containing an AMFIdentifierAssociation record shall set the Payload Direction field in the PDU header to not applicable (Direction Value 5, see ETSI TS 103 221-2 [8] clause 5.2.6).

*** NEXT MAIN DOCUMENT CHANGE ***

##### 6.3.2.2.2	MME identifier association

The IRI-POI present in the MME shall generate an xIRI containing an MMEIdentifierAssociation record when the IRI-POI present in the MME detects a new identifier association for a UE matching one of the target identifiers provided via LI_X1. Generation of this record is subject to this record type being enabled for a specific target (see clause 6.3.2.2.1).

In the case where the IRI-POI present in the MME detects that an ATTACH ACCEPT, TRACKING AREA UPDATE ACCEPT (including a GUTI) or GUTI REALLOCATION COMMAND message as defined in TS 24.301 [51] has been sent by the MME towards a target UE, the IRI-POI present at the MME shall immediately generate a MMEIdentifierAssociation xIRI. This xIRI shall be generated regardless of whether the ATTACH ACCEPT, TRACKING AREA UPDATE ACCEPT or GUTI REALLOCATION COMMAND procedure is subsequently successfully completed or not.

The IRI-POI present in the MME shall include the deassociationIndicator field when the IRI-POI present in the MME detects an identifier de-association for a UE matching one of the target identifiers provided via LI_X1.

NOTE 1:	The xIRI indicating a de-association of a GUTI from an IMSI is only generated if a new GUTI is not allocated to an IMSI to update a previous association.

Table 6.3.2-1: Payload for MMEIdentifierAssociation record

| Field name | Description | M/C/O |
| --- | --- | --- |
| iMSI | IMSI associated with the procedure. (see NOTE 1). | M |
| iMEI | IMEI used in the procedure, if available (see NOTE 1). | C |
| mSISDN | MSISDN used in the procedure, if available (see NOTE 1). | C |
| gUTI | LTE GUTI used in the procedure. | M |
| location | Location information available when identifier association occurs. Shall include all location information for the target UE available at the MME encoded as one of the following (see NOTE 3): - ePSUserLocationInformation parameter (location>fourGLocationInfo>ePSUserLocationInformation). - ePSLocationInformation parameter (location>fourGLocationInfo>ePSLocationInformation). When Dual Connectivity is activated, the additionalCellIDs parameter (location>fourGLocationInfo>ePSLocationInformation>mMELocationInformation>additionalCellIDs) shall also be populated, see clause 7.3.3 and Annex A. | M |
| tAIList | List of tracking areas associated with the registration area within which the UE is current registered. (see NOTE 2). | C |
| deassociationIndicator | Boolean which indicates if the xIRI is generated due to a de-association. This shall be encoded either as "deassociation" = True, or “association” = False. Shall be provided. This parameter is conditional only for backwards compatibility. | C |
| NOTE 1: IMSI shall always be provided, in addition to the warrant target identifier if different to IMSI. Other identifiers shall be provided if available. NOTE 2: List shall be included each time there is a change to the registration area. NOTE 3: The location information was sent as a userLocation (location>locationInfo>userLocation) between versions 17.0.0 and 17.7.0 of the present document. The location information may also be present in this field for backwards compatibility. |  |  |

The IRI-POI present in the MME generating an xIRI containing an MMEIdentifierAssociation record shall set the Payload Direction field in the PDU header to not applicable (Direction Value 5, see ETSI TS 103 221-2 [8] clause 5.2.6).

When transmitting the xIRI, the IRI-POI present in the MME shall set the payload format to 2, and provide the payload as a BER-encoded TS33128Payloads.XIRIPayloads structure.

*** NEXT MAIN DOCUMENT CHANGE ***

##### 6.3.2.3.3	Option B and Option C

For all messages except MMEIdentifierAssociation, the IRI messages shall include an IRI payload encoded according to TS 33.108 [12] Annex B.9.

The MDF2 shall encode the correct value of LIID in the IRI message, replacing the value "LIIDNotPresent" given in the xIRI (see clause 6.3.2.2).

For MMEIdentifierAssociation messages, the IRI message shall be encoded as an IRIEvent structure according to Annex A and used to populate the threeGPP33128DefinedIRI field in ETSI TS 102 232-7 [10] clause 15.

*** END OF MAIN DOCUMENT CHANGES ***

***FIRST ATTACHMENT CHANGE ***

START OF CHANGE 1

---a/33128/r17/TS33128Payloads.asn
+++b/33128/r17/TS33128Payloads.asn

@@ -3861,23 +3861,25 @@ PDSRSummaryTrigger ::= ENUMERATED

3861	3861

3862	3862		AMFIdentifierAssociation ::= SEQUENCE

3863	3863		{

3864		-	    sUPI             [1] SUPI,

3865		-	    sUCI             [2] SUCI OPTIONAL,

3866		-	    pEI              [3] PEI OPTIONAL,

3867		-	    gPSI             [4] GPSI OPTIONAL,

3868		-	    gUTI             [5] FiveGGUTI,

3869		-	    location         [6] Location,

3870		-	    fiveGSTAIList    [7] TAIList OPTIONAL

3864	+	    sUPI                    [1] SUPI,

3865	+	    sUCI                    [2] SUCI OPTIONAL,

3866	+	    pEI                     [3] PEI OPTIONAL,

3867	+	    gPSI                    [4] GPSI OPTIONAL,

3868	+	    gUTI                    [5] FiveGGUTI,

3869	+	    location                [6] Location,

3870	+	    fiveGSTAIList           [7] TAIList OPTIONAL,

3871	+	    deassociationIndicator  [8] BOOLEAN OPTIONAL

3871	3872		}

3872	3873

3873	3874		MMEIdentifierAssociation ::= SEQUENCE

3874	3875		{

3875		-	    iMSI        [1] IMSI,

3876		-	    iMEI        [2] IMEI OPTIONAL,

3877		-	    mSISDN      [3] MSISDN OPTIONAL,

3878		-	    gUTI        [4] GUTI,

3879		-	    location    [5] Location,

3880		-	    tAIList     [6] TAIList OPTIONAL

3876	+	    iMSI                    [1] IMSI,

3877	+	    iMEI                    [2] IMEI OPTIONAL,

3878	+	    mSISDN                  [3] MSISDN OPTIONAL,

3879	+	    gUTI                    [4] GUTI,

3880	+	    location                [5] Location,

3881	+	    tAIList                 [6] TAIList OPTIONAL,

3882	+	    deassociationIndicator  [7] BOOLEAN OPTIONAL

3881	3883		}

3882	3884

3883	3885		-- =================================

END OF CHANGE 1

***END OF ATTACHMENT CHANGES ***