14tTD172

ETSI TS 187 005 V2.0.2 (2008-02)

Technical Specification

Telecommunications and Internet converged Services and Protocols for Advanced Networking (TISPAN);

NGN Release 2 Lawful Interception;

Stage 2 definition

Reference

RTS/TISPAN-07031-NGN-R2

Keywords

IP, Lawful Interception, security, telephony

ETSI

650 Route des Lucioles

F-06921 Sophia Antipolis Cedex - FRANCE

Tel.: +33 4 92 94 42 00   Fax: +33 4 93 65 47 16

Siret N° 348 623 562 00017 - NAF 742 C

Association à but non lucratif enregistrée à la

Sous-Préfecture de Grasse (06) N° 7803/88

Important notice

Individual copies of the present document can be downloaded from:
http://www.etsi.org

The present document may be made available in more than one electronic version or in print. In any case of existing or perceived difference in contents between such versions, the reference version is the Portable Document Format (PDF). In case of dispute, the reference shall be the printing on ETSI printers of the PDF version kept on a specific network drive within ETSI Secretariat.

Users of the present document should be aware that the document may be subject to revision or change of status. Information on the current status of this and other ETSI documents is available at http://portal.etsi.org/tb/status/status.asp

If you find errors in the present document, please send your comment to one of the following services:
http://portal.etsi.org/chaircor/ETSI_support.asp

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© European Telecommunications Standards Institute 2007.

All rights reserved.

DECTTM, PLUGTESTSTM and UMTSTM are Trade Marks of ETSI registered for the benefit of its Members.
TIPHONTM and the TIPHON logo are Trade Marks currently being registered by ETSI for the benefit of its Members. 3GPPTM is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners.

Contents

Intellectual Property Rights	4

Foreword	4

1	Scope	5

2	References	5

3	Definitions, and abbreviations	6

3.1	Definitions	6

3.2	Abbreviations	7

4	Interception in the NGN	7

4.1	LI architecture model	7

4.2	LI reference model	8

4.3	Content of interception	9

5	Interception of non-IMS PSTN/ISDN Emulation Subsystem (PES)	9

5.1	Architecture for interception of PES	9

6	Interception of IMS	10

6.1	Architecture for interception of IMS	10

6.2	Activation, deactivation and interrogation	11

6.3	Intercept Related Information	11

6.4	Content of Communication	11

7	Identification of target of interception	12

7.1	ISDN/PSTN services	12

7.2	IMS services	12

8	Security considerations	12

Annex A (normative):	Endorsement statement for TS 133 107	13

Annex B (informative):	Endorsement statement for TS 133 108	14

Annex C (informative):	Endorsement statement for TS 102 232	16

Annex D (informative):	Endorsement statement for ES 201 671	17

Annex E (informative):	ISDN/PSTN LI reference configurations	19

Annex F (informative):	Selection of handover interface	22

Annex G (informative):	Bibliography	23

G.1	ETSI Specifications	23

G.2	3GPP specifications	23

G.3	ITU-T specifications	24

G.4	IETF specifications	24

G.5	ISO specifications	24

G.6	ANSI specifications	24

History	25

# Intellectual Property Rights

IPRs essential or potentially essential to the present document may have been declared to ETSI. The information pertaining to these essential IPRs, if any, is publicly available for ETSI members and non-members, and can be found in ETSI SR 000 314: "Intellectual Property Rights (IPRs); Essential, or potentially Essential, IPRs notified to ETSI in respect of ETSI standards", which is available from the ETSI Secretariat. Latest updates are available on the ETSI Web server (http://webapp.etsi.org/IPR/home.asp).

Pursuant to the ETSI IPR Policy, no investigation, including IPR searches, has been carried out by ETSI. No guarantee can be given as to the existence of other IPRs not referenced in ETSI SR 000 314 (or the updates on the ETSI Web server) which are, or may be, or may become, essential to the present document.

# Foreword

This Technical Specification (TS) has been produced by ETSI Technical Committee Telecommunications and Internet converged Services and Protocols for Advanced Networking (TISPAN).

# Introduction

Editor's note:	This text is placed here for now but it belongs to the introductory text in clause 4 (it appears there for the DR document).

The NGN is required to operate within a regulated environment. In Europe the privacy directive EC/2002/58 [17] applies and article 5 states:

- Member States shall ensure the confidentiality of communications and the related traffic data by means of a public communications network and publicly available electronic communications services, through national legislation. In particular, they shall prohibit listening, tapping, storage or other kinds of interception or surveillance of communications and the related traffic data by persons other than users, without the consent of the users concerned, except when legally authorised to do so in accordance with Article 15(1). This paragraph shall not prevent technical storage which is necessary for the conveyance of a communication without prejudice to the principle of confidentiality.

- Paragraph 1 shall not affect any legally authorised recording of communications and the related traffic data when carried out in the course of lawful business practice for the purpose of providing evidence of a commercial transaction or of any other business communication.

- Member States shall ensure that the use of electronic communications networks to store information or to gain access to information stored in the terminal equipment of a subscriber or user is only allowed on condition that the subscriber or user concerned is provided with clear and comprehensive information in accordance with Directive 95/ 46/EC, inter alia about the purposes of the processing, and is offered the right to refuse such processing by the data controller. This shall not prevent any technical storage or access for the sole purpose of carrying out or facilitating the transmission of a communication over an electronic communications network, or as strictly necessary in order to provide an information society service explicitly requested by the subscriber or user.

ETSI SR 002 211 [23] identifies those aspects of standardisation that are required to ensure compliance with the European Framework Directive. In some instances the right to privacy can be withheld as suggested in paragraph 2 of article 5 of the privacy directive [17] (see clause 5.1). Provisions for the lawful interception of traffic, and for retention of signalling data are allowed exceptions as defined in Article 15(1) of the privacy directive:

- Member States may adopt legislative measures to restrict the scope of the rights and obligations provided for in Article 5, Article 6, Article 8(1), (2), (3) and (4), and Article 9 of this Directive when such restriction constitutes a necessary, appropriate and proportionate measure within a democratic society to safeguard national security (i.e. State security), defence, public security, and the prevention, investigation, detection and prosecution of criminal offences or of unauthorised use of the electronic communication system, as referred to in Article 13(1) of Directive 95/46/EC. To this end, Member States may, inter alia, adopt legislative measures providing for the retention of data for a limited period justified on the grounds laid down in this paragraph. All the measures referred to in this paragraph shall be in accordance with the general principles of Community law, including those referred to in Article 6(1) and (2) of the Treaty on European Union.

The obligations from the directive are placed on member states but may be met by the provision of specific capabilities in the NGN and for LI and DR these are as follows:

- An NGN operator should provide mechanisms to ensure the interception and handover of signalling of specific NGN users if required to by a lawful authority

- An NGN operator should provide mechanisms to ensure the interception and handover of the content of communication of specific NGN users if required to by a lawful authority

- An NGN operator should provide mechanisms to ensure the retention and handover of signalling of specific NGN users if required to by a lawful authority

# 1	Scope

The requirement for provision of lawful interception for all Communication Service Providers (CSP) is described in TS 101 331 [3] and the present document gives the stage 2 definition for provision of an interception capability in TISPAN NGN R2.

The present document specifies the stage 2 model for Lawful Interception (LI) of TISPAN NGN services as specified by TR 180 001 [5].

The provisions in the present document apply only when the target of interception is an NGN user identified as specified in TS 184 002 [6], and when the network supplying services on behalf of the CSP is an NGN as specified by TISPAN in TR 180 001 [5] and ES 282 001 [1].

A guide to the application of the handover specifications is given in informative annexes.

NOTE:	Handover aspects are not specified in the present document but are described in TS 133 108 [8], ES 201 671 [2] and TS 102 232 [4].

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

- References are either specific (identified by date of publication and/or edition number or version number) or non-specific.

- For a specific reference, subsequent revisions do not apply.

- For a non-specific reference, the latest version applies.

Referenced documents which are not found to be publicly available in the expected location might be found at http://docbox.etsi.org/Reference.

NOTE:	While any hyperlinks included in this clause were valid at the time of publication ETSI cannot guarantee their long term validity.

[1]	ETSI ES 282 001: "Telecommunications and Internet converged Services and Protocols for Advanced Networking (TISPAN); NGN Functional Architecture Release 1".

[2]	ETSI ES 201 671 (V2.1.1): "Telecommunications security; Lawful Interception (LI); Handover Interface for the lawful interception of telecommunications traffic".

[3]	ETSI TS 101 331 (V1.2.1): "Lawful Interception (LI); Requirements of Law Enforcement Agencies".

[4]	ETSI TS 102 232 V1.4.1: " Lawful Interception (LI); Handover specification for IP delivery".

[5]	ETSI TR 180 001: "Telecommunications and Internet converged Services and Protocols for Advanced Networking (TISPAN); NGN Release 1; Release definition".

[6]	ETSI TS 184 002: "Telecommunications and Internet Converged Services and Protocols for Advanced Networking (TISPAN); Identifiers (IDs) for NGN".

[7]	ETSI TS 133 107: "Universal Mobile Telecommunications System (UMTS); 3G security; Lawful interception architecture and functions".

[8]	ETSI TS 133 108: "Universal Mobile Telecommunications System (UMTS); 3G security; Handover interface for Lawful Interception (LI)".

[9]	ETSI TR 102 528: " Lawful Interception (LI); Interception domain Architecture for IP networks".

[10]	ETSI ES 282 002: "Telecommunications and Internet converged Services and Protocols for Advanced Networking (TISPAN); PSTN/ISDN Emulation Sub-system (PES); Functional architecture".

[11]	ETSI ES 282 007: "Telecommunications and Internet converged Services and Protocols for Advanced Networking (TISPAN); IP Multimedia Subsystem (IMS); Functional architecture".

[12]	ETSI TS 182 012: "Telecommunications and Internet converged Services and Protocols for Advanced Networking (TISPAN); IMS-based PSTN/ISDN Emulation Subsystem; Functional architecture".

[13]	ITU-T I.130: "Method  for  the  characterization of  telecommunication  services supported  by  an  ISDN  and network  capabilities  of  an  ISDN"

[14]	ITU-T F.703: "ITU-T Recommendation F.703: Multimedia conversational services"

[15]	ETSI TR 180 002: "Telecommunications and Internet converged Services and Protocols for Advanced Networking (TISPAN); Release 2 definition"

[16]	ETSI TR 180 000: "Telecommunications and Internet converged Services and Protocols for Advanced Networking (TISPAN); NGN Terminology"

[17]	Directive 2002/58/EC of the European Parliament and of the council of 12 July 2002 concerning the processing of personal data and the protection of privacy in the electronic communications sector (Directive on privacy and electronic communications)

[18]	ETSI TS 102 165-1: "Telecommunications and Internet converged Services and Protocols for Advanced Networking (TISPAN); Methods and protocols; Method and proforma for Threat, Risk, Vulnerability Analysis"

[19]	ETSI ES 282 001: "Telecommunications and Internet converged Services and Protocols for Advanced Networking (TISPAN); NGN Functional Architecture Release 2"

[20]	ETSI TS 187 027: "Telecommunications and Internet converged Services and Protocols for Advanced Networking (TISPAN); IPTV Architecture; IPTV functions supported by the IMS subsystem"

[21]	ETSI TS 187 028: "Telecommunications and Internet converged Services and Protocols for Advanced Networking (TISPAN); IPTV Architecture; Dedicated subsystem for IPTV functions"

[22]	ETSI EG 202 238: "Telecommunications and Internet Protocol Harmonization Over Networks (TIPHON); Evaluation criteria for cryptographic algorithms"

[23]	ETSI SR 002 211v1.1.1 (2004-02): "Electronic communications networks and services; Candidate list of standards and/or specifications in accordance with Article 17 of Directive 2002/21/EC"

# 3	Definitions, and abbreviations

## 4	Definitions

For the purposes of the present document, the terms and definitions given in ES 201 671 [2] and the following apply:

Content of Communication (CC): information exchanged between two or more users of a telecommunications service, excluding intercept related information

NOTE:	This includes information which may, as part of some telecommunications service, be stored by one user for subsequent retrieval by another.

Handover Interface (HI): physical and logical interface across which the interception measures are requested from Communications Service Provider (CSP), and the results of interception are delivered from a CSP to a law enforcement monitoring facility

interception: action (based on the law), performed by a CSP, of making available certain information and providing that information to a law enforcement monitoring facility

interception interface: physical and logical locations within the CSP telecommunications facilities where access to the content of communication and intercept related information is provided

NOTE:	The interception interface is not necessarily a single, fixed point.

intercept related information: collection of information or data associated with telecommunication services involving the target identity, specifically communication associated information or data (e.g. unsuccessful communication attempts), service associated information or data and location information

internal network interface: network's internal interface between the Internal Intercepting Function (IIF) and a mediation device

Law Enforcement Agency (LEA): organization authorized by a lawful authorization based on a national law to request interception measures and to receive the results of telecommunications interceptions

Law Enforcement Monitoring Facility (LEMF): law enforcement facility designated as the transmission destination for the results of interception relating to a particular interception subject

mediation device: equipment, which realizes the mediation function

Mediation Function (MF): mechanism which passes information between a network operator, an access provider or service provider and a handover interface, and information between the internal network interface and the handover interface

target identity: technical identity (e.g. the interception's subject directory number), which uniquely identifies a target of interception

NOTE:	One target may have one or several target identities.

## 5	Abbreviations

For the purposes of the present document, the following abbreviations apply:

AF	Administration Function

CC	Content of Communication

CCCI	Content of Communication Control Interface

CCTF	Content of Communication Trigger Function

CCTI	Content of Communication Trigger Interface

CR	Change Request

CSP	Communications Service Provider

DF	Delivery Function

FE	Functional Entity

FFS	For Further Study

GSN	GPRS Support Node

HI	Handover Interface

HI1	Handover Interface Port 1 (for Administrative Information)

HI2	Handover Interface Port 2 (for Intercept Related Information)

HI3	Handover Interface Port 3 (for Content of Communication)

IIF	Internal Interception Function

IMS	IP Multimedia Core Network Subsystem

INI	Internal Network Interface

IRI	Intercept Related Information

LEA	Law Enforcement Agency

LEMF	Law Enforcement Monitoring Facility

LI	Lawful Interception

MF	Mediation Function

NGN	Next Generation Network

NGN-R1	NGN Release 1

PES	PSTN/ISDN Emulation Subsystem

PoI	Point of Interception

PSTN	Public Switched Telephone Network

RTP	Real Time Protocol

URI	Universal Resource Identifier

URL	Universal Resource Locator

# 6	Interception in the NGN

## 4.0	Structure of analysis

The analysis presented in the present document follows the recommendations for stage 2 of the method  for  the  characterization of  telecommunication  services supported  by  an  ISDN  and network  capabilities  of  an  ISDN defined in ITU-T I.130 [13]. The steps in expanding a stage 2 specification are listed below:

- Step 2.1: Derivation of a functional model from requirements stated in stage 1

- Step 2.2: Information flow diagrams

- Step 2.3: SDL diagrams for functional entities

- Step 2.4: Functional entity actions

- Step 2.5: Allocation of functional entities to physical locations

The primary points of the stage 1 requirements are stated in subclause 4.0.1 as a starting point for the further development of stage 2.

The structure for LI within the NGN should be mapped to the structure for handover of telecommunications defined in ES 201 158 [] and provisioned by each of ES 201 671 [2], 3GPP TS 133 208 [8] and TS 102 232 [4].

### 4.0.1	Review of stage 1 requirements

The stage 1 analysis approach is defined in ITU-T I.130 [] and consists of the following steps:

- Step 1.1 – Service prose definition and description

- Step 1.2 – Static description of the service using attributes

- Step 1.3 – Dynamic description of the service using graphic means

For the purposes of this document only step 1.1 is summarised.

#### 4.0.1.1	Provision/withdrawal

The LI service shall always be provided.

#### 4.0.1.2	Activation/deactivation/registration

The LI service shall be activated upon issue of a valid interception warrant from an LEA. The LI service shall be deactivated when the interception warrant expires or as defined by the LEA.

#### 4.0.1.3	Invocation and operation

The LI service shall be invoked on any communication from or to the target visible in the network.

Editor's note:	Placeholder for CR from BT (et al) to ad the stage 1 requirement for "result of interception"

#### 4.0.1.4	Interrogation

Interrogation shall be possible only from an authorized user.

An authorized user for the purposes of interrogation is one who is allowed by both LEA and the CSP to administer the LI interface.

#### 4.0.1.5	Interaction with other services

There shall be no interaction.

NOTE:	This means that the invocation of LI shall not alter the operation of any service.

## 7	LI architecture model

The architecture for lawful interception consists of a Point of Interception (PoI) for each of the signalling plane and the transport plane, collocated with an NGN Functional Entity (NGN FE) (the specific NGN FE varies with the service being intercepted), that delivers intercepted material to a Mediation Function (MF). The MF acts to mediate between the nationally specified handover interface and the internal interception interface of the NGN as specified in the present document.

The target is a specialist NGN user that receives service from the NGN.

NOTE 1:	A service offered to the NGN user may invoke many NGN-FEs.

NOTE 2:	There are a number of terms used across ETSI to refer to the various functions outlined in the first paragraph of this clause (4.1). The MF is also known as a Delivery Function (DF) in 3GPP documents, the Internal Network Interception interfaces are also referred to in 3GPP as X interfaces.

The LI capability in the NGN shall always be available and shall be invoked on receipt of instruction from the Law Enforcement Agency or its authorizing agency. The functions of the LI capability shall only be visible to, and their operation shall only be invoked by, authorized parties within the NGN and shall not alter or be impacted by the operation of any other functional entity in the NGN.

## 8	LI reference model

The present document adopts the generic reference model for the interception domain from TR 102 528 [9], its internal intercept functions, IRI-IIF, CCTF, and CC-IIF, and the internal interfaces INI1, INI2, INI3, CCTI and CCCI as shown in figure 1.

NOTE:	Interfaces INI1, INI1a, INI1b, INI1c, CCTF, CCTI and CCCI are not defined in the present document but are shown in the figure for completeness.

Figure 1: Reference Model for Lawful Interception from TR 102 528 [9]

The reference model depicts the following functions and interfaces:

- Intercept Related Information Internal Intercept Function (IRI-IIF) generates signalling intercept material.

- Content of Communication Internal Intercept Function (CC-IIF) generates content intercept material.

- Content of Communication Trigger Function (CCTF) controls the CC-IIF.

- Internal interface INI1 carries provisioning information from the Lawful Interception Administration Function (AF) to the Internal Intercept Functions (IIF).

- Internal interface INI2 carries Intercept Related Information (IRI) from the IRI-IIF to the MF.

- Internal interface INI3 carries Content of Communication (CC) information from the CC-IIF to the MF.

- Content of Communication Trigger Interface (CCTI) carries trigger information from the IRI-IIF to the CCTF.

- Content of Communication Control Interface (CCCI) carries controls information from the CCTF to the 
CC-IIF.

The reference model introduces the CCTF FE that may be used to in a number of configurations to allow for the provisioning of CC-IIF in an IP network. The location of the CCTF is not defined in the present document but considered configuration options are as follows:

- CCTF co-located with the LIAF: INI1b is internal to the AF and CCTF.

- CCTF co-located with the IRI-IIF: CCTI is internal to the IRI-IIF and CCTF.

- CCTF co-located with the IRI-IIF and CC-IIF: CCTI and CCCI are internal to the IRI-IIF, CCTF and CC-IIF.

- CCTF co-located with the MF: CCTI is merged with INI2.

- A stand alone CCTF: Both CCTI and CCCI are external interfaces.

A complete explanation of the functions and interface is found in clause 4 of TR 102 058 [9].

## 9	Content of interception

The point of interception shall provide the following data when available in the intercepting network from the communication of the target:

- Date and time of interception.

- Identity of the target and the correspondents of the target.

- Location of the target if available.

- Activity of the target.

NOTE:	Whilst each element of the interception may be described separately they may be delivered in combination, or may be delivered implicitly if no changes are reported (e.g. if location is not changed this knowledge may be delivered implicitly but when location is changed it may be explicitly reported).

# 10	Interception of non-IMS PSTN/ISDN Emulation Subsystem (PES)

NOTE:	This clause addresses ITU-T I.130 [] "Step 2.5: Allocation of functional entities to physical locations" with the specific application of NGN for non-IMS PES.

## 5.0	Allocation of functional entities to physical locations in non-IMS PES

The following scenarios are considered for deployment of non-IMS PES in the NGN:

…

Table 5.1: Allocation of functional entities to physical locations for non-IMS PES

| FE | Scenario 1 | Scenario 2 | Scenario 3 |
| --- | --- | --- | --- |
| FE1 |  |  |  |
| FE2 |  |  |  |
| FE3 |  |  |  |
| FE4 |  |  |  |

## 11	Architecture for interception of PES

The Point of Interception shall be at premises of the CSP, i.e. IRI-IIF and CC-IIF shall reside in equipment under full control (physical access, etc.) of the CSP.

Figure 1 (in clause 4.2) identifies the generic architecture for LI. The specific provision of the CC-IIF and IRI-IIF in the NGN for PES services is as shown in figure 2. This architecture for LI applies to a PES whose architecture conform to ES 282 002 [24].

NOTE:	Location of CC-IIF is subject to implementation and not all possible options need to be implemented.

Figure 2: Reference architecture for interception in the PES environment

The point of interception (as defined in clause 4) with respect to IRI, the IRI-IIF, should be implemented in the 
NGN-FE that hosts the service state machine.

The point of interception (as defined in clause 4) with respect to CC, the CC-IIF, should be implemented in a mediastream entity, e.g. Border Gateway.

# 6	Interception of IMS

Editor's note:	
The text that follows is marked for deletion from the present document pending its transfer to 3GPP SA3 LI (see also annex H) on the understanding the the interception of IMS is the responsibility of 3GPP-SA3-LI and that if the maintenance of the text that follows is agreed to by 3GPP-SA3-LI the text that follows will be deleted.

## 6.1	Architecture for interception of IMS

The Point of Interception shall be at premises of the CSP, i.e. IRI-IIF and CC-IIF shall reside in equipment under full control (physical access, etc.) of the CSP.

Figure 1 (in clause 4.2) identifies the generic architecture for LI. The specific provision of the CC-IIF and IRI-IIF in the NGN for IMS services is as shown in figure 3. This architecture for LI applies to the IMS subsystem as described in
ES 282 007 [11] and TS 182 012 [12].

Figure 3: Reference architecture for interception in the IMS environment

## 6.2	Activation, deactivation and interrogation

The principles described in TS 133 107 [7] apply, with the exception that the list of target identities is restricted to be a SIP URI or a TEL URL.

## 6.3	Intercept Related Information

Communications to or from a targeted subscriber and communications initiated on behalf of a targeted subscriber are intercepted at the P-CSCF or S-CSCF as described in TS 133 107 [7].

The stage 2 definition for interception of IMS found in TS 133 107 [7] shall apply for TISPAN NGN R1. Annex A specifies the parts of TS 133 107 [7] that apply.

NOTE 1:	When IMS is providing a PES service the interception service identified above and defined in
TS 133 107 [7] still applies. In addition, the AGCF may be used as an alternative point of interception.

Editor's note: The common IMS does not support the AGCF.

NOTE 2:	If the IMS is used for the support of transit communication and national LI requires their interception then the interception of communications in transit may take place at the IBCF or MGCF depending on the characteristics of the interconnected networks involved in the communication.

## 6.4	Content of Communication

Editor's note:	
It is noted that IMS does not of itself carry content of communication. Therefore this text may need to be maintained by ETSI TISPAN.

Interception of the content of communications takes place at transport processing functional entities identified in the TISPAN NGN R1 architecture (ES 282 001 [1]). Transport processing entities that may provide the CC-IIF are:

- An Access Media Gateway Function (A-MGF).

- A Core Border Gateway Function (C-BGF).

- An Interconnect Border Gateway Function (I-BGF).

- A Trunking Media Gateway Function (T-MGF).

- A Multimedia Resource Function Processor (MRFP).

NOTE 1:	Interception at an A-MGF, C-BGF or MRFP takes place when the target of interception is a subscriber of the IMS network.

NOTE 2:	If the IMS is used for the support of transit communication and national LI requires their interception then the interception of communications in transit may take place at the I-BGF or T-MGF depending on the characteristics of the interconnected networks involved in the communication.

When the interception of communication contents takes place at a C-BGF or I-BGF, interactions between the IRI-IIF and the CC-IIF takes place through the SPDF or the MF. The SPDF or the MF plays the role of a CCTF as identified in clause 4.3.

When the interception of communication contents takes place at an A-MGF or T-MGF, the AGCF or MGCF plays the role of the IRI-IIF, and the CCTF (as identified in clause 4.3) is located in the AGCF, MGCF or MF.

When the interception of communication contents takes place at an MRFP, the associated MRFC and an Application Server Function collectively, or the MF, play the role of a CCTF. The ASF controls the MRFC via the S-CSCF. In order to ensure that the Application Server gets involved in the communications subject to interception, the Administration Function (ADMF) provisions the S-CSCF with the address of the Application Server or creates an appropriate Initial Filter Criteria in the targeted subscriber's profile in the UPSF.

The instances in the network where the interception takes place (Point of Interception) shall be at premises of the CSP, i.e. IRI-IIF and CC-IIF shall reside in equipment under full control (physical access, etc.) of the CSP.

# 12	Identification of target of interception

## 13	ISDN/PSTN services

In the context of PSTN/ISDN emulation and services, the target shall be identified in the service domain by a globally unique E.164 identity.

NOTE:	The PES offers seamless ISDN/PSTN service to existing core network customers who will remain identified by their E.164 identity that may be mapped to a system unique SIP-identity.

Editor's note:	
The text that follows is marked for deletion from the present document pending its transfer to 3GPP SA3 LI (see also annex H) on the understanding the the interception of IMS is the responsibility of 3GPP-SA3-LI and that if the maintenance of the text that follows is agreed to by 3GPP-SA3-LI the text that follows will be deleted.

## 7.2	IMS services

IMS service users shall be identified by either a SIP-url or a tel-url [6].

# 14	Security considerations

The guidelines for security identified in TS 101 331 [3] apply. The provision and actions of the interception measure should be confidential and not discoverable, or alterable, by unauthorized parties.

# 15	NGN Network Entities role in interception

Each network entity may be a physical device, it may form part of a physical device, or it may be distributed over a number of physical devices.

## 16	Access Gateway (AGW)

The AGW is a network component that should support both the Multimedia and Legacy MS Domains. The CDMA20002 Access Gateway (AGW) consists of the Packet Data Service Node (PDSN) and other logical functions required to interface the core network to the CDMA2000 Radio Access Network (RAN).

The AGW provides the Core Network (CN) with access to the resources of the Access Network. The AGW presents the CN with a common interface to the specific capabilities, configuration, and resources of the numerous Access Network technologies. The AGW is connected via the following Reference Points:

Go. Policy Decision Function (signalling),

Mb. Media Resource Function Processor (bearer streams),

## 17	Access Network

The Access Network is a network component that may support both the Multimedia and Legacy MS Domains.

The access network may perform mobility management functions for registering, authorizing, authenticating and paging IP based terminals, independent of circuit based terminals. The access network may perform handoffs within an access network and between access networks of the same technology and may support handoffs between access networks of differing technologies.

## 18	Application Server

The Application Servers are network components that may support both the IMS and non-IMS NGN. These services may be accessed via the OSA Service Capability Server (OSA-SCS) or accessed directly from the user’s mobile station via other network entities. The OSA Application Server (OSA-AS) is connected via signalling Reference Points:

The SIP Application Server (SIP AS) is connected via signalling Reference Points:

Sh AAA,

ISC. CSCF

The distribution of an Application Server may consist of any of the following combinations:

a. all Application Servers may be hosted in a single server, or

b. one Application Server may be hosted in a constellation of servers.

An Application Server refers to:

a. 	a "Home Application Server" (i.e., owned and executed by the home service provider),

b. 	a "Third Party Application Server" (i.e., owned and executed by a third party), and/or

c. 	a "Local Application Server".

## 19	Border Router (BR)

The BR is a network component that should support both the IMS and non-IMS NGNs.

The BR connects the Core Network with peer networks (e.g., other service providers, corporate networks, Internet). The BR performs IP packet routing, exterior gateway routing protocols, and policing of incoming and outgoing traffic, ensuring traffic complies with defined Service Level Agreements established with peer networks. The BR may intercept any QoS allocation request, and issue a request to the Policy Decision Function (PDF), which should verify that the requested inbound and/or outbound QoS is available. A successful response from the PDF may cause the BR to forward the bandwidth allocation request to its final destination.

The BR is connected via Reference Points:

Mb. Media Gateway (bearer streams),

## 20	Breakout Gateway Control Function (BGCF)

The BGCF is a network component that applies only to the Multimedia Domain.

The BGCF selects the network in which PSTN breakout is to occur and, within the network where the breakout is to occur, selects the MGCF. The BGCF is connected via the following signalling Reference Points.

Mi. S-CSCF,

Mj. MGCF, and

Mk. BGCF.

## 21	Call Session Control Function (CSCF)

The CSCF establishes, monitors, supports, and releases Multimedia sessions and manages the user's service interactions. The CSCF is responsible for:

a. managing the allocation of required resources such as announcement servers, multi-party bridges, etc.,

b. maintaining knowledge of session states and user's service precedence,

c. querying the AAA for authentication information and user profile information, and

d. performing session processing tasks (e.g., network selection) required for session completion.

The CSCF entity in the home network is responsible for session control. The P-CSCF is used to proxy requests from the mobile station to an CSCF in the home network and to return responses from the home network to the mobile station.

The CSCF entity in the home network may be further divided into an Interrogating-Call Session Control Function (I-CSCF) and a Serving- Call Session Control Function (SCSCF). The reason for this could be load sharing and/or hiding of the internal network structure, or for allocation of an S-CSCF close to the mobile station. The I-CSCF would in such a case be the entry point to the network, responsible for locating the SCSCF serving the user, while the S-CSCF is the entity actually keeping the session state.

The communication between the CSCF and AAA may include the passing of addresses and security tokens necessary for the CSCF to communicate with other CSCF entities. Other techniques may also apply that allow an CSCF to determine the address of another CSCF entity.

The S-CSCF has access to Subscriber Profiles.

The CSCF uses information from various databases (e.g., subscriber profile) and invocation of various services applications to determine the exact service being requested.

The CSCF is connected via signalling Reference Points:

ISC. OSA Service Capability Server/SIP Application Server,

Cx. AAA,

Mg. Media Gateway Control Function,

Mr. Media Resource Function Controller,

Mw between P-CSCF and I-CSCF,

Mw between P-CSCF and S-CSCF,

Mi between BGCF and S-CSCF,

Mw between I-CSCF and S-CSCF, and

Gm between IP Multimedia Client and P-CSCF.

## 22	Public Switched Telephone Network (PSTN)

The PSTN is a network component that may support both the Multimedia and Legacy MS Domains.

The PSTN is defined in accordance with the appropriate applicable national and regional standards.

The PSTN is connected via Reference Points:

PSTN. Media Gateway Control Function (signalling), and

Mb. Media Gateway (bearer streams).

Annex A (normative):
Endorsement statement for TS 133 107

For the stage 2 definition of the interception of IMS and generic IP subsystem parts of TS 133 107 [7] apply normatively in the context of TISPAN NGN. This annex summarizes those parts of TS 133 107 [7] that apply in the context of TISPAN NGN LI.

NOTE:	Where no specific endorsement statement is given the text in the endorsed document is considered to have only background relevance and not to form part of the normative specification for NGN-R1 interception.

| Clause | Applicability in TISPAN NGN R1 |
| --- | --- |
| 1 |  |
| 2 |  |
| 3 |  |
| 4 | Figure 1d applies |
| 5 |  |
| 6 | Does not apply |
| 7 | Does not apply |
| 7A | Applies in full |
| 7A.1 | Does not apply (refers only to GSNs) |
| 7A.2 | Applies in full |
| 7A.3 | Applies in full with extensions defined in the present document |
| 7A.4 | Applies in full |
| 7A.5 | Does not apply (service defined only for cellular network) |
| 8 |  |
| 9 | Does not apply |
| Annex A | Does not apply |
| Annex B | Does not apply |
| Annex C |  |
| Annex D | Does not apply |
| Annex G | Does not apply (informative annex showing history of CRs applied to the document) |
| NOTE: Annexes E and F do not exist in TS 133 107 [7]. |  |

Annex B (informative):
Endorsement statement for TS 133 108

NOTE 1:	This annex is provided for information pending a full stage 3 mapping from TISPAN NGN to TS 133 108 [8].

NOTE 2:	The endorsements stated are indicative and further work is required to fully analyse the data and operations in TS 133.108 and how they should apply in TISPAN NGN.

This annex summarizes those parts of TS 133 108 [8] that apply in the context of TISPAN NGN LI.

NOTE 3:	Where no specific endorsement statement is given the text in the endorsed document is considered to have only background relevance and not to form part of the normative specification for NGN-R1 interception.

| Clause | Applicability in TISPAN NGN R1 |
| --- | --- |
| 1 |  |
| 2 |  |
| 3 |  |
| 4 |  |
| 4.1 |  |
| 4.2 |  |
| 4.3 |  |
| 4.4 |  |
| 4.4.1 | Applies in full |
| 4.4.2 |  |
| 4.5 | Applies in full |
| 4.5.1 | Applies in full |
| 4.5.2 | Applies in full |
| 4.5.3 | Applies in full |
| 5 | Does not apply |
| 6 |  |
| 7 |  |
| 7.1 |  |
| 7.1.1 |  |
| 7.1.2 |  |
| 7.1.3 | Correlation to the relevant CC is necessary |
| 7.2 |  |
| 7.3 |  |
| 7.4 |  |
| 7.5 | Yes / for correlation ASN.1 parameter correlation had to be used |
| 7.5.1 | Yes / for correlation ASN.1 parameter correlation had to be used |
| 7.6 |  |
| Annex A |  |
| Annex B | Requires detail study of ASN.1 to confirm applicability. May not apply |
| B.1 | May not apply |
| B.2 | May not apply |
| B.3 | May not apply |
| B.3a | May not apply |
| B.4 | May not apply |
| B.5 | May not apply |
| B.6 | May not apply |
| Annex C |  |
| Annex D |  |
| Annex E |  |
| Annex F |  |
| Annex G |  |
| Annex H | Applies in full |
| Annex J | Does not apply |
| Annex K | Does not apply (informative annex showing history of CRs applied to the document) |
| NOTE: Whereas the scope of the present document is interception in the NGN domain the scope of TS 133 108 [8] is handover for the PLMN, the data emanating from the PLMN is described and covers a range of capabilities more specific than those of the NGN (although the NGN is a superset of the capabilities that form the PLMN). |  |

Annex C (informative):
Endorsement statement for TS 102 232

NOTE 1:	This annex is provided for information pending a full stage 3 mapping from TISPAN NGN to TS 102 232 [4].

NOTE 2:	The endorsements stated are indicative and further work is required to fully analyse the data and operations in TS 102 232 [4] and how they should apply in TISPAN NGN.

Annex D (informative):
Endorsement statement for ES 201 671

NOTE 1:	This annex is provided for information pending a full stage 3 mapping from TISPAN NGN to ES 201 671 [2].

NOTE 2:	The endorsements stated are indicative and further work is required to fully analyse the data and operations in ES 201 671 [2] and how they should apply in TISPAN NGN.

| Clause | Title | Applicability in TISPAN NGN R1 |
| --- | --- | --- |
| 1 | Scope |  |
| 2 | References |  |
| 3 | Definitions and abbreviations |  |
| 3.1 | Definitions |  |
| 3.2 | Abbreviations |  |
| 4 | General requirements |  |
| 4.1 | Basic principles for the Handover Interface | Applies in full |
| 4.2 | Legal requirements | Applies in full |
| 4.3 | Functional requirements | Applies in full |
| 5 | Overview of Handover Interface | Applies in full |
| 5.1 | Handover Interface port 1 (HI1) | No: national implementation |
| 5.1.1 | Manual interface | No: national implementation |
| 5.1.2 | Electronic interface | No: national implementation |
| 5.2 | Handover Interface port 2 (HI2) | Applies in full |
| 5.3 | Handover Interface port 3 (HI3) | Applies in full |
| 6 | Specific identifiers for LI | Applies in full |
| 6.1 | Lawful Interception Identifier (LIID) | Applies in full |
| 6.2 | Communication Identifier (CID) | Applies in full |
| 6.2.1 | Network Identifier (NID) | Applies in full |
| 6.2.2 | Communication Identity Number (CIN) – optional | Applies in full |
| 7 | HI1: Interface port for administrative information |  |
| 7.1 | Information for the activation of lawful interception | Applies in full |
| 7.2 | LI notifications towards the LEMF | No: national implementation |
| 8 | HI2: Interface port for Intercept Related Information |  |
| 8.1 | Data transmission protocols |  |
| 8.1.1 | Application for IRI (HI2 information) |  |
| 8.2 | Types of IRI records |  |
| 9 | HI3: Interface port for Content of Communication |  |
| 10 | Performance and quality |  |
| 10.1 | Timing |  |
| 10.2 | Quality |  |
| 11 | Security aspects | Advisable |
| 11.1 | Security properties | Advisable |
| 11.2 | Security mechanisms | Advisable |
| 12 | Quantitative aspects | national matter |
| Annex A (normative): | Circuit switched network handover | Applies in full |
| Annex B (normative): | Packet switched network handover | Applies in full |
| Annex C (normative): | HI2 Delivery mechanisms and procedures |  |
| C.1 | ROSE |  |
| C.2 | FTP |  |
| Annex D (normative): | Structure of data at the Handover Interface (ASN.1) |  |
| Annex E (informative): | Use of subaddress and calling party number to carry correlation information | National matter |
| Annex F (informative): | GPRS HI3 Interface |  |
| Annex G (informative): | LEMF requirements – handling of unrecognized fields and parameters |  |
| Annex H (informative): | IP Multimedia Subsystem (IMS) handover |  |
| Annex I (informative): | Latest ASN.1 module versions | No |
| Annex J (informative): | Bibliography |  |
| Annex K (informative): | Change Request history | No |

Annex E (informative):
ISDN/PSTN LI reference configurations

The figures contained in this annex identify a number of reference configurations for lawful interception in TISPAN NGN networks. Interception configurations for communications to or from a targeted TISPAN NGN subscriber are shown in figures E.1, E.2 and E.3. Interception of communications in transit are shown in figures E.4 and E.5.

Figure E.1: Interception at the edge (case 1)

Figure E.2: Interception at the edge (case 2)

Figure E.3: Interception in the core

Figure E.4: Interception of communications in transit (TDM case)

Figure E.5: Interception of communications in transit (IP case)

Annex F (informative):
Selection of handover interface

Handover of intercepted material should be made by reference to one or more of the following specifications:

- ES 201 671 [2]: Handover Interface for the lawful interception of telecommunications traffic.

- TS 133 108 [8]: Handover interface for Lawful Interception.

- TS 102 232 [4]: Handover specification for IP delivery.

NOTE:	National specifications may be used instead of any of the ETSI specifications cited above.

Figure F.1 illustrates configurations of mediation function to map the Handover Interface to the intercepted data that are subject to bilateral agreement between Network Provider and LEA.

NOTE 1:	CS interception formats from the NGN may map to CS capabilities in TS 133 108 but there may be a requirement for extensions to TS 133 108 in some instances.

NOTE 2:	IMS interception formats from the NGN may map to IMS capabilities in TS 133 108 but there may be a requirement for extensions to TS 133 108 in some instances.

NOTE 3:	SIP interception formats from the NGN map to IMS capabilities in TS 133 108 but there may be a requirement for extensions to TS 133 108 in some instances.

Figure F.1: Reference Model for LI Mediation Function

Annex G (informative):
Bibliography

# G.1	ETSI Specifications

[A]	ETSI EN 300 356 (all parts): "Integrated Services Digital Network (ISDN); Signalling System No.7; ISDN User Part (ISUP) version 3 for the international interface".

[B]	ETSI EN 300 403-1 (V1.3.2): "Integrated Services Digital Network (ISDN); Digital Subscriber Signalling System No. one (DSS1) protocol; Signalling network layer for circuit-mode basic call control; Part 1: Protocol specification [ITU-T Recommendation Q.931 (1993), modified]".

[C]	ETSI ES 201 158: "Telecommunications security; Lawful Interception (LI); Requirements for network functions".

[D]	ETSI ETR 330: "Security Techniques Advisory Group (STAG); A guide to legislative and regulatory environment".

[F]	ETSI TS 101 671: "Handover Interface for the lawful interception of telecommunications traffic".

[G]	ETSI ES 283 002: "Telecommunications and Internet converged Services and Protocols for Advanced Networking (TISPAN); PSTN/ISDN Emulation Sub-system (PES); NGN Release 1 H.248 Profile for controlling Access and Residential Gateways"

# G.2	3GPP specifications

[H]	3GPP TS 29.002: "3rd Generation Partnership Project; Technical Specification Group Core Network; Mobile Application Part (MAP) specification".

[I]	3GPP TS 23.003: "3rd Generation Partnership Project; Technical Specification Group Core Network; Numbering, addressing, and identification".

[J]	3GPP TS 23.107: "3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Quality of Service QoS concepts and architecture".

[K]	3GPP TS 23.228: "3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; IP Multimedia Subsystem (IMS); Stage 2".

[L]	3GPP TS 24.008: "3GPP Technical Specification Group Core Network; Mobile radio interface Layer 3 specification, Core network protocol; Stage 3".

[M]	3GPP TS 29.060: "3rd Generation Partnership Project; Technical Specification Group Core Network; General Packet Radio Service (GPRS); GPRS Tunnelling Protocol (GTP) across the Gn and Gp interface".

[N]	3GPP TS 32.215: "3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Telecommunication Management; Charging Management; Charging data description for the Packet Switched (PS) domain)".

[O]	3GPP TS 33.106: "3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; 3G Security; Lawful Interception Requirements".

[P]	3GPP TS 23.032: "3rd Generation Partnership Project; Technical Specification Group Core Network; Universal Geographical Area Description (GAD)".

[Q]	3GPP TR 21.905: "3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Vocabulary for 3GPP Specifications".

# G.3	ITU-T specifications

[R]	ITU-T Recommendation Q.763: "Signalling System No. 7 – ISDN User Part formats and codes".

[S]	ITU-T Recommendation Q.931: "ISDN user-network interface layer 3 specification for basic call control".

[T]	ITU-T Recommendation X.680: "Abstract Syntax Notation One (ASN.1): Specification of Basic Notation".

[U]	ITU-T Recommendation X.681: "Abstract Syntax Notation One (ASN.1): Information Object Specification".

[V]	ITU-T Recommendation X.682: "Abstract Syntax Notation One (ASN.1): Constraint Specification".

[W]	ITU-T Recommendation X.683: "Abstract Syntax Notation One (ASN.1): Parameterization of ASN.1 Specifications".

[X]	ITU-T Recommendation X.690: "ASN.1 encoding rules: Specification of Basic Encoding Rules (BER), Canonical Encoding Rules (CER) and Distinguished Encoding Rules (DER)".

[Y]	ITU-T Recommendation X.880: "Information technology - Remote Operations: Concepts, model and notation".

[Z]	ITU-T Recommendation X.882: "Information technology - Remote Operations: OSI 
realizations - Remote Operations Service Element (ROSE) protocol specification".

# G.4	IETF specifications

[AA]	IETF STD 0005 (RFC 0791): "Internet Protocol".

[AB]	IETF STD 0007 (RFC 0793): "Transmission Control Protocol".

[AC]	IETF STD 0009 (RFC 0959): "File Transfer Protocol (FTP)".

[AD]	IETF RFC 1006: "ISO Transport Service on top of the TCP".

[AE]	IETF RFC 2126: "ISO Transport Service on top of TCP (ITOT)".

[AF]	IETF RFC 2806: "URLs for Telephone Calls".

[AG]	IETF RFC 3261: "SIP: Session Initiation Protocol".

# G.5	ISO specifications

[AH]	ISO 3166-1: "Codes for the representation of names of countries and their 
subdivisions – Part 1: Country codes".

# G.6	ANSI specifications

[AI]	ANSI/J-STD-025-A: "Lawfully Authorized Electronic Surveillance".

Annex H (normative):
Common IMS text for 3GPP review

NOTE 1:	The text that follows was deleted from the NGN-R1 version of the present document when preparing the NGN-R2 version on the understanding the the interception of IMS is the responsibility of 3GPP-SA3-LI and that if the maintenance of the text that follows is agreed to by 3GPP-SA3-LI the text that follows will be deleted.

NOTE 2: 	The clause numbering from the NGN-R1 document is maintained in this annex.

# 6	Interception of IMS

## 6.1	Architecture for interception of IMS

The Point of Interception shall be at premises of the CSP, i.e. IRI-IIF and CC-IIF shall reside in equipment under full control (physical access, etc.) of the CSP.

Figure 1 (in clause 4.2) identifies the generic architecture for LI. The specific provision of the CC-IIF and IRI-IIF in the NGN for IMS services is as shown in figure 3. This architecture for LI applies to the IMS subsystem as described in
ES 282 007 [11] and TS 182 012 [12].

Figure 4: Reference architecture for interception in the IMS environment

## 6.2	Activation, deactivation and interrogation

The principles described in TS 133 107 [7] apply, with the exception that the list of target identities is restricted to be a SIP URI or a TEL URL.

## 6.3	Intercept Related Information

Communications to or from a targeted subscriber and communications initiated on behalf of a targeted subscriber are intercepted at the P-CSCF or S-CSCF as described in TS 133 107 [7].

The stage 2 definition for interception of IMS found in TS 133 107 [7] shall apply for TISPAN NGN R1. Annex A specifies the parts of TS 133 107 [7] that apply.

NOTE 1:	When IMS is providing a PES service the interception service identified above and defined in
TS 133 107 [7] still applies. In addition, the AGCF may be used as an alternative point of interception.

Editor's note: The common IMS does not support the AGCF.

NOTE 2:	If the IMS is used for the support of transit communication and national LI requires their interception then the interception of communications in transit may take place at the IBCF or MGCF depending on the characteristics of the interconnected networks involved in the communication.

## 6.4	Content of Communication

Editor's note:	
It is noted that IMS does not of itself carry content of communication. Therefore this text may need to be maintained by ETSI TISPAN.

Interception of the content of communications takes place at transport processing functional entities identified in the TISPAN NGN R1 architecture (ES 282 001 [1]). Transport processing entities that may provide the CC-IIF are:

- An Access Media Gateway Function (A-MGF).

- A Core Border Gateway Function (C-BGF).

- An Interconnect Border Gateway Function (I-BGF).

- A Trunking Media Gateway Function (T-MGF).

- A Multimedia Resource Function Processor (MRFP).

NOTE 1:	Interception at an A-MGF, C-BGF or MRFP takes place when the target of interception is a subscriber of the IMS network.

NOTE 2:	If the IMS is used for the support of transit communication and national LI requires their interception then the interception of communications in transit may take place at the I-BGF or T-MGF depending on the characteristics of the interconnected networks involved in the communication.

When the interception of communication contents takes place at a C-BGF or I-BGF, interactions between the IRI-IIF and the CC-IIF takes place through the SPDF or the MF. The SPDF or the MF plays the role of a CCTF as identified in clause 4.3.

When the interception of communication contents takes place at an A-MGF or T-MGF, the AGCF or MGCF plays the role of the IRI-IIF, and the CCTF (as identified in clause 4.3) is located in the AGCF, MGCF or MF.

When the interception of communication contents takes place at an MRFP, the associated MRFC and an Application Server Function collectively, or the MF, play the role of a CCTF. The ASF controls the MRFC via the S-CSCF. In order to ensure that the Application Server gets involved in the communications subject to interception, the Administration Function (ADMF) provisions the S-CSCF with the address of the Application Server or creates an appropriate Initial Filter Criteria in the targeted subscriber's profile in the UPSF.

The instances in the network where the interception takes place (Point of Interception) shall be at premises of the CSP, i.e. IRI-IIF and CC-IIF shall reside in equipment under full control (physical access, etc.) of the CSP.

# 7	Identification of target of interception

…

## 7.2	IMS services

IMS service users shall be identified by either a SIP-url or a tel-url [6].

# History

| Document history |  |  |
| --- | --- | --- |
| V1.1.1 | December 2006 | Publication |
| V2.0.0 | August 2007 | First draft of NGN-R2 input |
| V2.0.1 | October 2007 | Output of rapporteur's meeting for approval at TISPAN#15bis |
| V2.0.2 | February 2008 | Input to TISPAN#16bis meeting |