Source:	Fraunhofer IIS

Title:	Initial Verification of the IVAS Fixed-Point Decoder Release

Document for:	Information

Agenda Item:	7.6

# Introduction

Now that there is a complete release of the IVAS decoder in fixed-point available [1] it was requested to verify this package. This contribution contains some assorted first observations of the source based on the requirements set forth in [2].

# Initial check for the requirements in [2]

The list of requirements in [2] has been taken as the  first set of things to look at, even though this is not the final complete release. It was however felt that an early analysis could be useful to ensure complete fulfillment at the end of the effort.

| Requirement | Observation |
| --- | --- |
| 3GPP Forge will be used as the platform of FX development, similarly to the development of FL code in Public Collaboration. | yes |
| Inclusion of 3rd party code or open-source code is not allowed; inclusion of code from generative AI tools is not allowed. | None observed. |
| Structure of FL code shall be preserved to the maximum extent to allow comparison between FL and FX code and easier maintainability of the two code bases. | Structure is very similar. |
| This includes the best preservation of function prototypes and comments in the code; | Seems to be the case |
| file names shall remain unchanged. | Generally the case with the inconsistency of the _fx suffix. Sometimes files were renamed with suffix "_fx". This seems to be acceptable but should be done consistently to either have the suffix always or never. What is the difference between files with and without the suffix if both are present? |
| In addition, all variables converted to FX shall be marked with precision information (integer, fractional) in Q-notation; | Barely present. This shall be added in a subsequent release to allow comparison to floating-point and further maintenance by other parties than the party doing the conversion. Also the definition of the nomenclature may need to be defined by SA4. Basic assumption could be TI nomenclature. If Q values have been observed they were often omitting the m of Qm.n if the size of the variable is known. This may be okay but should clarified. Example for TI syntax: 16 bit signed integer is Q15.0 or Q15. No unsigned Q values have been spotted during quick inspection. Would be good if there the “UQ” notation could be used, i.e. unsigned 16 bit integer is UQ16 or UQ16.0. |
| precision changes should be documented; | no documentation observed during quick inspection. This would be essential to allow verification vs. floating-point code. |
| the goal is that the original functions could be used interchangeably during development; function names of converted functions shall avoid symbol clashes during the development; | seems to be the case generally, fixed point functions have often suffix _fx, sometimes however _fixed and sometimes they are called the same. There may simply be cleanup missing. |
| the objective is to ultimately have maximum similarity to the FL code and eventually all variables and function names also in FX shall be unified to their FL counterpart. | requirement somewhat conflicts with the previous requirement, it would be good though if functions in fixed have the _fx suffix so that there are no symbol clashes due to identical symbol name but different prototype (and even implementation). This may be for SA4 to discuss. |
| The performance of the FX code should match the performance of the FL code, both in terms of subjective quality and complexity. | still to be verified |
| For decoding, the output synthesis of FX decoder (FL enc+ FX dec) compared to FL decoder (FL enc + FL dec) should be within the limit of segmental SNR (per frame and then averaged) > [60 dB], and MLD < 5. | documented by Ittiam. Still to be verified. |
| For bit-streams generated with the FX encoder, the output synthesis of the bit-stream decoded with the FL decoder compared to the synthesis of a bit-stream generated with the FL encoder and decoded with the FL decoder should be within the limit of MLD < [TBD], PEAQ (basic profile) < [0.1]. | Not applicable, this is a decoder-only release. |
| For non-EVS operation, the performance shall be measured using both the short and long test vector sets as specified in 3GPP Forge, with nominal level and +-10 dB. | documented by Ittiam, Still to be verified |
| Interoperability between FX and FL codes shall be guaranteed; for example, valid FL bitstreams shall be decodable by the FX- | Seems to be the case for the decoder as long as there is only the floating-point encoder and bistreams can be decoded with differences within the allowed range. |

# Additional observations

- some global variables not present in floating-point are now in the code. This should be easily fixable.

  - cosine_table_Q31

  - diff_norm_order1_table variable

  - diff_norm_order2_table variable

  - diff_norm_order3_table variable

  - ind_list

  - inv_table_145

  - inv_table_290

  - inv_table_298

  - inv_table_596

  - inv_time

  - local_result_table variable

  - local_result_table_2 variable

  - log_weights_music_compute variable

  - log_weights_noise_compute variable

  - log_weights_speech_compute variable

  - pFC_12k

  - pFC_16k

  - pFC_32k

  - pFC_48k

  - pFC_8k

  - power_table

  - release_cnst_table variable

  - side_gain_table

  - sine_table_Q31

  - slot_fx

  - square_root16_table

  - square_root30_q12

  - tableD1_145

  - tableD1_290

  - tableD1_298

  - tableD1_596

  - table_145

  - table_290

  - table_298

  - table_596

- Usage of STL/BASOP
stdint types are often still used, e.g. uint32_t - is this expected or should rather the STL types be used? This would need to be discussed as historically everything was fully converted to STL types while stdint types also have their merits.

# Conclusion

The fixed-point release for the IVAS decoder has been initially verified by the source, this should though only be seen as the first set of impressions without an in-depth analysis, which is planned for the future. The observations indicate that the package is not a final package but it also seems realistic that the observed points could be addressed in a subsequent release.

# References

- https://forge.3gpp.org/rep/sa4/audio/ivas-basop/-/releases/20240831_Ittiam_fixed_point_decoder_renderer_release

- 3GPP Tdoc SA4aA240038: Organization of IVAS Characterization Phase rev 1