Source:	Ericsson LM

Title:	On the status of the BASOP verification

Document for:	Discussion

Agenda Item:	1.5

## Introduction

The conversion of the IVAS floating point code to BASOP code is progressing. The second delivery, consisting of the first part of the encoder, was delivered on September 30th [2]. The verification of the decoder and renderer is ongoing in parallel.

## Progress

The time plan [3] stated that we would freeze the code of the decoder and renderer on October 15th, to allow subjective listening tests to be run on a stable and unique version. However, since there are still several open issues including crashes and quality artifacts, the code freeze was delayed to October 23rd. Further, it was found that the core decoder exceeds the computational complexity target by approximately 1.9 times. A part of this can be attributed to vector index operations being unnecessarily counted in BASOP, but there will likely be further changes required.

Since the verification of the decoder and renderer is consuming resources, the verification of the first delivery of the encoder may not have progressed that far. In addition, there are several intermediate conversions still in floating point, which means the encoding of the delivered formats is not fully completed. The background is that the floating-point code is kept operational for the non-converted formats, which makes it challenging to have a complete fixed-point path for a subset of the formats.

## PEAQ ODG for pass-through operation

One of the objective measures that is run in the automated tests is the PEAQ ODG score. The main setup compares the BASOP to the floating-point reference. An additional test, comparing to the input signal for the pass-through operation (same input and output format), has been set up. For the delivered encoder formats Stereo, ISM and Multichannel, delta ODG measures for a few operating points are shown in Figure 1, Figure 2 and Figure 3. The figures for ISM and Multichannel may however include parametric formats which are not yet converted to fixed point. Another issue with this test setup is that the comparison is done between the lowest observed scores across the channels. If the floating-point reference and the BASOP encoder produce a similar output for a low-quality channel, e.g. a zero-input LFE channel, the score will be reflecting this difference. For this reason, the figures below may not reflect the full picture and should be taken with a grain of salt.

Figure 1: Delta ODG scores for Stereo format, pass-through operation tests from the CI test suite.

Figure 2: Delta ODG scores for ISM format, pass-through operation tests from the CI test suite.

Figure 3: Delta ODG scores for Multichannel format, pass-through operation tests from the CI test suite.

## Core coder input overload

As mentioned in [5], the floating point IVAS encoder may produce signals for the core encoder that exceeds the 16-bit range -215,215-1=[-32768, 32767]. The signal overload is mostly pronounced for a few of the formats where a down-mix operation from a multi-channel input may result in a signal with a dynamic range that exceeds the limits. The problem is however not limited to these formats but may appear for most input formats. A solution to this issue is still under development.

## Summary

The BASOP conversion and verification of delivered parts is progressing, but there are several challenges to finalize the verification in time.

## References

- SA4aA240038, Organization of IVAS Characterization Phase rev 1, https://www.3gpp.org/ftp/TSG_SA/WG4_CODEC/3GPP_SA4_AHOC_MTGs/SA4_Audio/Docs/SA4aA240038.zip

- IVAS_BASOP_Release_Notes_30092024.pdf, https://forge.3gpp.org/rep/sa4/audio/ivas-basop/-/releases/20240930_Ittiam_fixed_point_encoder_release (also attached)

- Verification time plan, https://forge.3gpp.org/rep/sa4/audio/ivas-basop/-/wikis/Verification-time-plan

- Issue tracker on BASOP repository, https://forge.3gpp.org/rep/sa4/audio/ivas-basop/-/issues

- SA4aA240056, On the status of the BASOP conversion, https://www.3gpp.org/ftp/TSG_SA/WG4_CODEC/3GPP_SA4_AHOC_MTGs/SA4_Audio/Docs/S4aA240056.zip