3GPP TSG-SA WG4 Audio SWG post 130	SA4aA250001

Telco, 10 January 2025

Source:	HEAD acoustics GmbH

Title:	Test Method for Motion-to-Sound Latency

Agenda item:	1.6 (ATIAS_Ph2)

Document for:	Agreement

# Introduction

The IVAS codec supports binaural rendering with adaptation of the rendering to the listener's position and/or head orientation. When the listener turns their head, the acoustic scene appears fixed in space, which improves perceived externalization and immersion. A crucial system design criterion is a low latency between head movement and adapted sound at the ears. It is therefore necessary to measure and evaluate motion-to-sound (M2S) latency.

The present document

- discusses the technical background of measuring motion-to-sound latency,

- showcases the results of a proof of concept, in which M2S latency of a demo system featuring a real-time implementation of the IVAS renderer is measured, and

- proposes a candidate test method and requirements to be introduced in TS 26.260 [1] and TS 26.261 [2].

The proposed test method applies to headset user equipment (UE) in receive direction (RCV).

This document is a continuation of the discussion on input document S4-241964 [3] held at the SA4 #130 meeting in Orlando.

# Technical Background

## Scope and Literature Overview

Headtracking improves the perceived externalization and immersion by adapting binaural rendering to head orientation. As depicted in Figure 1, Motion-to-Sound (M2S) latency tM2S refers to the time difference between the time instant of movement of a listener's head t0 and the time instant t1 when the interaural cues present in the headphone signal are adapted. Perceptual thresholds for M2S latencies found in literature are in the order of about 60…100 ms [4], whereas lower values improve seamless and immersive listening experience.

Figure 1: Motion-to-sound latency - definition

Clause 4.2.3 of 3GPP TS 26.260 [1] ("Motion to Sound Latency in Dynamic Binaural Rendering Systems") already addresses a lab method to determine motion-to-sound latency. However, the method requires developer access to the rendering system (two parallel rendering chains, manipulation of headtracking data) and does not consider acoustic signal capture with a HATS. The method is thus not feasible for the "blackbox testing" approach, which is required for IVAS-based UE test methods in clause 5 of TS 26.260. Similar restrictions apply to several other test methods in literature, such as [6] and [7].

In contrast, the method from [8] specifically aims to assess M2S latency of black box rendering systems. The measurement setup uses an apparatus consisting of a pair of P.57-compliant [9] artificial ears mounted on a mechanical swivel platform that allows for manual yaw motion. The device under test is placed on the measurement apparatus such that the binaural rendering is adapted to the orientation of the swing apparatus. The binaural signal is captured with the artificial ears.

The approach presented in the present document adopts the basic approach from [8]. In particular, a similar measurement setup is used. However, several adjustments are made to improve measurement accuracy and meet the specific requirements of IVAS/ATIAS UE testing.

First, an impulsive motion is waived in favor of a continuous movement. Due to the mechanical inertia, it is not possible to generate arbitrarily short movement impulses, so that the temporal resolution is limited. Instead, it is preferred to choose a mechanical excitation that is closer to the actual application. UE is likely to be optimized for acceleration and velocity values that correspond to a human user.

Second, the signal analysis stage processes left and right ear signals independently by applying a fixed threshold to spectral differences over time. To evaluate spectral differences, a short time Fourier transform is applied, which again reduces temporal resolution bounds. This means that the actual spatial information, which is encoded in the interaural cues of the binaural signal, is not evaluated. Also, the selection of the threshold value is unclear. Instead, it is proposed to analyze both ears and evaluate interaural level differences, which again is inspired by human perception.

## Proposed Concept

The proposed measurement equipment is a motorized HATS but could in general also be realized with a comparable swivel apparatus with P.57-compliant artificial ears (such the one from [8]). The headset UE/device under test (DUT) is mounted on/fitted to the test equipment such that the binaural signals are rendered and played by the DUT and are acoustically captured by the artificial ears. The motorized HATS is able to perform yaw movements that are captured by the DUT's headtracking sensor.

The measurement apparatus requires some kind of position feedback, i.e., a sensor that tracks the exact point in time at which the head rotation passes 0° orientation. This can be realized, for instance, with an optical sensor or via feedback from the motor driver. An example setup is shown in Figure 2.

Figure 2: Example setup for measuring M2S latency of a DUT using a motorized HATS with position feedback using a laser pointer and an optical sensor

For the actual measurement, a test signal is rendered to an azimuth angle of 0° which corresponds to the front of listener for a 0° orientation. The HATS is then physically rotated from one side to the other and the time instant t0 is measured when the HATS passes 0° orientation. During motion, the binaural signals at the ears are recorded, from which the time instant t1 is determined at which the interaural cues of the binaural signal correspond to 0° orientation (signal analysis described below). The M2S latency is then determined via tM2S=t1-t0.

At an orientation of 0°, the monaural cues are monotonic when the angle changes and a change in sign occurs. Evaluating M2S latency at 0° is hence beneficial and robust to cope with different renderers and HRTF sets. However, with typical rotation velocities of the human head, even slight deviations of the interaural cues can lead to significant measurement deviations when determining time instant t1.

In order to improve the results, the measurement is continuously repeated, and the resulting values are averaged. Moreover, movements both from left to right and vice versa are performed as this allows to reduce the effect of a slight drift that the DUT's headtracking sensor may exhibit.

The rotation velocity needs to be sufficiently high for precise M2S latency measurements. Low rotation velocities lead to a low rate of change in the ILD curve, meaning that precise zero-crossing detection is not possible. According to the perceptual study in [5], humans are about equally sensitive for medium (~105°/s) and high head rotation velocities (~185°/s). For the current study, the rotation velocity of the motorized HATS was chosen in the same ballpark. To ensure comparability of the results, it seems also reasonable to specify a nominal velocity for such tests.

## Signal Analysis

Signal model and determination of interaural level difference

The following simplified signal model is used to derive the signal analysis. The binaural renderer and reproduction hardware is modeled as left and right ear FIR filters hl,φl and hr,φl with φ being the relative azimuth angle between source and head orientation and with sample index l. Given a test signal sl, the signals at the left and right ears xll and xrl, respectively, are given by
	xl|rl=hl|r,φl*sl+nl|r(l).

Additive noise, primarily introduced due to the movement of the mechanical apparatus, is modeled as terms nll and nr(l).

By applying a zero-phase matched filter hMFl specifically designed for the test signal sl, pre-processed left and right ear signals xll and xrl, respectively, are obtained that mitigate the impact of the noise, i.e.,
	xl|rl=xl|rl*hMFl≈hl|r,φl*sl.

Passing of frontal orientation is determined by evaluating the interaural level difference (ILD) on the pre-processed left and right ear signals xll and xrl, respectively. The ILD is calculated on short frames of length L and with a hop size of R and can be stated as
	ILDλ=clamp-ILDmax …ILDmax10log10l=0L-1xr2Rλ+ll=0L-1xl2Rλ+ldB.

Here, λ is the frame index that translates to frame times by means of
	tλ=1fsRλ+L-12	
with sampling rate fs.

By applying the clamp function
	clampξmin…ξmaxξ=maxξmin,minξmax, ξ,	
ILD is limited to ±ILDmax and improves numerical stability.

In order to cope with things like block-wise processing, finite headtracking update rates, noise etc., moving average smoothing is applied to obtain the smoothed ILD
	ILDλ=1Pr=-P-12P-12ILDλ+r.

Here, P is the odd-valued smoothing length stated in number of frames.

Choice of test signal

The above signal model and analysis lead to some design constraints for the test signal sl. The objective is that the ILD depends on hl|r,φl but is independent of sl. In particular, the following properties are pursued:

- sl must not contain any silent parts in order to yield reasonable ILDs for all frames of interest,

- sl needs be periodic with frame length L to avoid any impact of a non-zero interaural time difference on the result, and

- sl must be dominated by higher frequency content to introduce significant differences in ILD for varying angles φ.

- Characteristics of s(l) should be maintained also after encoding/decoding.

A pure sine signal
	sl=sin2πf0lfs	
with f0≫1kHz and R⋅f0fs∈N meets these requirements for a candidate test signal as is used in the current study. Other test signals might be possible as well but are for further study.

For such a signal, choosing hMFl as a narrowband zero-phase bandpass filter around f0 allows for a very good suppression of ambient or mechanical noise in the setup.

Detection of passing 0° orientation

Since even slight variance of the ILD can lead to significant measurement deviations when determining time instant t1 when passing the 0° orientation. It cannot be expected that the ILD perfectly vanishes at 0° due to various effects such as asymmetrical HRTF sets, left/right level imbalances of the UE (D/A, headphone drivers, ...) as well as fitting of earpieces in the artificial ears. Therefore, a precise calibration of the 0° orientation is required.

Calibration is performed by measuring the ILD for a certain period of time when the head orientation is 0° (no movements). The respective frames λ∈Λidle are evaluated and ILD0° is determined as the mean over the respective evaluation period, i.e.,
	ILD0°=1Λidleλ∈ΛidleILDλ,	
where the ⋅ operator determines the cardinality, i.e., the number of elements in Λidle.

Detection of passing 0° orientation can then be stated as finding zero-crossings in the compensated ILD
	ILDλ=ILDλ-ILD0°.

# Proof-of-Concept Experiment

## Objective

The objective of this proof of concept is to provide a realistic assessment of the capabilities of the test method described in this document. As there are no commercial off-the-shelf IVAS UEs available yet, a demo system was set up featuring a real-time implementation of the IVAS external renderer running on a PC. In order to be as close as possible to future IVAS UEs in terms of sensor technology, orientation sensor data from a standard smartphone is used that is fed into the renderer in real-time.

## Device under Test

Headtracking data is obtained from a Google Pixel 7 Pro smartphone mounted on top of the HATS. The smartphone has an orientation sensor, which can be accessed as "game rotation vector sensor" via the Android operating system API [10]. Using the Sensors2OSC app [11] and an IP connection over Wi-Fi, the data is sent to the IVAS renderer running on a separate PC.

The IVAS renderer implementation accounts for the headtracking data and renders a given spatial audio test signal to a binaural signal in real-time. The renderer implementation consists of a Python wrapper that interfaces the IVAS reference renderer [12].

The output of the renderer is played back on a pair of beyerdynamic DT°770 Pro headphones using a standard sound card.

## Measurement Setup

A motorized HATS (HEAD acoustics HMS II.3 with Move°S extension) equipped with ITU-T P.57-compatible Type 4.4 artificials ears were used. A photo of the measurement apparatus and DUT is shown in Figure 3.

Note: Unlike the example setup shown in Figure 2, no external optical sensor is used here as it is obsolete due to internal sensing mechanisms of the HATS motor control.

motorized HATS
(measurement apparatus)

wired headphones
(part of the DUT)

smartphone serving as headtracking sensor
(part of the DUT)

Figure 3: Experiment setup for measuring M2S latency of a demo system featuring a real-time implementation of the IVAS renderer

## Experiment Realization

The test signal was a 2 kHz sine rendered to 0° azimuth and 0° elevation in first-order Ambisonics (FOA) format. The signal was then encoded and decoded using the IVAS codec with a bit rate of 128 kbit/s and fed into the DUT.

First, the motorized HATS is oriented toward 0° for a few seconds in order to obtain ILD calibration data.

The motorized HATS is then rotated 12 times from right to left and vice versa, binaural signals are captured simultaneously via artificial ears. A velocity of 108°/s was used, corresponding to a medium-to-fast head rotation [5]. The HATS trajectory is shown in the upper plot in Figure 4.

The recorded binaural signal is pre-processed by applying a narrow zero-phase flattop bandpass filter to reduce noise of the mechanical apparatus. The signal analysis is performed with short frames with L=96 samples and a hop size of R=24 samples, which corresponds to 2 ms and 0.5 ms, respectively, at a sampling rate of 48 kHz. ILD smoothing is performed over P=101 frames, which corresponds to approximately 50 ms.

## Results

The HATS orientation over time and the ILD over time are shown in Figure 4. The time range, which is used for calibration, is highlighted in blue. The analysis time range, in which all zero-crossings are analyzed, is highlighted in red. In both plots, the zero crossings (upper plot: motion/orientation, lower plot: sound/level) are indicated with black vertical bars.

…

…

Figure 4: Measurement results plotted over time. Upper plot: HATS orientation. Lower plot: interaural level difference with zoom

First of all, it can be observed that the ILD for 0° shows an offset of about 2 dB. As the ILD slope is about 0.05 dB/ms, neglecting this offset would lead to a measurement error in the order of 40 ms.

Moreover, it can be seen that for angles around 0°, the smoothed ILD seems monotonic and exhibits approximately linear behavior. However, the microscopic behavior of the non-smoothed ILD is indeed non-monotonic. In the ILD plot, the region around the third zero-crossing is enlarged. One can observe abrupt ILD jumps due to adaptation of the direction by the DUT only every 20ms. Moreover, the ILD exhibits slight ILD fluctuations. Therefore, it becomes clear that ILD smoothing is necessary for robust detection of zero-crossings.

The resulting motion to sound latency measurements for each zero crossing, i.e., the offsets of the vertical bars in the upper and lower plots in Figure 4, are shown in Figure 5. One can see that all repetitions show similar values, which fluctuate only slightly around an average of 93.4 ms, which seems to be a reasonable magnitude of order for M2S latency of the given DUT. Due to the absence of outliers, the median value is close to the mean value with 94.6 ms.

Figure 5: Resulting M2S latency measurements

In the course of experimentation, the above experiment was repeated several times. Sometimes spurious outliers could be observed. Hence, in the favor of robustness of the measurement, it is proposed to consider the median instead of the mean value to determine a single-value M2S latency.

The experiment was also conducted with 3rd order HOA and 5.1 surround test signals, both coded with 128 kbit/s. Similar results were obtained as for the presented case with FOA.

## Effect of Sensor Drift

The above experiment is repeated with a lower quality head-tracking sensor. A dedicated head-tracking sensor with USB interface replaces the smartphone connected via Wi-fi.

The resulting M2S latency measurements are shown in Figure 6. For rotations from left to right (even indices), the measured values decrease, while for rotations from right to left (odd indicies) the values increase. This is the result of undesired sensor drift. However, the effect of sensor drift seems to be mitigated partially by averaging over both rotation directions. The mean M2S latency measurement is 101.4 ms and the median is 104.0 ms. Note that the value is expected to be in the same order of magnitude as in the previous experiment, but the exact value is unknown due to different hardware.

Figure 6: M2S latency measurements in the presence of sensor drift

Besides sensor drift, an effect that could arise in some IVAS UE use-cases is that a DUT intentionally adapts its reference/zero orientation over time, for instance, based on the average of past orientations (cf. options in [13]). Note that the measurement method presented here is not capable of dealing with such an adaptation and would lead to an underestimation of the actual M2S latency.

# Proposed Test Method and Requirements

Based on the theoretical background presented in Section 2 and the proof-of-concept in Section 3, it is proposed to consider the test method described in Annex A.1 as a candidate test method for TS 26.260.

In summary, the approach consists of the following stages:

Calibration: measure interaural features for 0° UE orientation (fitting, sensor, rendering, UE hardware)

Measurement: rotate UE from left to right and/or vice versa, record binaural signal, determine time t0 of passing 0°; repeat multiple times

Analysis: compare interaural features of recorded binaural signal to calibration data to determine t1 at which binaural signal corresponds to 0° orientation; latency is determined via tM2S=t1-t0.

Candidate requirements for TS 26.261 are given in Annex A.2.

# Summary

The source proposes a test method for M2S latency measurements of headset IVAS UE in receive direction using a test setup similar to the approach in [8]. The contribution provides technical background as well as an extensive proof-of-concept. Several adjustments to existing methods are made to meet the specific requirements for black-box IVAS UE testing. As there are currently no off-the-shelf IVAS UEs, a real-time demo setup based on the IVAS external renderer was set up in order to mimic functionalities of a UE. Using this demo setup, it could be shown that the proposed test method is well suited to measure M2S latency of IVAS UEs.

The source proposes to consider the test as stated in Annex A.1 with the corresponding requirements in Annex A.2 as candidates for the integration into 3GPP TS 26.260 and 3GPP TS°26.261, respectively.

# References

- 3GPP TS 26.260, “Objective test methodologies for the evaluation of immersive audio systems,” Release-18, 2024.

- 3GPP TS 26.261, “Terminal audio quality performance requirements for immersive audio services,” Release-18, 2024.

- HEAD acoustics GmbH, “Test methods for UE headtracking and binaural rendering”, 3GPP input document S4-241964 to 3GPP TSG-SA WG4 Meeting #130, USA, Orlando, November 2024.

- D. S. Brungart, B. D. Simpson, and A. J. Kordik, “The detectability of headtracker latency in virtual audio displays,” International Conference on Auditory Display (ICAD), Limerick, Ireland, 2005.

- C. Rappin, J. Palacino, P. Rueff, L. Feichter, and M. Paquier, “Latency detection threshold of head-tracking for different head rotation speeds in binaural rendering,” Forum Acusticum, Torino, Italy, Sept. 2023.

- J. Estrella and J. Plogsties, “Motion-to-Sound Latency Measurement Procedure for VR Sound Reproduction”, 142nd AES Convention, Berlin, Germany, May 2017.

- N. Meyer-Kahlen, M. Kastemaa, S. J. Schlecht, and T. Lokki, “Measuring Motion-to-Sound Latency in Virtual Acoustic Rendering Systems,” Journal of the Audio Engineering Society (JAES), vol. 71, no. 6, 2023. DOI: 10.17743/jaes.2022.0089.

- A. Tung, A. Genovese, G. Davis, A. Schevciw, „Motion to Sound Latency Measurement for Black-Box Spatial Audio Renderers,“ AES 5th International Conference on Audio for Virtual and Augmented Reality (AVAR), Redmond, WA, USA, August 2024.

- ITU-T, “Recommendation P.57, Artificial Ears,” International Telecommunication Union, 2021.

- Android developers documentation, “Position sensors”, URL: https://developer.android.com/develop/sensors-and-location/sensors/sensors_position (accessed 12/2024)

- Sensors2OSC product website, URL: https://sensors2.org/osc/ (accessed 12/2024)

- 3GPP TS 26.254, “Codec for Immersive Voice and Audio Services (IVAS); Rendering”, Release-18, 2024.

- IVAS codec public collaboration source code file “lib_rend/ivas_orient_trk.c”, URL: https://forge.3gpp.org/rep/ivas-codec-pc/ivas-codec/-/blob/main/lib_rend/ivas_orient_trk.c (accessed 12/2024)

- Annex

## Proposed Changes to TS 26.260

This section proposes changes to TS 26.260 to incorporate the test method described above.

Start change	[

### 4.0.2	Test equipment

Unless specified otherwise, the accuracy of electric and acoustic measurements made by test equipment shall meet the requirements defined in clause 5.3 of 3GPP TS 26.132 [27].

For tests with head tracking, HATS rotation around the vertical axis should be realized using a HATS with motorized head rotation or comparable equipment consisting of P.57-compliant artificial ears mounted on a turntable or swivel apparatus (denoted as "motorized HATS" in the following clauses). For dynamic head-tracking measurements, the equipment is subject to angular velocity and noise emission requirements as specified in the respective test directives. Moreover, dynamic head-tracking measurements require the equipment to provide some kind of sensing mechanism for tracking orientation over time. For motorized or manual rotations of HATS and/or UE, error in orientation (elevation and azimuth) shall not exceed ±2°.

NOTE 1:	A motorized rotation of HATS and/or UE is recommended even for static head-tracking measurements. Some UEs may not have a natural reference orientation (which, for instance, may be defined by the direction of a screen). In this case, the UE may reset the reference direction automatically after a span of time, e.g., to the current device orientation. This should be taken care of during the measurement. The measurement with the rotated HATS and/or UE should be performed quickly enough to prevent the reference direction from being spuriously readjusted. Therefore, it benefits from automation.

NOTE 2:	Dynamic head-tracking measurements mimic the behavior of typical human head rotation. Angular rotation velocities and noise emission limits are required that may not be met by standard turntables.

]	End change

Start change	[

5.7.5	Motion-to-Sound Latency

The test method applies to headset UE providing headtracking functionality.

Editor’s note: What about electrical interface testing (electrical interface intended for headset usage)?

The test setup is as described in clause 5.4.3.2. A motorized HATS or comparable apparatus suitable for dynamic head-tracking measurements as described in clause 4.02 is used. The measurement apparatus shall be capable of realizing the below-stated movement trajectory. It shall feature a sensing mechanism that tracks the point in time at which the apparatus passes 0° orientation with a tolerance of [±2 ms]. Moreover, its noise emission shall not exceed [-70 dBPa] measured at the artificial ears after signal filtering. An example setup is illustrated in Figure F1.

Figure F1: Example setup for measuring M2S latency using a motorized HATS with optical position feedback using a laser pointer and an optical sensor

Editor's note: Example figure from SA4aA250001, update will be provided later.

The test method evaluates interaural level differences that are calculated from the artificial ear signals with the following algorithm:

1)	The left and right ear signals are each filtered with a narrow zero-phase bandpass filter that is designed to suppress noise of the measurement apparatus while not impacting the test signal. The filtered signals are denoted as xll and xrl with sample index l and shall be made available at a sampling rate of fs=48 kHz.

2)	The interaural level difference (ILD) is determined on the pre-processed left and right ear signals xll and xrl, respectively. The ILD is calculated on short frames with index λ and can be stated as
	ILDλ=max-ILDmax,minILDmax, 10log10l=0L-1xr2Rλ+ll=0L-1xl2Rλ+ldB.

For the calculation, frame length L=[96] and hop size R=[24] are used. The ILD is limited to
-ILDmin≤ILD(λ)≤ILDmax, with ILDmin/max=±12 dB.

3)	The ILD is then smoothed over P=101 frames using a moving average with
	ILDλ=1Pr=-P-12P-12ILDλ+r.

Time instances per frame are determined by
	tλ=1fsRλ+L-12.

The motion-to-sound (M2S) latency measurement shall be conducted as follows:

1)	The test signal to be used for the measurements shall be a sine signal at [2 kHz] virtually positioned to 0° azimuth and 0° elevation (frontal position for 0° head orientation) as described in clause 5.5.2. The signal shall be calibrated to [-20 LFKS] as described in clause 5.5.1.

2)	The UE and the reference client are setup according to clauses 5.4.2 and 5.3.2, the source signal is encoded by the reference client and inserted at the POI to the UE.

3)	The motorized HATS is oriented to 0°. For ILD calibration, the ear signals are captured for a duration of [3 s] without applying any motion to the UE. Then, ILDλ is determined for the ILD calibration sequence and an average ILD (denoted as ILD0°) is calculated as the mean of ILDλ.

4)	The dynamic part of the measurement (in motion) is conducted as follows:

a)	The motorized HATS rotates continuously and repeatedly between [+60°] (start orientation) and [-60°]. The 0° orientation shall be passed Nmeas=[12] times, i.e., [6] times for each direction of rotation, so that a total of Nmeas partial estimates of M2S latency are determined.

b)	In the angle range between -10° and +10°, the angular velocity shall be constant with a nominal angular velocity of [108°/s].

c)	During motion, the left and right ear signals and the time stamps t0(i) (where the motorized HATS passes 0° orientation) are captured for i=1,…, Nmeas.

5)	After the last orientation is reached, the motorized HATS is immediately oriented to 0°. Step 3) is repeated to obtain the verification ILD, denoted as ILD0°,v.

6)	The M2S latency time is calculated as follows for each zero-crossing i=1,…, Nmeas of the orientation:

a)	The smoothed ILD versus time ILD(i)λ is determined.

b)	The time stamp t1(i) of the sound signal passing 0° orientation is determined by identifying the zero-crossing of ILD(i)λ- ILD0°.

c)	The M2S latency for the i-th measurement is determined by T(i)=t1(i)-t0(i).

7)	The overall M2S latency is determined by [calculating the median] of T(i).

Editor’s note/To Do: Outlier analysis of T(i) may help to detect erroneous measurements?

Editor’s note:	The measurement method presented requires a constant reference/zero orientation. In some IVAS UE use-cases, a DUT may intentionally adapt its reference/zero orientation over time, for instance, based on the average of past orientations. The measurement method is not suited for such cases.

Editor’s note:	The test method could also be extended to determine angular drift (with corresponding requirement for 26.261). This is important as a sufficiently low drift is needed to ensure a valid measurement of M2S latency.

]	End change

## Proposed Changes to TS 26.261

This section proposes changes to TS 26.260 to incorporate requirements:

Start change	[

6.5.3	Motion-to-Sound Latency

The M2S latency measured with the test method from clause 5.7.5 from TS°26.260 should not exceed [80 ms] and shall not exceed [100 ms].

The absolute difference between the initial ILD calibration (ILD0°) and the verification ILD (ILD0°,v) shall not exceed [0.5 dB].

]	End change