Source:	Bytedance

Title:	[ATIAS ph2] Objective Test Methods for Capturing Immersive Audio with Moving Sound Source

Document for:	Discussion and Agreement

Agenda Item:	1.6

# 1 Introduction

In previous specs, objective test methods were proposed for spatial audio capturing, including tests for user capture and spatial capture. In user capture tests, sound source is a loudspeaker placed at mouth of a dummy (HATS); In spatial capture tests, sound source is the same configured dummy placed around the tested UE.

Currently sound sources are placed at following locations relative to UE [2]:

| UE Type | Self-capture test | Spatial capture test |
| --- | --- | --- |
| Headset UE | Artificial mouth of HATS | 0-degree azimuth and elevation 1m relative to the reference point of the HATS |
| Handheld hands-free UE | At artificial mouth of HATS, 42 cm from the center point of the visual display of the UE | 0-degree azimuth and elevation 1m relative to the reference point of the HATS |
| Table-mounted UE | At artificial mouth of HATS, 40 cm between UE and lip reference plane and a height of 40 cm | 0° azimuth, 26.6° elevation and 89.4 cm relative to the reference point of the HATS |

Table 1. Existing sound source positions in capture tests of UEs

And in tests for directional information, several additional source positions are selected for spatial capture tests, including [2]:

| i | Source azimuthφi | Source elevation θi |
| --- | --- | --- |
| 1 | -90 | 0 |
| 2 | -60 | 0 |
| 3 | -30 | 0 |
| 4 | 0 | 0 |
| 5 | 30 | 0 |
| 6 | 60 | 0 |
| 7 | 90 | 0 |

Table 2: Additional source directions

However, all source locations are static.

From the information we gathered, it turns that we haven’t got a way to evaluate capturing fidelity when sound field contains moving sound sources, which arguably is the majority case in real life.

# 2 Discussions

In order to evaluate sound field capturing capability with moving sound sources, we propose the following objective test method:

## 2.1 Test Material

White Noise, whose loudness is calibrated using the method described in clause 5.5.1 in 3GPP TS 26.260 [2].

## 2.2 Test Candidate Signal

The method to produce the test candidate signal can be described as follows:

- The recording environment is an anechoic chamber that meets clause 4.0.3 of 3GPP TS 26.260 [2]

- The recording environment has a loudspeaker array that meets all criteria described in clause 4.1.1.4.1 of 3GPP TS 26.260 [2], with an extra requirement that there should be at least one semiarch loudspeaker array that can be placed horizontally, with its geometric center placed on the reference point of candidate UEs.

- Use VBAP [3] to pan a virtual sound source on the loudspeaker arch, with the following trajectory:

- Start from one end of the arch.

- On the arch, move towards the other end, with constant angular velocity of 108 degrees per second.

- Keep the same angular speed, move virtual source back to the starting point.

- Repeat the above trajectory once.

## 2.3 Test method

Our test method supports the following target audio formats , including ISM, stereo, MC, SBA, and binaural. MASA is not included since MASA comes from analysis of FOA and HOA signal.

In general, we calculate the following time-varying metrics from the test candidate signal and reference. All metrics are measured each 20 ms buffer.

### 2.3.1 ISM (Object)

#### 2.3.1.1 Reference Signal

A sound object that follows the moving trajectory in 2.2. The source trajectory is simulated by VBAP rendered to a virtual loudspeaker array, same as 2.2. The rendering method is aligned to clause 5.5.2 in 3GPP TS 26.260 [2].

#### 2.3.1.2 Candidate Signal

Candidate signal is recorded by placing the reference point of DUT at the geometric center of the loudspeaker array described in 2.2. DUT must be able to record ISM audio.

#### 2.3.1.3 Measurement

For ISM recordings, we measure the time-varying RMS curve for object signal, and the time-varying source direction in terms of azimuth angle. RMS and angle are measured once each audio buffer, which is 20 ms long.

yRMSk=t=0.02k0.02k+1y2t0.02

#### 2.3.1.4 Error Metric

We calculate the standard deviation of time-varying signal RMS of the candidate signal.

yRMSk=k=0KyRMSkK

σyRMSk=k=0KyRMSk-yRMSk2K

We also calculate time-varying difference between source angle of reference signal θt and candidate signal θt.

\Delta ∆θt\theta(t) =  θt-θt\Big|\theta(t) - \hat{\theta}(t)\Big|

### 2.3.2 SBA (Ambisonics)

#### 2.3.2.1 Reference Signal

For reference signal, we rendered the test material defined in 2.1 to ambisonic, with the same order as recorded SBA, and the same trajectory defined in 2.2. The source trajectory is simulated by VBAP rendered to a virtual loudspeaker array, same as 2.2. The rendering method is aligned to clause 5.5.2 in 3GPP TS 26.260 [2].

#### 2.3.2.2 Candidate Signal

Candidate signal is recorded by placing the reference point of DUT at the geometric center of the loudspeaker array described in 2.2. DUT must be able to record SBA audio.

#### 2.3.2.3 Measurement

For SBA recordings, we measure the following two metrics

- The time varying RMS of W channel WRMSk

WRMSk=YRMS0,0k=t=0.02k0.02k+1Y0,02t0.02

- The sound field directivity depicted by time-varying normalized spherical harmonics coefficientsNl,mkdefined by following processes

YRMSl,mk=t=0.02k0.02k+1Yl,m2t0.02,l∈0order,m∈-ll

Nl,mk=YRMSl,mkWRMSk+ϵ,l∈0order,m∈-ll,ϵ=10-20

#### 2.3.2.4 Error Metric

We calculate  the standard deviation ofWRMSk of the candidate signal.

WRMSk=k=0KWRMSkK

σWRMSk=k=0KWRMSk-WRMSk2K

We calculate the normalized spherical harmonics coefficients error by computing the 1 minus absolute value of Pearson correlation betweenNl,mk of reference and candidate signal.

- ΔNl,mk=1- rNl,mk=1- cov(Nl,mk, Nl,mk)σNl,mkσNl,mk

#### 2.3.2.5 Test Result

We first simulated recording error by attenuating the virtual loudspeakers at [-30,-20,-10,0,10] degrees by [-3.6, -10.8, -9.4, -8.9,-6.9] dB, respectively, then treating the rendered Ambisonics as candidate signal. The candidate signal is tested against the reference signal, getting the following error plot:

σWRMSk=0.010745

Figure 1: SBA Test Result with Simulation Data

To further verify the effectiveness of our test method, we also tested a legacy in-house spatial audio recording solution, and got the following results:

σWRMSk=0.006424

Figure 2: SBA Test Result with Recorded Data

### 2.3.3 Binaural

#### 2.3.3.1 Reference Signal

reference signal yl,ris recorded by HATS with the loudspeaker array defined in 2.2.

#### 2.3.3.2 Candidate Signal

- Headset put on HATS

- The DUT shall be configured such that the set of HRTFs used for binaural rendering correspond to the HATS used for testing

- The binaural signals recorded by UE are played over the headset and recorded by HATS as yl,r.

#### 2.3.3.3 Measurement

calculate the time varying ICLD difference between pl,r and pl,r

yRMSlk=t=0.02k0.02k+1yl2t0.02

yRMSrk=t=0.02k0.02k+1yr2t0.02

ICLDk=20logyRMSlk+ϵyRMSrk+ϵ

#### 2.3.3.4 Error Metric

We define the time-varying difference betweenICLDkand ICLDkin dB

Δ ICLD(k)=|ICLD(k)-ICLD(k)|

#### 2.3.3.5 Extra Requirement

Subjective tests are mandatory for binaural output format.

#### 2.3.3.6 Test Result

We first simulated recording error by attenuating the virtual loudspeakers at [-30,-20,-10,0,10] degrees by [-3.6, -10.8, -9.4, -8.9,-6.9] dB, respectively, then treating the rendered Ambisonics as candidate signal. The candidate signal is tested against the reference signal, getting the following error plots:

Figure 3: Binaural Test Result with Simulated Data

To further verify the effectiveness of our test method, we also tested a legacy in-house spatial audio recording solution, and got the following results:

Figure 4: Binaural Test Result with Recorded Data

# 3 Proposal

We kindly ask the SWG to accept our proposed objective test method, and then integrate them into 3GPP TS 26.260. Agreements can be made on an audio-format-to-format basis.

# References

[1] TS 26.259, Subjective test methodologies for the evaluation of immersive audio systems

[2] TS 26.260, Objective test methodologies for the evaluation of immersive audio systems.

[3] Pulkki, V. (1997). Virtual Sound Source Positioning Using Vector Base Amplitude Panning. Journal of The Audio Engineering Society.