3GPP TSG-SA WG4 Meeting #Ad hoc post #129-e	S4aI240139

# Online, 26th September 2024 - 24th October 2024			revision of S4-241699 and S4-241698

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.804 | CR | <CR#> | rev | <Rev#> | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [FS_AMD] New clause 5.24 QUIC-based Media Delivery |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Xiaomi |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_AMD |  |  |  |  |  | Date: |  |  | 10-10-2024 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | FS_AMD includes the topic about opportunities with QUIC for segmented streaming (topic “m)”). The corresponding new clause “QUIC-based segmented media delivery” needs to be developped. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR merges both previously endorsed as basis for further work CRs S4-241699 and S4-241698. The following changes are proposed: - Add clause 5.24 QUIC-based Media Delivery |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Change #1

## 2	References

[2]	Akamai Blog, "A QUICk Introduction to HTTP/3", April 2020, https://www.akamai.com/blog/developers/a-quick-introduction-http3

[5]	IETF RFC 9114: "HTTP/3", June 2022

[W3C-WT]	W3C Working Draft: "WebTransport", May 2024,
https://www.w3.org/TR/webtransport

Change #2

## 5.24	QUIC-based segmented media delivery

### 5.24.1	Description

#### 5.24.1.1	General

QUIC, specified in RFC 9000 [32], is a secure, reliable, multiplexed, connection-oriented transport protocol built on top of UDP. It is widely available and its impact on media streaming requires further study. In this clause we study QUIC-specific media streaming, not necessarily based on HTTP/3 [5] that is studied in clause 5.4.

A QUIC client establishes a connection with a server, and within this connection multiple concurrent streams can transport data . Thanks to a more efficient implementation of the TLS initial handshake, a QUIC connection is typically established faster than a TCP + TLS connection, therefore reducing initialization time. Additionally, by allowing QUIC streams to be multiplexed into a single QUIC connection, they can operate independently of each other, each with its own separate congestion window. Because packet retransmission occurs within a stream, a stall in one stream does not block the progress in the other streams. Similarly, packet loss in one stream does not affect the progress of data transfer in other streams multiplexed in the same QUIC connection. Finally, QUIC, like HTTP/2 [4], enables prioritisation capabilities at a stream level. However, QUIC does not provide the means to signal this prioritisation when multiplexing streams and it is up to the client implementation and the application to exchange and set the appropriate priorities.

Even though QUIC solves many issues compared to TCP (and HTTP/3 compared to HTTP/2 respectively), there are some open issues and shortcomings when it comes to media content delivery.

#### 5.24.1.2	Application access to QUIC protocol features

In a common scenario, QUIC is not directly exposed to the application but it is mediated through a QUIC-enabled application protocol such as HTTP/3 [5] or WebTransport [W3C-WT]. For that reason, the set of QUIC features available to an application is the subset exposed by the chosen application protocol (typically invoked via the public API of a client or server library). This approach facilitates the development of QUIC-based communication but at the cost of limiting the level of control by the application on the connection and stream management.

#### 5.24.1.3	Connection and stream management

Applications are encouraged to keep QUIC connections alive when it makes sense to do so, and HTTP/3 client libraries typically facilitate connection keep-alive behaviour for efficiency reasons. In addition, QUIC's "0-RTT" connection establishment procedure allows a client to reconnect to a server it has previously connected to, and to reuse a security context negotiated during a previous connection and cached by both parties to send application payload data to the server in the first UDP datagram of the new QUIC connection.

In the context of HTTP/3 [5], the mechanism for handling streams is tightly specified by the HTTP/3 protocol that is each HTTP request-response transaction consumes one stream in the QUIC connection.

In contrast, WebTransport [W3C-WT] allows the application to manage the connections and the streams without however being studied yet in the context of segmented media delivery.

Editor's Note: Some publications about WT and streaming were recently published. Further input needed.

WebTransport allows both the client and the server to request opening streams. Finally, WebTransport supports different congestion control algorithms and unreliable connections.

Editor's Note: Further content to be provided.

#### 5.24.1.4	Stream prioritisation

Even though QUIC supports prioritisation at a stream level, it does not provide the means to exchange the prioritisation information. The way this prioritisation is applied is thus not standardised and left to the implementation. Additionally, stream priority is nominated by the sender, and as such neither the network nor the recipient are aware of the applied mechanism.

#### 5.24.1.5	Key Issue objectives

Editor's Note: Focus the preceding set of open issues into a set of clear questions that this Key Issue aims to address.

##### 5.24.1.5.1	Introduction

The objectives listed below are targeting QUIC-specific delivery aspects;.

- Application access to QUIC connections

- Variability of QUIC implementations

Editor's Note: Further content to be provided.

### 5.24.2	Collaboration scenarios

#### 5.24.2.1	General

For the purpose of describing the following scenarios, it is only assumed that the 5GMS Client supports the QUIC protocol. Additionally, the 5GMS Client may support higher level protocols based on QUIC (for instance HTTP/3 or WebTransport) but this is not required and the analysis of those collaboration scenarios is still applicable to any of those cases.

#### 5.24.2.2	QUIC-agnostic 5GMS Client

In this scenario, the Media Stream Handler of the 5GMS Client operates a QUIC session over reference point M4 but the 5GMS Client has no specific feature regarding QUIC. This has the advantage that 5GMS Client is generic and implements the same logic whether or not QUIC is used for the delivery of the media.

#### 5.24.2.3	Media-independent QUIC-aware 5GMS Client

In this scenario, the Media Stream Handler of the 5GMS Client operates a QUIC session over reference point M4 and the 5GMS Client is able to detect whether QUIC is used and, in case it is used, the 5GMS Client can apply different logic. In this case, the QUIC client implementation is not specifically optimised for media transport (e.g. a generic off-the-shelf QUIC client library) and the set of QUIC protocol features exposed to the Media Stream Handler is limited by the richness of its API.

With some limited control over the QUIC streams, such a 5GMS Client would typically be able to:

- Set relative priorities between the different QUIC streams, especially if HTTP/3 is used by the Media Stream Handler as the application protocol on top of QUIC. Relative stream priorities can be useful to differentiate audio and video, base layer and enhancement layer, etc.

- Receive updates sent proactively by the 5GMS AS, especially if the HTTP/3 server push feature is used. For example, a 5GMSd AS could push MPD updates to a Media Player using this mechanism.

Editor’s note: More possible general features to be added.

#### 5.24.2.4	Media-optimised QUIC-aware 5GMS Client

In this scenario, the Media Stream Handler of the 5GMS Client operates a QUIC session over reference point M4 and the 5GMS Client is able to control the delivery of the media within the QUIC session. In this case, the QUIC client implementation is optimised for media transport and the set of QUIC protocol features exposed to the Media Stream Handler is therefore unlimited. Hence, the media-optimised QUIC-aware 5GMS Client provides the finest control over the delivery of media within the QUIC session.

With fine control over the QUIC streams, such a 5GMS Client would typically be able to:

- Set relative priorities between the different QUIC streams, especially if HTTP/3 is used by the Media Stream Handler as the application protocol on top of QUIC. Relative stream priorities can be useful to differentiate audio and video, base layer and enhancement layer, etc.

- Receive updates sent proactively by the 5GMS AS, especially if the HTTP/3 server push feature is used. For example, a 5GMSd AS could push MPD updates to a Media Player using this mechanism.

- Use one QUIC stream for all the media segments of a given component (e.g. per CMAF Track).

Editor’s note: More possible special features to be added.

### 5.24.3	Architecture mapping

#### 5.24.3.1	General

Editor's Note:	Further content to be provided.

### 5.24.4	High-level call flow

#### 5.24.4.1	General

Editor's Note: Further content to be provided.

### 5.24.5	Gap analysis and requirements

### 5.24.6	Candidate Solutions

Editor’s Note: Provide candidate solutions (including call flows) for each of the identified issues.

### 5.24.7	Summary and Conclusions

The study of this Key Issue has explored the ways in which QUIC can be deployed to support the 5G Media Streaming architecture, and the potential open issues arising from this deployment.

Editor's Note: Further content to be provided.

END OF CHANGES