## 3GPP TSG SA WG4  #129-e	S4aI240140

# Online, September 26 – October 23  2024	revision of S4-241609

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |  |
|  | 26.804 | CR | 0006 | rev | 9 | Current version: | 18.1.0 |  |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [FS_AMD] Multi-CDN and Multi-Access Media Delivery: client side CDN switching |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_AMD |  |  |  |  |  | Date: |  |  | 2024-08-13 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Document thecurrent approaches for multi-cdn and client side switching |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of new clause 5.19.6.X on client side CDN switching |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No baseline for Multi-CDN or accurate GAP analysis |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 5.x (NEW) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Separated out text on client side multi CDN |  |  |  |  |  |  |  |  |

## FIRST CHANGE

## 2	References

[DASH9]	Draft Text of ISO/IEC FDIS 23009-9 Information technology - Dynamic adaptive streaming over HTTP (DASH) – Part 9: Redundant Encoding and Packaging for segmented live media (REaP), ISO/IEC JTC 1/SC 29/WG 3 NO 1165, Jan. 26, 2024. [Online]. Available: https://www.mpeg.org/standards/MPEG-DASH/9/

[UNPKG24]	Emma Roth, "A popular open-source content delivery network went down for hours", The Verge, Apr. 12, 2024. [Online]. Available: https://www.theverge.com/2024/4/12/24128276/open-source-unpkg-cdn-down (accessed May 9, 2024).

[NET23]	Sebastian Moss, "Cloudflare recovers from service outage after power failure at core North American data center", Data Center Dynamics, Nov. 3, 2023. [Online]. Available: https://www.datacenterdynamics.com/en/news/cloudflare-recovers-from-service-outage-after-power-failure-at-core-north-american-data-center/ (accessed May 9, 2024).

[FSLY21]	Brian Barrett, "How an Obscure Company Took Down Big Chunks of the Internet", Wired, Jun. 8, 2021. [Online]. Available: https://www.wired.com/story/fastly-cdn-internet-outages-2021/ (accessed May 9, 2024).

[AKAM21]	Josh Fomon, "CDN Provider Akamai Takes Down Popular Internet Services During Outage", Ookla, Jul. 22, 2021. [Online]. Available: https://www.ookla.com/articles/akamai-outage-july-22-2021 (accessed May 9, 2024).

[NET22]	Charlotte Trueman, "Cloudflare outage brings hundreds of sites, services temporarily offline", Computer World, Jun. 21, 2022. [Online]. Available: https://www.computerworld.com/article/1627967/cloudflare-outage-brings-hundreds-of-sites-services-temporarily-offline.html (accessed May 9, 2024).

[VZ19]	Jim Salter, "The Internet broke today: Facebook, Verizon, and more see major outages", Ars Technica, Jul. 3, 2019. [Online]. Available: https://arstechnica.com/information-technology/2019/07/facebook-cloudflare-microsoft-and-twitter-suffer-outages/ (accessed May 9, 2024).

[DEMX01]	Marc Hoppner, "A content owner, a CDN and a player walk into a bar", (Jan. 6, 2023). Accessed: May 9, 2024. [Online Video]. Available: https://www.youtube.com/watch?v=S9EdoQFOQ9I&list
=PLkyaYNWEKcOf98lZxnCcL6y7ZIVU3oSYO&index=12

[DEMX02]	Guillaume du Pantavice, "Improving streaming experience with Bayesian optimization, from AB to AZ test", (Dec. 25, 2021). Accessed: May 9, 2024. [Online Video]. Available: https://www.youtube.com/
watch?v=t4nRrLygVwo&list=PLkyaYNWEKcOfD1GYFxFbZXDP03XM-cZPg&index=19

[IEEE01]	E. Ghabashneh and S. Rao, "Exploring the interplay between CDN caching and video streaming performance", IEEE INFOCOM 2020 – IEEE Conference on Computer Communications, Toronto, ON, Canada, 2020, pp. 516-525.

[ACM01]	K. Vermeulen, L. Salamatian, S. H. Kim, M. Calder, and E. Katz-Bassett, "The central problem with distributed content: common CDN deployments centralize traffic in a risky way", In Proceedings of the 22nd ACM Workshop on Hot Topics in Networks (HotNets ’23). Association for Computing Machinery, New York, NY, USA, 70-78.

[MHV01]	A. Bentaleb, R. Farahani, F. Tashtarian, C. Timmerer, H. Hellwagner, and R. Zimmermann, "Which CDN to Download From? A Client and Server Strategies", (Jan. 6, 2024). Accessed: May 9, 2024. [Online Video]. Available: https://www.youtube.com/watch?v=xCZmCnWgQRE

[VAS01]	Will Law, "Content steering with MPEG DASH", (May 4, 2023). Accessed: May 9, 2024. [Online Video]. Available: https://www.youtube.com/watch?v=o9Pa5y-Usxw

[MWS23]	W. Law and Y. Reznik, "DASH content steering at scale", Media Web Symposium (MWS’23), June 2023.

[DIFCS]	ETSI TS 103 998: "DASH-IF: Content steering for DASH".

[CMMF]	ETSI TS 103 973: "Coded multisource media format (CMMF) for content distribution and delivery".

[TS26501]	3GPP TS 26.501: "5G Media Streaming (5GMS) General description and architecture (Release 18)".

[TS26510]	3GPP TS 26.510: "Interactions and APIs for provisioning and media session handling (Release 18)".

[TS26512]	3GPP TS 26.512: "5G Media streaming (5GMS) protocols (Release 18)".

[TS103285]	Digital Video Broadcasting (DVB); MPEG-DASH Profile for Transport of ISO BMFF

Based DVB Services over IP Based Networks

## SECOND CHANGE

### 5.19.6	Candidate solutions

#### 5.19.6.1	Server-side CDN switching

These candidate solutions include approaches where a media streaming client or population of clients changes or switches between two or more CDNs based on recommendations from a remote server. An example includes the DASH Industry Forum’s content steering architecture [DIFCS].

Editor’s Note:	Inclusion and expansion on these sets of solutions is dependent on interest from working group.

#### 5.19.6.X Client-side CDN switching

##### 5.19.6.X.1          Overview

These candidate solutions summarize common approaches used for client side switching in Multi-CDN delivery scenarios.

These solutions can improve robustness and resilience against CDN outage/failures.

##### 5.19.6.X.2	Multi-CDN delivery with DNS Server

Multi-CDN delivery with a DNS server is a popular way to improve the delivery of services.

In this case the 5GMsd client follows the following steps:

1.	Segment or MPD Request: This is when the 5GMSd client aims to request the URL for a Media presentation description or a media segment.

2.	DNS Resolution: 5GMsd client initiates a DNS (Domain Name System) lookup to find the IP address associated with the domain name. This involves contacting the DNS resolver, that typically resides in the ISP network.

3.	Multi-CDN DNS: When the DNS resolver queries the authoritative DNS server for the domain, the server responds with a list of IP addresses from different CDN (Content Delivery Network) providers.

4.	CDN Selection: The DNS resolver uses an algorithm to select the best CDN based on several criteria like performance (latency, bandwidth), cost, and geographic location of the user.

5.	Edge Server: The user's request is forwarded to the selected edge server. Edge servers are strategically placed in various locations to reduce latency and increase speed by serving content from a location closer to the client.

6.	Content Delivery: The edge server delivers the requested content directly to the 5GMsd client. If the requested content is cached, it serves it immediately. If not, it fetches the content from the origin server or other CDN nodes.

7.	Fallback Mechanism: If the selected CDN fails to deliver the content due to an outage or other issue, the DNS resolver can quickly redirect the request to another CDN provider from the initial list. This ensures high availability and reliability.

##### 5.19.6.X.3	Multi-CDN delivery with DASH based switching

##### In case of DASH, the same steps as in 5.19.6.2.2 may be applied but some preceding steps can be implemented to select CDN or url in the DASH Media presentation description

##### The Media presentation description can contain different BaseURL elements that enable fetching segments from different locations.

##### In this case the DASH client can apply some additional logic, such as based on its historical data or assigned priorities in the Media presentation description to select a BaseURL.

##### This BaseURL can then be combined with the relative path for the media segment’s full URL.

##### This way the media Stream Handler can by interpreting the media presentation description decide on the URL to use for the request for the media segment or media presentation description.

##### After this step, the steps in 5.19.6.X.2 are done

##### To summarize, the following steps may be done at the 5GMSd client for media segment requests using the BaseURL in the Media Presentation Description before the step of multi CDN delivery with a DNS server in 5.19.6.X.2.

##### 1.  Check the different BaseURL in the media presentation description that apply to the segment

##### 2.  Check which BaseURL has the highest priority for usage, based on internal logic of the media stream handler, in  case a previous request has failed another BaseURL as before may be selected.

##### 3.  Combine the BaseURL with the relative path obtained for the media segment

##### 4.  Apply the steps as described in 5.19.6.2.2

##### This approach is applied in profiles for DASH used in the industry such as DVB-DASH [TS103285].

##### An example of handling error responses and use multiple BaseURL is shown in Figure Y, in this case the step may include reloading the media presentation description and recalculating the live edge to obtain again the relative request URL and make sure it is valid.

##### The media segment is requested again and in this case there should not be an error reponse.

##### In case of an error response the process may be repeated.

##### Figure Y a media access function combining baseURL changes, MPD update and live edge calculation as in [TS103285]

##### 5.19.6.X.4	Multi-CDN delivery with Content Steering,

##### For delivery using HTTP Live Streaming, there is no corresponding feature as the BaseURL in MPEG-DASH. Instead a mechanism known as content steering was developed to support downloads from different path ways.

##### This feature uses an external server to provide server paths to clients allowing them to change the path of the requests. This approach has recently also been considered for DASH in the DASH Industry Forum’s content steering architecture [DIFCS], so it is both applicable to DASH and HLS.

##### 5.19.6.X.5	Gap Analysis

##### Multi-CDN delivery using client side switching is supported in common streaming protocols and clients such as MPEG-DASH and HTTP Live Streaming (via content steering or DNS resolution).

#### By inclusion, this is also supported in 5G Media streaming (there are no restrictions in place).

#### It is not clear if text updates to the specifications are needed to clarify using 5G Media streaming for cases but a review of the normative text may be useful in case more explicit support for multi CDN in 5G media streaming is desired.

#### For example, the notion that Media Stream Handler and Media Session Handler cover many different IP port tuples could be made more explicit and it may be useful to document some example behaviour of the media session handler and or media stream handler to support these cases.

##### 5.19.6.X.6	Conclusion

#### Multi-CDN is commonly used in HTTP and adaptive streaming and potentially 5G Media streaming to combat error respobses and improve resilience.

#### Better support in 5G Media streaming enables better alignment with industry and improving delivery performance/user experience.

#### It is not clear what parts of the specifications need to be updated, but in case decision of normative work on this it may be useful to review text in different specifications and some example behaviour may be documented.

#### In this solution, an additional request to another CDN only happens after an error response was received.