3GPP TSG-SA4 Meeting #0-e (AH) MBS SWG post 129e	S4aI240153

# Online, , 26th Sep 2024 - 24th Oct 2024								revision of S4-241467

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.802 | CR | 0003 | rev | 3 | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [FS_AMD] Selected MBMS Functionalities not supported in MBS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Germany |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_AMD |  |  |  |  |  | Date: |  |  | 2024-10-11 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In completing TS 26.502 and TS 26.517, it is obvious that only a subset of the MBMS functionalities is supported in Rel-17. While many MBMS functionalities are likely not important to be supported for MBS, a systematic analysis of MBMS User Services features and their potential relevance for MBS should be completed and recommendations made on which ones to migrate to MBS User Services specifications and how best to achieve this. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addresses the work item objectives for this key issue - Documents the key issue in more detail, in particular how they relate to the 3GPP Media Delivery architecture and/or the MBS User Service architecture - Studies collaboration scenarios between the Application Service Provider and the 5G System and for each of the key topics. - Based on existing architectures, provides one or more deployment architectures that address the key topics and the collaboration models. - Maps the key topics to basic functions and develop high-level call flows. - Identifies the issues that need to be solved. - Provides candidate solutions including call flows, protocols and APIs for each of the identified issues. Identifies gaps and recommend potential normative work for stage-2 and stage-3, including which existing specifications would be impacted and/or if any new specifications would preferably be developed. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.11 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TR 26.802 CR 0001 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Updated scope, references and abbreviations are in CR 0001 References [29] 3GPP TS 26.502: "5G multicast-broadcast services; User service architecture". [30] 3GPP TS 26.517: "5G Multicast-Broadcast User Services; Protocols and Formats". Abbrevations: XXX XXXX |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | S4-241467; [FS_AMD] Selected MBMS Functionalities not supported in MBS; Qualcomm Germany; Thomas Stockhammer. E-mail Discussion: [8.6 FS_AMD; 1467; 21 August 1300 CEST] CR 26.804 [FS_AMD #08] Selected MBMS Functionalities not supported in MBS -> for agreement; Gabin, Frederic; Tue, 20 Aug 2024 07:58:05 +0000. [8.6 FS_AMD; 1467; 21 August 1300 CEST] CR 26.804 [FS_AMD #08] Selected MBMS Functionalities not supported in MBS -> for agreement; Rufael Mekuria; Tue, 20 Aug 2024 12:05:01 +0000. [8.6 FS_AMD; 1467; 21 August 1300 CEST] CR 26.804 [FS_AMD #08] Selected MBMS Functionalities not supported in MBS -> for agreement; Thomas Stockhammer; Tue, 20 Aug 2024 12:08:42 +0000. [8.6 FS_AMD; 1467; 21 August 1300 CEST] CR 26.804 [FS_AMD #08] Selected MBMS Functionalities not supported in MBS -> for agreement; Gabin, Frederic; Wed, 21 Aug 2024 12:19:08 +0000. Revisions: none Presenter: Thomas Stockhammer Online Discussion: - rev2 version presented. - Thomas: This is a skeleton. - Rufael: I am fine with endorsing this. Decision: Endorsed. S4-241467 is endorsed. |

## =====  CHANGE =====

## 5.11	Key Issue #10: Selected MBMS Functionalities not supported in MBS

### 5.11.1	Description

In completing TS 26.502 [29] and TS 26.517 [30], it is obvious that only a subset of the MBMS functionalities is supported in Rel-17. While many MBMS functionalities are likely not important to be supported for MBS, a systematic analysis of MBMS User Services features and their potential relevance for MBS should be completed and recommendations made on which ones to migrate to MBS User Services specifications and how best to achieve this.

### 5.11.2	Overview of MBMS Features

This clause lists a summary of the MBMS features:

-	User Service Discovery and Announcement

-	MBMS user service sessions (defined in sub-clause 7.3 and 8.3 of TS 26.346);

-	Associated delivery methods for File Repair and Reception Reporting (defined in sub-clauses 9.3 and 9.4 of TS 26.346, respectively);

-	Associated delivery methods for Consumption Reporting (defined in sub-clause 9.4A of TS 26.346);

-	Service protection (defined in sub-clause 11.3);

-	FEC repair data stream;

-	Content specific metadata

-	Media Presentation Description (defined in sub-clause 11.2.1.2);

-	Application Service Description (defined in sub-clause 11.2.1.2);

-	Initialization Segments (defined in sub-clause 11.2.1.2);

-	Schedule information (defined in sub-clause 11.2A);

-	Filtering data (defined in sub-clause 11.2B).

-	MBMS Download Delivery Functions

-	Basic protocol with FLUTE (as defined in clause 7.2 and 7.3 of TS 26.346)

-	OMA Push usage for MBMS Download (as defined in clause 7.4 of TS 26.346)

-	FLUTE session setup and control with RTSP (as defined in clause 7.5 of TS 26.346)

-	Hybrid Service Offerings for DASH-over-MBMS User Service and Generic Application Service (as defined in clause 7.6 of TS 26.346)

-	Keep-Updated Service (as defined in clause 7.7 of TS 26.346)

-	Location-specific delivery method

-	Partial File Handling

-	QoE Metrics (as defined in sub-clause 8.4 of TS 26.346).

-	MBMS Streaming Delivery Functions

-	Basic protocol with RTP (as defined in clause 8.2 and 8.3 of TS 26.346)

-	QoE Metrics (as defined in sub-clauses 8.3.2.1 and 8.4 of TS 26.346).

-	MBMS Streaming delivery on Unicast (as defined in sub-clause 8.5 of TS 26.346).

-	MBMS Group communication delivery functions  (as defined in sub-clause 8A of TS 26.346)

-	MBMS Transparent Delivery Functions (as defined in sub-clause 8B of TS 26.346)

-	Key Management functions (as defined in sub-clause 4.4.2 of TS 26.346)

-	Associated delivery functions: file repair (as defined in sub-clause 9.3 of TS 26.346)

-	Associated delivery functions: reception reporting (as defined in sub-clause 9.4 of TS 26.346)

-	Associated delivery functions: User Service Consumption Reporting (as defined in sub-clause 9.4A of TS 26.346)

-	Media Codecs (as defined in clause 10 of TS 26.346)

-	MBMS operation on Demand (MooD) as defined in clause 11 of TS 26.346

-	Time Synchronization as defined in TS 26.346, clause 4.6: MBMS metadata fragments and File Delivery Table (FDT) contain NTP encoded time values

-	Guidelines for linear audio/video streaming using DASH over MBMS broadcast as defined in Annex K of TS 26.346

-	MBMS Profile: MBMS User Service Discovery / Announcement Profile 1a (as defined in Annex L.2 of TS 26.346)

-	MBMS Profile: MBMS User Service Discovery / Announcement Profile 1b (as defined in Annex L.3 of TS 26.346)

-	MBMS Profile: MBMS User Service Discovery / Announcement Profile 1c (as defined in Annex L.3A of TS 26.346)

-	MBMS Profile: MBMS Download Profile (as defined in Annex L.4 of TS 26.346)

-	MBMS Profile: MBMS User Service Discovery / Announcement Profile for Transparent Delivery Services (as defined in Annex L.5 of TS 26.346)

-	FLUTE Profile (as defined in Annex L.6 of TS 26.346)

-	Guidelines for linear audio/video streaming using HLS over MBMS broadcasts as defined in Annex M of TS 26.346

### 5.11.3	Feature Comparison of MBMS and MBS User Services

In order to address the key issue as documented in clause 5.11.1, Table 5.11.3-1 provides an overview of the MBMS features as documented in clause 5.11.2, a equivalent functionality in MBS and related gaps as well as comments and potential next steps.

Table 5.11.3-1 Overview of the MBMS features as documented in clause 5.11.2, a equivalent functionality in MBS and related gaps as well as comments and potential next steps.

| MBMS Feature | TS 26.346 | Equivalent MBS Feature | Gaps, Comments and next steps |
| --- | --- | --- | --- |
| User Service Announcement and Discovery Metadata Fragments | Clause 4 and 11 | Announcement of MBS User Services See TS 26.502, clause 4.5.7 and 4.5.8, and TS 26.517, clause 5 | No gaps identified. |
| Download delivery: Basic Protocol | Clause 7.2 and 7.3 | Object Distribution Method, see clause 6.2 of TS 26.517 | No gaps identified. |
| Download delivery: OMA Push | Clause 7.4 | Not supported in TS 26.517. | Not considered relevant for MBS User Services |
| Download delivery: RTSP session setup | Clause 7.5 | Not supported in TS 26.517. | Not considered relevant for MBS User Services |
| Download delivery: Generic Application Service | Clause 7.6 | Partially supported in TS 26.517, clause 5.2.6. | Gaps: Metadata lacks the availablity to assign URLs to different distribution sessions, including unicast. Next Steps: define the association of URLs to distribution sessions – for more details see clause 5.11.4. |
| Download delivery: Keep-updated Service | Clause 7.7 | Supported through object manifest as defined in TS 26.517, clause 6.1.2 | No gaps identified. |
| Download delivery: Location-specific delivery method | Clause 7.8 | Not supported in TS 26.517. | <more details should be checked> |
| Download delivery: Partial File Handling | Clause 7.9 | Not supported in TS 26.517. | Gaps: Lack of support of the feature results in unnecessary loss of correctly received information Next steps: define the support of partial file handling for MBS – for more details see clause 5.11.5. |
| Download delivery: QoE Metrics | Clause 8.4 | Not supported in TS 26.517. | Gaps: Reporting mechanisms for MBS are minimally supported Comment: It is expected that some functionalities can be covered by the applicaton, but an analysis of reporting for MBS is needed. Next steps: analyse reporting options for MBS in the 5G context. |
| Streaming delivery: Basic protocol | Clause 8.2 | Not explicitely supported in TS 26.517, but covered in clause 7.2 of TS 26.517 as part of the packet distribution method. | Gaps: None identified Comment: more details on RTP based delivery may be checked, but are only considered as potential optimizations. Next steps: Nothing for now |
| Streaming delivery: QoE Metrics | Clause 8.3 and 8.4 | Not supported in TS 26.517. | Gaps: Reporting mechanisms for MBS are minimally supported Comment: It is expected that some functionalities can be covered by the applicaton, but an analysis of reporting for MBS is needed. Next steps: analyse reporting options for MBS in the 5G context – for more details see clause 5.11.6. |
| Streaming delivery: Unicast | Clause 8.5 | Not supported in TS 26.517. | Not considered relevant for MBS User Services |
| Group communication delivery | Clause 8A | Not explicitely supported in TS 26.517, but covered in clause 7.2 of TS 26.517 as part of the packet distribution method. | No gaps identified. |
| Transparent delivery | Clause 8B | Not explicitely supported in TS 26.517, but covered in clause 7.2 of TS 26.517 as part of the packet distribution method. | No gaps identified. |
| Key Management functions | Clause 4.4.2 | Supported in TS 26.517, see clause 5.2.10. | No gaps identified. |
| Associated delivery functions: file repair | Clause 9.3 | Object Repair mechanism for FLUTE as defined in TS 26.517, clause 6.2.4. | No gaps identified. |
| Associated delivery functions: reception reporting | Clause 9.4 | Not supported in TS 26.517. | Gaps: Reporting mechanisms for MBS are minimally supported Comment: It is expected that some functionalities can be covered by the applicaton, but an analysis of reporting for MBS is needed. Next steps: analyse reporting options for MBS in the 5G context - for more details see clause 5.11.6. |
| Associated delivery functions: Consumption Reporting | Clause 9.4A | Not supported in TS 26.517. | Gaps: Reporting mechanisms for MBS are minimally supported Comment: It is expected that some functionalities can be covered by the applicaton, but an analysis of reporting for MBS is needed. Next steps: analyse reporting options for MBS in the 5G context – for more details see clause 5.11.6. |
| Media Codecs | Clause 10 | Not supported in TS 26.517. MBS is not considered to operate as a full service and hence codecs are excluded. With the combination with 5GMS, the codecs from 5G Media streaming are relevant for MBS. | No gaps identified. |
| MBMS operation on Demand | Clause 11 | Not explicitely supported in TS 26.517. However, in combination with 5G Media Streaming, and 5G Media Streaming consumption reporting, an operation on On-demand may be implemented | Gaps: MooD not supported Comment: It is expected that some functionalities can be covered by the applicaton or by 5G Media Streaming, but an analysis of MooD for MBS is encouraged. Next steps: analyse MBS on demand operation in combination with 5G Media Streaming – for more details see clause 5.11.7. |
| Time Synchronization | Clause 4.6 | Not supported in TS 26.517. | Gaps: Time Synchronization not supported Next steps: analyse time synchronization requirements and add functionality if needed – for more details see clause 5.11.8. |
| Guidelines for DASH | Annex K | Not explicitely documented in TS 26.517. | No immediate actions needed, but may be combined with Application service extension – for more details see clause 5.11.4. |
| MBMS Profile 1a | Annex L.2 | <more details should be checked> |  |
| MBMS Profile 1b | Annex L.3 | <more details should be checked> |  |
| MBMS Profile 1c | Annex L.3A | <more details should be checked> |  |
| MBMS Profile: Download | Annex L.4 | <more details should be checked> |  |
| MBMS Profile: UA for Transparent delivery | Annex L.5 | <more details should be checked> |  |
| FLUTE Profile | Annex L.6 | Developed for TS 26.517, so no gaps. | No actions needed |
| Guidelines for HLS | Annex M | Not explicitely documented in TS 26.517. | No immediate actions needed, but may be combined with Application service extension – for more details see clause 5.11.4. |

### 5.11.4	Application Services – MBS and Hybrid

Editor’s Note: Study Issue in more details

### 5.11.5	Partial File Handling

Editor’s Note: Study Issue in more details

### 5.11.6	Reporting and Metrics

Editor’s Note: Study Issue in more details

### 5.11.7	MBS on Demand

Editor’s Note: Identify the issues that need to be solved.

### 5.11.8	Time Synchronization

Editor’s Note: Identify the issues that need to be solved.

### 5.11.9	Summary and Conclusions