3GPP TSG-SA4 MBS SWG post 129e	S4aI240156

# Online, 16th-18th October 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PSEUDO CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.942 | CR |  | rev |  | Current version: | 0.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [FS_MediaEnergyGREEN] Correction on collection and exposure at NF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Orange |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_MediaEnergyGREEN |  |  |  |  |  | Date: |  |  | 2024-10-16 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | TR 23.700-66 conclusions of the Study on Energy Efficiency and Energy Saving have been updated |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update of the clause on Collection and exposure of energy consumption information at NF according to new SA2 conclusions. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Wrong description of the work done by SA2. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st Change

#### 4.2.2.3		Collection and exposure of energy consumption information at NF

TR 23.700-66 [23700-66] studies and identifies potential enhancements to the 5G System (e.g., including network energy-related information exposure, enhancement for subscription and policy control to enable energy efficiency as a service criterion) to improve energy efficiency and to support energy saving in the network.

Three different key issues have been identified in that study:

-	KI#1: Network energy related information exposure

-	KI#2: Subscription and policy control to support energy efficiency and energy saving as service criteria

-	KI#3: 5GS enhancements for network energy saving and efficiency

KI#2 is not in scope of this study. The conclusions of KI#1 and KI#3 in clause 8 of TR 23.700-66 [23700-66] and the normative work following will be used for collection and exposure of Energy Consumption information at Network Functions (NFs) and are summarised as follows:

1.	A new network functionality will be defined to collect and calculate energy-related information and expose it to authorised consumers subject to operator's policy:

-	If the authorised consumer is a 5GC Network Function, the information exposure granularities that can be configured in this policy will include per application, per UE, per-UE-per-QoS Flow, per PDU session.

-	If the authorised consumer is an Application Function, the information exposure granularities that can be configured in this policy will include: per UE, per UE per application, per PDU session.

NOTE:	Whether the new network functionality can be deployed as a standalone Network Function or co-located with other Network Functions is for further study outside the scope of this document.

2.	The energy-related information  that can be exposed according to the above exposure granularities will include:

-	Energy Consumption information as defined in TS 28.310 [28310].

-	Renewable energy information, defined as energy from renewable non-fossil sources. For example (but not limited to) wind, solar, aerothermal, geothermal, hydrothermal.

3.	A consumer of energy-related information (i.e., 5GC NF or AF) may request different modes of exposure (e.g. periodic reporting or threshold-based reporting) as part of its subscription request.

4.	The new network functionality supporting the calculation of the Energy Consumption information includes the following aspects:

a)		OAM: provides the NF/Node-level Energy Consumption information at the gNB(s) and UPF(s) serving the UE.

b)			OAM: provides the overall data volume of the gNodeB.

c) 	The information of a) and b) received from OAM could be used by the new network functionality for all the UEs served by the NF/Node.

d)		UPF: provides the overall data volume of the UPF.

e)	UPF: provides the data volume for the QoS Flow or the Service Data Flow (SDF).

f)	When the gNodeB and/or the (I-)UPF(s) which are serving the UE change, the serving gNodeB ID and UPF ID will be sent to the new network functionality through AMF/SMF.

5.	The new network functionality determines the end-to-end energy consumption based on energy consumption per the granularities above at the serving Network Function (i.e. NG-RAN and UPF).

6.	In Release 19, only the energy-related information of user plane communication (not control plane signalling) is supported.

7.	Enhancements to NF discovery and (re-)selection procedures based on energy-related information:

- 	The NF profile may be extended (e.g. by including the new energy-related information or by reusing existing NF profile parameters) to allow an operator to influence NF discovery and selection based on its energy strategy.

-	NF discovery and (re-) selection will be enhanced to consider the energy-related information from the NF profiles and/or discovery requests from the NF consumer.

8. 	Enhancements to existing operations and procedures for energy saving and energy efficiency:

-	The User Plane path of a PDU session may be adjusted so that it consumes less energy.

The recommendations of the present document focusing on media services will need to be aligned with the conclusions in clause 8 of TR 23.700-66 [23700-66] impacting Application Functions used for media services.

End of Changes