3GPP TSG-SA WG4 Meeting post 129e	S4aI240159 is revision of S4aI240136

Adhoc,  26th September - 24th October 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.942 | pCR |  | rev | - | Current version: | 0.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | ### Update to Potential solution to Key Issue #1: Information exposure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Interdigital |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_MediaEnergyGREEN |  |  |  |  |  | Date: |  |  | 2024-10-14 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The latest draft of 3GPP TR 26.942 contains clause 7 on Potential Solutions to the already defined and described key issues. In this context, under KI #1; the following questions were defined: In this context, the subsequent analysis by this Key Issue should consider: 1. How should UE energy-related information be reported by a UE to the 5G System 2. Which reference points should be used to report UE energy-related information to the Data Collection AF 3. Would it be useful to expose energy-related information of the network to the Media Session Handler to help it optimize its media session in an energy-efficient way It is proposed to add the proposed content to the latest draft of TR 26.942 v 0.3.0 under clause 7.1 as one of the potential solutions so that it is not left incomplete. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR proposes new text to be added in TR 26.942 on “Clause 7 Potential Solutions”. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Proposed objectives will not be met. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7 (new), 7.1 (new), 7.1.2 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## 1st Change

# 2	References

[22261]	3GPP TS 22.261: "Service requirements for the 5G system".

[x] 	Electrochemical Energy Storage for Renewable Sources and Grid Balancing, Pages 411-435, 2015,

[y] 	R1-2206921: Summary for low power high accuracy positioning

[z]	Accubattery App: https://play.google.com/store/apps/details?id=com.digibites.accubattery&pcampaignid=web_share

## 2nd Change

## 7	Potential Solutions

## [Editor’s note: Description of potential solutions]

## 7.1 	Mapping of Solutions to Key issues

Table 7.1-1: Mapping of Solutions to Key Issues

| Solutions |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | KI#1 | KI#2 | KI#3 |  |
| #1 | X |  |  |  |
| #2 |  |  |  |  |
| #3 |  |  |  |  |
| #4 |  |  |  |  |
| #5 |  |  |  |  |
| #6 |  |  |  |  |
| #7 |  |  |  |  |
| #8 |  |  |  |  |
| #9 |  |  |  |  |

## 7.2	Solution #<1>: < Energy-related information exposure from UE :>

## 7.2.1	Key issue mapping

This solution addresses the key issue #1

This potential solution to Key Issue #1 determines the method of obtaining this additional energy-related information, enhancements to the entities involved in obtaining relevant information, and the impact of them on taking into consideration the media context (e.g., the 5G Media Streaming System according to TS 26.501 [26501], 5G Multicast–Broadcast User Services according to TS 26.502 [26502], the Real-time Media Communication according to TS 26.506 [26506]) including UE-related energy information exposure.

## 7.2.3	Functional Description

Energy credit is a service criterion introduced by TS 22.261 clause 6.15a.5 [22261] which can be used as an abstract measure of the energy impact on the network of delivering a service to a UE. According to this clause, subject to operator’s policy, the 5G system is required to support a mechanism to perform energy consumption credit limit control for services without QoS criteria. Energy credits, associated to a subscriber and used by the operator network’s “credit control”, are consumed depending on the UE behavior, e.g. depending on the number of active services, the data volume over a time period, the type of media data, area, and based on the amount of energy consumed by the network to provide the services.

This potential solution to Key Issue #1 determines the method of obtaining this additional energy-related information, enhancements to the entities involved in obtaining relevant information, and the impact of them on taking into consideration the media context (e.g., the 5G Media Streaming System according to TS 26.501 [26501], 5G Multicast–Broadcast User Services according to TS 26.502 [26502], the Real-time Media Communication according to TS 26.506 [26506], Split rendering for media Session Enabler (TS 26.565), etc.) including UE-related energy information exposure.

To obtain and maintain the UE energy-related information, a new generic logical entity in the UE called Energy Related Information Collection and Reporting entity is defined. It maintains the UE energy-related information data collection. As data collection it is proposed the UE battery level, the UE energy state, UE energy preference, UE battery capacity, UE energy supply and UE energy consumption rate. For reporting purposes, it also maintains power use, measurement duration (duration at which the power measurement is reported) and carbon intensity estimates. UE Energy Related Information Collection and Reporting entity can maintain the following UE energy-related information, for example:

- .

- UE battery discharge rate: the discharge rate is given by the battery capacity (in Ah) divided by the number of hours it takes to charge/discharge the battery. For example, a battery capacity of 500 Ah that is theoretically discharged to its cut-off voltage in 20 hours will have a discharge rate of 500 Ah/20 h = 25 A [x].

- UE battery life : It denotes how long the battery will power the device on a single charge. In simple terms, 'battery life' is about how long your phone can run before needing a recharge, which can be a matter of hours or a day, depending on usage [y].

- UE battery capacity: Battery capacity is defined as the total amount of electricity generated due to electrochemical reactions in the battery and is expressed in ampere hours. For example, a constant discharge current of 1 C (5 A) can be drawn from a 5 Ah battery for 1 hour [x]

- UE source of power supply: whether the UE is currently operating on battery or being powered by grid (“plugged-in”) or by renewable energy (e.g. solar panel). This can include the ratio of renewable energy over different time granularities (e.g. 30% renewable over the last 24h [TS 28.310]).

- UE energy consumption: integral of power consumption over time (defined in the TS 28.310 [28310])

- UE carbon intensity: Global greenhouse gases emitted per unit of generated electricity, measured in grams of CO₂ equivalents per watt-hour (for conversion to carbon emissions as defined in TS 22.261 [22261] and TS 23.700-66 [23700]).

UE energy-related information is private UE information, and it is possible that UE manufacturer does not want to share that information with 3rd parties, operators or service provider. Hence, we have introduced the term “UE energy index”.

  - Table 1. Example representation of UE energy index calculations

| UE energy index (integer values) | UE battery discharge rate (in %/h,) | UE battery life (in minutes) | UE battery capacity | UE source of power Supply | UE energy consumption rate at a voltage of 5V |
| --- | --- | --- | --- | --- | --- |
| a | 20 | 120 | 4000 mAh | Battery | 3 W |
| b | 50 | 30 | 4000 mAh | Battery | 1 W |
| c | 95 | 15 | 4000 mAh | Green | 7.5 W |
| d | - | - | 4000 mAh | Socket | 15 W |

The table above is a representative example of how UE energy index would allow to expose energy related information through the newly defined ‘UE Energy Related-Information Collection and Reporting entity”’inside the MSH. From the table above, we notice that the energy index has different values, e.g. a, b, c and d, each integer value corresponding to a particular UE battery capacity, UE supply and UE energy consumption rate. Each value of the energy index (a, b, c and d) is different from one another and the UE energy index of 2 UEs having similar UE energy capacity and UE energy supply, whether it is from the same manufacturer or different manufacturer will still also be different.

For example, a UE from manufacturer 1 may have 4000 mAh battery capacity and from the same manufacturer, there could be another UE 2 model, also having the same 4000 mAh battery capacity. But the way that each of these UEs consume their energy (drain their batteries) will be completely different from one another. This could be, for instance, (i) due to the age of each of the UEs – the older the UE gets the faster the battery is drained, the higher the energy is consumed, (ii) it is possible that one UE is premium flagship model, and the other UE is a basic model, both having 4000 mAh; the rate at which both these UEs consume energy will be totally different even if both are using the same application under the same conditions, (iii) one UE could be present in a region where the average temperature is 40 degree Celsius and another UE could be present in a region where the average temperature is -5 degree Celsius; both being the same model from the same manufacturer (iv) 2 UEs having the same battery capacity of 4000 mAh, but each user exhibiting different user behaviours, which would result in different energy consumption. The above table 1 could be for instance extended to incorporate other criteria into account while calcaulting the UE energy index. For instance, the Accubattery App [y] is a commonly used app, which allows the user to visualise the battery usage of each individual applications that are installed inside a UE, which is available only to the UE. Similar techniques could be used by the UE manufacturers to determine the battery consumption of individual media applications in order to convert to UE energy index.

Figure 7.2.3-1- Example of app showing the UE battery comsumption per application within UE (source: z)

And therefore, UE Energy index is unique to each UE, which allows to identify uniquely the energy consumption rate using a particular energy supply. How UE energy index is derived is implementation specific. It is left to the UE manufacturers to implement specific methods to derive such UE energy index, considering the UE energy capacity, UE energy supply, UE energy consumption rate of their individual UE models. This unique index value does not reveal any direct information about how a UE consumes its energy, but only allows to derive an abstract value. It is left for further study what is done by the UE Energy Related-Information Collection and Reporting entity upon derving the UE energy index.

Fig 7.2.3-1. shows different interfaces between UE and DN, including the new UE Energy Related-Information Collection and Reporting entityin the device. UE Energy Related-Information Collection and Reporting part is a logical entity and could interface as UE energy information handler inside the MSH. The “energy related information collection and reporting” is required to be part of Media Session Handler. For the sake of representation, this new logical entity is shown as a separate entity inside the direct data collection entity.

Figure 7. 2.3-2- UE energy information handler entity within UE

  - The energy information can be provided by the UE client through the configuration message. This can be transmitted as part of the “DeviceCapabilities” object, or through a dedicated new object. The new object could be defined as depicted below:

Table 2. Device energy information object

| DeviceEnergyInformation | Array | 0..1 | This contains a list of energy-related information from the device |
| --- | --- | --- | --- |
| UE energy state | Number | 0..1 | Level of the battery of the UE (e.g. in %) |
| UE energy consumption rate (power in W) | Number | 0..1 | UE charging/discharging rate or speed (e.g. in W, negative or positive) |
| UE energy preference | Number | 0..1 | Remaining time during which the UE wants to be considered in energy saving (e.g. in mn). “0” means that the end time in unknown. |
| UE energy capacity | Number | 0..1 | The total UE battery capacity (e.g. in mAh) |
| UE energy supply | enum | 0..1 | e.g. “battery”, “plug-in”, “renewable” |
| UE Energy | Number | 1..1 | e.g. between 0 and 1000 |
| UE measurement duration | Number | 1..1 | Energy Measurement interval |
| UE carbon intensity | Number | 1..1 | In g CO2-e / Wh |

## 7.X.3	Procedures

## 7.X.4	Impacts on existing services, entities and interfaces

## End of change