3GPP TSG-SA WG4 Meeting ad hoc post #129e	S4aI240162

Electronic, 26th September–24th October 2024	revision of S4aI240140

Title:	[FS_AMD] WT3a: Client-side switching for multi-CDN

Agenda Item:	2.6

Source:	Huawei, Hisilicon

Contact:	Rufael Mekuria

Abstract

Document some current approaches for multi-CDN and client-side switching under WT3a.

Background and motivation

Addition of new clause 5.19.1.X on client-side CDN switching:

- Multi-CDN delivery with DNS-based client-side switching

- Multi-CDN delivery with DASH-based client-side switching

- Multi-CDN delivery with Content Steering

Text proposal

The following text is proposed as a modification of TR 26.804 CR0006.

First change

## 2	References

…

[15]	3GPP TS 26.501: "5G Media Streaming (5GMS) General description and architecture (Release 18)".

[16]	3GPP TS 26.512: "5G Media streaming (5GMS) protocols (Release 18)".

…

[103285]	ETSI TS 103 285: "Digital Video Broadcasting (DVB); MPEG-DASH Profile for Transport of ISO BMFF Based DVB Services over IP Based Networks".

[DIFCS]

Next change

#### 5.19.1.2	Challenges Multi-CDN deployments aim to address

CDNs are often used by content distributors to globally scale delivery of their content to end-users. These networks consist of a number of Points of Presence (PoPs) located at various locations around the networks’ edge. These PoPs help load-balance delivery of content as well as improve Quality of Service (QoS) by reducing the distance/latency between every client and the content they are accessing. In many cases, content distributors employ multiple CDNs to leverage the strengths of one CDN over another in every location those CDNs have a PoP. For example, a client experiencing degraded performance while using one CDN may switch to another that is offering better performance at that time and location. As another example, a content distributor may prefer one CDN over another at a given time to reduce delivery costs and/or meet monthly contractual commitments. These multi-CDN deployments aim to solve content delivery issues that exist when only one CDN is used; but the benefits they provide may not be fully realized because of the various challenges experienced and underlying methods used to stream content to every client.

Challenges multi-CDN deployments and architectures aim to address may include:

1.	Sustained CDN-/network-wide service disruptions where network access, connectivity or QoS is severely degraded. Examples may include cases where an entire CDN’s network is degraded because of a network-wide misconfiguration or power failure. The duration of these events may last minutes to hours and affect a majority of the client population. Examples of recorded instances can be found in [UNPKG24], [NET23], [FSLY21], [AKAM21], [NET22], and [VZ19].

2.	Intermittent or short-term disruptions affecting QoS for an individual or small group of clients. Examples include short periods of congestion within the network, isolated HTTP request/response failures or delays caused by application server congestion, etc. The following discusses these in greater detail: [DEMX01], [DEMX02], [IEEE01], [ACM01], [MHV01], [VAS01], and [MWS23]

3.	Augmentation of one CDN's performance with that of another to achieve a level of performance that neither can provide on its own. An example is a peer-to-peer CDN where each peer has limited uplink capacity and is unable to satisfactorily service client demand on its own.

The following clauses summarise some common approaches used for client-side switching in multi-CDN delivery scenarios. These solutions can improve robustness and resilience against CDN outage/failures.

Next change

##### 5.19.1.2A	Multi-CDN delivery with DNS-based client-side switching

Multi-CDN delivery with a DNS server is a popular way to improve the delivery of services.

In this case the media player follows the following steps:

1.	Segment or MPD request: The media player aims to request the URL for a Media Presentation Description or a media segment.

2.	DNS resolution: The media player initiates a DNS (Domain Name System) lookup to find the IP address associated with the domain name. This involves contacting the DNS resolver, that typically resides in the ISP network.

3.	Multi-CDN DNS: When the DNS resolver queries the authoritative DNS server for the domain, the server responds with a list of IP addresses from different CDN (Content Delivery Network) providers.

4.	CDN selection: The DNS resolver uses an algorithm to select the best CDN based on several criteria like performance (latency, bandwidth), cost, and geographic location of the user.

5.	HTTP request to edge server: The media player's request is forwarded to the selected edge server. Edge servers are strategically placed in various locations to reduce latency and increase speed by serving content from a location closer to the client.

6.	Content delivery: The edge server delivers the requested content directly to the media player. If the requested content is cached, it serves it immediately. If not, it fetches the content from the origin server or other CDN nodes.

7.	Fallback mechanism: If the selected CDN fails to deliver the content due to an outage or other issue, the DNS resolver can quickly redirect the request to another CDN provider from the initial list. This ensures high availability and reliability.

##### 5.19.1.2B	Multi-CDN delivery with DASH-based client-side switching

In the specific case of MPEG-DASH, the same steps as in 5.19.1.2A may be applied but some preceding steps can be implemented to select CDN or URL in the DASH Media Presentation Description.

The Media Presentation Description can contain different BaseURL elements that enable fetching segments from different locations. In this case the media player (DASH client) can apply some additional logic, such as based on its historical data or assigned priorities in the Media Presentation Description to select a BaseURL. This BaseURL can then be combined with the relative path for the media segment’s full URL. This way, the media player can, by interpreting the media presentation description, decide on the URL to use for the request for the media segment or Media Presentation Description. After this step, the steps in clause 5.19.1.2A are followed.

To summarize, the following steps may be followed by the media player for media segment requests using the BaseURL in the Media Presentation Description before the step of multi CDN delivery with a DNS server in clause 5.19.6.X.2.

1.	Check the different BaseURLs in the media presentation description that apply to the segment.

2.	Check which BaseURL has the highest priority for usage, based on internal logic of the media player. In case a previous request has failed, another BaseURL may be selected, as before.

3.	Combine the BaseURL with the relative path obtained for the media segment.

4.	Apply the steps as described in clause 5.19.6.2.2.

This approach is applied in profiles for DASH used in the industry, such as DVB-DASH [103285].

An example of handling error responses and use multiple BaseURL is shown in figure 5.19.1.2B-1.

Figure 5.19.1.2B-1: A media player combining baseURL changes,
MPD update and live edge calculation as in [TS103285]

In this case, the step may include reloading the Media Presentation description and recalculating the live edge to obtain again the relative request URL and make sure it is valid.

The media segment is requested again, and in this case there should not be an error reponse.

In case of an error response the process may be repeated.

#### 5.19.1.2C	Multi-CDN delivery with Content Steering

There is no equivalent BaseURL feature available when using HTTP Live Streaming (HLS) [87] for media delivery. Instead, a mechanism known as HLS Content Steering was developed by Apple to support downloads from different pathways.

This feature uses an external Content Steering server to provide server paths to HLS clients allowing them to change the path of the requests. This approach has recently also been considered for use with MPEG-DASH by the DASH Industry Forum’s content steering architecture [DIFCS], so it is potentially applicable to DASH as well as HLS.

Next change

### 5.19.2	Collaboration scenarios

#### 5.19.2.1	Multi-CDN media delivery

In this scenario, the 5GMSd Client requests adaptive media streaming content from two or more 5GMSd Application Servers. The Client may choose one 5GMSd AS or use multiple simultaneously. This allows the client to distribute network load across Application Servers and M4 downlink transports, optimize costs, as well as improve QoS.

The 5GMS System Operator provides a video streaming service, but to manage increased demand it offloads some requests to an externally deployed 5GMSd AS, while handling others from an 5GMSd AS deployed in the Trusted DN. The quality of the streaming service is required to be equivalent regardless of whether the content is acquired from the externally deployed 5GMSd AS or from the 5GMSd AS deployed in the Trusted DN. By offering the service partly from its own Data Network, the MNO saves costs and only when demand increases on peak loads, the multiple CDNs can be exploited using third-party capacity to deliver the content in a stable manner.

The client’s Media Session Handler discovers the URLs of these Application Servers from the 5GMSd Application Function (AF), either through a Media Entry Point or from a separate piece of metadata. QoE metrics from the client may be used by the AF to determine the best Application Server(s) for each client to use when streaming media.

Figure 5.19.2.1-1 shows the Media Player communicating with multiple Application Server instances. Each AS instance has no direct communication with its peers; rather it communicates (minimally) with the Application Provider and with the 5GMSd AF (not depicted) via reference point M3d.

Figure 5.19.2.1-1: Multi-CDN media delivery within 5G system

Next change

### 5.19.3	Architecture mappings

#### 5.19.3.1	Server-side CDN switching

Editor’s Note:	Inclusion and expansion on these sets of solutions is dependent on interest from working group.

#### 5.19.3.2	Client-side CDN switching

No changes to the baseline 5GMSd architecture defined in clause 4.2 of TS 26.501 [15] are required to support client-side CDN switching.

Next change

### 5.19.4	High-level call flows

#### 5.19.4.1	Server-side CDN switching

Editor’s Note:	Inclusion and expansion on these sets of solutions is dependent on interest from working group.

#### 5.19.4.2	Client-side CDN switching

For progressive download or on-demand streaming, the call flows documented in clauses 5.7.3 and 5.7.4 of TS 26.501 [15] apply. In the case of client-side CDN switching, the Dynamic Policy resource in step 8 of clause 5.7.3 and step 15 of claue 5.7.4 declares a separate Service Data Flow description for each 5GMSd AS endpoint to which the Media Player connects at reference point M4d.

By having multiple Services Data Flow descriptions, Dynamic Policies can be insantiated that cover the multiple paths related to the mutli-CDN deployment.

NOTE:	In the case of on-demand streaming, it is straightforward to declare each 5GMSd AS endpoint address up front when the Dynamic Policy is instantiated, based on information in the Media Player Entry (MPD). In the case of live content and changing content, however, the 5GMSd AS endpoint addresses could change dynamically.

Next change

### 5.19.5	Gap analysis and requirements

#### 5.19.5.2	Client-side CDN switching

Multi-CDN delivery using client-side switching is supported in streaming protocols and clients such as MPEG-DASH and HTTP Live Streaming (via content steering or DNS resolution).

By inclusion, this is also supported in 5G Media Streaming (there are no restrictions in place).

Editor’s Note:	It is not clear if text updates to the specifications are needed to clarify using 5G Media Streaming for cases, but a review of the normative text may be useful in case more explicit support for multi-CDN in 5G Media Streaming is desired.

The Media Player dynamically switches between different 5GMSd AS instances. Given that the Media Player is based on an MPEG-DASH implementation, it can support this by implementing the multiple BaseURLs provided they are present in the supplied Media Player Entry.

For Dynamic Policy invocations and other service invocations at reference point M5, it is important that the Media Session Handler can share multiple service data flow descriptions with the 5GMSd AF covering each of the paths in scope of the Dynamic Policy.

This can be supported by using multiple Service Data Flow Descriptions when instantiating a Dynamic Policy. This is supported between the Media Session Handler and the 5GMSd AFat reference point M5 in TS 26.501 [15] and by the Dynamic Policies API specified in TS 26.510 [26510].

Hence, we may conclude from this analysis that there are no gaps in existing specifications to support client-side switching for multi-CDN, but documentation might still be helpful as this use case is not explicitly documented.

Next change

### 5.19.6	Candidate solutions

#### 5.19.6.1	Server-side CDN switching

These candidate solutions include approaches where a media streaming client or population of clients changes or switches between two or more CDNs based on recommendations from a remote server. An example includes the DASH Industry Forum’s content steering architecture [DIFCS].

Editor’s Note:	Inclusion and expansion on these sets of solutions is dependent on interest from working group.

#### 5.19.6.2	Client-side CDN switching

These candidate solutions include approaches where a media streaming client changes or switches between two or more 5GMSd AS instances based on decisions made locally.

Based on the gap analysis in clause 5.19.5.2, client-side switching is already supported for multi-CDN use cases, and therefore no candidate solution needs to be presented here.

Instead, it may be helpful to provide more explicit documentation to cover these use cases.

Next change

### 5.19.7	Summary and conclusions

#### 5.19.7.1	Server-side CDN switching

Editor’s Note:	Inclusion and expansion on these sets of solutions is dependent on interest from working group.

#### 5.19.7.2	Client-side CDN switching

Multi-CDN based on client-side switching is commonly used in HTTP and adaptive streaming and potentially 5G Media Streaming to combat error responses and improve error resilience.

Better support for client-side switching in 5G Media Streaming would enable better alignment with industry and improving delivery performance/user experience.

The study of this topic and the gap analysis conducted concludes that client-side switching for multi-CDN is already implicitly supported in the procedures for 5G Media Streaming defined in TS 26.501 [15] and specified in TS 26.510 [26510].

It may be useful to review text in different specifications and some example behaviour may be documented in addition to what is currently available. For example, the case of multi-CDN support could be explicitly mentioned and documented in some of the examples and/or call flows of TS 26.501 [15], or as an annex to it.

End of changes