3GPP TSG-SA WG4 MBS SWG Meeting	S4aI240165

# Online, 16th October 2024 - 18th October 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.804 | CR | <CR#> | rev | <Rev#> | Current version: | <Version#> |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [FS_AMD] WT #13: High-level call flow for QUIC-based segmented content |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Xiaomi |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_AMD |  |  |  |  |  | Date: |  |  | 14/10/2024 |
| Category: | B |  |  |  |  |  | Release: |  |  | 19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | No achitecture mapping yet in the QUIC-based segment delivery in 5GMS clause. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Propose three high-level call flows representing the impact of using QUIC Clients on the 5GMS procedure for DASH content delivery. Note that all this text is new compared to the baseline CR and as a result, there is no track change marks. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No progress of the QUIC topic in FS_AMD. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Change #1

### 5.24.4	High-level call flows

#### 5.24.4.1	General

This clause collects high-level call flows where the usage of QUIC or a QUIC-based protocol for the delivery of segmented content would have on a 5GMS system.

#### 5.24.3.2	Impact of a QUIC-agnostic 5GMS Client on the 5GMS high-level procedure for DASH content

In this context, the DASH MPD and the segments are requested and served within a QUIC connection. This is the case for example when HTTP/3 is used. The usage of QUIC in this case has no impact on the steps of the high-level procedure for DASH content. The same 19 steps are presented. However, the following notes can be made and are captured in the sequence diagram in Figure 5.24.3.2-1.

Figure 5.24.3.2-1 High-level call flow for DASH content using a QUIC-agnostic 5GMS Client

The specificities of using QUIC in this procedure are:

- 6: The Media Player establishes a QUIC connection with the 5GMS Application Server

- 7: The MPD request goes over the QUIC connection established in step 6.

- 8: The MPD is delivered over the same QUIC connection established in step 6.

- 13: If the QUIC connection of step 6 is no longer open, the QUIC Client establishes a new connection with the QUIC server.

- 15: The segment request goes over the open QUIC connection.

- 16: The segment response is delivered over the open QUIC connection.

- 17: The segment request goes over the open QUIC connection.

- 18: The segment response is delivered over the open QUIC connection.

Note that after step 18, the Media Player may have to establish a new connection whenever necessary. This can for instance be the case if the QUIC Server closed, if the Media Player stayed for period exceeding the time

#### 5.24.3.3	Impact of a QUIC-aware 5GMS Client on the 5GMS high-level procedure for DASH content

In this context, the DASH MPD and the segments are requested and served within the same QUIC connection which is kept alive by the 5GMS Client.

Compared to the 5GMS high-level procedure for DASH content, the following changes are:

- A new step 8a allows to optionally sent to the QUIC Client, the Initializations Segments.

- The step 15 and 16 are removed, unless the Media Player requires different Initializations Segments than those sent in step 8a.

- The new step 15 for segment requests comes with the ability for the Media Player to associate priorities to those requests.

However, the following notes can be made and are captured in the sequence diagram in Figure 5.24.3.2-1.

Figure 5.24.3.3-1 High-level call flow for DASH content using a QUIC-aware 5GMS Client

The specificities of using QUIC in this procedure are:

- 8a: The QUIC Server may send the Initialization Segments ahead of time.

- 8a: The QUIC connection remains open until segment requests

- 14: The Media Player selects from the Initialization Segments the one to be decoded when already pushed. Otherwise, it sends requests.

- 15: The Media Player sends requests for media segments along with their relative priority.

- 16: The Application Server responds to the requests according to the prioritisation.

About step 8a, the benefits is to decrease the start-up time by avoiding new requests for initialisation segments requests. For step 15 and after, a Media Player may send several requests for segments of the same Representation, of different Adaptation Sets, etc. In this case, the Media Player is able to give relative priorities and expect quicker receptions for the one that matters most, for instance the next segment on the media timeline compared to future ones, audio segments compared to video segments, etc. Those possibilities are listed in clause 5.24.2.3. Note that prioritisation by itself is not supported by the QUIC protocol natively but enabled by prioritising QUIC streams among them when multiplexing on the UDP connection. Also, HTTP/3 offer means for an HTTP/3 client to express its priorities. Those techniques are described in clause 5.24.1.4.

#### 5.24.3.4	Impact of a QUIC-optimised 5GMS Client on the 5GMS high-level procedure for DASH content

In this context, the DASH MPD and the segments are also requested and served within the same QUIC connection which is kept alive by the 5GMS Client.

Compared to the QUIC-aware 5GMS Client, the following changes are:

- The new steps 15 and 16 are grouped over the same QUIC stream.

- There can be one ore more repetition of the new steps 15 and 16 within each QUIC stream.

Figure 5.24.3.3-1 High-level call flow for DASH content using a QUIC-optimised 5GMS Client

The specificities of using QUIC in this procedure are:

- 8a: The QUIC Server may send the Initialization Segments ahead of time.

- 8a: The QUIC connection remains open until segment requests

- 14: The Media Player selects from the Initialization Segments the one to be decoded when already pushed. Otherwise, it sends requests.

- 15: The Media Player sends requests for media segments of the same group with the corresponding QUIC stream.

- 16: The Application Server responds to the requests of a same groups with the corresponding QUIC stream.

The concept of groups aims at emcompassing different approaches. A first approach is to use a single groups for all segments requests which effectively make all segment requests treated equally and the first-in-first-out approach is implemented. Another approach is to group the segment requests per DASH Representation, i.e. one group per quality of audio and video components. Yet another approach is to group segment requests per Adaptation Set. Finally another approach is to use one group per segment request which effectively corresponds to the HTTP/3 case.

END OF CHANGES