3GPP TSG-SA4 Meeting #0-e (AH) MBS SWG post 129e	S4aI240180

# Online, , 26th Sep 2024 - 24th Oct 2024	revision of S4aI240153

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.802 | CR | 0003 | rev | 4 | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [FS_AMD] Selected MBMS Functionalities not supported in MBS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Germany |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_AMD |  |  |  |  |  | Date: |  |  | 2024-10-18 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In completing TS 26.502 and TS 26.517, it is obvious that only a subset of the MBMS functionalities is supported in Rel-17. While many MBMS functionalities are likely not important to be supported for MBS, a systematic analysis of MBMS User Services features and their potential relevance for MBS should be completed and recommendations made on which ones to migrate to MBS User Services specifications and how best to achieve this. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addresses the work item objectives for this key issue - Documents the key issue in more detail, in particular how they relate to the 3GPP Media Delivery architecture and/or the MBS User Service architecture - Studies collaboration scenarios between the Application Service Provider and the 5G System and for each of the key topics. - Based on existing architectures, provides one or more deployment architectures that address the key topics and the collaboration models. - Maps the key topics to basic functions and develop high-level call flows. - Identifies the issues that need to be solved. - Provides candidate solutions including call flows, protocols and APIs for each of the identified issues. Identifies gaps and recommend potential normative work for stage-2 and stage-3, including which existing specifications would be impacted and/or if any new specifications would preferably be developed. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.11 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TR 26.802 CR 0001 |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  | S4aI240153; [FS_AMD] Selected MBMS Functionalities not supported in MBS; Qualcomm Germany; Thomas Stockhammer. E-mail Discussion: [FS_AMD] S4aI240153 "Selected MBMS Functionalities not supported in MBS"; Richard Bradbury; Wed, 16 Oct 2024 12:17:16 +0100. Revisions: S4aI240153_BBC.docx; 2024/10/16 11:15; 66 KB. Presenter: Thomas Stockhammer Online Discussion: October 18, 2024 - Thomas presents BBC version - Richard: Can you explain partial file and time sync - Thomas explains - Thorsten: Time Sync needs careful check to make sure that time stamps are synced. Expires and so on. - Richard: Partial file may be already there. - Small online edits to address remaining comments Decision: - Oct 18, 2024: online edited version agreed. Needs revision to new formal CR which is endorsed. S4aI240153 is revised to S4aI240180. |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## =====  CHANGE (new clause – no change marks shown) =====

## 5.11	Key Issue #10: Selected MBMS Functionalities not supported in MBS

### 5.11.1	Description

In completing TS 26.502 [29] and TS 26.517 [30], it is obvious that only a subset of the MBMS functionalities is supported in Release 18. While many MBMS functionalities are likely not important to be supported for MBS, a systematic analysis of MBMS User Services features and their potential relevance for MBS should be completed and recommendations made on which ones to migrate to MBS User Services specifications and how best to achieve this.

### 5.11.2	Overview of MBMS Features

Editor’s Note: The list is redundant to the table below and may be removed in a final version of the CR.

This clause lists a summary of the MBMS features defined or specified in TS 26.346 [?], along with the relevant clause number in that technical specification:

-	User Service Discovery and Announcement

-	MBMS user service sessions (clauses 7.3 and 8.3);

-	Associated delivery methods for File Repair and Reception Reporting (clauses 9.3 and 9.4 respectively);

-	Associated delivery methods for Consumption Reporting (clause 9.4A);

-	Service protection (clause 11.3);

-	FEC repair data stream;

-	Content-specific metadata

-	Media Presentation Description (clause 11.2.1.2);

-	Application Service Description (clause 11.2.1.2);

-	Initialization Segments (clause 11.2.1.2);

-	Schedule information (clause 11.2A);

-	Filtering data (clause 11.2B).

-	MBMS Download Delivery Functions

-	Basic protocol with FLUTE (clauses 7.2 and 7.3);

-	OMA Push usage for MBMS Download (clause 7.4);

-	FLUTE session setup and control with RTSP (clause 7.5);

-	Hybrid service offerings for DASH-over-MBMS User Service and Generic Application Service (clause 7.6)

-	Keep updated service (clause 7.7)

-	Location-specific delivery method

-	Partial file handling

-	QoE metrics (clause 8.4).

-	MBMS Streaming Delivery Functions

-	Basic protocol with RTP (clauses 8.2 and 8.3);

-	QoE metrics (clauses 8.3.2.1 and 8.4);

-	MBMS Streaming delivery on unicast (clause 8.5);

-	MBMS Group Communication delivery functions (clause 8A);

-	MBMS Transparent Delivery Functions (clause 8B);

-	Key Management functions (clause 4.4.2);

-	Associated delivery functions:

-	File repair (clause 9.3);

-	Reception reporting (clause 9.4);

-	User Service Consumption Reporting (clause 9.4A).

-	Media codecs (clause 10);

-	MBMS operation on Demand (MooD) (clause 11);

-	Time Synchronization (clause 4.6): MBMS metadata fragments and File Delivery Table (FDT) contain NTP encoded time values;

-	Guidelines for linear audio/video streaming using DASH over MBMS broadcast (annex K);

-	MBMS Profiles:

-	MBMS User Service Discovery / Announcement Profile 1a (clause L.2);

-	MBMS User Service Discovery / Announcement Profile 1b (clause L.3);

-	MBMS User Service Discovery / Announcement Profile 1c (clause L.3A);

-	MBMS Download Profile (clause L.4);

-	MBMS User Service Discovery / Announcement Profile for Transparent Delivery Services (clause L.5);

-	FLUTE Profile (clause L.6).

-	Guidelines for linear audio/video streaming using HLS over MBMS broadcasts (annex M).

### 5.11.3	Feature Comparison of MBMS and MBS User Services

In order to address the key issue as documented in clause 5.11.1, table 5.11.3-1 below provides an overview of the MBMS features as documented in clause 5.11.2, equivalent functionality in MBS. The final column identifies related gaps,  provides comments and suggests potential next steps.

Table 5.11.3-1: Overview of the MBMS features as documented in clause 5.11.2,
equivalent functionality in MBS and related gaps as well as comments and potential next steps.

| MBMS feature and sub-feature |  | TS 26.346 clause | Equivalent MBS Feature | Gaps, Comments and next steps |
| --- | --- | --- | --- | --- |
| User Service Announcement and Discovery Metadata Fragments |  | 4, 11 | Announcement of MBS User Services. See clauses 4.5.7 and 4.5.8 of TS 26.502 and clause 5 of TS 26.517. | No gaps identified. |
| Download delivery | Basic protocol | 7.2, 7.3 | Object Distribution Method, see clause 6.2 of TS 26.517. | No gaps identified. |
|  | OMA Push | 7.4 | Not supported in TS 26.517 Release 18. | Not considered relevant for MBS User Services. |
|  | RTSP session setup | 7.5 | Not supported in TS 26.517 Release 18. | Not considered relevant for MBS User Services. |
|  | Generic Application Service | 7.6 | Partially supported in clause 5.2.6 of TS 26.517,. | Gaps: Metadata lacks the availablity to assign URLs to different distribution sessions, including unicast. Next Steps: define the association of URLs to distribution sessions – for more details see clause 5.11.4. |
|  | Keep-updated Service | 7.7 | Supported through object manifest as specified in clause 6.1.2 of TS 26.517. | No gaps identified. |
|  | Location-specific delivery method | 7.8 | Not supported in TS 26.517 Release 18. | <more details should be checked> |
|  | Partial file handling | 7.9 | Not supported in TS 26.517 Release 18. | Gaps: Lack of support of the feature results in unnecessary loss of correctly received information Next steps: define the support of partial file handling for MBS – for more details see clause 5.11.5. |
|  | QoE metrics | 8.4 | Not supported in TS 26.517 Release 18. | Gaps: Reporting mechanisms for MBS are minimally supported Comment: It is expected that some functionalities can be covered by the applicaton, but an analysis of reporting for MBS is needed. Next steps: analyse reporting options for MBS in the 5G context. |
| Streaming delivery | Basic protocol | 8.2 | Not explicitely supported in TS 26.517, but covered in clause 7.2 of TS 26.517 as part of the packet distribution method. | Gaps: None identified Comment: more details on RTP based delivery may be checked, but are only considered as potential optimizations. Next steps: Nothing for now. |
|  | QoE metrics | 8.3, 8.4 | Not supported in TS 26.517 Release 18. | Gaps: Reporting mechanisms for MBS are minimally supported Comment: It is expected that some functionalities can be covered by the applicaton, but an analysis of reporting for MBS is needed. Next steps: analyse reporting options for MBS in the 5G context – for more details see clause 5.11.6. |
|  | Unicast | 8.5 | Not supported in TS 26.517 Release 18. | Not considered relevant for MBS User Services. |
|  | Group Communication delivery | 8A | Not explicitely supported in TS 26.517, but covered in clause 7.2 of TS 26.517 as part of the packet distribution method. | No gaps identified. |
|  | Transparent delivery | 8B | Not explicitely supported in TS 26.517, but covered in clause 7.2 of TS 26.517 as part of the packet distribution method. | No gaps identified. |
|  | Key management | 4.4.2 | Supported in clause 5.2.10 of TS 26.517. | No gaps identified. |
| Associated delivery functions | File repair | 9.3 | Post-session Object Repair mechanism for FLUTE as specified in clause 6.2.4 of TS 26.517. | No gaps identified. |
|  | Reception reporting | 9.4 | Not supported in TS 26.517 Release 18. | Gaps: Reporting mechanisms for MBS are minimally supported Comment: It is expected that some functionalities can be covered by the applicaton, but an analysis of reporting for MBS is needed. Next steps: analyse reporting options for MBS in the 5G context – for more details see clause 5.11.6. |
|  | Consumption Reporting | 9.4A | Not supported in TS 26.517 Release 18. | Gaps: Reporting mechanisms for MBS are minimally supported Comment: It is expected that some functionalities can be covered by the applicaton, but an analysis of reporting for MBS is needed. Next steps: analyse reporting options for MBS in the 5G context – for more details see clause 5.11.6. |
|  | Media codecs | 10 | Not supported in TS 26.517. MBS is not considered to operate as a full service and hence codecs are excluded. With the combination with 5GMS, the codecs from 5G Media streaming are relevant for MBS. | No gaps identified. |
|  | MBMS Operation on Demand (MooD) | 11 | Not explicitely supported in TS 26.517. However, in combination with 5G Media Streaming, and 5G Media Streaming consumption reporting, an operation on On-demand may be implemented | Gaps: MooD not supported Comment: It is expected that some functionalities can be covered by the applicaton or by 5G Media Streaming, but an analysis of MooD for MBS is encouraged. Next steps: analyse MBS on demand operation in combination with 5G Media Streaming – for more details see clause 5.11.7. |
| Time synchronization |  | 4.6 | Not supported in TS 26.517. | Gaps: Time synchronization not supported. Next steps: analyse time synchronization requirements and add functionality if needed – for more details see clause 5.11.8. |
| Guidelines for DASH |  | K | Not explicitely documented in TS 26.517. | No immediate actions needed, but may be combined with Application Service extension – for more details see clause 5.11.4. |
| Profiles | MBMS Profile 1a | L.2 | <more details should be checked> |  |
|  | MBMS Profile 1b | L.3 | <more details should be checked> |  |
|  | MBMS Profile 1c | L.3A | <more details should be checked> |  |
|  | MBMS Profile: Download | L.4 | <more details should be checked> |  |
|  | MBMS Profile: UA for Transparent delivery | L.5 | <more details should be checked> |  |
|  | FLUTE Profile | L.6 | Developed for TS 26.517, so no gaps. | No actions needed. |
| Guidelines for HLS |  | M | Not explicitely documented in TS 26.517. | No immediate actions needed, but may be combined with Application Service extension – for more details see clause 5.11.4. |

### 5.11.4	Application Services – MBS and Hybrid

Editor’s Note: Study Issue in more details

### 5.11.5	Partial File Handling

Editor’s Note: Study Issue in more details

### 5.11.6	Reporting and Metrics

Editor’s Note: Study Issue in more details

### 5.11.7	MBS on Demand

Editor’s Note: Identify the issues that need to be solved.

### 5.11.8	Time Synchronization

Editor’s Note: Identify the issues that need to be solved.

### 5.11.9	Summary and Conclusions