3GPP TSG-SA WG4 MBS SWG AhG Meeting post 129-e	S4aI240198

# Online, 16th October 2024 - 18th October 2024								revision of S4aI240165

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PSEUDO CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.804 | CR | 0019 | rev | <Rev#> | Current version: | <Version#> |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [FS_AMD] WT #13: High-level call flow for QUIC-based segmented content |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Xiaomi |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_AMD |  |  |  |  |  | Date: |  |  | 24/10/2024 |
| Category: | B |  |  |  |  |  | Release: |  |  | 19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | No high-level call flows yet in the QUIC-based segment delivery in 5GMS clause. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Based on the discussion on S4aI240165, representing call flows in a generalized manner, and thus abstracted from a concrete application layer protocol, creates ambiguities and confusion. Therefore, this contribution aims at proposing a solution to address this issue by: - Choosing one application layer protocol (HTTP/3 ) as example protocol for each 5GMS Client type call flow. - Changing the call flows to correspond to the example application layer protocol. This would keep the future conclusions of the WT application-layer-agnostic while giving example instantiations of the 5GMS client types. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No progress of the QUIC topic in FS_AMD. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Change #1 (new text)

### 5.24.4	High-level call flows

#### 5.24.4.1	General

This clause collects high-level call flows where the usage of QUIC or a QUIC-based protocol for the delivery of segmented content would have on a 5GMS system.

#### 5.24.3.2	Impact of a QUIC-agnostic 5GMS Client on the 5GMS high-level procedure for DASH content

To carry this impact analysis, a HTTP/3 client is used as an example of application layer protocol representing a QUIC-agnostic 5GMS Client.

In this context, the DASH MPD and the segments are requested and served within a QUIC connection. The usage of QUIC as transport protocol in this case has no impact on the steps of the high-level procedure for DASH content defined in clause 5.2 of TS 26.501 [15]. The same 19 steps are presented. However, the following notes describe the differences highlighted in the sequence diagram in figure 5.24.3.2-1.

Figure 5.24.3.2-1 High-level call flow for DASH content using a QUIC-agnostic 5GMS Client
based on figure 5.2-2 from TS 26.501 [15]

The specifics of using QUIC-based protocol in this procedure are:

6:	The Media Player establishes a QUIC connection with the 5GMSd Application Server.

7:	The MPD request goes over the QUIC connection established in step 6.

8:	The MPD is delivered over the same QUIC connection established in step 6.

13:	If the QUIC connection of step 6 is no longer open, the QUIC Client establishes a new connection with the QUIC server.

15:	The segment request goes over the open QUIC connection.

16:	The segment response is delivered over the open QUIC connection.

17:	The segment request goes over the open QUIC connection.

18:	The segment response is delivered over the open QUIC connection.

NOTE:	After step 18, the Media Player may have to establish a new connection whenever necessary. This can, for instance, be the case if the 5GMSd AS closes the QUIC connection due to inactivity by the Media Player.

#### 5.24.3.3	Impact of a Media-independent QUIC-aware 5GMS Client on the 5GMS high-level procedure for DASH content

To carry this impact analysis, a HTTP/3 client is used as an example of application layer protocol representing a Media-independent QUIC-aware 5GMS Client.

In this context, the DASH MPD and the segments are requested and typically served within the same QUIC connection which is kept alive by the 5GMS Client.

Compared to the 5GMS high-level procedure for DASH content defined in clause 5.2 of TS 26.501 [15], the possible backward compatible enhancements are :

-	A new step 8a allows the 5GMSd AS to optionally push Initializations Segments to the Media Player by using HTTP/3 server push.

-	Steps 15 and 16 can be omitted unless the Media Player requires different Initialization Segments than those pushed in step 8a or if the step 8a has not been performed.

-	In step 17, the request for each media segment by the Media Player may be enhanced with the ability to indicate a request priority.

However, the following notes can be made and are captured in the sequence diagram in figure 5.24.3.2-1.

Figure 5.24.3.3-1 High-level call flow for DASH content using a HTTP/3 client as media-independent QUIC-aware 5GMS Client

The specifics of using a HTTP/3 client in this procedure are:

8a:		The 5GMSd AS may send Initialization Segments to the Media Player ahead of time to the HTTP client cache using HTTP/3 server push functionality. (The QUIC connection can remain open until segment requests are sent by the Media Player.)

16a:	The Initialization Segments requested by the Media Player are served from the HTTP client cache since already pushed by the 5GMSd AS.

17:		The Media Player sends requests for media segments. If sent concurrently, the Media Player may send priority information along with the media segment requests using the Extensible Prioritization Scheme which is supported by HTTP/3.

18:		The 5GMSd AS responds to the requests according to the prioritisation.

Regarding step 8a, the benefit is to decrease the start-up time by avoiding new requests for initialization segments. In this case, the Media Player performs step 16a and not the steps 15 and 16.

For step 17 and after, a Media Player may send several requests for segments of the same Representation, of different Adaptation Sets, etc. In this case, the Media Player is able to express relative priorities and expects quicker reception of the ones that matter most, for instance the next segment on the media timeline compared to future ones, audio segments compared to video segments, etc. Those possibilities are listed in clause 5.24.2.3.

NOTE:	Prioritisation by itself is not supported by the QUIC protocol natively but enabled by prioritising QUIC streams when multiplexing on the UDP connection. Also, HTTP/3 support the Extensible Prioritization Scheme which allows an HTTP/3 client to express request priorities. These techniques are described in clause 5.24.1.4.

#### 5.24.3.4	Impact of a media-optimised QUIC-aware 5GMS Client on the 5GMS high-level procedure for DASH content

To carry this impact analysis,  a client sharing some commonalities with a Media-over-QUIC client is assumed as an example of application layer protocol representing a Media-optimised QUIC-aware 5GMS Client. However, this analysis deviates from the [MoQ] specification by assuming the presence of an MPD for content selection and the presence of segment request instead of subscription-based mechanism. What remains common is the mapping of media segment onto QUIC streams.

In this context, the DASH MPD and the media segments are also requested and served within the same QUIC connection which is kept alive by the Media Player.

Compared with the call flow for a media-independent QUIC-aware 5GMS Client presented in clause 5.24.3.3, the differences are:

-	Requests for media segments in steps 17 and 18 are grouped over the same QUIC stream.

-	There can be one or more repetitions of the steps 17i  and 18i within each QUIC stream over time.

The concept of groups aims at encompassing different approaches when using a media-optimised application layer protocol from which [MoQ] is an example of:

-	A first approach is to use a single group for all media segment requests, which has the effect of treating all segment requests equally on a first-in-first-out basis.

-	Another approach is to group media segment requests per DASH Representation, i.e. one group per quality of audio and video components.

-	Yet another approach is to group segment requests per Adaptation Set.

-	Finally another approach is to use one group per segment request. This approach corresponds to the HTTP/3 case by definition of the HTTP/3 specification.

Figure 5.24.3.3-1 High-level call flow for DASH content using a QUIC-based 5GMS Client

The specifics of using QUIC in this procedure are:

8a: The 5GMSd AS may send the Initialization Segments to the Media Player ahead of time. (The QUIC connection remains open until segment requests are received from the Media Player.)

16a: The Media Player selects from the Initialization Segments the one to be decoded from the ones already received. Otherwise, it sends requests.

17i: The Media Player sends requests for media segments of the same group with the corresponding QUIC stream. If sent simultaneously, the Media Player may send priority information with or apply the prioritization to the media segment requests.

18: The 5GMSd AS responds to the requests of a same groups within the corresponding QUIC stream.

END OF CHANGES