3GPP TSG-SA4 Meeting #0-e (AH) MBS SWG post 130	S4aI250022

# Online, , 19th Dec 2024 - 6th Feb 2025									revision of S4aI250015

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.502 | CR | 0033 | rev | 1 | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [AMD-ARCH-MED] In-session Unicast Repair for MBS Object Distribution |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Germany |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | AMD-ARCH-MED |  |  |  |  |  | Date: |  |  | 2025-01-06 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In-session Unicast Repair for MBS Object Distribution: For live and low-latency live services using the Object Distribution Method in MBS, in certain cases the transmission of an object is not successful. In this case, unicast repair for individual MBS Clients can improve the service quality. However, the timing of such requests needs to be carefully studied in order to avoid network overloads or significant latencies in the delivery. A study to extend MBS User Services and object streaming to address in-session repair is of relevance. For details refer to TR 26.802. In TR 26.802, In-session unicast repair for MBS Object Distribution as introduced in clause 5.9 and based on the conclusions in clause 5.9.7 to address Gap#1 in clause 5.9.5 by the candidate solution in clause 5.9.6 has been motivated. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In-session unicast repair for MBS Object Distribution as introduced in clause 5.9 of TR 26.802: i. Define a new reference point in TS 26.502 between the MBSTF and the MBS AS. ii. Document call flows and procedures for both post-session and in-session unicast repair. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Feature not supported |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 1, 2, 4.2.6, 4.3.3.1, 4.3.3.2, 4.3.4, 4.4.1, 5.2, 5.6, 7.1, 7.4 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | This revision 1 is updating the initial version to add substance. It is submitted for endorsement as basis for future work |  |  |  |  |  |  |  |  |

## =====  CHANGE =====

# 1	Scope

The present document defines an architecture and high-level procedures for User Services conveyed using the 5G multicast–broadcast capabilities of the 5G System defined in TS 23.501 [2], TS 23.502 [3] and TS 23.247 [5].

The design of the system is primarily motivated by the findings and conclusions in TR 26.802 [26802].

## =====  CHANGE =====

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System architecture for the 5G System (5GS)".

[3]	3GPP TS 23.502: "Procedures for the 5G System (5GS)".

[4]	3GPP TS 23.503: "Policy and charging control framework for the 5G System (5GS); Stage 2".

[5]	3GPP TS 23.247: "Architectural enhancements for 5G multicast-broadcast services; Stage 2".

[6]	3GPP TS 26.348: "Northbound Application Programming Interface (API) for Multimedia Broadcast/Multicast Service (MBMS) at the xMB reference point".

[7]	3GPP TS 26.501: "5G Media Streaming (5GMS); General description and architecture".

[8]	IETF RFC 3550: "RTP: A Transport Protocol for Real-Time Applications".

[9]	IETF RFC 2250: "RTP Payload Format for MPEG1/MPEG2 Video".

[10]	3GPP TS 26.247: "Transparent end-to-end Packet-switched Streaming Service (PSS); Progressive Download and Dynamic Adaptive Streaming over HTTP (3GP-DASH)".

[11]	3GPP TS 26.531: "Data Collection and Reporting; General Description and Architecture".

[12]	3GPP TS 23.468: "Group Communication System Enablers for LTE (GCSE_LTE)".

[13]	Void.

[14]	3GPP TS 23.468: "Group Communication System Enablers for LTE (GCSE_LTE)".

[15]	3GPP TS 29.522: "5G System; Network Exposure Function Northbound APIs; Stage 3".

[16]	OMA: "OMNA BCAST Service Class Registry", https://technical.openmobilealliance.org/OMNA/bcast/bcast-service-class-registry.html.

[17]	IANA: "Reliable Multicast Transport (RMT) FEC Encoding IDs and FEC Instance IDs", https://www.iana.org/assignments/rmt-fec-parameters/rmt-fec-parameters.xhtml#rmt-fec-parameters-1.

[18]	3GPP TS 33.501: "Security architecture and procedures for 5G system".

[19]	3GPP TS 33.246: "Security of Multimedia Broadcast/Multicast Service (MBMS)".

[26802]	3GPP TR 26.802: "Multicast Architecture Enhancement for 5G Media Streaming".

## =====  CHANGE =====

### 4.2.6	Object Repair

When using the Object Distribution Method, Object Repair procedures may be provisioned by the MBS Application Provider. This feature allows an MBS Client to retrieve from the MBS AS (via reference point MBS-4-UC) missing portions of objects not received intact from the MBSTF at reference point MBS-4-MC.

The followingObject Repair procedures are defined by the present document:

-	Post-session Object Repair after the completion of the MBS Distribution Session, including a randomized back-off period for different MBS Clients in order to avoid overloading the MBS AS.

-	In-session Object Repair during the MBS Distribution Session, including the ability to repair objects in streaming distribution.

The functions of the MBS AS are defined in clause 4.3.4.

## =====  CHANGE =====

#### 4.3.3.1	General

The functionality of the MBSTF is defined in clause 5.3.2.12 of TS 23.247 [5]. It receives User Plane data traffic at reference point Nmb8 and sends MBS data packets to the MB-UPF via reference point Nmb9.

NOTE:	The MBSTF may not be present in all deployments of the MBS System. However, the MBSTF is an integral and necessary component of MBS User Services, and the present document is not concerned with deployments in which it is absent.

The present document defines additional User Plane functionalities of the MBSTF to support MBS User Services as follows:

1.	Receiving MBS Distribution Session configurations from the MBSF at reference point Nmb2.

2.	Sending notification events to the MBSF, e.g., data ingest failure, session terminated, delivery started via reference point Nmb2.

3.	Based on the MBS Distribution Session configuration:

a)	MBS delivery of ingested objects or sequences of objects to the MBS Client using the Object Distribution Method (see clause 6.1). This may be used to support real-time distribution of media segments (as special objects) including CMAF segments.

b)	MBS delivery of ingested packet streams to the MBS Client using the Packet Distribution Method (see clause 6.2).

c)	Delivery of MBS User Service Announcement information (including updates as necessary) within the MBS Session.

4.	In case of 3a), potentially forwarding ingested objects to the MBS AS to support object repair.

## =====  CHANGE =====

#### 4.3.3.2	MBSTF subfunctions to support Object Distribution Method

The MBSTF subfunctions supporting the Object Distribution Method are depicted in figure 4.3.3.2-1 below.

Figure 4.3.3.2-1: MBSTF architecture overview for Object Distribution Method

The Object ingest subfunction supports:

-	Pull-based ingest at reference point MBS-11: The Object ingest subfunction in this case fetches one or more objects from the MBS AF using HTTPS for inclusion in the MBS User Service Announcement Channel. (The set of objects to be included is listed in an object manifest previously fetched from the MBS AF at the same reference point.)

-	Push-based ingest at reference point Nmb2: The Object ingest subfunction receives an object manifest from the MBSF that describes a set of objects for inclusion in the MBS User Service Announcement Channel. The MBSTF goes on to fetch the objects listed in the object manifest from the MBS AF via reference point MBS-11.

-	Pull-based ingest at reference point Nmb8: The Object ingest subfunction in this case fetches one or more objects from the MBS Application Provider (AF/AS).

-	Push-based ingest at reference point Nmb8: The Object ingest subfunction receives one or more objects from the MBS Application Provider (AF/AS).

-	Pull-based egest at reference point MBS-12: The Object ingest subfunction in this case forwards one or more objects from the MBS Application Provider (AF/AS) to the MBS-AS.

-	Push-based egest at reference point MBS-12: The Object ingest subfunction forwards one or more objects from the MBS Application Provider (AF/AS).

Object ingest procedures at reference point Nmb2 and MBS-11 shall be a subset of those at reference point Nmb8.

The Object segmentation subfunction supports the partitioning of an object into payload units suitable for MBS transmission.

The optional Application Layer FEC subfunction supports object recovery when some packets are not received by the MBMS Client.

The Packetisation subfunction places the payload units (and, optionally, the FEC data) into Nmb9 transmission packets according to clause 6.1.

The Packet scheduling subfunction schedules the outgoing packet stream according to target bit rate configuration.

The Control subfunction offers support for MBSTF service configuration, status query and notifications at reference point Nmb2.

## =====  CHANGE =====

### 4.3.4	MBS AS

The MBS AS is an optional entity that performs the following functions to support MBS User Services:

-	Providing a byte-range file repair service to the MBSTF Client (via reference point MBS-4-UC) for use with the Object Distribution Method.

The MBS AS is configured by the MBSF at reference point MBS-9. The MBS AS may acquire content from the MBSTF via MBS-12, by one of the two methods:

-	Pull-based ingest at reference point MBS-12: The MBS AS in this case fetches one or more objects from the MBSTF.

-	Push-based ingest at reference point MBS-12: The Object ingest subfunction receives one or more objects from the MBSTF.

The MBS AS may be deployed as a standalone entity, or its functions may be co-located with other Network Functions such as the MBSTF (see clause 4.3.3) or the 5GMS AS defined in TS 26.501 [7].

## =====  CHANGE =====

### 4.4.1	Overview

The following reference points defined in clause 5.1 of TS 23.247 [5] are relevant to MBS User Services architecture: Nmb1, Nmb2, Nmb5, Nmb8, Nmb9, Nmb10 and Nmb12.

The following additional reference points are defined by the present document:

-	MBS-3: Used by the MBSF to configure the MBS AF and to publish User Service Announcements to it. This reference point is not described further in the present document.

-	MBS-4-MC: Unidirectional multicast distribution of content from the MBSTF to the MBS Client.

-	MBS-4-UC: User Plane interactions between the MBSTF Client and the MBS AS for the purpose of file-based unicast repair.

-	MBS-5: User Plane interactions between the MBSF Client and the MBS AF for the purpose of MBS control plane and service handling.

-	MBS-6: API exposed by the MBSF Client and used by the MBS-Aware Application to manage and control MBS User Services.

-	MBS-7: API exposed by the MBSTF Client and used by the MBS-Aware Application to receive user data information distributed using MBS User Services.

-	MBS-8: Announcement of MBS User Services to the MBS-Aware Application by the MBS Application Provider. The procedures at this reference point are beyond the scope of 3GPP specification.

-	MBS-9: Used by the MBSF to configure the MBS AS.

-	MBS-10: User Plane interactions between the MBSF Client and the MBSSF for the purpose of authorising access to security-protected MBS data by means of the User Plane security procedure specified in clause W.4.1.3 of TS 33.501 [18].

-	MBS-11: Used by the MBSTF to retrieve object manifests and User Service Announcements listed in object manifests from the MBS AF.

-	MBS-12: Used by the MBS AS to retrieve objects from the MBSTF to support unicast repair.

In addition, the following reference points are defined inside the MBS Client function:

-	MBS-6′: API exposed by the MBSTF Client and used by the MBSF Client to (de)activate reception of an MBS Session by the MBSTF. The reception parameters are supplied by the MBSF Client.

This reference point is outside the scope of MBS User Services and is not described further in the present document.

-	MBS-7′: API exposed by the MSTF Client and used by the MBSTF to supply MBS Session configuration information that has been received from reference point MBS-4-MC.

This reference point is outside the scope of MBS User Services and is not described further in the present document.

## =====  CHANGE =====

## 5.2	High-level baseline procedures

The high-level baseline procedures for MBS User Services are shown in figure 5.2-1.

NOTE:	In the interests of brevity, the prefix MBS is omitted from the numbered steps in the figure.

Figure 5.2-1: MBS User Service high-level baseline procedures

The basic procedures are as follows:

1.	The MBS Application Provider provisions an Application Service as an MBS User Service by invoking the Nmbsf service on the MBSF.

2.	The MBSF provisions one or more MBS Distribution Session by invoking the Nmbstf on the MBSTF.

3.	Optionally, the MBSF provisions the MBS AS to support object repair via MBS-9.

4.	The MBSF provides an MBS User Service Announcement document that may be accessed by the MBS Client of interested UEs.

5.	The MBS Application Provider informs the MBS-Aware Application via MBS-8 that the specific Application Service can be accessed via an MBS User Service by means of an Application Service Announcement.

[Alternatively, the MBSF Client synthesises the Application Service Announcement from the User Service Announcement received in step 3 and informs the MBS-Aware Application via MBS-6 that the Application Service can be accessed via an MBS User Service.]

6.	The MBS Application Provider creates an MBS User Data Ingest Session with the MBSTF (via the MBSF) and the latter ingests the user data via Nmb8.

7.	Optionally, if object repair is provisioned, the MBSTF creates an object ingest session with the MBS AS and latter ingests the relevant user data via MBS-12.

8.	The MBS-Aware Application requests the MBS Client to access a specific MBS User Service by invoking procedures at MBS-6.

9.	Optional: If it has not already been received in step 3 above, the MBSF Client may discover additional access information about this MBS User Service by querying the MBSF via MBS-5.

10.	The MBSF Client provides the MBS Distribution Session information acquired as part of the User Service Announcement information to the MBSTF Client via MBS-6′.

11.	The MBSTF Client activates reception of the MBS Distribution Session.

12.	The MBSF Client continuously handles the MBS Distribution Session data based on updates received via MBS-5.

13.	The MBSTF Client receives MBS Distribution Session data via MBS-4-MC as part of the MBS Distribution Session. If unicast repair is provisioned, then the MBSTF client may use MBS-4-UC as part of the MBS Distribution Session to accomplish post-session or in-session repair.

14. The MBSTF provides the MBS Distribution Session data to the MBS-Aware Application via MBS-7 in an MBS Application Data Session.

15.	The MBS-Aware Application controls the MBS User Service by invoking MBS Application Service Control procedures on the MBSF Client via MBS-6.

## =====  CHANGE =====

## 5.6	Procedure for User Service data repair

### 5.6.1	General

In the case of the Object Distribution Method (as defined in clause 6.1), the MBSTF Client may collaborate with the MBS AS at reference point MBS-4-UC to recover lost portions of content corresponding to MBS data that was not successfully received by the MBSTF Client at reference point MBS-4-MC (see step 9 in clause 5.5).

The high-level procedure for data repair is illustrated in figure 5.6.1-1 below:

Figure 5.6.1-1: High-level call flow for MBS User Service data repair

In the following, more detailed procedures are provided for

-	General provisioning of object repair in clause 5.6.2.

-	Post-session Object Repair after the completion of the MBS Distribution Session in clause 5.6.3.

-	In-session Object Repair during the MBS Distribution Session, including the ability to repair objects in streaming distribution, in clause 5.6.4.

### 5.6.2	Object Repair provisioning

### Figure 5.6.2-1 provides an extension of the call flow in figure 5.3-2 to include object repair provisioning.

Figure 5.6.2-1: Provisioning of object repair

Editor’s Note: At this stage, MBS-9 is not defined as a service-based interface.

In particular, after step 12, the following steps are carried out to support object repair provisioning:

12a:	The MBSF creates an ingest session for repair objects

12b:	The MBS AS informs the MBSF on the creation of this ingest session

12c: 	The MBSF subscribes to status events from the MBS AS

In particular, after step 14, the following steps are carried out to support object repair ingest:

14a:	The MBSTF and the MBS AF establish a repair content ingest session

### 14c:	The MBS AS informs the MBSTF on the event of creation of the ingest session

### 5.6.3	Post-session Object Repair

For post-session object repair, the procedures in clause 5.5 are extended as shown in Figure 5.6.3-1. In particular, after the steps described in clause 5.5.2 and Figure 5.5-2, in case post-session object repair is configured for the MBS client, the following steps are carried out.

Figure 5.6.3-1: Call flow for Post Session Object repair

In particular, after step 13, when the content is received:

13a:	The MBSTF determines the end of session for each MBS distribution session

13b:	The MBSTF determines if any data is missing

13c: 	If missing, it determines some random backoff time

13d:	After backoff time has elapsed, the MBSTF requests missing data from the MBS AS

13e:	The MBSTF recovers the object

13f:	Once recovered, the MBSTF notifies the application on the available object

13g:	the application acquires the object from the MBSTF

### 5.6.4	In-session Object Repair

For in-session object repair, the procedures in clause 5.5 are extended as shown in Figure 5.6.4-1. In particular, after the steps described in clause 5.5.2 and Figure 5.5-2, in case in-session object repair is configured for the MBS client, the following steps are carried out.

Figure 5.6.4-1: Call flow for in session object repair

In particular, starting with an extended step 13 for each object:

13a0:	The MBSTF Client receives content for object 1

13a1:	The MBSTF Client determines the end of object 1 delivery

13a2:	The MBSTF Client determines if any data is missing for object 1

13b0:	The MBSTF Client receives content for object 2

13b1:	The MBSTF Client determines the end of object 2 delivery

13b2:	The MBSTF Client determines if any data is missing for object 2

In parallel to 13b0 – 13b2,

13a3: 	If missing, it the MBSTF Client requests missing data

13a4:	The MBSTF Client recovers the object

13a5:	The MBSTF Client informs the application on the recovered object

13a6:	The application acquires object 1 from the MBSTF Client

13c0:	The MBSTF Client receives content for object 3

13c1:	The MBSTF Client determines the end of object 3 delivery

13c2:	The MBSTF Client determines if any data is missing for object 3

In parallel to 13c0 – 13b3,

13b3: 	If missing, it the MBSTF Client requests missing data

13b4:	The MBSTF Client recovers the object

13b5:	The MBSTF Client informs the application on the recovered object

13b6:	The application acquires object 2 from the MBSTF Client

## =====  CHANGE =====

## 7.1	General

The Network Function Services exposed by the MBSF, the MBSTF and the MBS AS are defined in this clause.

## =====  CHANGE =====

## 7.4	MBS AS Services

### 7.4.1	General

The following table illustrates the set of NF services exposed by the MBS AS.

Table 7.3-1: NF services provided by MBS AS

| Service name | Service operation name | Operation semantics | Example consumer(s) |
| --- | --- | --- | --- |
| Nmbsas_MBSObjectRepairIngestSession | Create | Request/Response | MBSF |
|  | Retrieve | Request/Response | MBSF |
|  | Update | Request/Response | MBSF |
|  | Destroy | Request/Response | MBSF |
|  | StatusSubscribe | Subscribe/Notify | MBSF |
|  | StatusUnsubscribe |  | MBSF |
|  | StatusNotify |  | MBSF |
|  | StatusSubscribeMod |  | MBSF |