3GPP TSG-SA4 Meeting #0-e (AH) MBS SWG post 130	S4aI250027

# Online, , 19th Dec 2024 - 6th Feb 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.501 | CR | 0106 | rev | - | Current version: | 18.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [AMD-ARCH-MED] Improved QoS support for Media Streaming services |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson LM |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | AMD-ARCH-MED |  |  |  |  |  | Date: |  |  | 2025-01-06 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Improved QoS support: SA2 has defined a number of new features in the 5G System, especially in the PCF, from which media delivery may benefit. Examples documented in TS 23.501 include the use of Explicit Congestion Notification (ECN) to support Low Latency, Low Loss, Scalable Throughput (L4S) services (clause 5.37.3) and QoS Monitoring (clause 5.45). Solutions for improved QoS Support has been studied. For details see TR 26.804 clause 5.23 and the conclusions in clause 6.23. |  |  |  |  |  |  |  |  |
| Summary of change: |  | For Improved QoS support for Media Streaming services as introduced in clause 5.23 of TR 26.804: i. Integrate ECN marking for L4S into the architectures, high-level call flows and collaboration scenarios for both 5GMSd and 5GMSu. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Feature not supported |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## =====  CHANGE =====

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System architecture for the 5G System (5GS)".

[3]	3GPP TS 23.502: "Procedures for the 5G System (5GS)".

[4]	3GPP TS 23.503: "Policy and charging control framework for the 5G System (5GS); Stage 2".

[5]	Void

[6]	3GPP TS 26.307: "Presentation layer for 3GPP services".

[7]	3GPP TS 26.247: "Transparent end-to-end Packet-switched Streaming Service (PSS); Progressive Download and Dynamic Adaptive Streaming over HTTP (3GP-DASH)".

[8]	3GPP TS 26.234: "Transparent end-to-end Packet-switched Streaming Service (PSS); Protocols and codecs".

[9]	3GPP TS 23.003: "Technical Specification Group Core Network and Terminals; Numbering, addressing and identification".

[10]	3GPP TS 28.530: "Management and orchestration; Concepts, use cases and requirements".

[11]	3GPP TS 28.531: "Management and orchestration; Provisioning".

[12]	3GPP TS 28.541: "Management and orchestration; 5G Network Resource Model (NRM); Stage 2 and stage 3".

[13]	3GPP TS 23.222: "Common API Framework for 3GPP Northbound APIs".

[14]	IETF RFC 1034: "Domain names - concepts and facilities".

[15]	3GPP TS 23.548: "5G System Enhancements for Edge Computing; Stage 2".

[16]	3GPP TS 23.558: "Architecture for enabling Edge Applications".

[17]	3GPP TS 28.538: "Management and orchestration; Edge Computing Management".

[18]	3GPP TS 23.246: "Multimedia Broadcast/Multicast Service (MBMS); Architecture and functional description".

[19]	3GPP TS 26.346: "Multimedia Broadcast/Multicast Service (MBMS); Protocols and codecs".

[20]	3GPP TS 26.347: "Multimedia Broadcast/Multicast Service (MBMS); Application Programming Interface and URL".

[21]	3GPP TS 26.348: "Northbound Application Programming Interface (API) for Multimedia Broadcast/Multicast Service (MBMS) at the xMB reference point".

[22]	3GPP TS 26.531: "Data collection and reporting; General description and architecture".

[23]	3GPP TS 23.288: "Architecture enhancements for 5G System (5GS) to support network data analytics services".

[24]	3GPP TS 27.007: "AT command set for User Equipment (UE)".

[25]	CTA-5005: "Web Application Video Ecosystem – DASH-HLS Interoperability Specification".

[26]	3GPP TS 26.511: "5G Media Streaming (5GMS); Profiles, Codecs and Formats".

[27]	ISO/IEC 23000-19: "Information Technology Multimedia Application Format (MPEG-A) – Part 19: Common Media Application Format (CMAF) for segmented media".

[28]	IETF RFC 8216: "HTTP Live Streaming".

[29]	ISO/IEC 23009-1: "Information Technology – Dynamic Adaptive Streaming Over HTTP (DASH) – Part 1: Media Presentation Description and Segment Formats".

[30]	3GPP TS 26.502: "5G Multicast-Broadcast User Service Architecture".

[31]	Void.

[32]	3GPP TS 26.506: "5G Real-time Media Communication Architecture".

[33]	3GPP TS 23.222: "Common API Framework for 3GPP Northbound APIs".

[34]	3GPP TS 33.122: "Security aspects of Common API Framework (CAPIF) for 3GPP northbound APIs".

[35]	IETF RFC 6749: "The OAuth 2.0 Authorization Framework", October 2012.

[X1]	IETF RFC 3168: "The Addition of Explicit Congestion Notification (ECN) to IP".

[X2]	IETF RFC 9330:"Low Latency, Low Loss, Scalable Throughput (L4S) Internet Service: Architecture".

## =====  CHANGE =====

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

5GC	5G Core Network

5GMS	5G Media Streaming

5GMSd	5G Media Streaming downlink

5GMSu	5G Media Streaming uplink

5GS	5G Systems

AF	Application Function

ABR	Adaptive Bit Rate

AMF	Access and Mobility Function

ANBR	Access Network Bit rate Recommendation

API	Application Programming Interface

App	Application

AS	Application Server

CAPIF	Common API Framework

CDN	Content Delivery Network

DASH	Dynamic and Adaptive Streaming over HTTP

DN	Data Network

DNAI	Data Network Application Identifier

DNN	Data Network Name

DRM	Digital Rights Management

ECN	Early Congestion Notification

EPC	Evolved Packet Core

EPS	Evolved Packet System

EUTRAN	Evolved Universal Terrestrial Radio Access Network

FLUS	Framework for Live Uplink Streaming

FQDN	Fully-Qualified Domain Name

GPU	Graphics Processing Unit

GSM	Global System for Mobile communication

HPLMN	Home Public Land Mobile Network

HTTP	HyperText Transfer Protocol

HTTPS	HyperText Transfer Protocol Secure

L4S	Low Latency, Low Loss, Scalable Throughput

LTE	Long-Term Evolution

MBMS	Multimedia Broadcast Multicast System

MNO	Mobile Network Operator

MPD	Media Presentation Description

MSISDN	Mobile Station International Subscriber Directory Number

NA	Network Assistance

NEF	Network Exposure Function

NR	New Radio

NSMF	Network Slice Management Function

NSSAI	Network Slice Selection Assistance Information

NSSP	Network Slice Selection Policy

OAM	Operations, Administration and Maintenance

OTT	Over-The-Top

PCC	Policy and Charging Control

PCF	Policy and Charging Function

PDU	Packet Data Unit

PSS	Packet-switched Streaming Service

RAN	Radio Access Network

RTC	Real-Time media Communication

SBA	Service based Architecture

SLA	Service Level Agreement

TCP	Transmission Control Protocol

UPF	User Plane Function

URL	Unique Resource Identifier

URSP	UE Route Selection Policy

## =====  CHANGE =====

### 4.0.14	Using Explicit Congestion Notification (ECN) for Media Streaming sessions to support Low Latency, Low Loss, Scalable Throughput (L4S)

The 5G System allows using Explicit Congestion Notification (ECN) to support Low Latency, Low Loss, Scalable Throughput (L4S) for Media Streaming services as defined in TS 23.501, clause 5.37.3[2].

As described in RFC 9330 [149], RFC 9331 [150] and RFC 9332 [151], the purpose of ECN marking for L4S (Low Latency, Low Loss and Scalable Throughput) is to inform a recipient host at the earliest opportunity that an IP packet has experienced network congestion at some point in its routing path. It exposes congestion information by marking ECN bits in the IP header of the user IP packets between the UE and the Application Server. This early notification may be used by the receiving application to report the congestion to its sending peer using a suitable Layer 4 feedback mechanism. Based on this feedback, the sender should reduce the sending bit rate.

## =====  CHANGE =====

## 5.13	Downlink Media Streaming with Explicit Congestion Notification (ECN) to support L4S

An Application Function may request L4S support from the 5G Network for a certain QoS Flow, e.g. by invoking the Nnef_AfsessionWithQoS service. The concept of this solution is that an application only requests L4S support from the network when the application layer is able to adjust its load on the network depending on the L4S feedback (e.g. the percentage of congestion marks) it receives. The activation leverages the existing 5GMS Dynamic Policy invocation API, allowing the 5GMS-Aware Application to request L4S support as and when it is needed.

A high-level call flow for downlink media streaming is sketched in figure 5.13-1 below. The following is assumed:

-	The service here is a unicast downlink media streaming service with dynamic policy support, as described in clause 5.7.

-	The Layer 4 protocol used for application flows is TCP and the TCP stack used supports L4S.

-	The network supports L4S packet marking.

-	The application has specifically requested ECN marking for its media delivery session.

-	NG-RAN manipulates the ECN bits (per clause 5.37.3.2 of TS 23.501 [2]). It is equally possible that the PSA-UPF manipulates the ECN bits (per clause 5.37.3.3 of [2]).

Figure 5.13-1: Downlink media streaming call flow for L4S on request

The steps are as follows:

0:	Policy Template Provisioning. A Policy Template is provisioned with the requirement for L4S capability indicated by setting a flag.

1:	Dynamic Policy activation. The Media Session Handler within the 5GMSd Client obtains Service Access Information and triggers a dynamic policy activation. A Policy Template Binding is present within the Service Access Information for each provisioned Policy Template. Policy Template Bindings suitable for L4S are indicated by an L4S capability requirement flag being set. The 5GMSd Client detects that an L4S-capable media transport stack is present and in use. The selected Policy Template is one configured with L4S capability.

2:	QoS request. The 5GMSd AF requests QoS handling using e.g. the Nnef_AfSessionWithQoS service or the Npcf_PolicyAuthorization service. If the L4S capability requirement flag is set in the selected Policy Template, this indicates that the new QoS flow is required to be L4S-enabled. The new QoS flow with the L4S indication setting propagates through the 5G System.

3:	The Media Session Handler may inform the Media Player via the M11 interface API about the activation of L4S. Subject to availability of API access, the Media Player may use congestion notifications to perform early adaptation.

4: 	If the L4S capability requirement flag is set in the Policy Template Binding for the selected Policy Template, the 5GMSd Client selects/enables the L4S capability of the used transport protocol.

NOTE:	This step may happen implicitly by selecting an L4S-supporting transport protocol stack.

5:	The Media Player within the 5GMSd Client triggers the establishment of a TCP connection. The ECT(1) codepoint is set in the IP header, indicating an L4S-Capable Transport, and the SDAP entity ensures that the packet is forwarded via the matching QoS flow.

6:	The 5GMSd AS responds to the TCP connection establishment request. The 5GMSd AS sets ECT(1) in the IP headers, indicating an L4S-Capable Transport.

7:	The UPF finds the matching QoS flow identifier for the downlink packet and sends the packet via the according QoS flow to the UE. TCP Connection setup continues, with one ECT bit set in all packets.

8:.	When the RAN detects an upcoming congestion according to the congestion measurement (based on continuous congestion monitoring), the 5G System sets the CE (Congestion Experienced) codepoint in the IP header of the downlink packet.

9.	(Optional) The Media Player may generate a Congestion Notification based on, for example, calculating the proportion of CE marks over a sample period and the congestion trend, and notifies the Media Session Handler using the Network Assistance client API.

10.	(Optional) The Media Player sends the congestion information to the 5GMS AS in a congestion report.

11.	The TCP protocol stack used by the Media Player in the 5GMSd Client reflects the Early Congestion Notification to the TCP sender by setting the ECN-Echo (ECE) flag in the TCP header of an uplink PDU of the same TCP connection. The TCP sender reacts to the ECN-Echo accordingly (i.e., by reducing its sending congestion window).

NOTE 1:	The ECN-Echo flag is also acknowledged by the TCP sender setting the Congestion Window Reduced (CWR) flag in an outgoing TCP frame, but this acknowledgement is not illustrated in this call flow.

NOTE 2:	Classic ECN as specified in RFC 3168 [X1] requires an ECN signal to be treated as equivalent to a packet drop. L4S as specified in RFC 9330 [X2] specifies a more fine-grained response and an early congestion signal triggers a less severe reaction. How a TCP sender behaves “accordingly” is not in scope of the specification.

12. Based on the CE indication received in step 8, or by detecting a reduced bit rate in the downlink application flow, the Media Player in the 5GMSd Client reacts by, for example, changing the requested representation.

13. Based on the congestion report received in step 10, or the CE indication received in step 11, or detection of a reduced bit rate in the downlink application flow, the 5GMSd AS may react by, for example, updating the DASH presentation manifest to remove/add one or more representations.

## =====  CHANGE =====

## 6.10	Uplink Media Streaming with Explicit Congestion Notification (ECN) to support L4S

Support for uplink media streaming is very similar to that for downlink media streaming.

A high-level call flow for uplink media streaming is sketched in figure 6.10-1 below. The following is assumed:

-	The service here is a unicast uplink media streaming service with dynamic policy support, as described in clause 6.9.

-	The Layer 4 protocol used for application flows is TCP and the TCP stack used supports L4S.

-	The network supports L4S packet marking.

-	The application has specifically requested ECN marking for its media delivery session.

-	NG-RAN manipulates the ECN bits (per clause 5.37.3.2 of TS 23.501 [23]). It is equally possible that the PSA-UPF manipulates the ECN bits (per clause 5.37.3.3 of [23]).

Figure 6.10-1: Uplink media streaming call flow for L4S on request

The steps are as follows:

0:	Policy Template Provisioning. A Policy Template is provisioned with the requirement for L4S capability indicated by setting a flag.

1:	Dynamic Policy activation. The Media Session Handler within the 5GMSu Client obtains Service Access Information and triggers a dynamic policy activation. A Policy Template Binding is present within the Service Access Information for each provisioned Policy Template. Policy Template Bindings suitable for L4S are indicated by an L4S capability requirement flag being set. The 5GMSu Client detects that an L4S-capable media transport stack is present and in use. The selected Policy Template is one configured with L4S capability.

2:	QoS request. The 5GMSu AF requests QoS handling using e.g. the Nnef_AfSessionWithQoS service or the Npcf_PolicyAuthorization service. If the L4S capability requirement flag is set in the selected Policy Template, this indicates that the new QoS flow is required to be L4S-enabled. The new QoS flow with the L4S indication setting propagates through the 5G System.

3:	The Media Session Handler may inform the Media Streamer via the M11 interface API about the activation of L4S. Subject to availability of API access, the Media Player may use congestion notifications to perform early adaptation.

4:	If the L4S capability requirement flag is set in the Policy Template Binding for the selected Policy Template, the 5GMSu Client selects/enables the L4S capability of the used transport protocol.

NOTE:	This step may happen implicitly by selecting an L4S-supporting transport protocol stack.

5:	The Media Streamer within the 5GMSu Client triggers the establishment of a TCP connection. The ECT(1) codepoint is set in the IP header, indicating an L4S-Capable Transport, and the SDAP entity ensures that the packet is forwarded via the matching QoS flow.

6:	The 5GMSu AS responds to the TCP connection establishment request. The 5GMSu AS sets ECT(1) in the IP headers, indicating an L4S-Capable Transport.

7:	The UPF finds the matching QoS flow identifier for the downlink packet and sends the packet via the according QoS flow to the UE. TCP Connection setup continues, with one ECT bit set in all packets.

8:	When the RAN detects an upcoming congestion according to the congestion measurement (based on continuous congestion monitoring), the 5G System sets the CE (Congestion Experienced) codepoint in the IP header of the uplink packet.

9:	The TCP protocol stack used by the 5GMSu AS reflects the Early Congestion Notification to the TCP sender by setting the ECN-Echo (ECE) flag in the TCP header of a downlink PDU of the same TCP connection. The TCP sender reacts to the ECN-Echo accordingly (i.e., by reducing its sending congestion window).

NOTE 1:	The ECN-Echo flag is also acknowledged by the TCP sender setting the Congestion Window Reduced (CWR) flag in an outgoing TCP frame, but this acknowledgement is not illustrated in this call flow.

NOTE 2:	Classic ECN as specified in RFC 3168 [X1] requires an ECN signal to be treated as equivalent to a packet drop. L4S as specified in RFC 9330 [X2] specifies a more fine-grained response and an early congestion signal triggers a less severe reaction. How a TCP sender behaves “accordingly” is not in scope of the specification.

10:	Based on the CE indication received in step8, or by detecting a reduced bit rate in the uplink application flow, the Media Streamer in the 5GMSu Client may react by, for example, changing the requested representation.

11. Based on the CE indication received in step 8, or by detection of a reduced bit rate in the uplink application flow, the 5GMSu AS may react by, for example, updating the DASH presentation manifest to remove/add one or more representations.

## =====  CHANGE =====

# A.16	Collaboration scenarios for Media Streaming with Explicit Congestion Notification (ECN) to support L4S

## A.16.1	General

Collaboration scenarios for L4S ECN marking are depicted below. Both the Media AS and the 5GMS Client make use of an L4S-enabled protocol stack. Figure A.16.1-1 assumes that the Media AS resides within the external DN, while figure A.16.1-2 assumes the Media AS within the Trusted DN.

Figure A.16.1-1: Media AS in External DN

Figure A.16.1-2: Media AS in Trusted DN