3GPP TSG-SA4 Meeting #0-e (AH) MBS SWG post 130	S4aI250033

# Online, , 19th Dec 2024 - 6th Feb 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.802 | CR | 0007 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [FS_AMD] Generic Application Service |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Germany |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_AMD |  |  |  |  |  | Date: |  |  | 2025-01-07 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In the context of the Rel-19 AMD study, several cases for potential hybrid services have been identified, and are only partially addressed. This document addresses specifically the following case: - Application Services including hybrid services, as documented in clause 5.11.3.2 of TR 26.802, |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document addresses the above aspects generalizes the architectural and procedural aspects when combining unicast and broadcast when applied on the application level. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TR 26.802 CR 0006 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## =====  CHANGE =====

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[A331]	ATSC A/331, "Signaling, Delivery, Synchronization, and Error Protection".

[103972]	ETSI TR 103 972: "Deployment Guidelines for DVB-I services over 5G Systems".

[26804]	3GPP TR 26.804: " Study on 5G media streaming extensions"

[AWS-WM]	Kevin Yao, "Combining dynamic ad insertion and A/B watermarking", 22 FEB 2023 in AWS Elemental MediaTailor, Direct-to-Consumer & Streaming, Media & Entertainment, Media Services, available here: https://aws.amazon.com/blogs/media/combining-dynamic-ad-insertion-and-a-b-watermarking/

[104002]	ETSI TS 104 002: " Publicly Available Specification (PAS); DASH-IF Forensic A/B Watermarking An interoperable watermarking integration schema".

[DSS-2023]	DASH-IF Special Session: "A/B Watermarking", February 2023, slides available here: https://github.com/Dash-Industry-Forum/Dash-Industry-Forum.github.io/files/10720660/DASH-IF_ServerSideWM_2023.pdf

[CTA5007]	CTA 5007-A: "Web Application Video Ecosystem - Common Access Token"

[WM-API]	DASH-IF Candidate Specification, "Watermarking Encoder API", Community Review version available here https://dashif.org/news/cr-watermarking-api/

## =====  CHANGE =====

### 5.11.4	Summary and conclusions

It is recommended to address the following functionalities that are available in MBMS for MBS User Services:

1.	The generic Application Service as defined in clause 7.6 of TS 26.346 [16] based on the discussion in clause 5.11.3.2. A detailed discussion in provided in clause 5.13 with a new key issue.

2.	Partial file handling as defined in clause 7.9 of TS 26.346 [16] based on the discussion in clause 5.11.3.3,

3.	Reporting of metrics based on the discussion in clause 5.11.3.4,

4.	Time Synchronization as defined in TS 26.346 [16] in clause 4.6 based on the discussion in clause 5.11.3.6.

Other aspects identified in clause 5.11.2 for aligning MBS and MBMS are for further study.

## =====  CHANGE (assumed accepted in CR0006) =====

### 5.12.1	Description

In several deployment scenarios and specifications, the distribution of unicast streaming content such as DASH/HLS/CMAF content over a broadcast/multicast system is considered. Examples include, but are not limited to:

-	5GMS via eMBMS (see TS 26.501 [9], clause 4.9 and 5.10)

-	5GMS via MBS (see TS 26.501 [9], clause 4.11 and 5.12)

-	DVB ABR Multicast in ETSI TS 103 769 [12]

-	ATSC3.0 specification in A/331 [A331]

-	ETSI TS 103 720 [31] for 5G Broadcast

-	ETSI TR 103 972 [103972] for DVB-I over 5G Broadcast

Hybrid services have been well established, and examples include alternative languages requested over unicast, targeted ad insertion, coverage extensions, and several others.

In the context of the Rel-19 AMD study, several cases for potential hybrid services have been identified, and are only partially addressed:

-	In-session unicast repair as documented in clause 5.9 of this document,

-	Application Services including hybrid services, as documented in clause 5.11.3.2 of this document,

-	Reporting and metrics as documented in clause 5.11.3.4 of this document,

-	Support for DRM in clause 5.10 of TR 26.804 [26804], in particular as mentioned in clause 5.10.4: "The mapping to 5GMS via MBS/MBMS is for further study",

-	Common Media Client Data as introduced in clause 5.16 of TR 26.804 [26804], in particular as mentioned in clause 6.16, it is recommended "to further study the usage of CMCD when 5GMS is deployed over MBS and/or MBMS.

This clause addresses the above aspects generalizes the architectural and procedural aspects when combining unicast and broadcast. This aspect also allows to add new functionalities that require selective unicast requests.

In hybrid media cases, the Media Player may simultaneously request and consume resources from a unicast server (CDN) and a local Media Server, attached to the UE. In the example shown in Figure 5.12.1-1 taken ETSI TR 103 972 [103972], three options are considered, how the combination of unicast and multicast are carried out to support hybrid services.

1.	The broadcast receiver accesses unicast network

2.	The DASH client selects content from different networks

3.	The application (in this case DVB-I client) selects content from different networks

Figure 5.12.1-1: Different options to combine multicast/broadcast with unicast (from ETSI TR 103 972 [103972])

Many streaming services and experiences nowadays have a certain amount of personalization. In this case, two aspects are of importance:

-	Requests from the Media Player may include personalized information such as identifiers for the user, specific tokens, device identifiers, tracking data or other client metadata such as defined in CMCD.

-	Responses to requests may be customized by the CDN or some edge server based on information included in requests

Prominent examples are provided in Table 5.12.1-1.

Table 5.12.1-1: Examples for personalized unicast requests and responses

| Customized requests | Customized responses |
| --- | --- |
| Ad tracking and beaconing | Monetization incentives, ad skipping possible |
| DRM and License keys requests | License keys |
| Common Media Client Data | - |
| Identifiers used by Ad Decision systems | Targeted dynamic ad content |
| Tokens used by A/B Watermarking schemes | Variants of A/B watermarking |

For more details on A/B watermarking schemes and Ad Insertion, see for example [AWS-WM]. To fully support 5G Media Streaming and general media streaming services over MBS/MBMS, the above functionalities need to be supported as well in a hybrid manner.

Two high-level solutions may be considered to address the handling unicast requests.

- Solution 1: Media Player handles unicast requests

-	The manifest discriminates requests to the local media server (fed by broadcast/multicast) and the unicast requests by creating different service locations and typically different URLs

-	The Media Player handles the unicast requests directly to the network and deals with request issues

-	Generally, this is the preferred option for several use cases (alternative language requests, handoff to unicast for coverage extensions, etc.), but there are limitations

-	If the request to a broadcast resource is bundled with unicast related data, the information terminates in the local Media Server (CMCD, Tokens, etc.)

-	If the differentiated request needs to be obfuscated to the client and the response logic is provided be a network server, instead of done by the client.

-	Scalability, too many requests may go to unicast, so only subset needs to be addressed through unicast.

- Solution 2: MBS/MBMS/5G Broadcast client handles unicast request

- In this case, the Media Player sends conditioned requests to the local or gateway server

- The local/gateway server not only serves the requested resource, but also processes the additional information provided with the request and potentially issues unicast requests based on the resources.

-	The requested resource may be served from the broadcast distribution, or it may be served by issuing a unicast request or, as in the case of unicast repair, it may even be a combination of the two.

Solution 1 is well established in MBMS using the generic application service. A more detailed discussion is provided in clause 5.13 in a new key issue.. For high-level solution 2, no mechanism is yet provided. The remainder of this clause addresses a mapping of the solution 2 to MBS/MBMS architectures. The solution may be combined with 5G Media Streaming, or may support third-party media streaming services.

## =====  CHANGE (assumed accepted in CR0006) =====

## 5.13	Key Issue #12: Generic Application Service

### 5.13.1	Description

Based on the discussion in clause 5.12.1, there are cases when the Media Player is delegated to handle unicast requests. This corresponds to option 2 in figure 5.12.1-1. A specific case is mentioned in clause 5.11.4, namely the generic Application Service as defined in clause 7.6 of TS 26.346 [16] based on the discussion in clause 5.11.3.2.

The Generic Application service in clause 7.6 of TS 26.346 [16] is described as follows on a high-level:

-	The User service announcement contains an explicit Application Service Description, for example a DASH MPD or a HLS M3U8, or both. In this case, all resources that are directly or indirectly referenced in the application service entry point document instance of this metadata fragment, and are expected to be retrieved by HTTP GET, are delivered as an MBMS User Service. However, the user service may use MMBS broadcast or it may use unicast.

-	In order to support generic application services in MBMS, the User service description contains an Application Service Description which describes the service, including a content-type parameter to describe the information.

-	Now if the user service description contains a reference to an application service entry document containing broadcast-delivered objects, then

1)	The user service is a download delivery service using FLUTE.

2)	The MBMS download session delivers objects that are directly or indirectly referenced by the service entry document.

3)	If an object is delivered as a FLUTE object with an availability time defined by service is delivered then the MBMS download session delivers the objects such that the last packet of the delivered object is available at the UE latest at its availability time as announced in the application service document and the Content-Location element in the FDT for the delivered object matches the URL in the application service document.

4)	If an update to the application service document is delivered as a FLUTE object then the Content-Location element in the FDT for the delivered object matches the URI of the appropriate referenced application service document

For any application service which is not a DASH-over-MBMS service, a) its service definition and any specialized handling for service delivery over MBMS, and b) the content format with the exception that it is an HTML5 document, management and hosting of the associated Application Service Description are outside the scope of this specification.

In clause 7.6 of TS 26.346, more details on use cases are discussed. As conveyed by the application service document, an Application Service belonging to a MBMS User Service and carried by the MBMS download delivery method may be made available such that the resources are partly available on broadcast and are partly available in unicast.

Two main use cases are considered in this context:

1)	unicast fallback reception should the UE move outside the MBMS coverage area of the corresponding User Service.  Subsequently, should the UE move back into MBMS coverage, it may be required by network operator policy that only broadcast reception of the Service is permitted (network policy and the means for its delivery and execution is outside the scope of this specification).  It may also be desired by the MBMS service provider that reception of individual broadcast resources are restricted by MBMS service areas.

2)	unicast-supplemented service offerings, for which certain resources are only available on unicast and these resources provide an additional user experience and therefore should be accessible by the application, regardless whether the MBMS client is in the coverage for broadcast reception or not.

5.13.2	Collaboration scenarios and architecture mappings

#### 5.13.2.1	General

In order to address the functionality of handling a combination of requests from media players to unicast and broadcast, a new function in the Multicast Broadcast client is added that operates based on a policy to steer the client to specific service locations. The application service document is typically requested through the Media Service on the client, i.e. form service location 1. This allows to apply different policy enforcing mechanisms such SAND4M, Manifest rewrite or Content Steering. Figure 5.13.2.1-1 provides a basic overview of the considered architecture, initially independent of a specific implementation for MBS or MBMS.

Figure 5.13.2.1-1: Extension to Multicast Broadcast client to support generic application service

Examples include alternative languages requested over unicast, targeted ad insertion, coverage extensions, and several others.

#### 5.13.2.2	Mapping to MBS User Services

In TS 26.502, the MBS AS is already defined, but at this stage only has a single functionality, namely object repair. In order to support generic application services, the MBS AS is extended to host data not only for object repair.

NOTE: 	the physical location of the MBS AS may be collocated with a unicast server that is also used for regular unicast communication. The MBS AS is primarily considered a logical function.

Figure 5.13.2.2-1 now extends the MBS AS to support generic application services using the application unicast function.

Figure 5.13.2.2-1: Extensions to MBS User Services network architecture: Generic application unicast functionality in MBS AS

The above extension does not yet include the proposed extensions in considered in clause 5.9, namely definition of the reference point between MBSTF and MBS AS. However, it is considered a generalization of the discussion in clause 5.9. In addition, the client architecture may stay primarily unmodified, but some of the extensions are provided.

Figure 5.13.2.2-2: Extensions to MBS User Services reference architecture: Generic Application Service

In Figure 5.13.2.2-2, the MBS User Services reference architecture is extended also in the MBSTF client to support a steering policy for service locations. In this case, the MBSF client may configure the logic for steering via MBS-6', and the media server in the MBSTF client may also provide a configuration API, for example which requests are served via unicast or which ones are served via MBS-4-MC. While it is expected that the client can issue direct requests, the requests are virtually proxied through the MBSTF client.

The extensions for the high-level baseline procedures in TS 26.502 [26502] are a generalization to the procedure extensions in clause 5.9 for generic application service and are provided in Figure 5.13.2.2-3 in bold.

NOTE:	In the interests of brevity, the prefix MBS is omitted from the numbered steps in the figure.

Figure 5.12.2.4-3: Extended MBS User Service high-level baseline procedures

The following steps are updated or extended compared to the original high-level call flow:

-	The user service provisioning step 1 includes application unicast provisioning

-	The distribution session provisioning step 2 includes application unicast provisioning

-	A new step NEW1 (after step 2) is added to provision application unicast ingest

-	The user service announcement step 3 is extended to include instructions for application unicast

-	A new step NEW2 (after step 5) is added to ingest application unicast request content to the MBS AS

-	Step 8 and 9 are extended to provision the MBSTF client to support application unicast.

-	A new step NEW3 is added (after step 10) to support requests from the application that serve different service locations

-	Step 11 is extended such that in the distribution session also uses the MBS AS selectively for unicast requests.

-	A new step NEW4 is added (after step NEW3) that permits policy changes, for example unavailability of the MBS service. Based on this in step NEW5, the distribution is session now exclusively using unicast.

### 5.13.3	Gap analysis and requirements

The following aspects are identified to be missing:

1)	Formal definition of a named reference point between the MBSTF and the MBS AS in order to publish objects to the MBS AS for the purpose of application unicasts.

2)	User service provisioning to support application unicast requests.

3)	Distribution session provisioning to support application unicast requests.

4)	Provisioning of the MBS AS in order to support application unicast requests.

5)	User service announcements to support provisioning for application unicast requests for different purposes.

6)	Ingest session to support hosting unicast data on MBS AS

7)	Functional extension of MBSTF Client to support switching service locations.

8)	Support for differentiated requests from application to MBS client (and forward those to unicast or serve from local server).

### 5.13.4	Candidate solutions

In order to address the gaps identified in clause 5.12.3, the following solutions may be considered

1)	Formal definition of a named reference point between the MBSTF and the MBS AS in order to publish objects to the MBS AS for the purpose of application unicasts.

-	The same reference point as defined for object repair in clause 5.9 is expected to be used.

2)	User service provisioning to support application unicast requests.

-	the provisioning may be supported by template functions that are otherwise provided to a proxy/edge server to configure personalized requests.

3)	Distribution session provisioning to support application unicast requests.

-	This can be addressed by a generalization to what is needed for object repair in clause 5.9.

4)	Provisioning of the MBS AS in order to support application unicast requests.

-	This can be addressed by a generalization to what is needed for object repair in clause 5.9.

5)	User service announcements to support provisioning for application unicast requests for different purposes.

-	the provisioning may be supported by forwarding a template function that are otherwise provided to a proxy/edge server to configure personalized requests.

6)	Ingest session to support hosting unicast data on MBS AS

-	This can be addressed by a generalization to what is needed for object repair in clause 5.9.

7)	Functional extension of MBSTF Client to support switching service locations..

-	options include SAND4M, Manifest rewrite, Content Steering.

8)	Support for differentiated requests from application to MBS client with  different service locations.

-	no specific extensions are needed for typical clients.

9)	Support for application unicast requests from MBSTF client to MBS AS.

-	no specific extensions are needed except for parameters that are processed, including http header parameters, query parameters or other parts of HTTP requests.

### 5.13.5	Summary and conclusions

In order to address the functionality of handling a combination of personalized requests from media players, a new function in the Multicast Broadcast client is added that collects information and may communicate with the unicast Application Provider.

For stage-2 impact:

-	Gap#1-9 in clause 5.13.3 are expected to be addressed by:

a.	Defining a new reference point in TS 26.502 [29] (already considered in clause 5.9)

b.	Documenting call flows and procedures for application unicast requests based on the details in clause 5.13.4

Stage-3 impact is expected to address gaps #1 to #9 based on the candidate solutions in clause 5.13.4 and the stage-3 aspects. Full alignment with the MBMS application service is expected.