3GPP TSG-SA WG4 Meeting post #129e 	S4aI240193

Electronic Meeting, 26th September–24th October 2024

Title:	[FS-AMD WT 3b]: Further details on application layer considerations with multi-access media delivery

Agenda Item:	2.6

Source:	Samsung Electronics Co Ltd.

Introduction

Document S4aI240177 on the topic of application considerations with multi-access media delivery was agreed during Post SA4#129-e meeting, and is being included into the baseline CR#13 in S4aI240132. The merge proposal is discussed in S4-240192. Document S4aI240177 discussed about application consderations with two steering functionalities – MPQUIC and MPTCP, and introduced high level call flows, gaps, and a candidate solution for multi-access media delivery using ATSSS.

The candidate solution in S4aI240177 described two aspects on configuration of multipath delivery parameters by the 5GMS-Aware Application, and informing the 5GMS-Aware Application of status of multipath delivery connection. The present document proposes further details on these two aspects.

Background and motivation

Document S4aI240177, and subsequently clause 5.15.6.2 in candidate baseline CR#13 in S4aI240192, proposed to extend the media stream handling client API exposed by the Media Stream Handler to the 5GMS-Aware Application at reference point M7 and to the Media Session Handler at reference point M11 as follows:

1.	For the 5GMS-Aware Application to configure the following multipath delivery parameters:

-	Enable/disable multipath media delivery.

-	The number of MPQUIC paths or MPTCP subflows in the multipath delivery connection.

-	Add new or remove existing MPQUIC path or MPTCP subflows to/from the multipath delivery connection.

-	Which media application flows are mapped onto which MPQUIC path or MPTCP subflow.

These objectives may be achieved by modifying the Configurations and settings API specified in clause 13.2.4 of TS 26.512 [16].

2.	For the 5GMS-Aware Application to be informed of the following:

-	Connection endpoint information to each of the MPQUIC path or MPTCP subflow.

-	Status information of multipath delivery connection.

These objectives may be achieved by modifying the Dynamic Status Information specified in clause 13.2.6 of TS 26.512 [16].

The present document describes the modifications to the Configurations and settings API specified in clause 13.2.4 and Dynamic Status Information specified in clause 13.2.6 of TS 26512 as described above.

Text proposal

The following textual changes to TR 26.804 CR0013 are proposed.

# * * * * First change * * * *

### 5.15.6	Candidate solutions

Editor’s Note:	Candidate solutions for identified key issue.

#### 5.15.6.1	Multi-access downlink media streaming using CMMF

This candidate solution includes approaches where a 5GMSd Client accesses and downloads, via reference point M4d, CMMF-encoded media objects [CMMF – reference included within CR adding clause 5.19], and possibly original source media (e.g., MPEG-DASH or HLS media segments), over multiple access networks simultaneously from a single 5GMSd AS. CMMF (discussed in detail within clause 5.19.6.3) enables multi-access capabilities through application-layer implementations of the 5GMSd Client without requiring lower-layer (e.g., network, transport, etc.) multi-access integrations.

In this solution, multiple different CMMF-encoded bitstreams/objects (or representations) of the source media are stored/cached within a single logical 5GMSd AS. A CMMF-enabled 5GMSd Client requests and downloads a different CMMF-encoded representation (stripe) of the required original source media over each of the access networks available to it. These (potentially partially) received CMMF bitstreams/objects are decoded by a CMMF decoder in the 5GMSd Client yielding the required original source content once enough information has been received over all of the available access networks.

Unlike other multi-access technologies such as MPTCP, MPQUIC, ATSSS, etc., the responsibility to set up, request, and steer the delivery of content across each available access network rests with the application layer (e.g., Media Player) in this solution. For example, a Media Player sets up multiple HTTP connections in parallel, each one bound to a different network interface (each assigned with an IP address appropriate to that network). Requests to a single 5GMSd AS for different CMMF-encoded representations (stripes) of the original source media are sent from each of the HTTP connections over the different access networks. The HTTP response from the 5GMSd AS to each of these requests is routed appropriately over the appropriate access network, following standard network-layer/IP routing rules and procedures.

As mentioned above, traffic steering over each access network may be performed by the application layer (e.g., Media Player); and multiple policies can be defined/implemented based on the desired outcome. For example, a best-effort policy may evolve downloading as much CMMF-encoded content from each available access network until the CMMF decoder can successfully decode the required media. Networks that have lower latency, higher bandwidth, etc. will naturally contribute more to the download than those with higher latency, lower bandwidth, etc. Another policy may preference delivery of content from one access network over another. In this case, a schedular may be implemented so that requests of CMMF-encoded content made to the Application Server can be throttled over one access network so that the majority of the download is completed on the other(s).

Integration of CMMF within the 5GMS System is discussed in detail within clause 5.19.6.3.

#### 5.15.6.2	Multi-Access media delivery using ATSSS

To address the gaps identified in clause 5.15.5.2, it is proposed to extend the media stream handling client API exposed by the Media Stream Handler to the 5GMS-Aware Application at reference point M6 and to the Media Session Handler at reference point M11 as follows:

1.	For the 5GMS-Aware Application to configure the following multipath delivery parameters:

-	Enable/disable multipath media delivery.

-	The number of MPQUIC paths or MPTCP subflows in the multipath delivery connection.

-	Add new or remove existing MPQUIC path or MPTCP subflows to/from the multipath delivery connection.

-	Which media application flows are mapped onto which MPQUIC path or MPTCP subflow.

These objectives are achieved by enhancing the parameters to the Configurations and settings API specified in clause 13.2.4 of TS 26.512 [16] as follows:

### 13.2.4 Configurations and settings API DASH streaming for a particular downlink media delivery session may be configured by the 5GMSd-Aware Application at reference point M7d or by the Media Session Handler at reference point M11d with the parameters provided in table 13.2.4-1. Note that these parameters may be set and they may also be observed. Table 13.2.4-1: Media Player Configuration API Status; Type; Definition. sessionId; string; A media delivery session identifier for the downlink media streaming session that has been initialised using the method specified in clause 13.2.3.2.. source; Object; Provides the MPD and all contained information.. consumptionMode; Enum; Defines two modes: live: in this case the target latency is maintained, if specified in the service description, according to the parameters vod: in this case the latency is set by the application and the latency settings are ignored.. maxBufferTime; Integer; Maximum buffer time in milliseconds for the service.. serviceDescriptionId; id; Selects a service description by selecting an identifier.. serviceDescriptions[]; Service description parameters; Configures a service description as defined in annex K of ISO/IEC 23009-1 [32]. This allows the application to define additional service descriptions beyond those defined in the MPD.. id; id; Sets a service description identifier different from the ones available in the service descriptions in the MPD or modifies existing service descriptions.. serviceLatency; Object; Sets service description parameters for the service latency, as defined in table K.1 of ISO/IEC 23009-1 [32].. playBackRate; Object; Sets service description parameters for the playback rate, as defined in table K.2 of ISO/IEC 23009-1 [32] when the service is consumed in live mode.. operatingQuality; Object; Sets service description parameters for the operating quality, as defined in table K.3 of ISO/IEC 23009-1 [32].. operatingBandwidth; Object; Sets service description parameters for the operating bandwidth, as defined in table K.4 of ISO/IEC 23009-1 [32].. mediaSettings[]; Media type audio, video, subtitle; Sets the selected Adaptation Set based on the available Adaptation Sets for each media type.. metricsConfiguration[ ]; Object; Zero or more sets of settings for collecting metrics in relation to the downlink media streaming session.. enableMultipathDelivery; Boolean; Configures whether to use multipath delivery using multiple access network connection endpoints. pathsForMultipathDelivery; Integer; Number of paths used by the Media Stream Handler for multipath delivery connection: - In case of MPQUIC based multipath delivery, this parameter represents the number of MPQUIC paths - In case of MPTCP based multipath delivery, this parameter represents the number of MPTCP subflows. accessNetworkEndpoints[]; Array; Set of access network endpoints over which the multipath delivery connection is to be used.

2.	For the 5GMS-Aware Application to be informed of the following:

-	Connection endpoint information to each of the MPQUIC path or MPTCP subflow.

-	Status information of multipath delivery connection.

These objectives are achieved by enhancing the Dynamic Status Information specified in clause 13.2.6 of TS 26.512 [16] as follows:

### 13.2.6 Dynamic Status Information Table 13.2.6-1 provides a list of dynamically changing status information that can be obtained from the Media Player via reference point M7d or M11d. A separate set of Dynamic Status Information is provided for each active downlink media streaming session, indexed by its media delivery session identifier initialised per clause 13.2.3.2. Table 13.2.6-1: Media Player Dynamic Status information Status; Type; Parameter; Definition. state; Enumeration; An enumerated value from table 13.2.2-1 indicating the current state of the Media Player.. averageThroughput; float; none; Current average throughput computed in the ABR logic in bit/s.. bufferLength; float; MediaType "video", "audio" and "subtitle"; Current length of the buffer for a given media type, in seconds. If no type is passed in, then the minimum of video, audio and subtitle buffer length is returned. NaN is returned if an invalid type is requested, the presentation does not contain that type, or if no arguments are passed and the presentation does not include any adaption sets of valid media type.. liveLatency; float; none; Current live stream latency in seconds based on the latency measurement.. mediaSetting[ ]; MPDAdaptationSet; MediaType "video", "audio" and "subtitle"; Current media settings for each media type based on the CMAF Header and the MPD information based on the selected Adaptation Set for this media type.. mediaTime; float; None; Current media playback time from media playback platform. The media time is in seconds and is relative to the start of the playback and provides the media that is actually rendered.. playbackRate; float; None; The current rate of playback. For a video that is playing twice as fast as the default playback, the playbackRate value should be 2.00.. availableServiceDescriptions[ ]; Array of service descriptions; Provides the list of available selectable service descriptions with an id to select from. Those are either configured ones or the ones in the MPD.. availableMediaOptions[ ]; List of Adaptation Set or Preselection ids; MediaType "video", "audio" "subtitle" "all"; Provides the list of available media options that can be selected by the application based on the capability discovery and the subset information.. service‌Operation‌Points; array(Service‌Operation‌Point); The set of Service Operation Points declared in the presentation manifest (e.g. DASH MPD) of the current media presentation.. operative‌Service‌Operation‌Point; integer; A zero-based index into the service‌Operation‌Points array indicating the Service Operation Point currently operative in the playback session. Set to -1 if the array is empty.. metrics[ ][ ]; Metrics; A data blob of metrics for each configured metrics collecting scheme.. multipathDeliveryStatus; string; Status information of multipath delivery connection.

Editor's Note: Detailed formats of the above information are to be developed in conjunction with study progress documented in clause 5.24 of present document.

### 5.15.7	Summary and conclusions

# * * * * End of changes * * * *