3GPP SA4-e (AH) MBS SWG post 130	S4aI250009

# Online, December 19th 2024 – February 6th 2025

Source:	Orange

Title:	Pseudo-CR on potential solution to KI1 based Energy related information from the UE, network and other entities provided to an App

Spec:	3GPP TR 26.942 v1.0.0

Agenda item:	2.7 - FS_MediaEnergyGREEN (Study on Media enerGy consumption exposuRE and EvaluatioN framework)

Document for:	Agreement

1. Introduction

During SA4#130, S4-241927 was agreed with the following decision: “We should continue with the proposed approach in 241927 and document it”. This is the goal of this pCR.

2. Reason for Change

The regulatory goal to raise user awareness of environmental issues requires informing users of the environmental impact of audiovisual media consumption. This requirement is also relevant for MNO aiming to reduce their scope 3 (indirect) emissions, which is the most difficult aspect for reaching its net-zero strategy. In both cases, the energy consumption only has to be exposed to the user, not necessarily the network. The overall energy consumption (device + the network and other components of the content delivery) for the user’s audiovisual services could also be reported to the user by an App that is given access to the end-to-end consumption information. This pCR propose a solution where the 5GMS client is collecting the necessary information from the UE, network, and other entities, and provide this to the App for exposing to the user.

3. Conclusions

This document aims to propose a solution for KI1 (Energy-related Information exposure) allowing the 5GMS client to collect the necessary information from the UE, network, and other entities, and provide this to the App for exposing to the user.

4. Proposal

It is proposed to agree the following changes to 3GPP TR 26.942 v1.0.0.

* * * First Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	ETSI TS 28.310: "Management and orchestration; Energy efficiency of 5G".

[3]	ETSI EN 303 472: "Environmental Engineering (EE); Energy Efficiency measurement methodology and metrics for RAN equipment".

[4]	ETSI ES 202 706-1 V1.7.1 (2022-08): "Environmental Engineering (EE); Metrics and measurement method for energy efficiency of wireless access network equipment; Part 1: Power consumption - static measurement method".

[5]	ETSI TS 22.261: "Service requirements for the 5G system".

[6]		D Lundén, J Malmodin, P Bergmark and N Lövehagen, "Electricity Consumption and Operational Carbon Emissions of European Telecom Network Operators", Sustainability 14(5):2637, 2022.

[7]		J Malmodin, "The power consumption of mobile and fixed network data services-The case of streaming video and downloading large files." Electronics Goes Green, 2020.

[8]		J Malmodin, N Lövehagen, P Bergmark and D Lundén, "ICT sector electricity consumption and greenhouse gas emissions – 2020 outcome", Telecommunications Policy (2024): 102701.

[9]	The Greenhouse Gas Protocol, https://ghgprotocol.org

[10]	The Greenhouse Gas Protocol, “A Corporate Accounting and Reporting Standard, Revised Edition”

[11]	The Greenhouse Gas Protocol: "Corporate Value Chain (Scope 3) Accounting and Reporting Standard: Supplement to the GHG Protocol Corporate Accounting and Reporting Standard".

[12]	The European Commission: "Commission Delegated Regulation (EU) 2023/2772", The Official Journal of the European Union, 23-7-2023

[13]	Ember Climate: "Global Electricity Review 2023",
https://ember-climate.org/insights/research/global-electricity-review-2023/

[14]	3GPP TR 28.913: "Study on new aspects of Energy Efficiency (EE) for 5G phase 2"

[15]	3GPP TS 28.554: "5G end to end Key Performance Indicators (KPI)".

[16]	3GPP TS 28.552: "Management and orchestration; 5G performance measurements".

[17]	3GPP TS 28.533: "Management and orchestration; Architecture framework".

[18]	3GPP TS 28.622: "Telecommunication management; Generic Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)".

[19]	3GPP TS 28.532: "Management and orchestration; Generic management services".

[20]	3GPP TR 23.700-66: "Study on Energy Efficiency and Energy Saving".

[21]	3GPP TS 26.531: "Data collection and reporting; General description and architecture".

[22]	3GPP TS 26.532: "Data Collection and Reporting; Protocols and Formats".

[23]	3GPP TS 26.501: "5G Media Streaming (5GMS); General description and architecture".

[24]	3GPP TS 26.247: "Transparent end-to-end Packet-switched Streaming Service (PSS); Progressive Download and Dynamic Adaptive Streaming over HTTP (3GP-DASH)".

[25]	3GPP TS 38.300: "NR; NR and NG-RAN Overall description; Stage-2”.

[26]	3GPP TS 26.114: “IP Multimedia Subsystem (IMS); Multimedia telephony; Media handling and interaction”

[27]	3GPP TS 26.118: “Virtual Reality (VR) profiles for streaming applications”

[28]	3GPP TS 28.405: "Telecommunication management; Quality of Experience (QoE) measurement collection; Control and configuration”

[29]	International Telecommunication Union, Recommendation ITU-T L.1310: "Energy efficiency metrics and measurement methods for telecommunication equipment".

[30]	International Telecommunication Union, Series L Supplement 36, "ITU-T L.1310 – Study on methods and metrics to evaluate energy efficiency for future 5G systems", 11/2017

[31]	International Telecommunication Union, Recommendation ITU-T L.1450, "Methodologies for the assessment of the environmental impact of the information and communication technology sector", 9/2018

[32]	Jens Malmodin, Nina Lövehagen, Pernilla Bergmark, and Dag Lundén. "ICT sector electricity consumption and greenhouse gas emissions–2020 outcome." Telecommunications Policy (2024): 102701

[33]	International Telecommunication Union, Report ITU-R BT.2385-1, "Reducing the environmental impact of terrestrial broadcasting systems", 03/2022

[34]	International Telecommunication Union, Opinion ITU-R OP.104, "Advice for sustainability strategies incorporating carbon offsetting policies", 2022

[35]	International Telecommunication Union, Report ITU-R BT.2521-0, "Practical examples of actions to realize energy aware broadcasting", 3/2023

[36]		International Telecommunication Union, Report ITU-R BT.2540-0, "Display energy reduction through image signal processing", 03/2024

[37]	ISO/IEC 23001-11:2023 Information technology, MPEG systems technologies,  Part 11: Energy-efficient media consumption (green metadata)

[38]	DVB, "Study Mission report on Energy Aware service Delivery and Consumption", DVB Document S100, 11/2023

[39]	ATSC Planning Team 9, https://www.atsc.org/subcommittees/planning-team-9-sustainability/

[40]	ETSI ES 202 706-1: "Environmental Engineering (EE); Metrics and measurement method for energy efficiency of wireless access network equipment; Part 1: Power consumption – static measurement method 5G base station energy performance KPIs".

[41]	ETSI ES 203 700: "Environmental Engineering (EE); Sustainable power feeding solutions for 5G network".

[42]	ETSI ES 203 539: "Environmental Engineering (EE); Measurement method for energy efficiency of Network Functions Virtualisation (NFV) in laboratory environment".

[43]	ETSI EN 303 470: "Environmental Engineering (EE); Energy Efficiency measurement methodology and metrics for servers".

[44]	ETSI EN 303 471: "Environmental Engineering (EE); Energy Efficiency measurement methodology and metrics for Network Function Virtualisation (NFV)".

[45]	ETSI ES 203 475: "Environmental Engineering (EE); Standardization terms and trends in energy efficiency".

[46]	ETSI ES 203 136: "Environmental Engineering (EE); Measurement methods for energy efficiency of router and switch equipment Update standard for Energy efficiency for router and switch equipment".

[47]	ETSI EN 303 215: "Environmental Engineering (EE); Measurement methods and limits for power consumption in broadband telecommunication networks equipment".

[48]	ETSI ES 203 184: "Environmental Engineering (EE); Measurement Methods for Power Consumption in Transport Telecommunication Networks Equipment".

[49]	ETSI EN 301 575: "Environmental Engineering (EE); Measurement method for energy consumption of Customer Premises Equipment (CPE)".

[50]	ETSI ES 203 215: "Environmental Engineering (EE); Measurement Methods and Limits for Power Consumption in Broadband Telecommunication Networks Equipment".

[51]	Greening of Streaming, https://www.greeningofstreaming.org/

[52]	DIMPACT, https://dimpact.org/

[53]	DIMPACT, "Methodology: Estimating the carbon impacts of serving digital media and entertainment products", version 1.0, October 2022

[54]	DIMPACT, Draft paper "Literature review and policy principles for streaming and digital media carbon footprinting", March 2023

[55]	Ultra HD Forum, https://ultrahdforum.org/ibc2023-press-release-ultra-hd-forum-to-showcase-efficient-hdr-sdr-sustainability-demos/

[56]	3GPP TR 22.882: "Study on Energy Efficiency as a service criteria".

[57]	ARCEP, "Arcep publishes a draft decision for public consultation, with a view to enhancing its annual “Achieving digital sustainability” survey",
https://en.arcep.fr/fileadmin/cru-1714402758/user_upload/41-24-english-version.pdf

[58]	3GPP TS 26.502: "5G multicast-broadcast services; User service architecture".

[59]	3GPP TS 26.506: "5G Real-time Media Communication Architecture".

[60]	Directive (EU) 2024/825: "amending Directives 2005/29/EC and 2011/83/EU as regards empowering consumers for the green transition through better protection against unfair practices and better information", 28th February 2024.
https://eur-lex.europa.eu/legal-content/EN/TXT/?uri=CELEX:32024L0825

[61]	ARCEP, "Arcom and Arcep, in tandem with ADEME, publish an unprecedented study on the environmental impact of audiovisual media consumption in France in 2022, and up to 2030", https://en.arcep.fr/fileadmin/user_upload/57-24-english-version.pdf

[62]	3GPP TR 26.955: "5G Video Codec Characteristics"

[63]	Android BatteryManager API, https://developer.android.com/reference/kotlin/android/os/BatteryManager

[64]	3GPP TS 23.247: "Architectural enhancements for 5G multicast-broadcast services"

[65]	3GPP TS 26.565: "Split Rendering Media Service Enabler”

[66]	ETSI ES 203 228: "Environmental Engineering (EE); Assessment of mobile network energy efficiency".

[67]	International Telecommunication Union, Recommendation ITU-T L.1331, "Assessment of mobile network energy efficiency", 1/2022

[68]	ETSI ES 202 336-12: "Environmental Engineering (EE); Monitoring and control interface for infrastructure equipment (power, cooling and building environment systems used in telecommunication networks); Part 12: ICT equipment power, energy and environmental parameters monitoring information model".

[69]	Harmony OS Battery information API, https://developer.huawei.com/consumer/en/doc/harmonyos-references/js-apis-battery-info-V5

[70] 	3GPP TS 23.501: "System architecture for the 5G System (5GS) "

[71]	3GPP TS 23.502: “Procedures for the 5G System (5GS)”

* * * Next Change * * * *

(all new text)

## 7.5	Solution #4: Energy related information from the network and other Service Provider entities provided to a UE application and Application Service Provider

### 7.5.1	Key Issue mapping

This solution addresses Key Issue #1 (Energy-related Information exposure) described in clause 6.1.

### 7.5.2	Functional description

This solution addresses how energy-related information from the device, the network and other components of the content delivery, during media consumption can be provided to a UE application for exposing to the user.

Following requirement PR1-1, it is reusing existing architecture, especially the generic architecture for UE data collection, reporting and event exposure defined in TS 26.531 [21]. In this specification the Data Collection AF has already interfaces with components which are already providing information from the device (Direct Data Collection Client), the network (NWDAF) and other components from the application Service Provider (Indirect Data Collection Client), and the UE Application.

Figure 7.5.2-1: Reference architecture for data collection and reporting

In this architecture, the UE Application is responsible for sharing relevant data with the Direct Data Collection Client via reference point R7. As the goal of this solution is to provide energy-related information to the UE application, data collected by the UE application will be processed directly by the UE application and exposing of the energy-related information from the UE is not required over the reference point R7. But energy-related information from the network and other components of the content delivery is required.

In the reference architecture for data collection and reporting, no use case where energy-related information from different locations is provided to the UE Application has been taken into consideration. Interfaces and the role of each components need to be adapted:

- Data Collection AF receives reports from Direct Data Collection Client at reference point R2, from the Indirect Data Collection Client at reference point R3 or from the Application Server (AS) instances at reference point R4 but those reports do not contain energy-related information.

- Data Collection AF processes received data reports but expose processed UE data to subscribers both inside the trusted domain (such as the NWDAF) and outside it (such as the Event Consumer AF in the Application Service Provider). Data Collection AF does not process network or Service Provider information and reports are only exposed to consumer entities such as the NWDAF or Event Consumer AF of an Application Service Provider over reference points R5 and R6, not to Direct Data Collection Client over reference point R2.

The purposes of the reference points in the functional architecture have to be modified as follows:

- R2 used by the Data collection AF to send reports containing energy-related information from the network and other components from the Application Service Provider to the Direct Data Collection Client.

- R3 used by the Indirect Data collection Client to send reports containing energy-related information from the Application Service Provider to the Direct Data Collection Client, in addition of the data collection already defined in TS 26.531 [21].

- R4 used by the AS instance to send energy-related data reports to its Data Collection AF instance, in addition of the data collection already defined in TS 26.531 [21].

- A new reference point R9 is required between the Data Collection AF and the EIF (Energy Information Function), collecting data from OAM and 5GC NF(s) to assist the calculation of energy related information, calculating the energy related information at different granularities (per UE included), and exposing the Energy Consumption information to the authorized consumer NF(s) (AF/NEF or 5GC NF) as defined in TS 23.501 [70].

[Editor’s note: EIF is not yet available in the last version of TS 23.501 v19.1.0 but CR adding this function have already been agreed during SA2#166 (i.e.,  ]

The instantiation of the UE data collection, reporting and event exposure architecture in the 5GMS System is defined in clause 4.7 of TS 26.501 [23] and the reference architecture for this instantiation is reproduced in figure 7.5.2-2.

Figure 7.5.2-2: Data collection and reporting architecture instantiation for 5G Media Streaming

The principle is the same in this instantiation for R2 and R4, but not for R3. The Indirect Data Collection Client is not instantiated in the 5GMS architecture and R3 is not existing. In this case, no interface can be reused between the Data Collection AF and the Indirect Data Collection to exchange energy-related information from the Application Service Provider. Only the 5GMS AS, when instantiated by the 5GMS Application Provider can provide its energy-related information. As the energy-related information from the Application Provider is not specific to mobile networks (except for the 5GMS AS) and as the 5GMS Application Provider is also responsible of the 5GMS-Aware Application, no need for interoperability has been identify here. This exchange of energy-related information between the Application Service Provider and the 5GMS-Aware Application is out of scope of this study. Like in TS 26.531 [21], a reference point between the Data Collection AF and the Energy Information Function is missing.

Even if this solution addresses how energy-related information from the device, the network and other components of the content delivery, during media consumption can be provided to a UE application for exposing to the user. It seems also relevant to use the reference point R6 to also reports energy-related information from the network to Event Consumer AF.

The new architecture would be as follow:

Figure 7.5.2-3: Data collection and reporting architecture instantiation for 5G Media Streaming including energy-related information

### 7.5.3	Procedures

The procedures for data collection and reporting client configuration, including energy related information from the network, is similar clause 5.4 of TS 26.531 [21]. The principle is the same than in clause 5.4 of TS 26.531 [21] but the entities providing reports are not the same.

The different data collection clients proceed as follows:

9. If present in the instantiation, the UE Application creates a data collection and reporting context, including energy-related information with the Direct Data Collection Client. As part of this context, the UE Application may indicate consent for a UE identifier to be included in data reports submitted on its behalf by the Direct Data Collection Client.

10. As a consequence of step 9 or (if the UE Application is not present) during its own initialisation, the Direct Data Collection Client acquires its data collection and reporting configuration from the Data Collection AF, if relevant.

If the UE Application is instantiated, the Direct Data Collection Client provides it with a data collection and reporting configuration derived from that just obtained from the Data Collection AF.

11.	The Indirect Data Collection Client acquires its data collection and reporting configuration from the Data Collection AF, if relevant.

12.	The AS acquires its data collection and reporting configuration from the Data Collection AF, if relevant.

13.	The EIF acquires its data collection and reporting configuration from the Data Collection AF, if relevant.

14. The EIF may submit a data report to the Data Collection AF by invoking the Ndcaf_DataReporting service defined in the present document and specified in TS 26.532 [7].

15. The AS may submit a data report to the Data Collection AF by invoking the Ndcaf_DataReporting service defined in the present document and specified in TS 26.532 [7].

16. The Data Reporting AF processes the data report.

17. The Data Collection AF may expose an event to the Direct Data Collection Client by invoking the Naf_EventExposure_Notify service operation on the latter, as defined in clause 5.2.19.2.4 of TS 23.502 [71].

18. The Direct Data Collection Client may submit the processed report to the UE Application.

19. The Data Collection AF may expose an event to the Event Consumer AF by invoking the Naf_EventExposure_Notify service operation on the latter, as defined in clause 5.2.19.2.4 of TS 23.502 [71].

### 7.5.4	Summary

This solution indicates how energy-related information from the device, the network and other components of the content delivery, during media consumption can be provided to a UE application for exposing to the user or to an Event Consumer AF for exposing energy-related information from the network to the Application Service Provider.

This solution uses the same principle than Data collection and reporting defined in TS 26.531 [21] and instantiated in the 5GMS System in clause 4.7 of TS 26.501 [23]. But several reference points and entities are impacted:

- The Direct Data Collection Client is not only sending reports, it also becomes a consumer entity of reports including energy-related information from the Data Collection AF through R2. It includes a new Energy Collection and Reporting function to provide energy-related information from the network to the UE application.

- The AS provides energy-related information in addition of existing data.

- A new component, the Energy Information Function, is instantiated in 5GMS System for exposing energy-related information from the network. This entity exchanges with the Data Collection AF through a new reference point called R9.

- The Data Collection AF is able to provide energy-related information in addition of existing data to the Event Consumer AF managed by the Application Service Provider.

* * * End of Changes * * * *