3GPP TSG-SA4 Meeting #0-e (AH) MBS SWG post 130	S4aI250015

# Online, , 19th Dec 2024 - 6th Feb 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.502 | CR | 0033 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [AMD-ARCH-MED] In-session Unicast Repair for MBS Object Distribution |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Germany |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | AMD-ARCH-MED |  |  |  |  |  | Date: |  |  | 2024-12-18 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In-session Unicast Repair for MBS Object Distribution: For live and low-latency live services using the Object Distribution Method in MBS, in certain cases the transmission of an object is not successful. In this case, unicast repair for individual MBS Clients can improve the service quality. However, the timing of such requests needs to be carefully studied in order to avoid network overloads or significant latencies in the delivery. A study to extend MBS User Services and object streaming to address in-session repair is of relevance. For details refer to TR 26.802. In TR 26.802, In-session unicast repair for MBS Object Distribution as introduced in clause 5.9 and based on the conclusions in clause 5.9.7 to address Gap#1 in clause 5.9.5 by the candidate solution in clause 5.9.6 has been motivated. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In-session unicast repair for MBS Object Distribution as introduced in clause 5.9 of TR 26.802: i. Define a new reference point in TS 26.502 between the MBSTF and the MBS AS. ii. Document call flows and procedures for both post-session and in-session unicast repair. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Feature not supported |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## =====  CHANGE =====

To be completed