3GPP TSG-SA4 Meeting #0-e (AH) MBS SWG post 130	S4aI250021

# Online, , 19th Dec 2024 - 6th Feb 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.501 | CR | 0104 | rev | - | Current version: | 18.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [AMD-ARCH-MED] Improved QoS support for Media Streaming services |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | AMD-ARCH-MED |  |  |  |  |  | Date: |  |  | 2025-01-03 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In FS_AMD, several QoS features which could be beneficial to the Media Delivery System have been studied, including ECN marking for L4S, PDU Set handling and QoS monitoring. As concluded and recommended, this paper intends to integrate the ECN marking for L4S and QoS monitoring features into the architectures, high-level call flows and collaboration scenarios for both 5GMSd and 5GMSu. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Integrate ECN marking for L4S into the architectures, high-level call flows and collaboration scenarios for both 5GMSd and 5GMSu. - Integrate the QoS monitoring feature into the architectures, high-level call flows and collaboration scenarios for both 5GMSd and 5GMSu. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Feature not supported |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

=====  CHANGE =====

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System architecture for the 5G System (5GS)".

[3]	3GPP TS 23.502: "Procedures for the 5G System (5GS)".

[4]	3GPP TS 23.503: "Policy and charging control framework for the 5G System (5GS); Stage 2".

[5]	Void

[6]	3GPP TS 26.307: "Presentation layer for 3GPP services".

[7]	3GPP TS 26.247: "Transparent end-to-end Packet-switched Streaming Service (PSS); Progressive Download and Dynamic Adaptive Streaming over HTTP (3GP-DASH)".

[8]	3GPP TS 26.234: "Transparent end-to-end Packet-switched Streaming Service (PSS); Protocols and codecs".

[9]	3GPP TS 23.003: "Technical Specification Group Core Network and Terminals; Numbering, addressing and identification".

[10]	3GPP TS 28.530: "Management and orchestration; Concepts, use cases and requirements".

[11]	3GPP TS 28.531: "Management and orchestration; Provisioning".

[12]	3GPP TS 28.541: "Management and orchestration; 5G Network Resource Model (NRM); Stage 2 and stage 3".

[13]	3GPP TS 23.222: "Common API Framework for 3GPP Northbound APIs".

[14]	IETF RFC 1034: "Domain names - concepts and facilities".

[15]	3GPP TS 23.548: "5G System Enhancements for Edge Computing; Stage 2".

[16]	3GPP TS 23.558: "Architecture for enabling Edge Applications".

[17]	3GPP TS 28.538: "Management and orchestration; Edge Computing Management".

[18]	3GPP TS 23.246: "Multimedia Broadcast/Multicast Service (MBMS); Architecture and functional description".

[19]	3GPP TS 26.346: "Multimedia Broadcast/Multicast Service (MBMS); Protocols and codecs".

[20]	3GPP TS 26.347: "Multimedia Broadcast/Multicast Service (MBMS); Application Programming Interface and URL".

[21]	3GPP TS 26.348: "Northbound Application Programming Interface (API) for Multimedia Broadcast/Multicast Service (MBMS) at the xMB reference point".

[22]	3GPP TS 26.531: "Data collection and reporting; General description and architecture".

[23]	3GPP TS 23.288: "Architecture enhancements for 5G System (5GS) to support network data analytics services".

[24]	3GPP TS 27.007: "AT command set for User Equipment (UE)".

[25]	CTA-5005: "Web Application Video Ecosystem – DASH-HLS Interoperability Specification".

[26]	3GPP TS 26.511: "5G Media Streaming (5GMS); Profiles, Codecs and Formats".

[27]	ISO/IEC 23000-19: "Information Technology Multimedia Application Format (MPEG-A) – Part 19: Common Media Application Format (CMAF) for segmented media".

[28]	IETF RFC 8216: "HTTP Live Streaming".

[29]	ISO/IEC 23009-1: "Information Technology – Dynamic Adaptive Streaming Over HTTP (DASH) – Part 1: Media Presentation Description and Segment Formats".

[30]	3GPP TS 26.502: "5G Multicast-Broadcast User Service Architecture".

[31]	Void.

[32]	3GPP TS 26.506: "5G Real-time Media Communication Architecture".

[33]	3GPP TS 23.222: "Common API Framework for 3GPP Northbound APIs".

[34]	3GPP TS 33.122: "Security aspects of Common API Framework (CAPIF) for 3GPP northbound APIs".

[35]	IETF RFC 6749: "The OAuth 2.0 Authorization Framework", October 2012.

[X1]	IETF RFC 9330:"Low Latency, Low Loss, Scalable Throughput (L4S) Internet Service: Architecture".

[X2]	IETF RFC 9331: "Explicit Congestion Notification (ECN) Protocol for Very Low Queuing Delay (L4S)".

[X3]	IETF RFC 9332: "Dual-Queue Coupled Active Queue Management (AQM) for Low Latency, Low Loss, and Scalable Throughput (L4S)".

[X4]	IETF RFC 3168: "The Addition of Explicit Congestion Notification (ECN) to IP".

=====  CHANGE =====

3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

5GC	5G Core Network

5GMS	5G Media Streaming

5GMSd	5G Media Streaming downlink

5GMSu	5G Media Streaming uplink

5GS	5G Systems

AF	Application Function

ABR	Adaptive Bit Rate

AMF	Access and Mobility Function

ANBR	Access Network Bit rate Recommendation

API	Application Programming Interface

App	Application

AS	Application Server

CAPIF	Common API Framework

CDN	Content Delivery Network

DASH	Dynamic and Adaptive Streaming over HTTP

DN	Data Network

DNAI	Data Network Application Identifier

DNN	Data Network Name

DRM	Digital Rights Management

ECN	Explicit Congestion Notification

EPC	Evolved Packet Core

EPS	Evolved Packet System

EUTRAN	Evolved Universal Terrestrial Radio Access Network

FLUS	Framework for Live Uplink Streaming

FQDN	Fully-Qualified Domain Name

GPU	Graphics Processing Unit

GSM	Global System for Mobile communication

HPLMN	Home Public Land Mobile Network

HTTP	HyperText Transfer Protocol

HTTPS	HyperText Transfer Protocol Secure

L4S	Low Latency, Low Loss and Scalable Throughput

LTE	Long-Term Evolution

MBMS	Multimedia Broadcast Multicast System

MNO	Mobile Network Operator

MPD	Media Presentation Description

MSISDN	Mobile Station International Subscriber Directory Number

NA	Network Assistance

NEF	Network Exposure Function

NR	New Radio

NSMF	Network Slice Management Function

NSSAI	Network Slice Selection Assistance Information

NSSP	Network Slice Selection Policy

OAM	Operations, Administration and Maintenance

OTT	Over-The-Top

PCC	Policy and Charging Control

PCF	Policy and Charging Function

PDU	Protocol Data Unit

PSS	Packet-switched Streaming Service

RAN	Radio Access Network

RTC	Real-Time media Communication

SBA	Service based Architecture

SLA	Service Level Agreement

TCP	Transmission Control Protocol

UPF	User Plane Function

URL	Unique Resource Identifier

URSP	UE Route Selection Policy

=====  CHANGE =====

4.0.6	Dynamic policies

The dynamic policies feature is applicable to both downlink media streaming and uplink media streaming. It enables the 5GMS Client in the UE to manipulate the network traffic handling policies for an ongoing media streaming session.

NOTE:	The PCF is accessed via the NEF when the 5GMS network services are deployed outside the Trusted DN.

Figure 4.0.6-1: High-level arrangement for dynamic policies

Figure 4.0.6-2: Domain model for dynamic policies

With reference to figure 4.0.6-2, dynamic policies work as follows:

1.	A conceptual Service Operation Point is an abstract set of requirements that support a media streaming service (e.g., SD, HD, UHD). It is identified by an External reference that is used to tag Policy Template resources provisioned in the 5GMS System and Service Descriptions included in Media Entry Point documents.

2.	The Service Operation Point is embodied in the 5G System by a Policy Template which is provisioned in the 5GMS network services by the 5GMS Application Provider within the scope of an umbrella Provisioning Session. A Policy Template may be defined as being applicable to a particular Data Network and/or Network Slice. The Policy Template carries the External reference and Network QoS parameters corresponding to a single Service Operation Point. (Any number of Policy Templates provisioned for different Data Networks and/or Network Slices may reference the same Service Operation Point.) The 5GMS network services may reject attempts to provision a Policy Template that specifies Network QoS parameters outside acceptable bounds imposed by local system configuration.

In addition, the Policy Template may include a reference to an existing Background Data Transfer policy. If no previously defined Background Data Transfer policy exists, the Policy Template may instead include the parameters that are used by the 5GMS network services to provision a Background Data Transfer policy for the current Provisioning Session. These parameters may include desired time windows when Background Data Transfer may be advertised to 5GMS Clients, a quota representing the maximum number of 5GMS Clients that are permitted to take advantage of Background Data Transfers in each such time window and a quota representing a ceiling for the aggregate volume of data that all 5GMS Clients are permitted to transfer in each Background Data Transfer window. Hence, an advertised time window is not a guarantee that a request for Background Data Transfer will actually be granted by the 5GMS System.

The Policy Template may include a L4S enablement flag and this Policy Template is used to enable ECN marking for L4S in the 5G system as described in clause 5.37.3 of TS 23.501 [2]. As described in RFC 9330 [X1], RFC 9331 [X2] and RFC 9332 [X3], the purpose of ECN marking for L4S (Low Latency, Low Loss and Scalable Throughput) is to inform a recipient host at the earliest opportunity that an IP packet has experienced network congestion at some point in its routing path. It exposes congestion information by marking ECN bits in the IP header of the user IP packets between the UE and the Application Server.

The Policy Template may include a QoS monitoring configuration and this Policy Template is used to enable the QoS monitoring in the 5G system as described in clause 5.45 of TS 23.501 [2] for measurements and reporting of QoS parameters. The QoS monitoring configuration shall include the parameters to be monitored, reporting frequency (event triggered, periodic), optionally target entity of reporting and optionally the notification via UPF.

3.	The 5GMS Application Provider makes one or more Media Entry Point documents (e.g., DASH MPDs) available for use by the 5GMS Client. To take advantage of the dynamic policies feature, a Media Entry Point document includes one or more Service Descriptions, each identifying the streaming requirements of a presentation that correspond to a single Service Operation Point (e.g., SD, HD, UHD) and identified by means of an External reference. The same Service Description may be included in more than one Media Entry Point document in case a common Service Operation Point is applicable to multiple media presentations.

4.	When a Media Entry Point is selected by the 5GMS Client at the start of a media streaming session, the 5GMS Client retrieves Service Access Information from a network-side component of the 5GMS System describing the set of available Policy Templates provisioned in step 2 and exposes this to a controlling application on the UE.

4a.	If Background Data Transfer was provisioned as part of any Policy Templates in step 2 above, the Service Access Information includes details of the advertised time windows when Background Data Transfers are available and the data volume quota (if any). Maximum bit rates for the 5GMS Client in either or both the uplink and downlink direction may also be nominated by the 5G System and signalled to the 5GMS Client in the Service Access Information. Finally, an endpoint in the 5GMS network services may be provided allowing the 5GMS Client to subscribe to receive real-time notifications of Background Data Transfer warning notifications.

5.	At the start of a media streaming session, the controlling application on the UE selects one of the Service Descriptions listed in the Media Entry Point document that realises its preferred Service Operation Point. Either the Media Player (when the Service Descriptions are within the Media Entry Point document) or the controlling application (when the Service Descriptions are not within the Media Entry Point document) informs the 5GMS Client of its choice by passing the corresponding External reference to it.

6.	If there is a Policy Template available for the current media streaming session with the indicated External reference, the 5GMS Client instantiates this Policy Template by interacting with a network-side component of the 5GMS System in order to realise the Service Operation Point described by the Policy Template and the Service Description. The effect of this is that the corresponding network Quality of Service is applied to the media streaming session.

7.	At any point during one of the advertised Background Data Transfer time windows the 5GMS Client may request a Background Data Transfer by instantiating a Policy Template with a Background Data Transfer specification in the 5GMS network services, including an estimate of the data volume it intends to transfer. The 5GMS network services may grant the request for the Background Data Transfer if the data volume estimate is acceptable and if the quota of requests for the time window in question has not already been exceeded. If the request is granted, the 5GMS network services apply the appropriate Background Data Transfer Quality of Service policy to the media streaming session from the Policy Template in question. The Background Data Transfer grant returned to the 5GMS Client includes an estimate of the time period for which Background Data Transfer is available for the 5GMS Client to use. After this period has expired, the 5GMS network services automatically revert the network Quality of Service back to its state before the grant.

8.	The 5GMS media services also subscribe to receive Background Data Transfer warning notifications from the PCF related to the individual Background Data Transfer policy as defined in clause 4.16.7.3 of TS 23 502 [3]. The 5GMS media services shall notify the 5GMS Client when the network performance of that particular media streaming session degrades below the Background Data Transfer policy currently in force or when the aggregate data volume for all data transfers during the current Background Data Transfer time window has been reached.

In addition, the use of dynamic policies by 5GMS Clients is logged by the 5GMS System and, if suitably provisioned, is exposed by it to subscribing 5GMS Application Providers in the form of events (see also clause 4.0.12).

=====  CHANGE =====

5.7.X	Dynamic Policy of ECN marking for L4S for downlink media streaming

Figure 5.7.X-1 below shows a high-level call flow for downlink media streaming for configuration and usage of ECN marking for L4S. The following is assumed:

-	The service here is a unicast downlink media streaming service with dynamic policy support.

-	As an example, the Layer 4 protocol used for application flows is TCP and the TCP stack used supports ECN marking for L4S.

-	The network supports ECN marking for L4S packet marking.

-	The application has specifically requested ECN marking for L4S for its media delivery session.

-	NG-RAN manipulates the ECN bits (per clause 5.37.3.2 of TS 23.501 [2]). It is equally possible that the PSA-UPF manipulates the ECN bits (per clause 5.37.3.3 of [2]).

Figure 5.7.X-1: Downlink media streaming call flow for ECN marking for L4S

The steps are as follows:

0:	Policy Template Provisioning. A Policy Template is provisioned with the requirement for L4S capability that shall be indicated by setting an L4S enablement flag.

1:	Dynamic Policy activation. The Media Session Handler within the 5GMSd Client obtains Service Access Information and triggers a dynamic policy activation. A Policy Template Binding is present within the Service Access Information for each provisioned Policy Template. Policy Template Bindings suitable for L4S are indicated by an L4S enablement flag being set. The 5GMSd Client detects that an L4S-capable media transport stack is present and in use. The selected Policy Template is configured with the L4S enablement flag.

2:	QoS request. The 5GMSd AF requests QoS handling using e.g. the Nnef_AfSessionWithQoS service or the Npcf_PolicyAuthorization service. If the L4S enablement flag is set in the selected Policy Template, this indicates that the new QoS flow is required to be L4S-enabled. The new QoS flow with the ECN marking for L4S indication setting propagates through the 5G System.

3:	The Media Sesssion Handler may inform the Media Player via the M11d interface API about the activation of L4S. Subject to availability of API access, the Media Player may use congestion notifications to perform early bit-rate adaptation.

4: 	If the L4S enablement flag is set in the Policy Template Binding for the selected Policy Template, the 5GMSd Client selects/enables the L4S capability of the used transport protocol.

NOTE:	This step may happen implicitly by selecting an L4S-supporting transport protocol stack.

5:	The Media Player within the 5GMSd Client triggers the establishment of a TCP connection. The ECT(1) codepoint is set in the IP header, indicating an L4S-Capable Transport, and the SDAP entity ensures that the packet is forwarded via the matching QoS flow.

6:	The 5GMSd AS responds to the TCP connection establishment request. The 5GMSd AS sets ECT(1) in the IP headers, indicating an L4S-Capable Transport.

7:	The UPF finds the matching QoS flow identifier for the downlink packet and sends the packet via the corresponding QoS flow to the UE. TCP Connection setup continues, with one ECT bit set in all packets.

8:	When the RAN detects an upcoming congestion according to the congestion measurement (based on continuous congestion monitoring), the 5G System sets the CE (Congestion Experienced) codepoint in the IP header of the downlink packet.

9.	The TCP protocol stack used by the Media Player in the 5GMSd Client reflects the Early Congestion Notification to the TCP sender by setting the ECN-Echo (ECE) flag in the TCP header of an uplink PDU of the same TCP connection. The TCP sender reacts to the ECN-Echo accordingly (i.e., by reducing its sending congestion window).

NOTE 1:	The ECN-Echo flag is also acknowledged by the TCP sender setting the Congestion Window Reduced (CWR) flag in an outgoing TCP frame, but this acknowledgement is not illustrated in this call flow.

NOTE 2:	Classic ECN as specified in RFC 3168 [X4] requires an ECN signal to be treated as equivalent to a packet drop. L4S as specified in RFC 9330 [X1] specifies a more fine-grained response and an early congestion signal triggers a less severe reaction. How a TCP sender behaves “accordingly” is not in scope of the specification.

10. Based on the CE indication received in step 8, or by detecting a reduced bit rate in the downlink application flow, the Media Player in the 5GMSd Client reacts by, for example, changing the requested representation.

5.7.Y	Dynamic Policy of QoS monitoring for downlink media streaming

Figure 5.23.4.2-1: High-level call flow for QoS monitoring for downlink Media Streaming

- Policy Template Provisioning. A Policy Template is provisioned and shall include the QoS monitoring configuration. The QoS monitoring configuration includes the parameters to be monitored, the reporting frequency (event triggered, periodic), and optionally the target entity of reporting. and optionally the notification via local UPF.

NOTE:	In case the 5GMSd AS is deployed as an EAS instance in the Edge DN, a local UPF can also be inserted for local access to the 5GMSd Edge AS. In order to reduce the latency used for exposure of the QoS monitoring results, the local UPF is expected to provide the notifications of network status directly to the 5GMSd AF and 5GMSd AS, or via a locally deployed NEF as defined in clause 5.8.2.17 of TS 23.501 [2].

- Service Access Information retrieval. The Media Session Handler retrieves Service Access Information from 5GMSd AF via M5d.

- Dynamic Policy activation. The Media Session Handler within the 5GMSd Client obtains Service Access Information and triggers a dynamic policy activation. A Policy Template Binding is present within the Service Access Information for each provisioned Policy Template. Policy Template Bindings suitable for QoS monitoring shall be indicated by a QoS monitoring configuration. The selected Policy Template is configured with the QoS monitoring configuration.

- QoS Monitoring request. The 5GMSd AF invokes the Npcf_PolicyAuthorization service or the Nnef_AFsessionWithQoS service with the requested QoS monitoring configurations. In the case where the 5GMSd AS is deployed in the Edge DN, the 5GMSd AF may additionally enable the exposure of QoS montoring results via the local UPF or local NEF in this step.

- The PCF accepts the request and enables QoS monitoring within the 5G System, i.e., by configuring the RAN and/or the (local) UPF for monitoring and reporting of target QoS parameters for the downlink media streaming.

6.	Following the QoS monitoring request(s), the PCF may expose the QoS monitoring results to the 5GMSd AF periocially or by event triggers.

7.	Alternatively, the QoS monitoring results may be exposed to the 5GMSd AF by the UPF directly using the Nupf_EventExposure_Notify service or via a locally deployed NEF using the Nnef_EventExposure_Notify service at reference point N33.

8.	If QoS monitoring was requested by the Media Session Handler, the 5GMSd AF shall send the notifications of the QoS monitoring results to the Media Session Handler via the MQTT notification channel at reference point M5d.

9.	The Media Session Handler shall provide the QoS monitoring results to the Media Stream Handler at reference point M11d.

10.	The Media Stream Handler may use the notified QoS monitoring results to modify its behaviour.

For example, in the case of downlink media streaming, the Media Player may use the monitored packet latency to determine when to request the next media segment, and/or to change the bit rate of the next media segment based on the monitored congestion status.

NOTE:	Whether notification of network status by the 5GMSd AF to the 5GMSd AS at reference point M3d is practical, useful and desirable is for further study. This could, for example, use the same asynchronous MQTT-based notification mechanism available at reference point M5d.

=====  CHANGE =====

6.9.X	Dynamic Policy of ECN marking for L4S for uplink media streaming

Figure 6.9.X-1 below shows a high-level call flow for downlink media streaming for configuration and usage of ECN marking for L4S. The following is assumed:

-	The service here is a unicast uplink media streaming service with dynamic policy support.

-	As an example, the Layer 4 protocol used for application flows is TCP and the TCP stack used supports L4S.

-	The 5GS network supports ECN marking for L4S.

-	The application has specifically requested ECN marking for its media delivery session.

-	NG-RAN manipulates the ECN bits (per clause 5.37.3.2 of TS 23.501 [2]). It is equally possible that the PSA-UPF manipulates the ECN bits (per clause 5.37.3.3 of [2]).

Figure 5.23.4.4-1: Uplink media streaming call flow for ECN marking for L4S

The steps are as follows:

0:	Policy Template Provisioning. A Policy Template is provisioned with the requirement for L4S capability that is indicated by setting a L4S enablement flag.

1:	Dynamic Policy activation. The Media Session Handler within the 5GMSu Client obtains Service Access Information and triggers a dynamic policy activation. A Policy Template Binding is present within the Service Access Information for each provisioned Policy Template. Policy Template Bindings suitable for L4S are indicated by an L4S enablement flag being set. The 5GMSu Client detects that an L4S-capable media transport stack is present and in use. The selected Policy Template is configured with the L4S enablement flag.

2:	QoS request. The 5GMSu AF requests QoS handling using e.g. the Nnef_AfSessionWithQoS service or the Npcf_PolicyAuthorization service. If the L4S enablement flag is set in the selected Policy Template, this indicates that the new QoS flow is required to be L4S-enabled. The new QoS flow with the ECN marking for L4S indication setting propagates through the 5G System.

3:	The Media Session Handler may inform the Media Streamer via the M11u interface API about the activation of L4S. Subject to availability of API access, the Media Player may use congestion notifications to perform early adaptation.

4:	If the L4S enablement flag is set in the Policy Template Binding for the selected Policy Template, the 5GMSu Client selects/enables the L4S capability of the used transport protocol.

NOTE:	This step may happen implicitly by selecting an L4S-supporting transport protocol stack.

5:	The Media Streamer within the 5GMSu Client triggers the establishment of a TCP connection. The ECT(1) codepoint is set in the IP header, indicating an L4S-Capable Transport, and the SDAP entity ensures that the packet is forwarded via the matching QoS flow.

6:	The 5GMSu AS responds to the TCP connection establishment request. The 5GMSu AS sets ECT(1) in the IP headers, indicating an L4S-Capable Transport.

7:	The UPF finds the matching QoS flow identifier for the downlink packet and sends the packet via the corresponding QoS flow to the UE. TCP Connection setup continues, with one ECT bit set in all packets.

8:	When the RAN detects an upcoming congestion according to the congestion measurement (based on continuous congestion monitoring), the 5G System the CE (Congestion Experienced) codepoint in the IP header of the uplink packet.

9:	The TCP protocol stack used by the 5GMSu AS reflects the Early Congestion Notification to the TCP sender by setting the ECN-Echo (ECE) flag in the TCP header of a downlink PDU of the same TCP connection. The TCP sender reacts to the ECN-Echo accordingly (i.e., by reducing its sending congestion window).

NOTE 1:	The ECN-Echo flag is also acknowledged by the TCP sender setting the Congestion Window Reduced (CWR) flag in an outgoing TCP frame, but this acknowledgement is not illustrated in this call flow.

NOTE 2:	Classic ECN as specified in RFC 3168 [X4] requires an ECN signal to be treated as equivalent to a packet drop. L4S as specified in RFC 9330 [X1] specifies a more fine-grained response and an early congestion signal triggers a less severe reaction. How a TCP sender behaves “accordingly” is not in scope of the specification.

10:	Based on the CE indication received in step 8, or by detecting a reduced bit rate in the uplink application flow, the Media Streamer in the 5GMSu Client may react by, for example, changing the bit rate of the representation.

6.9.Y	Dynamic Policy of QoS monitoring for uplink media streaming

Figure 5.23.4.2-1: High-level call flow for QoS monitoring for uplink Media Streaming

- Policy Template Provisioning. A Policy Template is provisioned and shall include the QoS monitoring configuration. The QoS monitoring configuration includes the parameters to be monitored, the reporting frequency (event triggered, periodic), and optionally the target entity of reporting and optionally the notification via UPF.

NOTE:	In case the 5GMSu AS is deployed as an EAS instance in the Edge DN, a local UPF can also be inserted for local access to the 5GMSu Edge AS. In order to reduce the latency used for exposure of the QoS monitoring results, the local UPF is expected to provide the notifications of network status directly to the 5GMSu AF and 5GMSu AS, or via a locally deployed NEF as defined in clause 5.8.2.17 of TS 23.501 [2].

- Service Access Information retrieval. The Media Session Handler retrieves Service Access Information from 5GMSu AF via M5u.

- Dynamic Policy activation. The Media Session Handler within the 5GMSu Client obtains Service Access Information and triggers a dynamic policy activation. A Policy Template Binding is present within the Service Access Information for each provisioned Policy Template. Policy Template Bindings suitable for QoS monitoring are indicated by a QoS monitoring configuration included. The selected Policy Template is one configured with the QoS monitoring configuration.

- QoS Monitoring request. The 5GMSu AF invokes the Npcf_PolicyAuthorization service or the Nnef_AFsessionWithQoS service with the requested QoS monitoring configurations. In the case where the 5GMSu AS is deployed in the Edge DN, the 5GMSu AF may additionally enable the exposure of QoS montoring results via the local UPF or local NEF in this step.

- The PCF accepts the request and enables QoS monitoring within the 5G System, i.e., by configuring the RAN and/or the (local) UPF for monitoring and reporting of target QoS parameters for the uplink media streaming.

6.	Following the QoS monitoring request(s), the PCF may expose the QoS monitoring results to the 5GMSu AF periocially or by event triggers.

7.	Alternatively, the QoS monitoring results can be exposed to the 5GMSu AF by the UPF directly using the Nupf_EventExposure_Notify service or via a locally deployed NEF using the Nnef_EventExposure_Notify service at reference point N33.

8.	If QoS monitoring was requested by the Media Session Handler, the 5GMSu AF sends the notifications of the QoS monitoring results to the Media Session Handler via the MQTT notification channel at reference point M5u.

9.	The Media Session Handler shall provide QoS monitoring results to the Media Stream Handler at reference point M11u.

10.	The Media Stream Handler may use the notified QoS monitoring results to modify its behaviour.

For example, in the case of uplink media streaming, the Media Player may use the monitored packet latency, congestion status, etc. to determine the bit rate of the uplink streaming.

NOTE:	Whether notification of network status by the 5GMSu AF to the 5GMSu AS at reference point M3u is practical, useful and desirable is for further study. This could, for example, use the same asynchronous MQTT-based notification mechanism available at reference point M5u.