SA4-e (AH) RTC SWG post 129-e	S4aR240052

25th Sept 2024, online

Source:	InterDigital Europe

Title:	Haptics media definition and pipeline

Spec:	3GPP TR 26.854

Document for:	Agreement

1. Introduction

This contribution proposes to define haptics as a media in the context of 3GPP SA4, and provide an overview of the haptics pipeline, creation and rendering.

2. Reason for Change

Provide initial text for Haptics definition, pipeline and background to populate section 4 for TR 26.854.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 26.854

* * * First Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TR22.847: “Study on supporting tactile and multi-modality communication services”

[3]	3GPP TR26.813: “Study of Avatars in Real-Time Communication Services”

[4]	3GPP TR22.856: “Study on Localized Mobile Metaverse Services”

[HFX]	Haptic editor for full-body immersive experiences, In Proceedings of the 24th ACM Symposium on Virtual Reality Software and Technology 2018

[OPENXR]	xxxx

* * * Second Change * * * * All new.

# 4	Background

## 4.1	Introduction

… Editor’s note: general intro to haptics media, characteristics, work done in SA1…

## 4.A Definition of Haptics Media

Haptics relates to the sense of touch. As a media, Haptics represents information describing physical feedback rendered for a specific user device and body location. This information is defined as a new media type and is described by a time-based signal (a haptic effect) or a spatial signal (a physical property of an object). The rendering is triggered by a timing information or an interaction. Different Haptic modalities are considered, targeting different human mechanoreceptors (tactile, kinesthetic, proprioception) and thermoreceptors. [ISO-90-31]

## 4.B Haptics media pipeline

Haptics can be used as a media type at the same level as audio and video. Figure 1 depicts an end-to-end pipeline associating audio, video and haptics in the most complete scenario of interactive 3D scenes. A simpler version of the pipeline consists in 2D AVH (Audio-Video-Haptic media) with only 2D assets and no interaction.

Figure 1- Audio-visual Haptic end-to-end pipeline.

This figure also illustrates the different formats and some existing APIs supporting haptics media. .

## 4.B.1 Haptics media creation

For the creation of haptics effects and their association with A/V content, several tools have been developed and can be used. In Figure 2 an example of such tool, the HFX studio [HFX] is illustrated. It shows how to create timeline for haptics, with several channels and different haptic effects. It also shows how to design effects for particular body parts on the user.

The main principle of authoring tools is to create effects for the user and the targeted experience, and let the application manage this experience regarding the available rendering devices. For instance, in this picture a haptic effect is generated on the user torso assuming several actuators. If the receiving application does not support a haptic suit as a rendering device but may just be run on a smartphone with a single actuator, the application will select only the first vibrotactile signal from the distributed file or stream. On the opposite more complex receivers might use the full file or stream with more complex setups.

Figure 2- Haptic studio editing tool.

The HFX studio supports the HJIF interchanged format (or mezzanine format) defined in section 6 of [ISO90-31] for the creation, editing and distribution of haptic effects.

Other commercial tools exist, and they are generally dedicated to their associated proprietary format or platform:

- Designer | bHaptics Developer

- Meta Haptics Studio on Meta Quest | Quest VR games | Meta Store

- Haptic Composer - Design, Test, & Play Haptics (interhaptics.com)

## 4.B.2 Haptic media rendering

Several sdks have been provided to integrate haptics into standalone applications or for developing dedicated applications such as Unity and Unreal. Often provided with their proprietary format such as the meta sdk for unity or unreal, bHaptics or Apple Core Haptics.

- Haptics SDK for Unity

- Haptics SDK for Unreal

- SDK | bHaptics Developer

- Core Haptics | Apple Developer Documentation

The Interhaptics platform (Interhaptics Platform - Partners) is providing both authoring tools and Software Development Kit (SDK) for software providers and OEMs manufacturers.

APIs with the devices are often based on OpenXR [OPENXR]. However the current haptic support in OpenXR is limited, but some work is on going to extend capabilities.

## 4.C Example of Haptics media devices

A large number of haptic devices exist. The simpler devices are the smartphone and wearables integrating vibrotactile devices, usually one. Chipsets support mono or stereo haptics, for instance the Snapdragon G3X gen2. When several wearables are supported by an application some form of spatialization is then possible.

An example of a stereo haptic device is provided with the Razer headset with one vibrotactile actuator per ear.

Gaming Headset Powered by Intelligent Haptics with Razer HyperSense 🎧 | Razer United States

More complex devices including a set of actuators allow for spatialization of haptic effects on the user body, such as the Razer Esther cushion for gaming seats with 16 actuators on the back and seat.

Project Esther: World's First HD Haptics Gaming Seat Cushion | Razer United States

VR and game controllers always integrated haptic feedback with vibrotactile feedback, but recent controllers also provide kinaesthetic feedback such as the SONY DualSense.

DualSense wireless controller | The innovative new controller for PS5 | PlayStation

Other haptic modalities are also available like thermal and friction with the Weart haptic glove or simple fan like the Dyson heat fan.

WEART haptic solutions | WEART

Electrotactile feedback has been integrated into special suits such as the Teslasuit and gloves using electro muscle stimulations.

Teslasuit | Meet our Haptic VR Suit and Glove with Force Feedback

Editor’s note: add figure pending check on copyright.

* * * End of Changes * * * *