3GPP SA4 RTC Telco Post #129-e	S4aR240057

Online, 25 September 2024

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PSEUDO CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.822 | CR | pseudo | rev | - | Current version: | 1.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [FS_5G_RTP_Ph2] AL-FEC awareness at RAN for the Downlink and the Uplink |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_5G_RTP_Ph2 |  |  |  |  |  | Date: |  |  | 09/25/2024 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | This paper addresses Note 3 for Solution #10 to Key Issue #4: AL-FEC awareness for PDU Set handling A difficulty in resolving Note 3 is packet loss estiation for the gNB to AS segment (5G core network and N6 segment). |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added a practical method on packet loss estimation for the gNB to AS segment (5G core network and N6 segment). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Note 3 is not resolved, and only stays as a concept. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * 1st change * * * *

## 6.10	Solution #10: AL-FEC awareness at RAN while considering upstream and downstream packet losses for the downlink and the uplink

### 6.10.1	Key Issue mapping

This maps to Key Issue #4.

### 6.10.2	Description

### 6.10.2.1	Motivation

As explained in clause 5.4.2.2, there is an upstream network segment before the RAN and there may be a downstream network segment after the RAN (e.g., a tethered link). For DL data originated from the AS, when the RAN drops extra PDUs of a PDU Set encoded with AL-FEC, it needs to consider how many of the PDUs have already been lost in the upstream network segment. The RAN also needs to consider how many packets may get lost in the downstream network segment. Otherwise, the application receiver may not receive enough PDUs for reconstructing all the source PDUs.

NOTE 1:	Although the solution below is on how the RAN drops extra PDUs, it applies to a generic network device if the network device receives the downstream packet loss probability.

### 6.10.2.2	Solution

Figure 6.10.2.2-1: The RAN drops extra PDUs by considering packet losses in the upstream network segment and downstream network segment.

The RAN derives the PDUs that have been lost in the upstream network segment, e.g., by taking the difference between the number of PDUs in the PDU set (NPDS) in the RTP header extension for PDU Set marking for a PDU Set and the number of PDUs of the PDU Set that it has received.

The downlink network segment informs the RAN of the packet loss rate over the tethered link. This is shown in Figure 6.10.2.2-1.

NOTE 2:	Although in Figure 6.10.2.2-1 it is shown that the UE provides the packet loss feedback, the XR Tethered Endpoint can also provide the packet loss feedback.

The RAN then decides on how many PDUs are extra PDUs given (1) the upstream packet losses, (2) the packet losses at the RAN (not dropped as extra PDUs), and (3) the downstream packet loss rate.

The following example shows how the RAN decides on the number of extra PDUs that it can drop while still ensuring that the receiver can reconstruct all the source packets with a high probability. Denote the following:

(1)	the upstream packet losses: N1

(2)	the packet losses at the RAN (not dropped as extra PDUs): N2

(3)	the downstream packet loss rate: p

It is assumed that the packet losses in the downsteam are independent. Consider a PDU Set with be K=30 source PDUs, and 15 repair PDUs, hence a total of N=45 PDUs. Let the packet losses at the RAN due to wireless link error (not dropped as extra PDUs) be N2. Let the AL-FEC code be a MDS code. For simplicity each PDU is assumed to be sent in a separate transport block over the RAN air interface. Let N1=2, p=2%. Let the minimum reconstruction success probability be 0.995. Then the allowed values for the number of extra PDUs N3 that the RAN can drop must satisfy the following inequality:

n=Km-N3m-N3n(1-p)n pm-N3-n> 0.995, where m=N-N1-N2.

In the above, the summand gives the probability that n packets out of m-N3 packets are a success and the remaining m-N3-n packets are a failure. The probabilities are summed across all values of n which allow the application receiver to reconstruct all the source packets.

Figure 6.10.2.2-2: The RAN can drop up to N3=6 extra PDUs while still meeting the reconstruction probability threshold 0.995 with the packet losses in the upsteam, at the RAN (due to wireless link error, not dropped as extra PDUs), and in the downstream.

NOTE 3:	Although the solution presented above is for DL traffic, a similar solution can be devised for UL traffic, in which case the tethered link becomes upstream and the gNB to the AS segment (5G core network and N6 segment) becomes downstream.

For the uplink traffic, the UE needs to determine the number of PDUs that it needs to successfully deliver over the air between the UE and the gNB. The number depends on the packet loss rate for the gNB to the AS segment. It is difficult to directly measure the packet loss rate. One possible method is to send probing packets (e.g., ICMP ping messages), but the AS may not recognize the IP address of the gNB and may reject the probing packet for security reasons. Even if transmitting probing packets is enabled, the probing packets may be treated differently in the network from the media packets, resulting inaccurate estimation of the packet loss rate. As an alternative, the packet loss rate may be estimated indirectly: the gNB provides feedback on the uplink packet reception, the AS provides feedback on the end-to-end packet reception, and the UE estimates the packet loss rate for the gNB to the AS segment, denoted as pr2a, which is the ratio of the number of packets lost as observed by the AS but not observed as lost by the gNB to the number of packets successfully received by the gNB. This is illustraged with an example in Figure 6.10.2.2-3.

Figure 6.10.2.2-3:  Indirect method for calculating the packet loss rate for the gNB to AS segment, where an X sign means the corresponding packet is lost.

Once the packet loss rate for the gNB to the AS segment is estimated, a similar formulation can be used to derive the number of PDUs that the UE can drop to meet a minimum reconstruction success probability at the AS.

The above assumes that the extra PDUs dropped at the RAN are not taken as signals of network congestion. Network congestion can occur in the upstream network segment, at the RAN or in the downstream network segment.

NOTE 4:	It is FFS under what conditions the RAN can drop extra PDUs considering network congestion in the upstream network segment, at the RAN or in the downstream network segment.

# * * * * end of 1st change * * * *