SA4-e (AH) RTC SWG post 129-e	S4aR240059

# 25th September 2024, Online

Source:	Nokia

Title:	[FS_5G_RTP_Ph2] Updates to the gap analysis on lone PDUs

Spec:	TR 26.822

Agenda item:	4.5

Document for:	Agreement

1. Introduction

TR 26.822 currently includes a gap analysis on the QoS requirements for lone PDUs in clause 6.2. During the previous meeting, a solution for PSI signaling for lone PDUs was incorporated, which includes a note on the potential impacts of differentiating RTCP and RTP into separate QoS flows. However, this note would be more appropriately placed within the gap analysis section.

2. Reason for Change

Move the considerations on multiplexing from the solution #15 to the gap analysis in solution #2.

Edits in the gap analysis to improve the text.

3. Proposal

It is proposed to agree the following changes to TR 26.822.

* * * First Change * * * *

## 6.2	Solution #2: Gap analysis on the QoS requirements for lone PDUs

### 6.2.1	Key Issue mapping

This solution intends to give gap analysis on the KI#2: QoS handling requirements for lone PDUs.

### 6.2.2	Description

According to TS 23.501 [3], in case a single PDU doesn't belong to a PDU Set based on the Protocol Description for PDU Set identification, the UPF still maps it to a PDU Set and determines the PDU Set Information accordingly. In this case, both the lone PDU and the PDUs belonging to a PDU Set are in the same service data flow and the lone PDU is delivered to the UE in the DL direction following the PDU Set QoS parameters.

There could be different scenarios where the application server may send the PDU Sets and lone PDUs in the same service data flow which can be detected by the 5GS. For video data, as described in Annex A.2.2.1 of TS 26.522 [2], it is generally recommended that the network function considers non-VCL NAL units (e.g. SPS NAL unit) as part of the PDU Set of the associated VCL NALUs, e.g. identified by the same timestamp.. However, there are other scenarios where lone PDUs and PDUs belonging to a PDU Set are multiplexed in a single service data flow as following.

-	Scenario #A: RTP streams multiplexed in a single RTP session. In this scenario, multiple RTP streams are multiplexed in a single RTP session which is carried over a single service data flow. For example, the audio and video streams are multiplexed in a single RTP session, while the PDU Set handling is needed only for the video streams. Similarly, when FEC or RTP retransmission feature is enabled, the corresponding repair packets or retransmission packets may also be multiplexed with the original video stream. As of Rel-18, the 5GS cannot distinguish different RTP streams multiplexed in a single service data flow and has to treat the PDUs in other RTP streams as lone PDUs.

-	Scenario #B: RTP data and control packets are multiplexed on a single port. In this scenario, the RTP and RTCP flows are carried over a single service data flow. When the PDU Set handling is needed for the RTP flow(s), the 5GS has to treat the RTCP traffic as lone PDUs since it cannot distinguish between the RTP and RTCP traffic.

As can be seen from the above, one key reason for the lone PDU handling is that the PDUs belonging to a PDU Set and the lone PDUs are carried over a single service data flow and as of Rel-18, the 5GS cannot differentiate the multiplexed data flows in a single service data flow.

Therefore, it is clear that

-	Coexistence of lone PDUs and PDUs belonging to a PDU Set in a single service data flow can be due to the lack of the capability to differentiate multiplexed media flows in 5GS.

Editor’s Note:	Other scenarios for the co-existence of lonely PDUs and PDU Set is FFS.

However, the scenario where lone PDUs may exist, is still possible due to the multiplexed RTP and RTCP or RTP audio and video traffic flows are in a single QoS Flow as requested by the application layer, e.g., the QoS requirements for them are the same.

And the QoS requirements for multiplexed media streams could be different. For example, the QoS requirements for audio and video streams could be different.

For PDU Set based QoS handling, the PDU Set QoS parameters are introduced in TS 23.501 [3] as following:

-	PDU Set Delay Budget, which defines an upper bound for the delay that a PDU Set may experience for the transfer between the UE and the N6 termination point at the UPF.

-	PDU Set Error Rate, which defines an upper bound for the rate of PDU Sets that have been processed by the sender of a link layer protocol (e.g., RLC in RAN of a 3GPP access) but that are not successfully delivered by the corresponding receiver to the upper layer (e.g., PDCP in RAN of a 3GPP access).

-	PDU Set Integrated Handling Information, which indicates whether all PDUs of the PDU Set are needed for the usage of the PDU Set by the application layer in the receiver side.

If the NG-RAN receives PDU Set QoS Parameters, it enables the PDU Set based QoS handling and applies PDU Set QoS Parameters. When the PDU Set QoS parameters are available, they will supersede the PDU QoS parameters (i.e. PSDB/PSER supersedes the PDB/PER).

For the corresponding PDU QoS parameters, they are at a per packet granularity including the per-packet latency requirement (i.e. packet delay budget), the per-packet loss rate requirement (i.e. packet loss rate), etc. From the application perspective, the PDU Set QoS parameters and the PDU QoS parameters should reflect the same network requirements while at different granularities. Therefore, QoS requirements for multiplexed media streams could be different and applying the PDU Set QoS parameters to a single PDU could be an issue.

The solution to KI#4 in TR 23.700-70 [6] enables the network to differentiate multiplexed streams sent in the same media transport such that they can be mapped into distinct QoS flows. However, in some cases this may result in unintended behavior, e.g. RTCP packets mapped to a different QoS flow would no longer measure the RTP media QoS flow characteristics which may result in errors e.g. in measuring the roundtrip delay using RTCP sender/receiver reports. On the other hand, it could be problematic to apply the PDU Set QoS to lone PDUs, as described above.

Editor’s Note: How to balance this trade-off is FFS.

In addition, as discussed in draft TR 23.700-70 [6], how to support the traffic detection and QoS mapping for multiplexed data flows is ongoing in SA2 Rel-19 FS_XRM_Ph2 as shown below:

This key issue proposes study traffic detection and QoS Flow mapping in 5GS for different media streams multiplexed within a single end-to-end transport connection.

-	How to identify multiplexed traffic flows with different QoS requirements within a single transport connection.

-	How to do QoS Flow mapping for traffic flows with different QoS requirements.

-	Whether and what information needs to be provided from AF for traffic detection.

-	Whether and how AF provides QoS requirements of different traffic flows to the 5GS.

Via the potential R19 enhancements in 5GS, it is possible to differentiate the multiplexed RTP streams or RTP/RTCP flows, which may avoid the co-existence of lone PDUs and PDUs belonging to a PDU Set.

### 6.2.3	Conclusion

Based on the gap analysis in the above, it is proposed to make the following conclusions.

-	QoS requirements for multiplexed media streams could be different and applying the PDU Set QoS parameters to a single PDU could be an issue.

-	Coexistence of lone PDUs and PDUs belonging to a PDU Set in a single service data flow may be due to the lack of the capability to differentiate multiplexed media flows in 5GS.

Editor’s Note: 	Whether multiplexing is the only reason for lonely PDUs and whether the handling of multiplexed data flows in R19 SA2 FS_XRM_Ph2 can avoid this issue are FFS.

* * * Second Change * * * *

## 6.15	Solution #15: PSI signaling for lone PDUs

### 6.15.1	Key Issue mapping

This solution addresses the key issue #2.

### 6.15.2	Description

#### 6.15.2.1	Background

As of Rel-18, there is no mechanism to mark PDUs carrying protocol data other than RTP. Thus, PDUs belonging to protocols such as RTCP, STUN, etc. cannot be marked.

In Rel-18, SA2 has agreed that the PSA UPF marks, in the downlink, each N6-unmarked PDU ("lone PDU") with PDU Set Information into a PDU Set. If the UPF receives a PDU that does not belong to a PDU Set based on Protocol Description for PDU Set identification, the UPF still maps it to a PDU Set and determines the PDU Set Information by implementation-specific means.

Observation 1: PDUs of non-RTP protocols (e.g. RTCP) are mapped by the UPF into PDU Sets. The associated PDU Set Information is determined by the UPF.

When the PDU Set Information is not provided by the sender in an RTP HE, the UPF may be able to reliably obtain some parts of the PDU Set Information based on the UPF implementation. Annex A of TS 26.522 [2] describes how a Network Function can obtain the PDU Set Information from the RTP header and RTP payload, respectively.

When the RTP HE for PDU Set marking is not available, the UPF may derive some parts of the PDU Set Information from the (S)RTP header, as described in clause A.2.1 of TS 26.522. However, PDU Set Importance (PSI) cannot be obtained and PDU Set Size (PSSize) can only be obtained after the reception of the last PDU of the PDU Set. Also, the derivation in clause A.2.1 only applies for the case when a PDU Set is a video frame. It is not applicable to e.g. audio PDU Sets or PDU Sets that are video slices.

When the RTP payload is not encrypted (i.e., SRTP is not used), the UPF may derive some parts of the PDU Set Information from the RTP payload. Clause A.2.2 of TS 26.522 describes how this can be done for RTP payloads carrying H.264/AVC and H.265/HEVC coded bitstreams. In summary, the UPF needs to parse the NAL unit header, which is the first one and two byte(s) of the RTP payload for H.264/AVC and H.265/HEVC, respectively. Similar to the derivation from the RTP header, the PSSize can only be obtained after the reception of all PDUs of the PDU Set. PSI can be obtained by using the same logic the RTP sender uses to populate the RTP HE for PDU Set marking (i.e., parsing the NAL unit header), as described in the relevant guidelines in clause 4.2.6.2 of TS 26.522. However, this means more operations and thus more processing load for the UPF since it would need to check one or more of the NAL unit header fields. This is not feasible considering that the UPF processes data from several endpoints simultaneously under tight latency constraints.

Observation 2: For RTP PDUs, if the RTP HE for PDU Set marking is not present, the UPF may derive some parts of the PDU Set Information from the RTP header or RTP payload (if unencrypted), albeit with some restrictions. In particular, PSI cannot be obtained from the RTP header, and deriving PSI from the RTP payload imposes a significant processing overhead for the UPF given its high processing load and tight latency constraints.

Signaling the PDU Set Information in an RTP HE or deriving it from the RTP header/payload is not possible for PDUs carrying protocol data other than RTP (e.g. RTCP). Such PDUs are considered as lone PDUs by the UPF and placed into a PDU Set in a way that is determined by the UPF implementation. The UPF also has to define the PDU Set Information for the PDU Sets containing the lone PDUs. For some parts of the PDU Set Information (PSN, PSSN, End PDU), this operation is straightforward.

For example, consider an RTCP PDU that is placed by the UPF into its own PDU Set (i.e., the PDU Set contains only that PDU). It is described below whether/how the PDU Set Information can be obtained for that PDU Set.

-	Number of PDUs in the PDU Set (NPDS) is set to 1.

-	PSN is set to 0 and the End PDU flag is set to 1 (since NPDS=1).

-	PSSN is trickier since it depends on the transmission order of the PDU Set by the sender and is monotonically increased by the sender by 1 for each subsequent PDU Set. Since there is no gap between the PSSNs assigned by the sender for the RTP PDUs, the UPF would have to either assign an existing PSSN (i.e., a PSSN that is already used by another PDU Set) to the new PDU Set that contains the lone PDU, or a predetermined value that indicates that PSSN is undefined for that PDU Set.

-	PSSize is equal to the size of the RTCP packet since that packet would be the only PDU in the PDU Set.

-	To determine the PSI, the UPF must resort to preconfiguration (i.e. use a pre-defined value) since it has no means to obtain the PSI from the packet header or payload for non-RTP PDUs.

Any default PSI setting at the UPF may not be accurate since the importance of different PDU Sets is application- and codec-dependent. For example, some RTCP message types may be considered more important for low latency applications. In another example, RTCP feedback messages for viewport signaling may be crucial for the functionality of an immersive application, and thus it would be beneficial to be able to indicate a low PSI value for the RTCP packets carrying those.

Editor’s note:	Other potential benefits and limitations of PDU Set handling for non-RTP protocol types (e.g. RTCP, STUN) is FFS.

Observation 3: For lone PDUs, some parts of the PDU Set Information must be determined by the UPF. However, the UPF cannot reliably determine the PSI and may only assign a pre-defined PSI value (e.g. by the network operator). Sender applications are in the best position to determine the PSI.

Editor’s note:	Whether it is questionable that the UPF can reliably determine PDU Set Information for lone PDUs based on local configuration needs to be verified with SA2.