3GPP TSG-SA4 RTC SWG Ad hoc telco	S4aR240078

# 9th October 2024                                                                           revision of S4aR240061

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PSEUDO CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.567 | CR |  | rev |  | Current version: | 0.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [SR_IMS] Metrics definition and formats |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, InterDigital Communications |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | SR_IMS |  |  |  |  |  | Date: |  |  | 2024-10-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The definition and formats of split rendering metrics is missing in the TS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add sub clauses titles in clause 6 Add the metrics definition and formats in clause 6.1 Add metrics configuration in clause 6.2 Add metrics reporting format in clause 6.3 Add reference in clause 2 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete Technical Specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Clause 6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st Change

# 6	Split Rendering Metrics

Editor’s note: this clause defines a set of metrics that are relevant to the operation of a split rendering session.

## 6.1 Metrics definition and formats

SR-DCMTSI client and Media Function supporting the QoE metrics feature shall support the collection and reporting of the metrics in clause 16.2 of TS 26.114 [7], which include corruption duration metric, successive loss of RTP packets, frame rate, jitter duration, sync loss duration, round-trip time, average codec bitrate, codec information, call setup time.

SR-DCMTSI client and Media Function supporting the QoE metrics feature shall also support the collection and reporting of the split-rendering latency metrics defined in clause 9.3.4 of TS 26.565 [8] according to the QoE metrics configuration, which include pose-to-render-to-photon, render-to-photon, round-trip interaction delay, user interaction delay, age of content, scene update delay, metadata delay, and data frame delay metrics. A Media Function may use the “QoE timing information” defined in clause 9.3.2 of TS 26.565 [8] to transmit the timing information required for measuring the QoE latency metrics to an SR-DCMTSI client.

The metrics listed above are valid for one or more media types such as speech/audio, video, and text, and are calculated for each measurement resolution interval "Measure-Resolution". They are reported to the QoE server according to the measurement reporting interval "Sending-Rate" and after the end of the session.

## 6.2 Metrics Configuration

SR-DCMTSI client and Media Function supporting the QoE metrics feature shall support the usage of an OMA-DM solution for configuration of QoE metrics and their activation. As an alternative, metrics configuration can be specified by the QoE Measurement Collection (QMC) functionality.

## 6.3 Metrics Reporting

## 6.3.1 General

The metrics reporting procedure specified in clause 16.4 of TS 26.114 [7] allows the SR-DCMTSI client to send QoE metrics reports to the QoE server.

An SR-DCMTSI Client shall report QoE metrics specified in clause 6.2 for the real-time media it has received using the protocol specified in clause 16.4 of TS 26.114 [7] according to the QoE metrics reporting configuration obtained in a 3GPP MTSIQOE (MTSI QoE metrics) management object (see clause 16.3.1 of TS 26.114 [7]) or in an RRC message (see clause 16.5.1 of TS 26.114 [7]).

The quality metrics report follows the XML-based report format defined in clause 6.3.2.

SR-DCMTSI Clients shall use the MIME type "application/3gprtc-qoe-report+xml" for an XML-formatted QoE report. The metrics report format is defined in clause 6.3.2.

## 6.3.2 Report format

Editor’s note: this clause defines the metrics report formatted as an XML document.

_____________________ End of 1st Change________________________________

2nd Change

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[7]	3GPP TS 26.114: "IP Multimedia Subsystem (IMS); Multimedia Telephony; Media handling and interaction".

[8]	3GPP TS 26.565: "Split Rendering Media Service Enabler".

_____________________ End of 2nd Change________________________________