3GPP SA4-e (AH) RTC SWG post 129-e	S4aR240088

# Online 23rd of October 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Pseudo CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.822 | CR | - | rev | - | Current version: | 1.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | FS_5G_RTP_Ph2 Solution KI#2 lone/unmarked PDU without SSRC packet filter in 5G System |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA WG 4 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_5G_RTP_Ph2 |  |  |  |  |  | Date: |  |  | 11-10-2024 |
| Category: | B |  |  |  |  |  | Release: |  |  | 19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Solution for guidelines for PDU Set marking in case of lone/unmarked PDU #2 and Multiplexing #9 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Solution with guidelines for handling lone pdu and multiplexing case for RTP HE marking The solution for #9 builds |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | KI not addressed |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4, 4.1, 4.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# References

........

[ZZ]			Real-Time Transport Protocol (RTP) Parameters (iana.org)

[ZZa]		3GPP TS 29.571 3rd Generation partnership project; technical specification group core network and terminals; 5G System; common data types for Service based interfaces; stage 3

[ZZb]		3GPP TS 38.415 NG-RAN; PDU Session User Plane Protocol(Release 18)

CHANGE 1

## 6.X	Solution #X: Guidelines for PDU Set Marking in Multiplexing Scenarios

### 6.X.1	Key Issue mapping

Solution to key issue #2.

### 6.X.2	Description

#### 6.X.2.1	General

The RTP Header Extension for PDU Set marking aims to support enabling PDU Set QoS for RTP in the 5G System as defined in [3].

It enables components in the 5G System to identify PDU Sets and apply PDU Set QoS, i.e. see 5.37.5 in [3].

This clause proposes a guidelines for applying and using the RTP Header Extension for PDU Set Marking when lone/unmarked PDU’s are present to enable effective PDU Set based QoS handling.

NOTE:      This solution covers the case where 5G System cannot apply packet filter based on RTP SSRC in a QoS 				 flow.

For PDU Set based QoS handling, the PDU Set QoS parameters are introduced in TS 23.501 [3] as follows:

-	PDU Set Delay Budget, which defines an upper bound for the delay that a PDU Set may experience for the transfer between the UE and the N6 termination point at the UPF.

-	PDU Set Error Rate, which defines an upper bound for the rate of PDU Sets that have been processed by the sender of a link layer protocol (e.g., RLC in RAN of a 3GPP access) but that are not successfully delivered by the corresponding receiver to the upper layer (e.g., PDCP in RAN of a 3GPP access).

-	PDU Set Integrated Information, which indicates whether all PDUs of the PDU Set are needed for the usage of the PDU Set by the application layer in the receiver side.

If the NG-RAN receives PDU Set QoS Parameters, it enables the PDU Set based QoS handling and applies PDU Set QoS Parameters. When the PDU Set QoS parameters are available, they will supersede the PDU QoS parameters (i.e. PSDB/PSER supersedes the PDB/PER).

Additional guidelines to support this case for the following scenarios is considered:

- The case of unmarked Packets in a stream to which RTP Header Extensions are applied including marking for data bursts. A guideline is presented in clause 6.X.2.2 addressing KI #2.

#### 6.X.2.2	Unmarked packets PDU Set handling Guideline.

It is recommended that when the RTP HE for PDU set marking is enabled, the RTP HE is applied to each RTP packet that belongs to a PDU Set.  This enables effective identification of all packets belonging to a PDU Set by the 5G System. This can subsequently enable suitable PDU Set QoS based Handling for each PDU Set in the NG-RAN.

In some cases, packets may exist that do not belong a PDU Set, but are instead a single independent packet intended for transmission.

For example the case when a single frame is carried in a single packet that is not marked as a PDU Set. An example could be an audio RTP packet combined with a video RTP Stream to which RTP HE are applied that benefits from PDU Set QoS based transmission.  As audio frames may consist of just a single packet, adding signalling to indicate groupings of these packets introduces unnecessary overhead and complexity, but RTP HE was used for the video stream packets.

Another example could be a control packet such as RTCP transmitted over the same QoS flow / IP Tuple. While original RTP RFC recommends transmitting these packets on a different IP tuple, in practice these packets are often transferred on the same flow using control see [5].

In the 5G System, such an unmarked PDU will be treated as a separate PDU Set and PDU Set QoS parameters are applied instead of PDU QoS Parameters to a single packet.

NOTE: 	Due to this behaviour of the 5G System PDU Set QoS parameters such as delay budget, when configured, have to be suitable for the unmarked data as well as the marked data.

A practical example could be an audio and a video stream on a QoS flow IP tuple following [4]. In this case the video stream RTP packets include RTP Header Extension for PDU Set marking for each RTP Packet but on the other hand the audio stream RTP Packets do not contain the RTP Header Extension. The PDU Set delay budget is set to 20 milliseconds (as example) in order to support the real-time video frame transmission. This delay budget is as a consequence used in the 5G System for video PDU sets but also for the independent audio packets that are treated as a single PDU Set in the 5G System. In this practical case, a delay budget of 20 milliseconds is also suitable for the audio stream as this delay leads to a satisfactory end-to-end performance.

Other PDU Set information such as PDU set importance may use a default value may be applied by the 5G System in case no RTP Header Extension is present.

The unmarked packets do not count toward the PDU set size of marked PDU Sets as optionally signalled in the RTP Header extension. In the case of an unmarked packet, it is handled as a separate PDU Set instead and the size of the packet is identical to the PDU Set Size of this separate PDU Set (which should equal the size of the PDU).

End of data burst signalling may still apply in the RTP Header Extension to all packets (both marked and unmarked).

In case the last packet of the data burst is an unmarked packet there are two possible solutions

-	add RTP HE for PDU Set marking to the packet if packet if possible (i.e is an RTP Packet)

-	In case of a control packet is the last packet of a data burst and this is not possible, the end of data burst should have been set in an earlier already marked packet.

The guideline for determining PDU Set information at the UPF from either RTP HE or unmarked PDU is given in Table 6.X.2.2-1.

Table 6.X.2.2-1: Determining PDU Set information at UPF from RTP HE and unmarked PDU

| PDU Set information | RTP HE | without RTP HE i.e. lone PDU |
| --- | --- | --- |
| PDU Set importance | set by interpreting PSI field in [2] | set by 5G System to default value |
| PDU Set Size | optionally transmitted in additional PSSize field | PDU Size |
| End of Data Burst | can be set by EoDB flag | N/A for lone PDU |
| PDU Set Sequence number | from PDU set sequence number | separate space for lone PDU's maybe used |
| PDU Sequence number | wrap on 9 bits of PSSN most significant bit is set to 0 | 9 but counter at UPF with most significant bit is set to 1 |
| end of PDU set | End of the PDU Set in RTP HE | always 1 |

The different PDU Set information can be determined by the UPF as shown in table 6.X.2.2-1 with different PDU Set information in the left most column. The middle column indicates how the UPF can derive PDU Set information for packets that include RTP HE for PDU Set information. The right most column indicates how UPF can derive PDU Set information for unmarked packets (lone PDU’s).

The PDU Set Sequence Number has some additional steps applied by the UPF (these steps are an example, an actually UPF could apply different steps, this is just to give how this scenario can be addressed).

The UPF can only use the 9 least significant bits of the RTP HE mapping PSSN from RTP HE to PSSN in PDU Set Information and set the most significant bit of PSSN in PDU Set information to 0.

The UPF can then use the 9 least significant bits of PSSN in PDU Set information based on a counter for unmarked PUD ‘s, and set the most significant bit to 1.  This is an example of how the UPF can deal with both PSSN from RTP HE and unmarked PDU and still get PSSN to identify the PDU Set.

Signalling presence of unmarked PDU could be enabled in TS 26.510.

The definition of PSSN in 5G RTP [2] and [ZZb] are currently not aligned, the strict increment by one only applies for 5G RTP not for PSSN generated by UPF as part of PDU Set information.

NOTE: This guideline does not require changes to the definition of PSSN in [2].

### 6.X.3	Proposal

Develop the guidelines for handling unmarked PDU’s by the UPF and handling unmarked PDU sets in TS 26.522 following clause 6.X.2.2.

Develop signalling for the presence of lone PDU’s in TS 26.510.