3GPP SA4 RTC Telco Post #129-e	S4aR240097

Online, 23 October 2024

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PSEUDO CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.822 | CR | pseudo | rev | - | Current version: | 1.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [FS_5G_RTP_Ph2] PSSN for the lone PDU |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_5G_RTP_Ph2 |  |  |  |  |  | Date: |  |  | 10/23/2024 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | This addresses Key Issue #2: QoS handling requirements for lone PDU TS23.501 does not specify how to determine the PSSN for the lone PDU. The straightforward method seems to be that the UPF assigns the PSSN contiguously as a PDU Set or lone PDU arrives. However, this will increase the jitter, limit the throughput, and break the PDU Set correlation information. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added an analysis of the issues and an observation. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Negative impact on application performance. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * 1st change (All New) * * * *

### 6.X.1	Key Issue mapping

This maps to Key Issue #2.

### 6.X.2	Description

According to clause 5.37.5 of TS23.501 [3], on handling the lone PDU in the DL, the UPF maps a lone PDU to a PDU Set without specifying how to set the PSSN, and what is specified is to ‘determine’ the PDU Set Information which includes the PSSN. Specifically, clause 5.37.5.1 of TS23.501 [3] states:

If the PSA UPF receives a PDU that does not belong to a PDU Set based on Protocol Description for PDU Set identification, then the PSA UPF still maps it to a PDU Set and determines the PDU Set Information as described in clause 5.37.5.2.

Clause 5.37.5.2 of TS23.501 [3] further states:

To support PDU Set based QoS handling, the PSA UPF identifies PDUs that belong to PDU Sets and determines the below PDU Set Information which it sends to the NG-RAN in the GTP-U header.

The method of determining the PSSN for the lone PDU has implications on the application performance.

The most straightforward method seems to be that the UPF assigns a PSSN continguously to every incoming PDU Set or lone PDU. For example, if the PSSN of the last departing PDU Set is x, and at the next moment a lone PDU arrives, the UPF assigns x+1 as the PSSN to the lone PDU.

While this method ensures the continuity of the PSSNs, it will cause three issues impacting the application.

Issue #1 (modification of too many PSSNs): if the UPF receives a lone PDU, it must modify the PSSN of every PDU Set to be received afterwards. This increases the complexity of the UPF and may introduce delay jitter and limit the throughput given that the UPF typically handles a very large amount of traffic.

Issue #2 (maintaining state information at UPF to support PDU Set correlation information): to convey the PDU Set correlation information (Sol#23) in FS_XRM_Ph2 [6], a dependent PDU Set carries the PSSN of the PDU Set on which it depends. For convenience, the PSSN is called correlated PSSN here. With the lone PDU, the UPF needs to modify both PSSNs in a dependent PDU Set. Moreover, the difference between the two PSSNs may change, preventing the UPF from simply using the gap to derive the new value for the correlated PSSN. As a result, the UPF needs to memorize the PSSN mappings that it has done. This is shown in Figure 6.X.2-1. For the PDU Set with PSSN=102 and correlated PSSN=101 entering the UPF, the difference between the two PSSNs is 1. The UPF maps PSSN=102 to 104. Previously, it mapped PSSN=101 to 102. Now seeing correlated PSSN=101 again in PDU Set with PSSN=102, the UPF needs to recall how it mapped PSSN=101. The UPF cannot simply map the correlated PSSN=101 to 103 using the difference 1. This requirement of maintaining per-PSSN state information can significantly increase the UPF complexity and introduce even more jitter and limit more on the throughput suffered in the first issue.

Figure 6.X.2-1 The UPF has the burden of tracking PSSN mappings to support PDU Set correlation information, where an arrow indicates dependence.

Issue #3 (breaking media dependence information in the case of out-of-order delivery): this straightforward method cannot correctly convey the PDU Set correlation information when there is out-of-order delivery of the PDU Sets. This is shown in Figure 6.X.2-2. PDU Set with PSSN=99 is out of order. When the PSU Set with PSSN=100 reaches the UPF, the UPF does not know in the future whether there will be a lone PDU before the PDU Set with PSSN=99, i.e., it cannot distinguish the top scenario and the bottom scenario.

If the UPF assumes one scenario, the other scenario may occur, in which case the PDU Set correlation information will be wrong, as illustrated with an example in Figure 6.X.2-3. Alternatively, the UPF can memorize that PSSN=99 is mapped to 100 even before the PSU Set with PSSN=99 arrives, and this requires the UPF to maintain a per-PSSN state, which is undesirable.

Figure 6.X.2-2 The UPF cannot distinguish the two scenarios because it does not know whether there will be a lone PDU before PDU Set with PSSN=99.

Figure 6.X.2-3 The PDU Set correlation information is wrong after the PDUs go through the UPF because the UPF assumes that there is no lone PDU before PDU Set with PSSN=99 but there actually is one lone PDU.

Observation: To avoid added jitter or reduced troughput and not to corrupt PDU Set correlation information, it is desirable that the UPF does not change the PSSN of a PDU Set.

# * * * * End of 1st change * * * *