3GPP TSG-SA4 Meeting #129-e (AH) RTC SWG post 129-e	S4aR240103

# online, 25th Sep 2024 - 23rd October 2024

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.113 | CR | 0004 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [iRTCW] Clarifications on consumption reporting |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | InterDigital Communications, BBC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | iRTCW, TEI18 |  |  |  |  |  | Date: |  |  | 2024-10-23 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | TS 26.113 describes the QoE consumption reporting API in clause 10.6 but does not provide clear details on some of the properties of the ConsumptionReport and ConsumptionReportingUnit data types. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Specify that the mediaPlayerEntry property included in the consumption reporting data type represents the WebSocket URI of a swapEndpoint and the mediaConsumed property included in the ConsumptionReportUnit represents the received media information in clause 10.6 of TS 26.113. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Specification does not fully address the consumption reporting feature. |  |  |  |  |  |  |  |  |
| Q |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

## 10.6	Consumption Reporting API

### 10.6.1	General

The Consumption Reporting API allows the RTC Media Session Handler to report media consumption to the RTC AF, as configured by the ServiceAccessInformation resource (see clause 10.2).

The relevant procedures are specified in clause 5.3.6 of TS 26.510 [3].

The media consumption reporting API and consumption report format are specified in clause 9.6 of TS 26.510 [3]. The population of the data types to report consumption of RTC sessions is profiled in the following clauses.

### 10.6.2	ConsumptionReport data type

The ConsumptionReport data type is specified in clause 9.6.3.1 of TS 26.510 [3].

In the case of real-time media communication:

-	The mediaPlayerEntry shall be populated with the URI of the WebSocket connection for the swapEndpoint listed in rtcClientConfiguration of the ServiceAccessInformation resource.

-	A separate ConsumptionReportUnit shall be reported in the consumptionReportingUnits array for each media component actively being received by the RTC endpoint, as identified by the "m=" line in the negotiated SDP information.

### 10.6.3	ConsumptionReportingUnit type

The ConsumptionReportUnit type is specified in clause 9.6.3.2 of TS 26.510 [56].

In the case of real-time media communication:

-	The mediaConsumed property shall indicate the media identifier (mid) or the synchronization source identifier (SSRC) value of the received media by an RTC Client.

NOTE:	The media identifier and SSRC values of the received media content are present in the negotiated SDP information.

When both mid and SSRC values are present in the SDP for a specific RTC session, the mid value of that media content shall be used to populate the mediaConsumed property in preference.

-	In cases where consumption of a particular media component was reported in a previous ConsumptionReport, the duration property is simply extended in the ConsumptionReportingUnit to reflect the total time for which that media component has been consumed during the RTC session, while the mediaConsumed and startTime properties remain the same as in the previously reported ConsumptionReportingUnit.

End of changes