Agenda item: 	4.6

Source: 	Qualcomm Inc.

Title: 	[5G_RTP_Ph2] On the definition of Time To the Next Data Burst

Document for	Discussion and Agreement

# Introduction

In clause 6.25 of TR26.822 [1], the following two options on the definition for the time to the next data burst (TTNB) are proposed:

-	Option 1: the starting time is the departure time of the first packet of the first PDU Set.

-	Option 2: the starting time is the departure time of the last packet of the last PDU Set.

NOTE: This needs to be coordinated with SA2/RAN2.

and their respective pros and cons are analyzed therein.

However, the data bust is not necessarily related to PDU Sets [2], and either of the above options assumes on the contrary that a data burst consists of one or multiple PDU Sets. Therefore, the definition needs to be revisited.

# Proposed definitions

### 2.1 A first definition

To decouple PDU Sets from the data burst, a first definition is proposed as follows:

-	Option 1: the starting time is the departure time of the first packet of the current data burst.

-	Option 2: the starting time is the departure time of the last packet of the current data burst.

It is not clear which of the above two is necessarily better given the analysis of the pros and cons in [1], which apply to this new definition.

Moreover, if the first packet is lost, then with Option 1, there will be no TTNB information about the next data burst available to the UPF, considering the DL case. Similarly, the same issue occurs if the last packet is lost, and Option 2 is adopted.

### 2.2 A second definition

To increase the error resilience, we consider the following definition:

The definition of TTNB is one of the following options:

-	Option 1: the starting time is the departure time of the first packet of the current data burst.

-	Option 2: the starting time is the departure time of the last packet of the current data burst.

Additionally, one or more packets can carry the TTNB information. How many packets in a data burst carry the TTNB information can be based on packet loss rate, and left to UE implementation.

Figure 1 illustrates Option 1 for the second definition. Note that all packets carry the same TTNB value.

Figure 1: Option 1 of the 2nd definition.

However, this definition has two drawbacks:

- Increased UPF complexity. This is because, if Option 1 is used, the UPF needs to save the arrival time of the first packet to figure out when the next data burst will arrive.

- It is impossible for the UPF to know the TTNB, if the reference packet (the first packet in Option 1 and the last packet in Option 2) is lost.

### 2.3 	A third definition

Proposal: For the definition of TTNB, the starting time is the departure time of the current packet which carries the TTNB information. Additionally, how many packets carry the TTNB information is up to UE implementation. This definition includes the first definition as a special case.

This is illustrated in Figure 2:

Figure 2: The third definition when packets depart the traffic source.

With this definition, different packets generally carry different TTNB information based on when the respective packets depart from the traffic source.

Another benefit of this definition is that it allows the UPF to estimate the N6 jitter (see clause 5.37.8.2 of TS23.501[3], where the scenario is periodic traffic but the need applies equally to dynamic traffic). This is shown in Figure 3. The N6 jitter can be estimated as (|d1’-d1| + |d3’-d3| |)/2.

Figure 3: The third definition when packets arrive at the UPF.

# Proposal

Proposal: For the definition of TTNB, the starting time is the departure time of the current packet which carries the TTNB information. Additionally, how many packets carry the TTNB information is up to UE implementation.

# References

[1] 	TR26.822, Study on 5G Real-time Transport Protocol Configurations, V1.2.0, Rel-19, Nov 2024.

[2]		TS26.522, 5G Real-time Media Transport Protocol Configurations, Rel-18.0.0, March 2024.

[3]		TS 23.501, "System architecture for the 5G System (5GS)", Rel-19, V19.1.0, Sep 2024.