3GPP SA4 RTC SWG Telco Post #130	S4aR250018

# Online, 15 January 2025

Source:	InterDigital France

Title:	Pseudo-CR on Haptics Media in messaging service

Spec:	3GPP TS 26.854 v1.0.0

Agenda item:	4.9

Document for:	Agreement

1. Introduction

This contribution provides for the integration of haptics media in 3GPP messaging services.

2. Proposal

Agree to the additional details to finalize section 8.3

* * * First Change All new * * * *

### 8.3.4	Messaging Media services

#### 8.3.4.1 System description

To support the use-cases in section 5 for haptics media enhanced messaging services, the following figures from TS 26.143 [143] are modified to illustrate haptics media functionalities in the overall Messaging Media system, Figure 8.3.4-1, and in the Messaging Media player model, Figure 8.3.4.-2.

Haptic Engine

Haptic Engine

Figure 8.3.4-1 Example system for Haptics enhanced Messaging multimedia message exchange

Haptic Engine

Figure 8.3.4-2 Haptics media enhanced MMBP Player Model

#### 8.3.4.2 Media Capabilities and Profiles

The MMBP media capabilities in [143] currently does not support haptics media type. The inclusion of a top level haptic media type would happen at the same level as other media formats (text, speech, audio image etc) and would not require modification to the multipart MMBPs and container formats (single, mixed, alternative, parallel, related and 3GP9) with the exception that the sample entry for haptics needs to be added to the supported sample entries in 26.244 [244].  The Haptics media needs to be synchronized and spatially aligned with other media types. For Messaging services, it is expected that a single track is used. Time synchronization is provided by using the ISO BMFF/MP4 file format.

In addition, when the haptics media effect is included in a scene description in association with a particular node, the scene description capabilities of the messaging service need to be extended to support MPEG_haptic and MPEG_haptic_material as well as SD-Rendering-glTF-Interactive [mecar]

8.3.4.2.1 Player and Decoding capabilities

For instance, the capability 26143_HAPTICSMEDIA_MIHS can be defined as the capability of playing back (decoding and rendering) a file that:

- is decodable by a decoder capable of the MIHS decoding and rendering capabilities as defined in [7],

- is encapsulated in an ISO BMFF track with the sample entry ‘mih1’ as defined in [25]

- -	is contained in a 3GP file that conforms to the 26143_CONTAINER_MP4_3GP9 capability defined in clause 5.2 of [143] and extended with a sample entry set for haptics media.

The media type for files with this capability 26143_HAPTICSMEDIA_MIHS shall be signalled with haptics/mp4, profile="3gp9" codecs="mih1" or an equivalently compatible media type.

##### 8.3.4.2.2 MMBP content generator capabilities:

For instance, the capability 26143_HAPTICSMEDIA_ENC_MIHS for a content generator can be defined as the capability of:

- Generate a haptics media file from an actuator or from a local stored file, such that the file can be played back by a player with the capability 26143_HAPTICSMEDIA_MIHS

- Generate an ISO BMFF track that conforms with the requirements of the sample entry 'mih1' as defined in [7].

- Generate a 3GP file from the ISO BMFF track that conforms to the 26143_CONTAINER_MP4_3GP9 capability with the sample entry set to haptics media

- Signal the haptic media capability with the generated file  haptics/mp4, profile="3gp9" codecs="mih1" or an equivalently compatible media type

Note: TR 26.841 [841] studies media messaging enhancement, evaluation of haptics media as part of MMBP media capabilities in IETF MIMI [MIMI], and in the advance file format would need to be done.

* * * End of Changes * * * *