SA4-e (AH) RTC SWG post 130	S4aR250021

# 15 January 2025, Online

Source:	Nokia

Title:	[5G_RTP_Ph2] On the definition of data burst

Agenda item:	4.6

Document for:	Agreement

# Introduction

The term “data burst” currently has slightly different definitions in TS 26.522 (SA4) and TS 23.501 (SA2).

SA4 definition:

Data Burst: A data burst is a set of multiple PDUs generated and sent by the application such that there is an idle period between two data bursts. A Data Burst can be composed of one or multiple PDU Sets.

SA2 definition:

Data Burst: A set of multiple PDUs generated and sent by the application in a short period of time.

NOTE 1:	A Data Burst can be composed of one or multiple PDU Sets.

Neither the “idle period between two data bursts” in the SA4 definition nor the “short period of time” in the SA2 definition provides sufficient accuracy for identifying data bursts, that is, classifying a sequence of packets as a data burst.

SA4 has already recognized the issue regarding the data burst definition in TS 26.522 and reached the following conclusion in TR 26.822:

-	Revisit definition of a data burst in TS 26.522 [2] to indicate what is meant by idle time and if it is required.

Thus, there is a need to adopt a more accurate definition and resolve the misalignment between the current definitions of a data burst in TS 26.522 and TS 23.501.

# Discussion

To provide a formal definition of a data burst, the following questions need to be answered:

- How many packets constitute a burst?

- How close should the packets be to each other, i.e. what is inter-packet gap?

Number of Packets: A data burst consists of multiple packets. The exact number of packets that constitute a burst can vary depending on the application properties (e.g. number of packets comprising a video frame) and specific transport protocol (e.g. flight behavior of RTP vs QUIC).

Inter-Packet Gap: The inter-packet gap, or the time interval between consecutive packets within a burst, is usually very short. This gap is often measured in milliseconds (ms) or microseconds (µs). For a transmission to be considered a burst, the inter-packet gap should be shorter than the interarrival time of packets outside the burst.

The author of [1] provides an “intuitive” definition of burst as follows:

Burst is a group of consecutive packets with shorter interpacket gaps than packets arriving before or after the burst of packets. Packets may be of the same flow or of different flows.

The need for a formal definition is also recognized in [1] and the following definition is offered:

Burst – a sequence of consecutive packets whose inter-packet arrival time ta is shorter than the interarrival time of packets arriving before or after these packets and is in a range (td-d1, td+d2) where di is a tolerance and td is a predefined interpacket arrival time. The interarrival time is counted from last bit of packet 1 to first bit of packet 2. Thus, consecutive packets form a burst if their interarrival times (ta) are in the range (td-d1, td+d2).

The following figures from [1] demonstrates the effect of changing the predefined interarrivel time td on the number of detected bursts.

The authors of [2] define burst as “some sequence of x packets, each arriving within y bit times of each other”. They use the term bit time [3] to take into account the effect of link utilization on the definition of a burst:

Because the meaning of a microsecond or nanosecond depends on the data rate, we instead refer to “bit time”. A given amount of data transmitted over a microsecond at 10 Gbps will be 10 times denser than the same amount of data transmitted at 100 Gbps. In other words, a gap of y ns corresponds to less burstiness at 100 Gbps than at 10 Gbps.

Combining the ideas from [1] and [2], a formal definition can be established.

A data burst is a sequence consecutive packets, where:

- Each packet in the sequence arrives within ta  (td-d1, td+d2) of the previous packet.

- ta ​is shorter than the interarrival times of packets arriving before or after the burst.

- ta​ can be normalized in terms of bit times to account for varying data rates.

However, one problem with this definition is that it defines data burst from a receiver perspective. However, the burst related signaling is provided by the RTP sender, therefore a data burst should rather be defined from a sender perspective (excluding any network jitter). Therefore, although the above receiver centric definition is not fully appropriate from a practical point of view.

A compromise could be found by incorporating the aspects discussed above into the data burst definition yet keeping it sender centric. The following definition is an attempt to do so:

Data Burst: A data burst is a set of consecutive PDUs with shorter inter-PDU gaps than the PDUs sent before or after the data burst.

NOTE 1: A data burst can be composed of one or multiple PDU Sets.

NOTE 2: Meaning of a short inter-PDU gap needs to be determined by the application and can vary depending on different factors including the available bandwidth and link utilization.

# Proposal

- Update the data burst definition in TS 26.522 as per the discussion above

- Coordinate with SA2 to align the data burst definitions in TS 23.501 and TS 26.522

# References

[1] Krzanowski, Roman. "Burst (of packets) and burstiness." 66th IETF meeting. 2006. https://www.ietf.org/proceedings/66/slides/ippm-10.pdf

[2] Woodruff, Jackson, Andrew W. Moore, and Noa Zilberman. "Measuring burstiness in data center applications." Proceedings of the 2019 Workshop on Buffer Sizing. 2019. https://www.cl.cam.ac.uk/~nz247/publications/woodruff2019measuring.pdf

[3] https://en.wikipedia.org/wiki/Bit_time