3GPP TSG-SA4 RTC SWG Ad hoc telco post#130	S4aR250026

Online, 15th January 2025	revision of S4aR250005/0006/0007

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Pseudo CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.567 | CR | - | rev | - | Current version: | 0.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [SR_IMS] Procedure for delay adaptation based on QoE metrics |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | InterDigital Communications |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | SR_IMS |  |  |  |  |  | Date: |  |  | 15-01-2025 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | QoE latency metrics, and their measurements during the rendering loop in an XR Baseline Client have been addressed in 3GPP specifications. But the procedures for an efficient delay adaptation to maintain an acceptable QoE for the user when the network conditions are changing is missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add the Delay adaptation procedure based on QoE metrics information in Clause 7.3. The Delay adaptation based on QoE metrics consists of adapting a delay metric (e.g. pose-to-render-to-photon, roundtrip interaction delay etc..) to maintain an acceptable QoE for the user. - Add related terms definition - Add Configuration format - Add Metadata format |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The 4th Objective of the WI is not fulfilled. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.1, 7.3 and Annex A |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Introduction

User Quality-of-Experience (QoE) is a key indicator for the adoption of eXtended Reality (XR) applications over a 3GPP network.

Delay adaptation based on QoE latency metrics is essential to maintain the quality of the user experience at an acceptable level when the network conditions are changing. This delay adaptation based on QoE metrics may be performed by the MF. For example, it relies on different scene representations to modify the delay of various processing tasks involved in the rendering loop such as:

- the update of the XR scene graph representation by considering animation, user interactivity, physics simulation,

- the culling techniques (e.g., camera frustrum, back-face) process with respect to the user current point of view to exclude objects or portions of the XR scene that are not visible to the user,

- the rendering of the XR scene with the proper lighting conditions

The proposal is to add the new procedures for delay adaptation based on QoE latency metrics information.

## Changes from previous contributions S4aR250005/0006/0007:

Taking into account to comments received during the 3GPP SA4 RTC SWG Telco on Dec 17 in S4aR250014, the following changes have been made:

- simplify the procedure to include the delay adaptation in the rendering loop. A delay adaptation information message may be sent with pose and user inputs metadata.

- simplify the delay adaptation configuration message (remove the threshold values and the type)

- simplify the delay adaptation information message (remove the acknowledgement)

- Clarify on when to reset the delay adaptation information message periodicty

- merge the 3 previous contributions into one including the call flow, the message format for the configuration and the metadata.

CHANGE 1

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

SR-DCMTSI Client	: a Split-Rendering capable DCMTSI Client

Delay adaptation based on QoE metrics: The process of adjusting processing delay, for example by adjusting the Level of Detail (LoD) of objects being processed in an XR scene to maintain an acceptable QoE for the user.

Split rendering session: a media session running the split rendering process between two or more entities. A split rendering session may include a data channel, consisting of application data channel, and one or more RTP streams for delivering split rendered media.

CHANGE 2

## 7.3	Network support procedures

## 7.3.1	split rendering adaptation

An SR-DCMTSI client or an MF may trigger further procedures during a split rendering, One such procedure may be to adapt the split of rendering operations between the SR-DCMTSI client and the MF during a split rendering session. This split may be due to change in operating conditions of the split rendering session, for example operating conditions of the UE, the MF or changes in the application or scene being rendered, for example changes in the scene description. Split adaptation may include data exchange, for example, exchange of adaptation messages, application state information and assets needed for the split rendering of an DC application. The following generic procedure shall apply, while the exact details may depend on the DC-application being rendered.

Figure7.3-1: General procedures for adaptation of split rendering

The steps are as follows:

Step 1: The IMS session is established between the SR-DCMTSI client in terminal and a terminating SR-DCMTSI Client which may be in a terminal. For Person to Person calls, procedures in clause 7.1 are followed.

Step 2: A split rendering session is set up between the SR-DCMTSI client and a serving MF.

Step 3: Assets related to the application being split rendered may be delivered to participants of the split rendering session. The asset delivery may include javascript assets, scene descriptions and graphical objects needed for the session.

Rendering Loop:

The rendering loop is executed continuously during the duration of the split rendering session, for each frame.

Step 4: The SR-DCMTSI client in terminal sends metadata required for rendering to the MF. The metadata may include pose, pose predictions, user inputs etc.

Step 5 and 6: The SR-DCMTSI client in terminal and the MF render the frame.

Step 7: The frame rendered by the MF is transmitted to the SR-DCMTSI client in terminal as well as possible metadata.

Step 8: The SR-DCMTSI client in terminal composes a display frame from the received rendered media and media rendered locally.

NOTE: Steps 5,6,7 although ordered above, may occur in any order. Step 8 may include pose-correction. Step 8 and 6 may be executed as a single step.

Further Procedures:

Split Adaptation:

NOTE: Split Adaptation refers to adaptation of split rendering operations in an ongoing split rendering session between the SR-DCMTSI client and MF, without impacting the MF resources provisioned by IMS AS in step 9 of clause 7.1

Step 9: A trigger to adapt the split occurs at the SR-DCMTSI client in terminal; the trigger may be, for example, a change in available UE resources (e.g. battery, compute), changes in QoE of the split rendering session, changes in the scene/application being rendered.

Step 10: The SR-DCMTSI client in terminal decides if a new split of the rendering operations is needed and determines the new split.

Step 11: The SR-DCMTSI client in terminal sends a request to the MF to adapt the split to the new split.

Step 12: The MF actuates the new split of the rendering operations.

Step 13: The MF sends an acknowledgment of the new split to the SR-DCMTSI client in terminal.

Step 14: The MF and UE may exchange messages and data to support the new split of operations. This may include exchange of messages, for example, for synchronization of the state of the scene being split rendered or exchange of assets, for example, those in Step 3.

The meta-data messages exchanged shall follow the formats specified in clause 5.4

Step 15: The rendering loop (steps 4 through 8) continues.

Note:  Split adaptation is shown to be initiated by the SR-DCMTSI client in terminal for clarity, the procedure may be triggered by the MF. Further, other procedures to actuate the new split may be executed during the split rendering session.

## 7.3.2	Delay adaptation based on QoE metrics

An SR-DCMTSI client or an MF may trigger further procedures during a split rendering. An additional procedure may include adjusting various round-trip delays between the SR-DCMTSI client and the MF during a split rendering session. The delay adaptation information may be sent from the SR-DCMTSI client to the MF periodically when the measured QoE metrics (e.g., poseToRenderToPhoton, roundtripInteractionDelay) goes out of the target range. Upon receiving the notification, an MF may adjust the delay involved in some of the processing tasks. For example, an MF may change the Level of Detail (LoD) of the objects in an XR scene which impacts the processing complexity of the XR scene. Delay adaptation procedure may include data exchange, for example, exchange of notification and acknowledgment messages for delay adaptation. The following generic procedure may apply, while the exact details may depend on the DC-application being rendered.

Figure7.3-2: General procedures for delay adaptation based on QoE metrics information

Steps 1 to 3 are as described in clause 7.3-1.

The steps for delay adaptation based on QoE metrics information during the rendering loop are as follows:

The SR-DCMTSI client measures and collects the QoE metrics negotiated in the configuration during the rendering loop. This includes the active QoE metric currently considered by the delay adaptation (e.g., poseToRenderToPhoton, roundtripInteractionDelay).

Step 4: The SR-DCMTSI client in terminal sends metadata required for rendering to the MF. In addition to the pose, pose predictions and user input metadata, the metadata may include a delay adaptation information message to the MF. The trigger to send the delay adaptation information message is based the configured periodicity to inform the MF that the measured delay of the active QoE latency metric is out of the target range. For example, the measured poseToRenderToPhoton QoE latency metric goes out of the target range due to new network conditions. The delay adaptation information message consists of the signed difference between the target value and the measured value of the active QoE metric.

Step 5a: The MF may adjust the processing delay based on the delay adaptation information message.

NOTE: The MF may change the LoD of the objects that are part of the scene for the delay adaptation.

Step 6 and 5b: The SR-DCMTSI client in terminal and the MF render the frame.

Steps 7 to 8 are as described in clause 7.3-1.

CHANGE 3

Annex A (normative):

# A.1	Supported Message Types

This clause defines the message types supported by this specification.

Editor’s Note: The clause may contain all message types for XR and other services. The suitable message types from TS 26.567 need to be referred or modified and imported to this spec as appropriate.

## A.1.X	Delay Adaptation based on QoE metrics

## A.1.X.1	Configuration format

The MF shall share the configuration information of the delay adaptation procedure with the SR-DCMTSI client in terminal during the split rendering session negotiation and establishment processes. The configuration information of the delay adaptation procedure shall be in JSON format according to the Metadata Data Channel Message Format defined in clause 5.4.3. The message type shall be “urn:3gpp:split-rendering:v1:daqoe:configuration”.

Table A.1.X.1-1 – Configuration message format for Delay adaptation based on QoE metrics

| Name | Type | Cardinality | Description |
| --- | --- | --- | --- |
| id | string | 1..1 | A unique identifier of the message in the scope of the data channel session. |
| type | string | 1..1 | urn:3gpp:split-rendering:v1:daqoe:configuration |
| message | Object | 1..1 | Message content |
| qoeMetrics | array | 1..1 | An array of the QoE metrics for which scene adaptation is available. This qoeMetrics array may contain all or a subset of the QoE metrics negotiated in the metrics configuration message in clause 6.3.1. |
| qoeMetricId | string | 1..1 | A unique identifier of the QoE metric within the scope of the split rendering session. The name of that QoE metric is chosen as unique ID, this name should be consistent with the name provided in the metrics reporting configuration defined in clause 6.3.1. |
| targetDelay | number | 1..1 | The target delay for the QoE metric in milliseconds. The targetDelay shall be in the range of minimumThreshold and maximumThreshold values. |
| periodicity | string | 1..1 | The periodicity of the delay adaptation information for that QoE metric. It may be expressed as a multiple of the "Measure-Resolution" defined in clause 16.3.2 of TS 26.114 [7]. Whenever a delay adaptation information message is sent, the SR-DCMTSI client in terminal shall reset its reporting interval timer to the value of the periodicity property and it shall begin countdown of the timer again. |
| activeQoeMetricId | string | 1..1 | A unique identifier of the QoE metric that is considered for the delay adaptation within the scope of the split rendering session. The name of that metric shall be consistent with the name provided in the metrics reporting configuration defined in clause 6.3.1. |

NOTE: 	The target delay range for a QoE metric is delimited by minimumThreshold and maximumThreshold delay values. The thresholds, for each QoE metric in the qoeMetrics array, can be provided by the DC Application Server to the SR-DCMTSI client in terminal.

#### A.1.X.2	Metadata format

During a IMS-based split rendering session, the operating environment of the serving MF, the resources of SR-DCMTSI client or the network conditions may change. Consequently, the roundtrip delay may need to be adjusted to deliver a consistent QoE.

When delay adaptation procedure is enabled, the SR-DCMTSI client in terminal checks for the active QoE metric being monitored whether the measured delay is within the target delay range or not. The SR-DCMTSI client in terminal may decide to invoke the delay adaptation procedure for that QoE metric based on the periodicity when the measured delay is outside the target delay range. The SR-DCMTSI client in terminal may determine and send to the MF a delay offset value for the active QoE metric.

The delay adaptation information message format that is used for IMS-based split rendering shall comply with the format defined in Table A.1.X.2-1. The delay adaptation information message shall be carried as part of the data channel messaging mechanism. The metadata data channel message shall be in JSON format according to the Metadata Data Channel Message Format defined in clause 5.4.3. The message type shall be “urn:3gpp:split-rendering:v1:daqoe:information”.

Table A.1.X.2-1 Metadata format for Delay Adaptation information message based on QoE metrics

| Name | Type | Cardinality | Description |
| --- | --- | --- | --- |
| id | string | 1..1 | A unique identifier of the message in the scope of the IMS-based split rendering session. |
| type | string | 1..1 | urn:3gpp:split-rendering:v1:daqoe:information |
| message | object | 1..1 | Message content |
| adaptationtId | string | 1..1 | An identifier of the adaptation message unique within the scope of the SR session. |
| activeQoeMetricId | string | 1..1 | The unique identifier of the QoE metric currently considered by the delay adaptation procedure within the scope of the IMS-based split rendering session. This identifier was exchanged between the SR-DCMTSI client in terminal and the MF during the configuration step. The name of the metric shall be consistent with the name provided in the metrics reporting configuration defined in clause 6.3.1. |
| delayOffsetValue | number | 1..1 | Signed delay offset value (relative to the targetDelay) derived from the equation (1): delayOffsetValue = targetDelay – measuredDelay (1) The measuredDelay is the value of the active QoE metric. |

END OF CHANGES