Agenda item: 	4.6

Source: 	Qualcomm Inc.

Title: 	[5G_RTP_Ph2] On the LS on AL-FEC RAN Awareness

Document for	Discussion and Agreement

# Introduction

SA2 sent an LS S2-2410999 [1] to RAN2, and CC’ed SA4, on the feasibility of AL-FEC RAN awareness:

Question 1: Please provide feedback on feasibility of use of content ratio information for discarding of DL PDUs during congestion for RLC AM mode based on the above principles. Please also provide feedback on applicability of RLC AM for XR services, also considering the RAN-level enhancements of RLC Acknowledged Mode (RLC AM) with small packet delay budget as per RP-240791.

With respect to RLC UM, RAN indicated in R2-2405781 that "RAN2 does not think it is possible for NG-RAN to determine reliably whether a PDU has been successfully delivered over an unacknowledged-mode data bearer." After sending the previous LS in S2-2405625, SA2 clarified the principles for potential use of content ratio information for discarding of PDUs during congestion as listed above.

Question 2: Please provide feedback on feasibility of use of content ratio information for discarding of DL PDUs during congestion for RLC UM based on the above principles.

RAN2 replied in R2-2411002 [2] to SA2, and CC’ed to SA4:

Within RAN2, there is no consensus on the usefulness of using AL-FEC content ratio information for discarding DL PDUs by NG-RAN during congestion. It is up to NG-RAN how/whether to use content ratio information, should it be provided.

From the above, RAN2 has no concern on the feasibility of AL-FEC RAN awareness. Rather, it has not reached consensus on the usefulness of AL-FEC awareness.

SA4 has studied on the usefulness of AL-FEC awareness during FS_5G_RTP_Ph2 in Rel-19, and the outcomes have been documented in TR26.822 [3]. The study encompassed the benefits of overhead reduction and UE power saving, as captured in Solutions #17 and #19, respectively, in TR26.822 [3].

It would be beneficial to bring our study to the attention of SA2, hopefully helping SA2 make a more informed decision about whether to support AL-FEC RAN awareness. For this purpose, a companion paper S4aR250025 [4] is submitted for consideration at this meeting.

SA2 is expected to make a decision on AL-FEC RAN awareness at the SA2 #167 meeting Feb 17-21, 2025, which overlaps with the SA4 #131 meeting. Should an SA4 LS be sent to inform SA2 of the study carried out within SA4 on the usefulness of AL-FEC RAN awareness, it is important to send the LS at the beginning of the SA4 #131 meeting.

# Proposals

Proposal 1: SA4 sends an LS to SA2 on the usefulness of AL-FEC awareness in response to the RAN2 LS R2-2411002.

Proposal 2: If Proposal 1 is agreed, SA4 RTC SWG will work on and agree to the LS and send it to SA4 plenary for SA4 agreement/approval on the first day of SA4 #131.

Proposal 3: SA4 uses the draft LS in S4aR250025 as a staring point for the LS conceived in Proposal 1.

References

[1] S2-2410999, "Reply LS on Application-Layer FEC Awareness at RAN", 3GPP SA WG2 Meeting #165, Hyderabad, India, 14-18 October 2024.

[2] R2-2411002, "Reply LS on Application-Layer FEC Awareness at RAN", 3GPP SA WG2 Meeting #128, Orlando, USA, November 18-22, 2024.

[3] 3GPP TR 26.822, V.19.0.0, January 2025.

[4] S4aR250024, Draft Reply to LS on Application-Layer FEC Awareness at RAN, RTC Ad Hoc Meeting Post SA4 #130 Meeting, Online, 15 January 2025.