Source:	HEAD acoustics GmbH

Title:	Preliminary results of HaNTE round robin test results for Lab2

Document for:	Discussion

# Introduction

This contribution reports on the Handsets featuring Non-Traditional Earpieces (HaNTE) round robin results at Laboratory 2 (HEAD acoustics GmbH). Tests were conducted according to the agreed test plan [1] of the 3GPP work item HaNTE [2]. In addition, initial results and definitions from lab 1 of the round robin test [3] are reviewed.

# Devices

Seven commercially available mobile phones were evaluated as devices under test (DUT), which all provide a non-traditional earpiece. For reference, an additional device with a traditional earpiece was evaluated as an eighth device. Since the form factor of all devices is similar (smart phones), the alternative handset position (also called flat handset position) according to ITU-T P.64 [4], where B-axis is rotated by -5° (∆B=5°).

For the HaNTE-devices (DUT1 to DUT7), a common ECRP of Ye=-21mm was used for testing, as already specified in the test results from lab 1 [3]. ECRP of DUT8 (non-HaNTE device) was determined via visible acoustic outlet.

# Test setup

## Overview

As head and torso simulator (HATS), HEAD acoustics HMS II.3 (complies with ITU-T P.58 [5], equipped with ear simulators of Type 3.3 [6]) was used in conjunction with the automated handset positioner HHP IV (complies with ITU-T P.64 [4]). For privacy tests (see clause 3.3), HEAD acoustics turntable HRT I was used for the realization of HATS rotation.

For testing of receive loudness rating and frequency response, 3GPP TS 26.132 [7] specifies the usage of the British English single talk sequence according to clause 7 of ITU-T P.501 [8]. In order to reduce testing time, in most cases a shorter measurement signal according to Annex D of ITU-T P.501 [8]

In order to further minimize testing time and to re-use recordings as often as possible, all HATS measurements were conducted without DRP-ERP or DF-correction. For the analysis of receive frequency responses (RFR) and speech quality according to ITU-T P.863 [9], DF-correction according to ITU-T P.58 [5] was applied. For the calculation of receive loudness ratings, DRP-ERP-correction according to ITU-T P.57 [6] was applied.

According to the test plan [1], All DUTs were connected via 3G/UMTS to the test equipment, calls were setup with AMR-WB codec at 12,65 kbit/s.

An application force of 8 N was used for all tests.

The test suite used for the evaluation provided an optional automated volume control check, which sweeps all possible volume settings of a device (via Bluetooth remote control). Volume control settings for maximum (MAX) and nominal (NOM) loudness ratings were determined with the short British English test sequence according to Annex D of ITU-T P.501 [8]. In addition, the resulting maximum volume steps were manually double-checked by counting the possible volume steps (from minimum to maximum) during a call.

## Test 1: Speech Quality

For speech quality testing at nominal and maximum volume, eight sentences of British English speech (two female, two male talkers) from ITU-T P.501 [8] were concatenated to an overall sequence of 32.0s. The source signal for the receive direction was pre-filtered to wideband and then calibrated to an active speech level (ASL) according to ITU-T P.56 [10] of -16 dBm0.

The recording is then analyzed with the speech quality prediction method according to ITU-T P.863 [9] in super-wideband mode (version 2.4), analyzing sentences pairs. The resulting four MOS values are averaged to an overall result value.

## Test 2: Privacy

As a measure for acoustic sound radiation of the devices, the differences in RLR between handset mode and far-field (measured at a radius of 42 cm in front of the HATS EEP) is evaluated as shown in Figure 1. Different angles between symmetry plane of the HATS and the measurement microphone are used for testing, i.e. points A, B, C, D and E as defined in Table 1. The test is run with maximum volume setting.

dHF = 42cm

Figure 1: Test setup for privacy

Table 1: Angles for testing privacy

| Measuring position | Measuring angle (starting from E) |
| --- | --- |
| A | -180° |
| B | -135° |
| C | -90° |
| D | -45° |
| E | 0° |

## Test 3a: Robustness (variation of ECRP)

To investigate the robustness of the (chosen) ECRP, several shifts of 1 cm in Ze and Ye direction are evaluated. Figure 2 shows the resulting grid located around the default ECRP (marked in green). As defined in the test plan [1], four mandatory (marked in red) and four optional (marked in blue) points are specified. Table 2 provides unique labels (S0-S8) for the shifts to be applied.

RLR and RFR are evaluated with the short speech sequence as defined in clause 3.1. The test is run with nominal volume setting.

Figure 2: Variation of ECRP

Table 2: Shifts around ECRP (=S0)

| Shift | Offset Ze [mm] | Offset Ye [mm] |
| --- | --- | --- |
| S0 | 0 | 0 |
| S1 | -10 | -10 |
| S2 | +10 | -10 |
| S3 | +10 | +10 |
| S4 | -10 | +10 |
| S5 | 0 | -10 |
| S6 | +10 | 0 |
| S7 | 0 | +10 |
| S8 | -10 | 0 |

## Test 3b: Robustness (variation of fork position)

To investigate the impact of the clamping fork positions, the round robin test evaluates three different positions. Initial results from lab1 [3] defined these three positions as offsets in Ye-axis. The positioning strategy for the forks in shown in Table 3 and is not bound to a vendor-specific handset positioner.

Table 3: Fork positions

|  | Bottom | Middle | Top |
| --- | --- | --- | --- |
| Fork position #1 |  |  |  |
| Fork position #2 |  |  |  |
| Fork position #3 |  |  |  |

All measurements except Test 3b are conducted with position #1. Only Test3b evaluates positions #2 and #3. Values for bottom, middle and top fork position are provided in Table 4. During the testing, several positions were found to be not reachable with the utilized handset positioner and slightly different positions were chosen. In addition, the mounting of forks has a limited reproducibility (~ +/- 3mm). Note that the number of possible fork positions can be limited due to multiple buttons present at one or both sides of the phones and/or construction of the handset positioner.

Table 4: Fork positions for DUTs

|  | Bottom [mm] | Middle [mm] | Top [mm] |
| --- | --- | --- | --- |
| DUT1 | 25 | 95 | 145 |
| DUT2 | 25 | 95 | 155 |
| DUT3 | 25 | 95 | 145 |
| DUT4 | 25 | 95 | 150 |
| DUT5 | 30 | 100 | 140 |
| DUT6 | 70 | 105 | 150 |
| DUT7 | 25 | 105 | 150 |
| DUT8 | 25 | 80 | 130 |
| NOTE 1: DUT6 could not adequately be mounted in fork positions #2 and #3 due to the slippery surface, arrangement of button positions and overall size (very long and wide). Alternative middle and top positions are currently investigated. |  |  |  |

RLR and RFR are evaluated with the short speech sequence as defined in clause 3.1. The test is run with nominal volume setting.

# Results

## Preparations

As described in clause 3.1, the volume steps for nominal and maximum loudness ratings were determined. Table 5 provides the results of these preparation measurements, as well as the nominal (requirement: -1 to 5 dB, target: 2 dB) and maximum RLR (requirement: >-13 dB). Note that the nominal RLR was measured with the default test sequence according to 3GPP TS 26.132 [7]. RLR values not meeting the requirements are marked in red. Some DUTs showed strange behaviour here, which is described in the table notes below.

Table 5: Volume settings for MAX and NOM

|  | MAX | NOM | RLR-MAX | RLR-NOM |
| --- | --- | --- | --- | --- |
| DUT1 | 7 | 4 | -8.3 | 1.9 |
| DUT2 | 7 | 6 | -5.7 | -3.0 |
| DUT3 | 14 | 6 | -13.3 | 2.1 |
| DUT4 | 14 | 7 | -15.9 | 0.9 |
| DUT5 | 6 | 2 | -10.8 | 1.8 |
| DUT6 | 10 | 6 | -10.7 | 2.6 |
| DUT7 | 5 | 1 | -10.4 | 3.0 |
| DUT8 | 15 | 6 | -7.1 | 1.9 |
| NOTE 1: DUT2 does not provide a valid nominal volume setting. During incremental testing, RLR "jumps" from ~8.5 dB at step 5/7 to ~-2.5 dB at step 6/7. NOTE 2: DUT4 seems to have an incorrect implementation of volume control. At a certain volume increase step, the recorded signal level decreased. This was confirmed by measurements as well as by expert listening. NOTE 3: DUT5 provides a “boost mode” in the volume control. This mode was disabled / not used for testing. |  |  |  |  |

## Test 1: Speech Quality

Table 6 provides the results of the speech quality testing according to ITU-T P.863 for NOM and MAX volume settings. In several cases, the decrease in quality between these two volume settings is more than 0.5 MOS.

Table 6: P.863 results for MAX and NOM settings

|  | MAX [MOS] | NOM [MOS] | MAX-NOM [MOS] |
| --- | --- | --- | --- |
| DUT1 | 2.7 | 3.4 | -0.7 |
| DUT2 | 3.0 | 3.1 | -0.1 |
| DUT3 | 3.6 | 4.1 | -0.5 |
| DUT4 | 3.6 | 3.7 | -0.1 |
| DUT5 | 3.6 | 3.8 | -0.2 |
| DUT6 | 2.7 | 2.8 | -0.1 |
| DUT7 | 2.9 | 3.5 | -0.6 |
| DUT8 | 3.0 | 3.1 | -0.1 |

## Test 2: Privacy

Table 7 provides the results in RLR for privacy testing. Note that the RLR at the DUT and at the microphone positions is measured with the short test sequence (see clause 3.1). In order to fully assess the sound radiation of the devices, Table 8 provides results in terms of an attenuation in dB (RLR@Mic - RLR-MAX@DUT).

Table 7: RLR results for privacy testing (in dB)

|  | RLR-MAX @DUT | RLR at measurement microphone |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | A | B | C | D | E |  | MIN | MAX | MAX-MIN |
| DUT1 | -8.4 | 30.1 | 29.6 | 27.3 | 26.8 | 30.9 |  | 26.8 | 30.9 | 4.1 |
| DUT2 | -5.4 | 28.6 | 26.9 | 24.8 | 26.5 | 26.4 |  | 24.8 | 28.6 | 3.8 |
| DUT3 | -13.3 | 30.6 | 27.6 | 25.4 | 26.8 | 29.6 |  | 25.4 | 30.6 | 5.2 |
| DUT4 | -15.7 | 25.1 | 21.4 | 19.7 | 23.0 | 26.3 |  | 19.7 | 26.3 | 6.6 |
| DUT5 | -11.3 | 24.0 | 20.9 | 18.4 | 21.1 | 23.1 |  | 18.4 | 24.0 | 5.7 |
| DUT6 | -9.9 | 29.8 | 27.4 | 25.0 | 27.2 | 29.1 |  | 25.0 | 29.8 | 4.8 |
| DUT7 | -11.1 | 21.4 | 19.5 | 18.6 | 21.8 | 21.8 |  | 18.6 | 21.8 | 3.2 |
| DUT8 | -7.2 | 31.3 | 28.3 | 26.3 | 27.8 | 30.8 |  | 26.3 | 31.3 | 4.9 |

Table 8: RLR attenuation results for privacy testing (in dB)

|  | RLR-MAX @DUT | Attenuation at measurement microphone |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | A | B | C | D | E |  | MIN | MAX | MAX-MIN |
| DUT1 | -8.4 | 38.5 | 38.0 | 35.7 | 35.2 | 39.2 |  | 35.2 | 39.2 | 4.1 |
| DUT2 | -5.4 | 34.0 | 32.3 | 30.2 | 31.9 | 31.8 |  | 30.2 | 34.0 | 3.8 |
| DUT3 | -13.3 | 43.9 | 41.0 | 38.8 | 40.2 | 42.9 |  | 38.8 | 43.9 | 5.2 |
| DUT4 | -15.7 | 40.8 | 37.1 | 35.4 | 38.7 | 42.0 |  | 35.4 | 42.0 | 6.6 |
| DUT5 | -11.3 | 35.3 | 32.2 | 29.6 | 32.4 | 34.3 |  | 29.6 | 35.3 | 5.7 |
| DUT6 | -9.9 | 39.7 | 37.3 | 34.9 | 37.1 | 39.0 |  | 34.9 | 39.7 | 4.8 |
| DUT7 | -11.1 | 32.5 | 30.6 | 29.6 | 32.8 | 32.8 |  | 29.6 | 32.8 | 3.2 |
| DUT8 | -7.2 | 38.5 | 35.6 | 33.5 | 35.0 | 38.1 |  | 33.5 | 38.5 | 4.9 |

## Test 3a: Robustness (variation of ECRP)

Table 9 provides the results of the RLR measurements for the robustness against shifts of ECRP. For RLR values meeting the requirements of TS 26.132 [7] (-1 to +5 dB), cells are colored in green (or in red for failing).

Table 9: RLR vs shifts

|  | S0 | S1 | S2 | S3 | S4 | S5 | S6 | S7 | S8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DUT1 | 1.4 | 6.1 | 1.6 | 4.8 | 7.1 | 4.9 | -0.4 | 6.6 | 2.2 |
| DUT2 | -2.8 | -4.9 | -3.1 | 5.8 | 6.4 | -4.1 | -1.5 | 6.6 | -3.1 |
| DUT3 | 3.1 | 6.6 | 2.1 | 8.9 | 14.4 | 4.0 | 1.1 | 11.1 | 6.5 |
| DUT4 | 0.9 | 0.3 | -3.5 | 9.8 | 11.4 | -1.5 | -0.5 | 11.0 | 2.7 |
| DUT5 | 2.7 | 2.3 | 7.7 | 14.9 | 12.6 | 4.2 | 6.1 | 13.3 | 2.1 |
| DUT6 | 3.2 | 6.7 | 8.4 | 9.3 | 9.2 | 6.8 | 3.4 | 9.1 | 3.9 |
| DUT7 | 2.3 | 5.4 | 11.9 | 7.3 | 5.8 | 7.6 | 3.8 | 6.8 | 2.2 |
| DUT8 | 1.7 | -3.1 | -5.5 | 36.4 | 49.7 | -3.8 | 40.0 | 21.9 | 1.8 |

## Test 3b: Robustness (variation of fork position)

Table 10 provides the results of the RLR measurements for the robustness against different fork positions. For RLR values meeting the requirements of TS 26.132 [7] (-1 to +5 dB), cells are colored in green (or in red for failing).

Table 10: RLR vs fork positions

|  | FP#1 | FP#2 | FP#3 |
| --- | --- | --- | --- |
| DUT1 | 1.4 | 0.4 | 0.8 |
| DUT2 | -2.8 | -5.7 | -5.2 |
| DUT3 | 3.1 | 2.8 | 2.8 |
| DUT4 | 0.9 | 1.2 | 0.9 |
| DUT5 | 2.7 | -2.4 | -2.5 |
| DUT6 | 3.2 |  |  |
| DUT7 | 2.3 | 0.5 | 2.7 |
| DUT8 | 1.7 | 2.0 | 2.0 |
| NOTE 1: Since the RLR of DUT2 determined at position #2 and #3 equals the maximum RLR of the position #1, the measurements will be repeated and paying attention to the volume control. NOTE 2: DUT6 could not adequately be mounted in fork positions #2 and #3 (see Table 4). |  |  |  |

# Conclusion

The present document presented preliminary results of the HaNTE round robin test from lab2, which were obtained according to the test plan [1].

For Tests 3a and 3b, the RFR measurements are available as well and will be provided in a revision of this contribution (or as a follow-up contribution).

As a slight modification to the original test plan  [1], the source proposes to use common volume settings for the round robin test in lab 3 and lab 4 (see Table 5).

# References

| [1] | 3GPP S4-201164, „Proposals for data collection of HaNTE – test methods“. |
| --- | --- |
| [2] | 3GPP SP-191212, „New WID on Handsets Featuring Non-Traditional Earpieces (HaNTE)“. |
| [3] | 3GPP S4-201096, „Results of HaNTE round robin tests in Lab 1“. |
| [4] | ITU-T Recommendation P.64, „Determination of sensitivity/frequency characteristics of local telephone systems,“ 06/2019. |
| [5] | ITU-T Recommendation P.58, „Head and torso simulator for telephonometry,“ 05/2013. |
| [6] | ITU-T Recommendation P.57, „Artificial ears,“ 12/2011. |
| [7] | 3GPP TS 26.132, „Speech and video telephony terminal acoustic test specification,“ Release 16.0.0. |
| [8] | ITU-T Recommendation P.501, „Test signals for use in telephonometry,“ 03/2017. |
| [9] | ITU-T Recommendation P.863, „Perceptual objective listening quality prediction,“ 03/2018. |
| [10] | ITU-T Recommendation P.56, „Objective measurement of active speech level,“ 12/2011. |