Source:	Samsung Electronics Co., Ltd.

Title:	Updated device types and functional structures for FS_5GSTAR

Agenda Item:	6.3

Document for:	Discussion and Agreement

# Introduction

This contribution is a revised and updated version of AHVIC-295, reflecting the comments received during the Video SWG telco on 15th September 2020.  Changes include:

- Updated device functional structure figures

- Addition of interface descriptions between functions

- Addition of references to Microsoft HoloLens 2, to understand the capabilities of a specific state of the art device currently available in the market

Objective #1 of FS_5GSTAR relates to defining device types and their functional structures:

- Provide formal definitions for the functional structures of AR glasses, classified as device types of XR5G-A4 (standalone) and XR5G-A2, A5 (wirelessly tethered) in TR 26.928, including their capabilities and constraints with respect to communication, computing and graphics processing, tracking, sensors, display, and power consumption

NOTE 1: Device type of XR5G-A3 (video see-through HMD) are not the primary scope of this study, but are not excluded per se.

Accordingly, a skeleton clause named Device Functional Structure was agreed in both the draft skeleton for TR and the Permanent Document.

This contribution aims to introduce descriptions on such device functional structures as a starting point for discussions on the topic.

# Device types

The three device types highlighted in objective #1 are referenced from TR 26.928; the relevant definitions and corresponding descriptions in TR 26.928 are copied below for ease of reference:

------------------------------------------------------------------------------------------------------------------------------------------------

Typical components of HMDs are listed as follows:

- Optical systems: Display and lenses

- Tracking sensors and possibly additional sensors

- Cameras

- XR related processing (summarized as XR engine) including GPUs, CPUs, ASICs (e.g. dedicated media encoding and decoding), etc.

- Communication functionalities, as for example provided by a 5G System

XR5G-A2 - Simple AR Wearable Glass wireless: Such device types are not available in 2019 and are still far out. They would include a minimum number of sensors, possibly cameras for AR localization, power supply, as well as a wireless modem for connectivity. XR processing, AR localization as well as network connectivity is provided by external means, e.g. a puck or a smartphone.

XR5G-A4 - AR Wearable Glass standalone: Such device types are not available in 2019 but are under consideration. For such devices, the 5G modem, power supply, as well as all media/XR processing is expected to be integrated in a single device.

XR5G-A5 - Smart AR Wearable Glass wireless: Such device types are not available in 2019 and are still far out. In addition to an XR5G-A2 device, such a device would include at least a certain amount of XR/Media processing capabilities such as encoders/decoders and XR processing.

| XR Type Number | XR Device Type Name | Tethering Examples | 5G Uu Modem | XR Engine Localization | Power Supply | Typical Max Avail Power |
| --- | --- | --- | --- | --- | --- | --- |
| XR5G-A2 | Simple AR Wearable Glass wireless tethering | 802.11ad/y, 5G sidelink. etc. | External | External | Internal | 0.5 – 2 W |
| XR5G-A4 | AR Wearable Glass standalone | n/a | XR device | XR device or Split | Internal | 2 - 4 W |
| XR5G-A5 | Smart AR Wearable Glass wireless tethering | 802.11ad/y, 5G sidelink. etc. | External | XR device or Split | Internal | 0.5 – 2 W |

------------------------------------------------------------------------------------------------------------------------------------------------

TR 26.928 gives a helpful overview of the device types that include AR wearable glasses, namely standalone AR glasses (XR5G-A4) and wireless tethered AR glasses (XR5G-A2 and XR5G-A5).

An important difference to highlight between XR5G-A2 and XR5G-A5 is the location of where the AR/MR related processing takes place (in the tethering device vs the AR glasses), although it may be difficult to mandate all AR/MR processing related functions into either one device or the other.

The fine line between XR5G-A2 and XR5G-A5 (the level of “split”) lies in the processing requirements of each AR/MR processing function, as well as the whole processing flow for each specific AR glasses service.  This can be further clarified by identifying the possible functions within the device functional structures.

It should also be noted that “split” also includes the leverage of cloud entities (e.g. MEC) for certain processing functions, implying that for the wireless tethered device types, AR/MR processing functions may be split between 3 different physical entities: the cloud, the tethering device (e.g. smartphone, puck), and the AR glasses.

# Device functional structures

In order to dive further into the device functional structures, we identify the different functions that are required for enabling AR glasses related services:

- Tracking and sensing

- Capturing

  - Vision camera: capturing (in addition to tracking and sensing) of the user’s surroundings for vision related functions

  - Media camera: capturing of scenes or objects for media data generation where required

NOTE: vision and media camera logical functions may be mapped to the same physical camera, or to separate cameras. Camera devices may also be attached to other device hardware (AR glasses or smartphone), or exist as a separate external device.

- AR/MR media processing

  - Codecs: encoder and decoders for the encoding and decoding of media data

  - Vision engine: engine which performs processing for AR related localisation, mapping etc., i.e. SLAM

  - Renderer: for the rendering of AR/MR media to a display

- Tethering and network interfaces for AR/MR content delivery

# Device configurations

Depending on the terminal device configuration, the functions identified above may exist exclusively in different physical entities, or may be duplicated between the different entities.

Example device functional structures are shown below.

NOTE: The intention of these figures is to provide a generic basis for device functional structures on which discussions and further work can be carried out (such as further mapping to service architectures etc.).

Device configuration #1

Device configuration #2

Device configuration #3

# Interfaces

# Based on device configuration #1, we provide further details on the interfaces between the different functions of the device configuration:

- Tracker/sensor output interface

Raw outputs from various tracking and sensor device components, namely IMUs (inertial measurement unit).  These outputs are typically sent as inputs to the vision engine, or to a cloud entity when cloud-based processing is utilised.

Current commercialised state of the art AR/MR devices might contain the following related components [X]:

- Accelerometer – used by the system to determine linear acceleration along the X, Y and Z axes and gravity

- Gyro – used by the system to determine rotations

- Magnetometer – used by  the system to estimate absolute orientation

- Camera output to Vision engine interface

Depending on the device hardware, different types of camera outputs can be used as inputs into the vision engine, these may include [X]:

- Visible light environment tracking cameras – typically gray-scale cameras used by a system for head tracking and map building

- Depth cameras (ToF)– these may be short-throw (near-depth) or long-throw (far-depth), depending on the application desired (e.g. hand tracking, or spatial mapping)

- Infrared (IR cameras) – used for eye tracking

- World facing RGB camera – used for image processing tasks and locating the camera’s position in and perspective on the scene

Microsoft HoloLens 2 includes 4 visible light cameras for head tracking, 2 IR cameras for eye tracking, 1-MP ToF depth camera for capturing depth, and one world facing RGB camera capable of 8-MP stills and 1080p30 video [X].

- Vision engine to AR/MR renderer interface

The vision engine provides all the information required for the AR/MR renderer to adapt the rendering for a consistent combination of virtual content with the real world [Y].  This information may be the output of vision engine processes such as spatial mapping, scene understanding, and room scan visualization [X].  Vision engine processes are typically SLAM related, differing depending on the specific device and/or platform implementation.

One typical output of the vision engine is the device/user pose information, which may include estimation properties depending on the service and application.

- AR/MR renderer to Display interface

The AR/MR rendering typically outputs a rendered 2D frame (per eye) for a given time instance according to the device/user’s current pose in his/her surrounding environment.  This rendered 2D frame is sent as an input to the device display.  In the future, it can be predicted that non-2D displays will require a different output from the AR/MR renderer in order to support 3D displays e.g., Light Field 3D display.

- Camera output to Media processor interface

RGB and depth cameras are be used to capture RGB/depth images and videos, which can be consumed as regular images and videos, or may be used as inputs to a media processor for further media processing.

- Media processor to AR/MR media codec interface

A media processor performs processes such as 3D modelling, in order to output uncompressed media data into the AR/MR media codec for encoding.  One example of the media data at this interface is raw point cloud media data (in a format such as a ply file).

- AR/MR media codec to AR/MR renderer interface

Compressed AR/MR media content intended for rendering and display is decoded by the AR/MR media codec, and fed into the AR/MR renderer.  This is typically a decoded video bitstream (for 2D AR/MR media), or a decoded 3D media bitstream (for 3D AR/MR media).

- AR/MR media codec to Network delivery interface

Media contents that are captured or generated by the device (from interface 5 and 6) are encoded by the AR/MR media codec before being passed onto the network delivery entity for packetization and delivery over the 5G network.  For 2D AR/MR media contents, this is typically a compressed video bitstream that is conformant to the video codec used by the AR/MR media codec.

Network delivery to AR/MR media codec interface

Media contents that are received through the network delivery interface over the 5G network are depacketized by the network delivery entity and fed into the AR/MR media codec.  The subsequent decoded bitstream is handled through interfaces 7 and 4.

- Network delivery interface

Network interface for AR/MR content delivery.

# Proposal

We propose to include the text and figures in section 3 of this document into clause 3 of the Permanent Document as a starting point for discussions on device functional structures.

# References

[X] https://docs.microsoft.com/en-us/windows/mixed-reality

[Y] ETSI GS ARF 003 V1.1.1 “Augmented Reality Framework (ARF); AR framework architecture”