3GPP TSG SA post WG4#114-e meeting	S4aV210701

8th June 2021

Agenda item: 	4.6

Source: 	InterDigital

Title: 	[5G Video] ETM bug

Document for	Information

# Introduction

In the context of the study on 5G Video codecs characteristics (FS_5GVideo) some ETM 6.1  issues were reported in S4-210205 and S4aV200643 and consisted of two aspects:

- Failing to encode some sequences at certain QP.

- A saturation at high bit rate (low PSNR at high bitrates).

This contribution provides an update now that ETM 7.2 and ETM 7.3 are available.

# ETM 6.1 vs ETM 7.2 / ETM 7.3

## QP issue for some sequences

This has been fixed, and we have been able to encode all sequences at all QPs with ETM 7.2.

## High bit rate behavior

We observe the same behavior between ETM 6.1 and ETM 7.2 at high bitrate:

| LDP-IDR |  | 5QP BD-rate (piecewise cubic) ETM 6.1 |  |  |  | 5QP BD-rate (piecewise cubic) ETM 7.2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Y | U | V | Y |  | U | V |
| MovingText2 | MovingText2-FullHD-420-8bit | -26.50% | -17.02% | -16.33% | -30.41% |  | -20.19% | -16.20% |
|  | MovingText2-FullHD-420-10bit | -26.13% | -16.40% | -16.75% | -30.06% |  | -19.70% | -17.04% |
|  | MovingText2-4K-420-8bit | -15.93% | -22.67% | -18.54% | -19.41% |  | -23.32% | -17.33% |
|  | MovingText2-4K-420-10bit | -15.75% | -22.98% | -21.08% | -19.15% |  | -23.26% | -17.51% |
| TextMixTransitions | TextMixTransitions-FullHD-420-8bit | -19.71% | -24.34% | -24.36% | -19.14% |  | -21.48% | -21.91% |
|  | TextMixTransitions-FullHD-420-10bit | -19.35% | -25.84% | -24.80% | -19.50% |  | -23.57% | -23.69% |
|  | TextMixTransitions-4K-420-8bit | -6.33% | -26.14% | -15.87% | -7.51% |  | -28.51% | -16.06% |
|  | TextMixTransitions-4K-420-10bit | -6.09% | -31.28% | -21.47% | -7.39% |  | -29.65% | -23.19% |

To clarify if the absence of transform skip in ETM was the reason for this behavior we ran some partial simulations on some of the JVET CTC sequences: one frame at QP:2, 7, 12 and 17.

ETM 6.1 versus HM 16.22:

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | #REF! |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class B | 1.94% | 1.50% | 2.14% | 2180% | #NUM! |
| Class C | 3.36% | 2.17% | 2.48% | 2763% | #NUM! |
| Class E | 1.99% | 1.16% | 1.89% | 2425% | #NUM! |
| Overall | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class D | 4.41% | 3.41% | 3.57% | 2893% | #NUM! |
| Class F | 13.53% | 11.40% | 11.94% | 2549% | #NUM! |

Partial simulations ETM 7.3 versus HM 16.22 without TS (one frame) :

Normal QP

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | #REF! |  |  |
|  | Y | U | V | EncT | DecT |
| Class B | -9.55% | -12.56% | -14.34% | 3162% | #NUM! |
| Class C | -7.02% | -6.65% | -9.00% | 4087% | #NUM! |
| Class E | -12.18% | -17.04% | -16.46% | 2616% | #NUM! |
| Overall | -9.36% | -11.71% | -13.09% | 3285% | #NUM! |
| Class D | -5.45% | -6.09% | -6.80% | 4134% | #DIV/0! |
| Class F | -7.55% | -9.67% | -10.23% | 3759% | #NUM! |

Low QP

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | #REF! |  |  |
|  | Y | U | V | EncT | DecT |
| Class B | 1.90% | 1.53% | 2.15% | 2502% | #NUM! |
| Class C | 3.20% | 2.08% | 2.43% | 3192% | #NUM! |
| Class E | 1.90% | 1.30% | 1.93% | 2623% | #NUM! |
| Overall | 2.33% | 1.66% | 2.19% | 2746% | #NUM! |
| Class D | 4.23% | 3.05% | 3.45% | 3169% | #NUM! |
| Class F | 5.74% | 3.02% | 3.62% | 3017% | #NUM! |

Slide Editing Low QP:

ETM 7.3 versus ETM 6.1 in LD-B (one frame):

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | #REF! |  |  |
|  | Y | U | V | EncT | DecT |
| Class B | 0.00% | 0.00% | 0.00% | 101% | #NUM! |
| Class C | 0.01% | -0.05% | 0.01% | 99% | #NUM! |
| Class E | -0.02% | 0.10% | -0.06% | 97% | #NUM! |
| Overall | 0.00% | 0.01% | -0.01% | 99% | #NUM! |
| Class D | -0.01% | -0.13% | -0.03% | 96% | #NUM! |
| Class F | -0.04% | 0.02% | 0.13% | 99% | #NUM! |

Gaëlle Martin-Cocher, Rémi Jullian, Tangi poirier