Source:	PEREY Research & Consulting, B<>Com, Orange, Nokia, Deutsch Telekom

Title:	FS_5GSTAR: Section 4.4.3 Spatial computing tracking components

Agenda Item:	4.8

Contact:	Jerome Royan jerome.royan@b-com.com, Christine Perey, cperey@perey.com, Padmakumar Subramani Padmakumar.subramani@nokia.com,  Igor Curcio igor.curcio@nokia.com, Jens Johann J.Johann@telekom.de

Document for:	Discussion and Agreement

# Introduction & Proposal

TR26.998 (version 0.1.0 released September 8, 2021) does not currently describe all the required components for the spatial computing service scenario. Trackables have not been defined or described in section 4.4 entitled “AR Content Formats and Codecs.”

This contribution is designed to answer the open questions that were unanswered in prior meetings when the Spatial computing service scenario description was proposed.

The purpose of this contribution is to describe the principles of Trackables, including 3D Maps, and to propose text to be used in TR26.998.

The text included in this contribution is adapted from the ETSI GS ARF 004-2 V0.0.4 (2021-06) Augmented Reality Framework (ARF) Interoperability Requirements for AR components, systems and services Part 2: World Storage and AR Authoring functions. The GS004-2 specification is available from the following link: https://docbox.etsi.org/ISG/ARF/Open/gs_arf00402v0.0.4.pdf

| Reason for change: | To include descriptions of Trackables and Anchor, including 3D Maps, which are required for the full description of the new spatial computing service scenario |
| --- | --- |
| Summary of change: | Definitions of trackables, including but not limited to 3D Maps and Anchor, and a clause about trackables |
| Consequences if not approved: | The spatial computing service scenario and all the required components will not be fully described. |

# Proposed changes

******CHANGE #1*****

Trackable: A trackable is a digital element with calculated features (e.g., mesh trackable, learnt trackable, map trackable, CAD trackable) or an element of the real world of whose features are available and/or could be extracted (e.g., fiducial marker, radio, UWB, image, sound). Features can be made available from an analysis of the element itself (fiducial markers, natural images, 3D point cloud) or processed from a representation of the element (3D CAD model).

Anchor: position and orientation of the real world to which AR assets are attached.

****END OF CHANGE #1****

******CHANGE #2*******

### 4.4.3 Trackable

### Trackables are used by a vision engine or AR function for the purpose of estimating the pose (pose estimation) of the trackable or the AR device and for world understanding (e.g. object recognition, or user activity analysis). Trackables provide a Coordinate Reference System in which a pose can be expressed. Due to different use case requirements, trackables for use by AR systems are available in a range of types. Nine types of trackables, shown in Figure 4.4.3-1, have been identified.

Figure 4.4.3-1: Types of trackables

### .

#### 4.4.3.1		Fiducial trackable

### A fiducial marker (or fiducial) is an object placed in the field of view of an imaging system that appears in the image produced, for use as a point of reference or a measure [source is Wikipedia].

### In order to provide scale, fiducial trackables include information about the trackable’s original size.  Patterns, such as a grid of black and white boxes, can be used to define the ID of the fiducial trackable.

#### 4.4.3.2	Image trackable

### An image trackable is an image combined with its size and resolution. 2D geometric primitives of interest (e.g. points or segments) are detected in an image trackable. Each 2D geometric primitive is associated to a descriptor to be used by the computing system [e.g. Scale-Invariant Feature Transform (SIFT), Speeded Up Robust Features (SURF), Binary Robust Independent Elementary Features (BRIEF), Oriented FAST and rotated BRIEF (ORB), or Accelerated-KAZE (AKAZE)] to characterize the 2D geometric primitive in the most discriminating way while being invariant to rotation, scale or illumination. Image geometric primitives with their associated descriptors represent the features extracted from an image trackable. At runtime, these features are matched to the ones extracted from a given image captured by a camera of an AR device. From these matches, and knowing the position of the features extracted from the image trackable in relation to the real world, the 3D pose of the vision sensor is estimated.

#### 4.4.3.3	Mesh and CAD trackable

#### Mesh and Computer-aided Design (CAD) trackables are composed of a 3D model from which 2D or 3D features have been extracted. At runtime, these features are matched with the ones extracted from the vision sensor embedded in the AR device. From these matches, and knowing the position of the features extracted from the mesh or CAD trackable in relation to the real world, the 3D pose of the vision sensor is estimated.

#### 4.4.3.4	3D Map trackable

### A 3D Map trackable is a set of 3D features previously built by an AR device.

### Most modern AR system are able to build sparse 3D maps using Simultaneous Localization And Mapping (SLAM). These sparse 3D maps are created during the mapping step by triangulating matching features extracted from images captured by the vision sensor embedded in the AR device. This mapping process can be performed either at runtime or offline. A 3D map trackable is then used at runtime to relocalize the AR device by matching the features extracted from the current image with the ones stored in the previously built 3D map. In order to accelerate the searching and matching step for large environment, an optimization is necessary. The process of optimization consists of retrieving an image used during the mapping step from a requested image captured at runtime, and then matching keypoints common to both the current and the retrieved image instead of all the 3D points of the map. Thus, in addition to the 3D feature cloud, this optimization requires storing the retrieval image model (e.g. a bag of word model), the 2D features for each image used during the reconstruction (to match features between the current and the retrieved image) as well as the correspondences between these 2D features and the 3D points (to get the correspondences between the 2D features of the current image and the 3D points stored in the 3D Map Trackable). In this case, the map file size for an area of a hundred square metres may reach or exceed the storage capacity of the AR device.

### Some AR systems are using a dense 3D map of the real world. Using readings generated by a device’s depth sensor, pose of the AR device can be calculated by comparing the depth image capture by the AR device with the dense 3D map. Although this approach is processing intensive, the fact that it is based purely on the 3D geometry of the scene means using this trackable with depth sensing does not suffer as heavily from changes in appearance of the user’s environment over time.

#### 4.4.3.5	Radio and UWB trackable

#### Radio and UWB trackables make use of the distribution characteristics of electromagnetic waves and typically differ in frequency range and achievable pattern resolutions. Two types of Radio and UWB trackables can be used for positioning an AR device. The first solution is to register a fingerprint of the wave signal received by the Radio or UWB receptor (tag) from the Radio or UWB emitters (anchors) for a large number of positions in the real environment. In real-time, the wave signal captured by the receiver is compared to the fingerprint to estimate the position of the AR device. The second approach is to measure the distance between the tag and the anchors based on the time-of-flight of the signal. Knowing the position of the anchors, the position of the tag can be estimated by triangulation.

#### 4.4.3.5	Learnt trackable

#### Learnt trackables are trackables based on machine learning approaches, and are composed of a learnt model (e.g. deep neural network or a random forest).

### Some models using learnt descriptors characterize the features used by previous trackable types. Others learnt 2D/2D, 2D/3D or 3D/3D correspondences to improve the matching step. Finally, some learnt models can directly infer the pose from one or several images captured by the vision sensors in an end-to-end manner. Thus, a learnt model can be used as a trackable to estimate the pose of an AR device, or to recognize an object in the real environment.

****END OF CHANGE #2****