3GPP TSG-SA/WG4 Video SWG 	S4aV210809

Electronic meeting, October 19, 2021

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PSEUDO CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.955 | CR | - | rev | - | Current version: | 1.3.4 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Configuration Settings for AV1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: |  |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_5GVideo |  |  |  |  |  | Date: |  |  | 2021-10-17 |
| Category: |  |  |  |  |  |  | Release: |  |  | 17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Enable verification of AV1 performance tests |  |  |  |  |  |  |  |  |
| Summary of change: |  | Identification of command line options used to generate performance evaluation results for AV1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## 8.4	AV1

### 8.4.3	Test Configurations and Results for Libaom

#### 8.4.3.1	Introduction

...

Change 1

Several changes had to be introduced to the libaom reference software to match the requirements and needs of 3GPP and for this test. A specific release of that software containing these changes, which should be used for the 3GPP testing, can be found by using the following git release tag:

3gpp-2021-10-15

The AV1 encoder that is built using the AV1 libraries is aomenc. Command line options are used to configure the encoding performed by aomenc.

A complete description of all command line options used to generate the bitstreams referenced in this technical report can be found here:

Editor’s note: 	A link to the online publication of “aomenc - command line parameters” will be added shortly. In the meantime, a draft of the specification has been provided as a contribution.

Depending on the testing scenario, libaom needs to be configured differently with appropriate command line configuration settings. Such configuration settings are documented individually and provided in independent Linux shell script files for convenience. These attached configuration scripts take arguments to generate the tuples. For example, S1-AV1-01.sh, S5-AV1-SCC-01.sh, etc. Arguments to be used with each configuration script are listed in the sections, below.

Here are the common command line options used by all of AV1 configuration file scripts, along with an explanation of their use.

| --cpu-used=0 | Instruct the encoder to use settings to achieve the best compression efficiency. |
| --- | --- |
| --enable-keyframe-filtering=1 | Enable temporal filtering for Key frame |
| --enable-tpl-model=0 | disable RDO based temporal layer model so that fixed hierarchy is used. |
| --end-usage=q | Instruct encoder to use the constant quality mode |
| --frame-parallel=0 | disable the support for parallel decoding, to achieve the best compression efficiency. |
| --gf-min-pyr-height=5 --gf-max-pyr-height=5 | set the number of hierarchical layers to be 6 [0-5] |
| --min-gf-interval=32 --max-gf-interval=32 | Set the sub-GOP size to 32 frames. |
| --passes=1 | Use single pass encoding |
| --tile-columns=0 | disable column tiles, so that the entire frame is encoded as a single tile to achieve the best compression efficiency. |
| --codec=av1 | Specify that the Alliance for Open Media codec version to be used is AV1 |
| --verbose, -v | display encoding parameters for information only. |
| --psnr | enabled encoder to calculate PSNR using a reconstructed frame after encoding. This is for information only. |
| --i420 | all input video is in 420 color format |
| --input-bit-depth=10 | specifies the bit depth of the input video |
| --bit-depth=10 | specifies the bit depth of the encoded video to be 10 bit. since all input videos specified by 3GPP are converted to 10 bit before encoding, so both input bit depth and encoding bit depth are specified to be 10 bit. |
| --threads=1 | using single threading mode for encoding to achieve the best compression efficiency. |
| --obu | Open Bitstream Unit (OBU) container format is used as the output bitstream container format. |
| --fps | Specifies the frame rate of the input video. Frame rate value is represented as rational number as “numerator/denominator”. For example, if the frame rate is 29.97, 30000/1001 should be used. If frame rate is 59.94, 60000/1001 should be used. If frame rate is 30, 30000/1000 should be used. |

#### 8.4.3.2	Scenario 1: Full HD Streaming

##### 8.4.3.2.1	Overview

Table 8.4.3.2.1-1 provides an overview of the AV1 test tuples. For provided bitstreams, the AV1 encoder aomenc is built from the libaom libraries using the tag provided in the introduction. Keys are identified to refer to the bitstreams in the context of the scenario.

Table 8.4.3.2.1-1  Test Tuple generation with AV1 for Full HD Scenario

| Key | Clause | Reference Sequence | Reference Encoder | Configuration | Variations | Anchor Key |
| --- | --- | --- | --- | --- | --- | --- |
| S1-A01-AV1 | 8.4.3.2.3 | S1-R1 | aomenc <tag> | S1-AV1-01 | QP: [27,39,49,59,63] | S1-A01-AV1-<QP> |
| S1-A02-AV1 | 8.4.3.2.3 | S1-R2 | aomenc <tag> | S1-AV1-01 | QP: [27,39,49,59,63] | S1-A02-AV1-<QP> |
| S1-A03-AV1 | 8.4.3.2.3 | S1-R3 | aomenc <tag> | S1-AV1-01 | QP: [27,39,49,59,63] | S1-A03-AV1-<QP> |
| S1-A04-AV1 | 8.4.3.2.3 | S1-R4 | aomenc <tag> | S1-AV1-01 | QP: [27,39,49,59,63] | S1-A04-AV1-<QP> |
| S1-A05-AV1 | 8.4.3.2.3 | S1-R5 | aomenc <tag> | S1-AV1-01 | QP: [27,39,49,59,63] | S1-A05-AV1-<QP> |
| S1-A06-AV1 | 8.4.3.2.3 | S1-R6 | aomenc <tag> | S1-AV1-01 | QP: [27,39,49,59,63] | S1-A06-AV1-<QP> |
| S1-A07-AV1 | 8.4.3.2.3 | S1-R7 | aomenc <tag> | S1-AV1-01 | QP: [27,39,49,59,63] | S1-A07-AV1-<QP> |
| S1-A08-AV1 | 8.4.3.2.3 | S1-R8 | aomenc <tag> | S1-AV1-01 | QP: [27,39,49,59,63] | S1-A08-AV1-<QP> |
| S1-A11-AV1 | 8.4.3.2.4 | S1-R11 | aomenc <tag> | S1-AV1-02 | QP: [27,39,49,59,63] | S1-A11-AV1-<QP> |
| S1-A12-AV1 | 8.4.3.2.4 | S1-R12 | aomenc <tag> | S1-AV1-02 | QP: [27,39,49,59,63] | S1-A12-AV1-<QP> |
| S1-A13-AV1 | 8.4.3.2.4 | S1-R13 | aomenc <tag> | S1-AV1-02 | QP: [27,39,49,59,63] | S1-A13-AV1-<QP> |
| S1-A14-AV1 | 8.4.3.2.4 | S1-R14 | aomenc <tag> | S1-AV1-02 | QP: [27,39,49,59,63] | S1-A14-AV1-<QP> |
| S1-A15-AV1 | 8.4.3.2.4 | S1-R15 | aomenc <tag> | S1-AV1-02 | QP: [27,39,49,59,63] | S1-A15-AV1-<QP> |
| S1-A16-AV1 | 8.4.3.2.4 | S1-R16 | aomenc <tag> | S1-AV1-02 | QP: [27,39,49,59,63] | S1-A16-AV1-<QP> |
| S1-A17-AV1 | 8.4.3.2.4 | S1-R17 | aomenc <tag> | S1-AV1-02 | QP: [27,39,49,59,63] | S1-A17-AV1-<QP> |

##### 8.4.3.2.2	Common Parameters and Settings

For both SDR and HDR the following common Scenario 1 command line options are used.

| --auto-alt-ref=1 | parameter to specify how alternative reference frames are determined in random access configuration. only 1 is supported. |
| --- | --- |
| --lag-in-frames=35 | This parameter is typically set to be the sub-GOP size + 3. Since sub-GOP size is 32, this parameter is set to be 35. |
| --use-fixed-qp-offsets=1 | use fixed ratios to determine the QP for frames at different hierarchical layers. In av1 encoder, cq-level specified is only used for the frames at the highest temporal layer (non-reference frames), for Key frames and Inter frames at other layers, a fixed ratio is used to determine the quantization parameter. |

IntraPeriod: Intra Period aligned with GOPSize such that approximately 1 second is achieved, i.e.

- "frameRate": 23.98 or 24.0 or 25 or 30 => IntraPeriod set to 32,

- "frameRate": 50.0 or 59.94 or 60 => IntraPeriod set to 64

##### 8.4.3.2.3	S1-AV1-01: SDR Settings

For SDR content the following command line option is used.

| --deltaq-mode=0 | disable block based adaptive quantization, so all blocks within a frame use the same quantization parameter |
| --- | --- |

S1-AV1-01.sh <num_frame> <fps_norm> <fps_denom> <width> <height> <cq-level> <intra_period> <input> <output>

##### 8.4.3.2.4	S1-AV1-02: HDR PQ Settings

For SDR content the following command line option is used.

| --deltaq-mode=5 | enable chroma QP offset adjustment for HDR video. Only frame level chroma QP offset is enabled. |
| --- | --- |
| --chroma-sample-position=colocated | specify chroma sample position to be colocated |

S1-AV1-02.sh <num_frame> <fps_norm> <fps_denom> <width> <height> <cq-level> <intra_period> <input> <output>

##### 8.4.3.2.5	AV1 Results

Editor’s Note: to be completed.

#### 8.4.3.3	Scenario 2: 4K-TV

##### 8.4.3.3.1	Overview

Table 8.4.3.3.1-1 provides an overview of the AV1 test tuples. For provided bitstreams, the AV1 encoder aomenc is built from the libaom libraries using the tag provided in the introduction. Keys are identified to refer to the bitstreams in the context of the scenario.

Table 8.4.3.3.1-1  Test Tuple generation with AV1 for 4K-TV Scenario

| Key | Clause | Reference Sequence | Reference Encoder | Configuration | Variations | Anchor Key |
| --- | --- | --- | --- | --- | --- | --- |
| S2-A01-AV1 | 8.4.3.3.3 | S2-R1 | aomenc <tag> | S2-AV1-01 | QP: [27,39,49,59,63] | S2-A01-AV1-<QP> |
| S2-A02-AV1 | 8.4.3.3.3 | S2-R2 | aomenc <tag> | S2-AV1-01 | QP: [27,39,49,59,63] | S2-A02-AV1-<QP> |
| S2-A03-AV1 | 8.4.3.3.3 | S2-R3 | aomenc <tag> | S2-AV1-01 | QP: [27,39,49,59,63] | S2-A03-AV1-<QP> |
| S2-A04-AV1 | 8.4.3.3.3 | S2-R4 | aomenc <tag> | S2-AV1-01 | QP: [27,39,49,59,63] | S2-A04-AV1-<QP> |
| S2-A05-AV1 | 8.4.3.3.3 | S2-R5 | aomenc <tag> | S2-AV1-01 | QP: [27,39,49,59,63] | S2-A05-AV1-<QP> |
| S2-A06-AV1 | 8.4.3.3.3 | S2-R6 | aomenc <tag> | S2-AV1-01 | QP: [27,39,49,59,63] | S2-A06-AV1-<QP> |
| S2-A07-AV1 | 8.4.3.3.3 | S2-R7 | aomenc <tag> | S2-AV1-01 | QP: [27,39,49,59,63] | S2-A07-AV1-<QP> |
| S2-A08-AV1 | 8.4.3.3.3 | S2-R8 | aomenc <tag> | S2-AV1-01 | QP: [27,39,49,59,63] | S2-A08-AV1-<QP> |
| S2-A11-AV1 | 8.4.3.3.4 | S2-R11 | aomenc <tag> | S2-AV1-02 | QP: [27,39,49,59,63] | S2-A11-AV1-<QP> |
| S2-A12-AV1 | 8.4.3.3.4 | S2-R12 | aomenc <tag> | S2-AV1-02 | QP: [27,39,49,59,63] | S2-A12-AV1-<QP> |
| S2-A13-AV1 | 8.4.3.3.4 | S2-R13 | aomenc <tag> | S2-AV1-02 | QP: [27,39,49,59,63] | S2-A13-AV1-<QP> |
| S2-A14-AV1 | 8.4.3.3.4 | S2-R14 | aomenc <tag> | S2-AV1-02 | QP: [27,39,49,59,63] | S2-A14-AV1-<QP> |
| S2-A15-AV1 | 8.4.3.3.4 | S2-R15 | aomenc <tag> | S2-AV1-02 | QP: [27,39,49,59,63] | S2-A15-AV1-<QP> |
| S2-A16-AV1 | 8.4.3.3.4 | S2-R16 | aomenc <tag> | S2-AV1-02 | QP: [27,39,49,59,63] | S2-A16-AV1-<QP> |
| S2-A17-AV1 | 8.4.3.3.4 | S2-R17 | aomenc <tag> | S2-AV1-02 | QP: [27,39,49,59,63] | S2-A17-AV1-<QP> |

##### 8.4.3.3.2	Common Parameters and Settings

For both SDR and HDR the following common Scenario 2 command line options are used.

| --auto-alt-ref=1 | parameter to specify how alternative reference frames are determined in random access configuration. only 1 is supported. |
| --- | --- |
| --lag-in-frames=35 | This parameter is typically set to be the sub-GOP size + 3. Since sub-GOP size is 32, this parameter is set to be 35. |
| --use-fixed-qp-offsets=1 | use fixed ratios to determine the QP for frames at different hierarchical layers. In av1 encoder, cq-level specified is only used for the frames at the highest temporal layer (non-reference frames), for Key frames and Inter frames at other layers, a fixed ratio is used to determine the quantization parameter. |

IntraPeriod: Intra Period aligned with GOPSize such that approximately 1 second is achieved, i.e.

- "frameRate": 23.98 or 24.0 or 25 or 30 => IntraPeriod set to 32,

- "frameRate": 50.0 or 59.94 or 60 => IntraPeriod set to 64

##### 8.4.3.3.3	S2-AV1-01: SDR Settings

For SDR content the following command line option is used.

| --deltaq-mode=0 | disable block based adaptive quantization, so all blocks within a frame use the same quantization parameter |
| --- | --- |

S2-AV1-01.sh <num_frame> <fps_norm> <fps_denom> <width> <height> <cq-level> <intra_period> <input> <output>

##### 8.4.3.3.4	S2-AV1-02: HDR PQ Settings

For SDR content the following command line option is used.

| --deltaq-mode=5 | enable chroma QP offset adjustment for HDR video. Only frame level chroma QP offset is enabled. |
| --- | --- |
| --chroma-sample-position=colocated | specify chroma sample position to be colocated |

S2-AV1-02.sh <num_frame> <fps_norm> <fps_denom> <width> <height> <cq-level> <intra_period> <input> <output>

##### 8.4.3.3.5	Test Results

Editor’s Note: to be completed.

#### 8.4.3.4	Scenario 3: Screen Content Scenario

##### 8.4.3.4.1	Overview

Table 8.4.3.4.1-1 provides an overview of the AV1 test tuples. For provided bitstreams, the AV1 encoder aomenc is built from the libaom libraries using the tag provided in the introduction. Keys are identified to refer to the bitstreams in the context of the scenario.

Table 8.4.3.4.1-1 Test Tuple generation with AV1 for Screen Content Scenario

| Key | Clause | Reference Sequence | Reference Encoder | Configuration | Variations | Anchor Key |
| --- | --- | --- | --- | --- | --- | --- |
| S3-A01-AV1 | 8.4.3.4.3 | S3-R01 | aomenc <tag> | S3-AV1-01 | QP=[27,39,49,59,63] | S3-A01-AV1-<QP> |
| S3-A02-AV1 | 8.4.3.4.3 | S3-R02 | aomenc <tag> | S3-AV1-01 | QP=[27,39,49,59,63] | S3-A02-AV1-<QP> |
| S3-A03-AV1 | 8.4.3.4.3 | S3-R03 | aomenc <tag> | S3-AV1-01 | QP=[27,39,49,59,63] | S3-A03-AV1-<QP> |
| S3-A04-AV1 | 8.4.3.4.3 | S3-R04 | aomenc <tag> | S3-AV1-01 | QP=[27,39,49,59,63] | S3-A04-AV1-<QP> |
| S3-A05-AV1 | 8.4.3.4.3 | S3-R05 | aomenc <tag> | S3-AV1-01 | QP=[27,39,49,59,63] | S3-A05-AV1-<QP> |
| S3-A06-AV1 | 8.4.3.4.3 | S3-R06 | aomenc <tag> | S3-AV1-01 | QP=[27,39,49,59,63] | S3-A06-AV1-<QP> |
| S3-A07-AV1 | 8.4.3.4.3 | S3-R07 | aomenc <tag> | S3-AV1-01 | QP=[27,39,49,59,63] | S3-A07-AV1-<QP> |
| S3-A08-AV1 | 8.4.3.4.3 | S3-R08 | aomenc <tag> | S3-AV1-01 | QP=[27,39,49,59,63] | S3-A08-AV1-<QP> |
| S3-A09-AV1 | 8.4.3.4.3 | S3-R09 | aomenc <tag> | S3-AV1-01 | QP=[27,39,49,59,63] | S3-A09-AV1-<QP> |
| S3-A10-AV1 | 8.4.3.4.3 | S3-R10 | aomenc <tag> | S3-AV1-01 | QP=[27,39,49,59,63] | S3-A10-AV1-<QP> |
| S3-A11-AV1 | 8.4.3.4.3 | S3-R11 | aomenc <tag> | S3-AV1-01 | QP=[27,39,49,59,63] | S3-A11-AV1-<QP> |
| S3-A12-AV1 | 8.4.3.4.3 | S3-R12 | aomenc <tag> | S3-AV1-01 | QP=[27,39,49,59,63] | S3-A12-AV1-<QP> |
| S3-A13-AV1 | 8.4.3.4.3 | S3-R13 | aomenc <tag> | S3-AV1-01 | QP=[27,39,49,59,63] | S3-A13-AV1-<QP> |
| S3-A14-AV1 | 8.4.3.4.3 | S3-R14 | aomenc <tag> | S3-AV1-01 | QP=[27,39,49,59,63] | S3-A14-AV1-<QP> |
| S3-A15-AV1 | 8.4.3.4.3 | S3-R15 | aomenc <tag> | S3-AV1-01 | QP=[27,39,49,59,63] | S3-A15-AV1-<QP> |
| S3-A16-AV1 | 8.4.3.4.3 | S3-R16 | aomenc <tag> | S3-AV1-01 | QP=[27,39,49,59,63] | S3-A16-AV1-<QP> |
| S3-A17-AV1 | 8.4.3.4.4 | S3-R01 | aomenc <tag> | S3-AV1-02 | QP=[27,39,49,59,63] | S3-A17-AV1-<QP> |
| S3-A18-AV1 | 8.4.3.4.4 | S3-R02 | aomenc <tag> | S3-AV1-02 | QP=[27,39,49,59,63] | S3-A18-AV1-<QP> |
| S3-A19-AV1 | 8.4.3.4.4 | S3-R03 | aomenc <tag> | S3-AV1-02 | QP=[27,39,49,59,63] | S3-A19-AV1-<QP> |
| S3-A20-AV1 | 8.4.3.4.4 | S3-R04 | aomenc <tag> | S3-AV1-02 | QP=[27,39,49,59,63] | S3-A20-AV1-<QP> |
| S3-A21-AV1 | 8.4.3.4.4 | S3-R05 | aomenc <tag> | S3-AV1-02 | QP=[27,39,49,59,63] | S3-A21-AV1-<QP> |
| S3-A22-AV1 | 8.4.3.4.4 | S3-R06 | aomenc <tag> | S3-AV1-02 | QP=[27,39,49,59,63] | S3-A22-AV1-<QP> |
| S3-A23-AV1 | 8.4.3.4.4 | S3-R07 | aomenc <tag> | S3-AV1-02 | QP=[27,39,49,59,63] | S3-A23-AV1-<QP> |
| S3-A24-AV1 | 8.4.3.4.4 | S3-R08 | aomenc <tag> | S3-AV1-02 | QP=[27,39,49,59,63] | S3-A24-AV1-<QP> |
| S3-A25-AV1 | 8.4.3.4.4 | S3-R09 | aomenc <tag> | S3-AV1-02 | QP=[27,39,49,59,63] | S3-A25-AV1-<QP> |
| S3-A26-AV1 | 8.4.3.4.4 | S3-R10 | aomenc <tag> | S3-AV1-02 | QP=[27,39,49,59,63] | S3-A26-AV1-<QP> |
| S3-A27-AV1 | 8.4.3.4.4 | S3-R11 | aomenc <tag> | S3-AV1-02 | QP=[27,39,49,59,63] | S3-A27-AV1-<QP> |
| S3-A28-AV1 | 8.4.3.4.4 | S3-R12 | aomenc <tag> | S3-AV1-02 | QP=[27,39,49,59,63] | S3-A28-AV1-<QP> |
| S3-A29-AV1 | 8.4.3.4.4 | S3-R13 | aomenc <tag> | S3-AV1-02 | QP=[27,39,49,59,63] | S3-A29-AV1-<QP> |
| S3-A30-AV1 | 8.4.3.4.4 | S3-R14 | aomenc <tag> | S3-AV1-02 | QP=[27,39,49,59,63] | S3-A30-AV1-<QP> |
| S3-A31-AV1 | 8.4.3.4.4 | S3-R15 | aomenc <tag> | S3-AV1-02 | QP=[27,39,49,59,63] | S3-A31-AV1-<QP> |
| S3-A32-AV1 | 8.4.3.4.4 | S3-R16 | aomenc <tag> | S3-AV1-02 | QP=[27,39,49,59,63] | S3-A32-AV1-<QP> |
| S3-A33-AV1 | 8.4.3.4.5 | S3-R01 | aomenc <tag> | S3-AV1-SCC-01 | QP=[27,39,49,59,63] | S3-A33-AV1-<QP> |
| S3-A34-AV1 | 8.4.3.4.5 | S3-R02 | aomenc <tag> | S3-AV1-SCC-01 | QP=[27,39,49,59,63] | S3-A34-AV1-<QP> |
| S3-A35-AV1 | 8.4.3.4.5 | S3-R03 | aomenc <tag> | S3-AV1-SCC-01 | QP=[27,39,49,59,63] | S3-A35-AV1-<QP> |
| S3-A36-AV1 | 8.4.3.4.5 | S3-R04 | aomenc <tag> | S3-AV1-SCC-01 | QP=[27,39,49,59,63] | S3-A36-AV1-<QP> |
| S3-A37-AV1 | 8.4.3.4.5 | S3-R05 | aomenc <tag> | S3-AV1-SCC-01 | QP=[27,39,49,59,63] | S3-A37-AV1-<QP> |
| S3-A38-AV1 | 8.4.3.4.5 | S3-R06 | aomenc <tag> | S3-AV1-SCC-01 | QP=[27,39,49,59,63] | S3-A38-AV1-<QP> |
| S3-A39-AV1 | 8.4.3.4.5 | S3-R07 | aomenc <tag> | S3-AV1-SCC-01 | QP=[27,39,49,59,63] | S3-A39-AV1-<QP> |
| S3-A40-AV1 | 8.4.3.4.5 | S3-R08 | aomenc <tag> | S3-AV1-SCC-01 | QP=[27,39,49,59,63] | S3-A40-AV1-<QP> |
| S3-A41-AV1 | 8.4.3.4.5 | S3-R09 | aomenc <tag> | S3-AV1-SCC-01 | QP=[27,39,49,59,63] | S3-A41-AV1-<QP> |
| S3-A42-AV1 | 8.4.3.4.5 | S3-R10 | aomenc <tag> | S3-AV1-SCC-01 | QP=[27,39,49,59,63] | S3-A42-AV1-<QP> |
| S3-A43-AV1 | 8.4.3.4.5 | S3-R11 | aomenc <tag> | S3-AV1-SCC-01 | QP=[27,39,49,59,63] | S3-A43-AV1-<QP> |
| S3-A44-AV1 | 8.4.3.4.5 | S3-R12 | aomenc <tag> | S3-AV1-SCC-01 | QP=[27,39,49,59,63] | S3-A44-AV1-<QP> |
| S3-A45-AV1 | 8.4.3.4.5 | S3-R13 | aomenc <tag> | S3-AV1-SCC-01 | QP=[27,39,49,59,63] | S3-A45-AV1-<QP> |
| S3-A46-AV1 | 8.4.3.4.5 | S3-R14 | aomenc <tag> | S3-AV1-SCC-01 | QP=[27,39,49,59,63] | S3-A46-AV1-<QP> |
| S3-A47-AV1 | 8.4.3.4.5 | S3-R15 | aomenc <tag> | S3-AV1-SCC-01 | QP=[27,39,49,59,63] | S3-A47-AV1-<QP> |
| S3-A48-AV1 | 8.4.3.4.5 | S3-R16 | aomenc <tag> | S3-AV1-SCC-01 | QP=[27,39,49,59,63] | S3-A48-AV1-<QP> |
| S3-A49-AV1 | 8.4.3.4.6 | S3-R01 | aomenc <tag> | S3-AV1-SCC-02 | QP=[27,39,49,59,63] | S3-A49-AV1-<QP> |
| S3-A50-AV1 | 8.4.3.4.6 | S3-R02 | aomenc <tag> | S3-AV1-SCC-02 | QP=[27,39,49,59,63] | S3-A50-AV1-<QP> |
| S3-A51-AV1 | 8.4.3.4.6 | S3-R03 | aomenc <tag> | S3-AV1-SCC-02 | QP=[27,39,49,59,63] | S3-A51-AV1-<QP> |
| S3-A52-AV1 | 8.4.3.4.6 | S3-R04 | aomenc <tag> | S3-AV1-SCC-02 | QP=[27,39,49,59,63] | S3-A52-AV1-<QP> |
| S3-A53-AV1 | 8.4.3.4.6 | S3-R05 | aomenc <tag> | S3-AV1-SCC-02 | QP=[27,39,49,59,63] | S3-A53-AV1-<QP> |
| S3-A54-AV1 | 8.4.3.4.6 | S3-R06 | aomenc <tag> | S3-AV1-SCC-02 | QP=[27,39,49,59,63] | S3-A54-AV1-<QP> |
| S3-A55-AV1 | 8.4.3.4.6 | S3-R07 | aomenc <tag> | S3-AV1-SCC-02 | QP=[27,39,49,59,63] | S3-A55-AV1-<QP> |
| S3-A56-AV1 | 8.4.3.4.6 | S3-R08 | aomenc <tag> | S3-AV1-SCC-02 | QP=[27,39,49,59,63] | S3-A56-AV1-<QP> |
| S3-A57-AV1 | 8.4.3.4.6 | S3-R09 | aomenc <tag> | S3-AV1-SCC-02 | QP=[27,39,49,59,63] | S3-A57-AV1-<QP> |
| S3-A58-AV1 | 8.4.3.4.6 | S3-R10 | aomenc <tag> | S3-AV1-SCC-02 | QP=[27,39,49,59,63] | S3-A58-AV1-<QP> |
| S3-A59-AV1 | 8.4.3.4.6 | S3-R11 | aomenc <tag> | S3-AV1-SCC-02 | QP=[27,39,49,59,63] | S3-A59-AV1-<QP> |
| S3-A60-AV1 | 8.4.3.4.6 | S3-R12 | aomenc <tag> | S3-AV1-SCC-02 | QP=[27,39,49,59,63] | S3-A60-AV1-<QP> |
| S3-A61-AV1 | 8.4.3.4.6 | S3-R13 | aomenc <tag> | S3-AV1-SCC-02 | QP=[27,39,49,59,63] | S3-A61-AV1-<QP> |
| S3-A62-AV1 | 8.4.3.4.6 | S3-R14 | aomenc <tag> | S3-AV1-SCC-02 | QP=[27,39,49,59,63] | S3-A62-AV1-<QP> |
| S3-A63-AV1 | 8.4.3.4.6 | S3-R15 | aomenc <tag> | S3-AV1-SCC-02 | QP=[27,39,49,59,63] | S3-A63-AV1-<QP> |
| S3-A64-AV1 | 8.4.3.4.6 | S3-R16 | aomenc <tag> | S3-AV1-SCC-02 | QP=[27,39,49,59,63] | S3-A64-AV1-<QP> |
| S3-A65-AV1 | 8.4.3.4.3 | S3-R17 | aomenc <tag> | S3-AV1-01 | QP=[27,39,49,59,63] | S3-A65-AV1-<QP> |
| S3-A66-AV1 | 8.4.3.4.4 | S3-R17 | aomenc <tag> | S3-AV1-02 | QP=[27,39,49,59,63] | S3-A66-AV1-<QP> |
| S3-A67-AV1 | 8.4.3.4.5 | S3-R17 | aomenc <tag> | S3-AV1-SCC-01 | QP=[27,39,49,59,63] | S3-A67-AV1-<QP> |
| S3-A68-AV1 | 8.4.3.4.6 | S3-R17 | aomenc <tag> | S3-AV1-SCC-02 | QP=[27,39,49,59,63] | S3-A68-AV1-<QP> |

##### 8.4.3.4.2	Common Parameters and Settings

The following common Scenario 3 command line options are used.

| --lag-in-frames=0 | No future frame is used as reference in low delay configuration. |
| --- | --- |
| --deltaq-mode=0 | disable block based adaptive quantization, so all blocks within a frame use the same quantization parameter |

##### 8.4.3.4.3	S3-AV1-01: Main 10 Profile with no fixed Intra

For these clips the following additional command line options are used.

| --kf-min-dist=9999 --kf-max-dist=9999 | Only the first frame is encoded as Intra (Key) frame. |
| --- | --- |
| --use-fixed-qp-offsets=1 | use fixed ratios to determine the QP for frames at different hierarchical layers. In av1 encoder, cq-level specified is only used for the frames at the highest temporal layer (non-reference frames), for Key frames and Inter frames at other layers, a fixed ratio is used to determine the quantization parameter. |

S3-AV1-01.sh <num_frame> <fps_norm> <fps_denom> <width> <height> <cq-level> <input> <output>

##### 8.4.3.4.4	S3-AV1-02: Main 10 Profile with fixed Intra every second

For these clips the following additional command line options are used.

| --min-q=<cq-level> --max-q=>cq-level> | Specifies min/max q equal to the cq-level to make sure all frames use the same QP |
| --- | --- |
| --disable-warning-prompt | This parameter is used to bypass the warning prompt that min-q and max-q are set to be the same value. |

IntraPeriod such that exactly 1 second is achieved (i.e., IntraPeriod value equal to the sequence fps value rounded to the nearest integer).

S3-AV1-02.sh <num_frame> <fps_norm> <fps_denom> <width> <height> <cq-level> <intra_period> <input> <output>

##### 8.4.3.4.5	S3-AV1-SCC-01: Screen Content Profile with no fixed Intra

For these clips the following additional command line options are used.

| --kf-min-dist=9999 --kf-max-dist=9999 | Only the first frame is encoded as Intra (Key) frame. |
| --- | --- |
| --use-fixed-qp-offsets=1 | use fixed ratios to determine the QP for frames at different hierarchical layers. In av1 encoder, cq-level specified is only used for the frames at the highest temporal layer (non-reference frames), for Key frames and Inter frames at other layers, a fixed ratio is used to determine the quantization parameter. |
| --tune-content=screen | specify content type to be screen content. In AV1, screen content coding tools are enabled in all profiles. The encoder internally has an algorithm to detect if the input video is screen content or not. By default, if the encoder detects the input video is screen content, it will enable the screen content coding tool. When tune-content is set to screen, the encoder will bypass the screen content detection and assume the input video is screen content and enable screen content coding tools. |

S3-AV1-SCC-01.sh <num_frame> <fps_norm> <fps_denom> <width> <height> <cq-level> <input> <output>

##### 8.4.3.4.6	S3-AV1-SCC-02: Screen Content Profile with fixed Intra every second

For these clips the following additional command line options are used.

| --min-q=<cq-level> --max-q=>cq-level> | Specifies min/max q equal to the cq-level to make sure all frames use the same QP |
| --- | --- |
| --disable-warning-prompt | This parameter is used to bypass the warning prompt that min-q and max-q are set to be the same value. |
| --tune-content=screen | specify content type to be screen content. In AV1, screen content coding tools are enabled in all profiles. The encoder internally has an algorithm to detect if the input video is screen content or not. By default, if the encoder detects the input video is screen content, it will enable the screen content coding tool. When tune-content is set to screen, the encoder will bypass the screen content detection and assume the input video is screen content and enable screen content coding tools. |

IntraPeriod such that exactly 1 second is achieved (i.e., IntraPeriod value equal to the sequence fps value rounded to the nearest integer).

S3-AV1-SCC-02.sh <num_frame> <fps_norm> <fps_denom> <width> <height> <cq-level> <intra_preiod> <input> <output>

##### 8.4.3.4.7	Test Results

Editor’s Note: to be completed.

#### 8.4.3.5	Scenario 4: Messaging and Social Sharing

##### 8.4.3.5.1	Overview

Table 8.4.3.5.1-1 provides an overview of the AV1 test tuples. For provided bitstreams, the AV1 encoder aomenc is built from the libaom libraries using the tag provided in the introduction. Keys are identified to refer to the bitstreams in the context of the scenario.

Table 8.4.3.5.1-1 Test Tuple generation with AV1, Messaging and Social Sharing Scenario

| Key | Clause | Reference Sequence | Reference Encoder | Config | Variations | Anchor Key |
| --- | --- | --- | --- | --- | --- | --- |
| S4-A01-AV1 | 8.4.3.5.3 | S4-R01 | aomenc <tag> | S4-AV1-01 | QP=[27,39,49,59,63] | S4-A01-AV1-<QP> |
| S4-A02-AV1 | 8.4.3.5.3 | S4-R02 | aomenc <tag> | S4-AV1-01 | QP=[27,39,49,59,63] | S4-A02-AV1-<QP> |
| S4-A03-AV1 | 8.4.3.5.3 | S4-R03 | aomenc <tag> | S4-AV1-01 | QP=[27,39,49,59,63] | S4-A03-AV1-<QP> |
| S4-A04-AV1 | 8.4.3.5.3 | S4-R04 | aomenc <tag> | S4-AV1-01 | QP=[27,39,49,59,63] | S4-A04-AV1-<QP> |
| S4-A05-AV1 | 8.4.3.5.4 | S4-R01 | aomenc <tag> | S4-AV1-02 | QP=[27,39,49,59,63] | S4-A05-AV1-<QP> |
| S4-A06-AV1 | 8.4.3.5.4 | S4-R02 | aomenc <tag> | S4-AV1-02 | QP=[27,39,49,59,63] | S4-A06-AV1-<QP> |
| S4-A07-AV1 | 8.4.3.5.4 | S4-R03 | aomenc <tag> | S4-AV1-02 | QP=[27,39,49,59,63] | S4-A07-AV1-<QP> |
| S4-A08-AV1 | 8.4.3.5.4 | S4-R04 | aomenc <tag> | S4-AV1-02 | QP=[27,39,49,59,63] | S4-A08-AV1-<QP> |

##### 8.4.3.5.2	Common Parameters and Settings

The following common Scenario 4 command line options are used.

| --lag-in-frames=0 | No future frame is used as reference in low delay configuration. |
| --- | --- |
| --deltaq-mode=0 | disable block based adaptive quantization, so all blocks within a frame use the same quantization parameter |

##### 8.4.3.5.3	S4-AV1-01: no Intra

For these clips the following additional command line options are used.

| --kf-min-dist=9999 --kf-max-dist=9999 | Only the first frame is encoded as Intra (Key) frame. |
| --- | --- |
| --use-fixed-qp-offsets=1 | use fixed ratios to determine the QP for frames at different hierarchical layers. In av1 encoder, cq-level specified is only used for the frames at the highest temporal layer (non-reference frames), for Key frames and Inter frames at other layers, a fixed ratio is used to determine the quantization parameter. |

S4-AV1-01.sh <num_frame> <fps_norm> <fps_denom> <width> <height> <cq-level> <input> <output>

##### 8.4.3.5.4	S4-AV1-02: Intra

For these clips the following additional command line options are used.

| --min-q=<cq-level> --max-q=>cq-level> | Specifies min/max q equal to the cq-level to make sure all frames use the same QP |
| --- | --- |
| --disable-warning-prompt | This parameter is used to bypass the warning prompt that min-q and max-q are set to be the same value. |

IntraPeriod = power of 2 value that is greater than or equal to the frame rate (fps), such that near 1 second is achieved: 32 for 30fps sequences and 64 for 60fps sequences

S4-AV1-02.sh <num_frame> <fps_norm> <fps_denom> <width> <height> <cq-level> <intra_period> <input> <output>

##### 8.4.3.5.5	Test Results

Editor’s Note: to be completed.

#### 8.4.3.6	Scenario 5: Online Gaming

##### 8.4.3.6.1	Overview

Table 8.4.3.6.1-1 provides an overview of the AV1 test tuples. For provided bitstreams, the AV1 encoder aomenc is built from the libaom libraries using the tag provided in the introduction. Keys are identified to refer to the bitstreams in the context of the scenario.

Table 8.4.3.6.1-1 Test Tuple generation with AV1, Online Gaming Scenario

| Key | Clause | Reference Sequence | Reference Encoder | Configuration | Variations | Anchor Key |
| --- | --- | --- | --- | --- | --- | --- |
| S5-A01-AV1 | 8.4.3.6.3 | S5-R01 | aomenc <tag> | S5-AV1-01 | QP=[27,39,49,59,63] | S5-A01-AV1-<QP> |
| S5-A02-AV1 | 8.4.3.6.3 | S5-R02 | aomenc <tag> | S5-AV1-01 | QP=[27,39,49,59,63] | S5-A02-AV1-<QP> |
| S5-A03-AV1 | 8.4.3.6.3 | S5-R03 | aomenc <tag> | S5-AV1-01 | QP=[27,39,49,59,63] | S5-A03-AV1-<QP> |
| S5-A04-AV1 | 8.4.3.6.3 | S5-R04 | aomenc <tag> | S5-AV1-01 | QP=[27,39,49,59,63] | S5-A04-AV1-<QP> |
| S5-A05-AV1 | 8.4.3.6.3 | S5-R05 | aomenc <tag> | S5-AV1-01 | QP=[27,39,49,59,63] | S5-A05-AV1-<QP> |
| S5-A06-AV1 | 8.4.3.6.3 | S5-R06 | aomenc <tag> | S5-AV1-01 | QP=[27,39,49,59,63] | S5-A06-AV1-<QP> |
| S5-A07-AV1 | 8.4.3.6.3 | S5-R07 | aomenc <tag> | S5-AV1-01 | QP=[27,39,49,59,63] | S5-A07-AV1-<QP> |
| S5-A08-AV1 | 8.4.3.6.3 | S5-R08 | aomenc <tag> | S5-AV1-01 | QP=[27,39,49,59,63] | S5-A08-AV1-<QP> |
| S5-A09-AV1 | 8.4.3.6.3 | S5-R09 | aomenc <tag> | S5-AV1-01 | QP=[27,39,49,59,63] | S5-A09-AV1-<QP> |
| S5-A10-AV1 | 8.4.3.6.3 | S5-R10 | aomenc <tag> | S5-AV1-01 | QP=[27,39,49,59,63] | S5-A10-AV1-<QP> |
| S5-A11-AV1 | 8.4.3.6.3 | S5-R11 | aomenc <tag> | S5-AV1-01 | QP=[27,39,49,59,63] | S5-A11-AV1-<QP> |
| S5-A12-AV1 | 8.4.3.6.3 | S5-R12 | aomenc <tag> | S5-AV1-01 | QP=[27,39,49,59,63] | S5-A12-AV1-<QP> |
| S5-A13-AV1 | 8.4.3.6.3 | S5-R13 | aomenc <tag> | S5-AV1-01 | QP=[27,39,49,59,63] | S5-A13-AV1-<QP> |
| S5-A14-AV1 | 8.4.3.6.4 | S5-R01 | aomenc <tag> | S5-AV1-02 | QP=[27,39,49,59,63] | S5-A14-AV1-<QP> |
| S5-A15-AV1 | 8.4.3.6.4 | S5-R02 | aomenc <tag> | S5-AV1-02 | QP=[27,39,49,59,63] | S5-A15-AV1-<QP> |
| S5-A16-AV1 | 8.4.3.6.4 | S5-R03 | aomenc <tag> | S5-AV1-02 | QP=[27,39,49,59,63] | S5-A16-AV1-<QP> |
| S5-A17-AV1 | 8.4.3.6.4 | S5-R04 | aomenc <tag> | S5-AV1-02 | QP=[27,39,49,59,63] | S5-A17-AV1-<QP> |
| S5-A18-AV1 | 8.4.3.6.4 | S5-R05 | aomenc <tag> | S5-AV1-02 | QP=[27,39,49,59,63] | S5-A18-AV1-<QP> |
| S5-A19-AV1 | 8.4.3.6.4 | S5-R06 | aomenc <tag> | S5-AV1-02 | QP=[27,39,49,59,63] | S5-A19-AV1-<QP> |
| S5-A20-AV1 | 8.4.3.6.4 | S5-R07 | aomenc <tag> | S5-AV1-02 | QP=[27,39,49,59,63] | S5-A20-AV1-<QP> |
| S5-A21-AV1 | 8.4.3.6.4 | S5-R08 | aomenc <tag> | S5-AV1-02 | QP=[27,39,49,59,63] | S5-A21-AV1-<QP> |
| S5-A22-AV1 | 8.4.3.6.4 | S5-R09 | aomenc <tag> | S5-AV1-02 | QP=[27,39,49,59,63] | S5-A22-AV1-<QP> |
| S5-A23-AV1 | 8.4.3.6.4 | S5-R10 | aomenc <tag> | S5-AV1-02 | QP=[27,39,49,59,63] | S5-A23-AV1-<QP> |
| S5-A24-AV1 | 8.4.3.6.4 | S5-R11 | aomenc <tag> | S5-AV1-02 | QP=[27,39,49,59,63] | S5-A24-AV1-<QP> |
| S5-A25-AV1 | 8.4.3.6.4 | S5-R12 | aomenc <tag> | S5-AV1-02 | QP=[27,39,49,59,63] | S5-A25-AV1-<QP> |
| S5-A26-AV1 | 8.4.3.6.4 | S5-R13 | aomenc <tag> | S5-AV1-02 | QP=[27,39,49,59,63] | S5-A26-AV1-<QP> |
| S5-A27-AV1 | 8.4.3.6.5 | S5-R01 | aomenc <tag> | S5-AV1-SCC-01 | QP=[27,39,49,59,63] | S5-A27-AV1-<QP> |
| S5-A28-AV1 | 8.4.3.6.5 | S5-R02 | aomenc <tag> | S5-AV1-SCC-01 | QP=[27,39,49,59,63] | S5-A28-AV1-<QP> |
| S5-A29-AV1 | 8.4.3.6.5 | S5-R03 | aomenc <tag> | S5-AV1-SCC-01 | QP=[27,39,49,59,63] | S5-A29-AV1-<QP> |
| S5-A30-AV1 | 8.4.3.6.5 | S5-R04 | aomenc <tag> | S5-AV1-SCC-01 | QP=[27,39,49,59,63] | S5-A30-AV1-<QP> |
| S5-A31-AV1 | 8.4.3.6.5 | S5-R05 | aomenc <tag> | S5-AV1-SCC-01 | QP=[27,39,49,59,63] | S5-A31-AV1-<QP> |
| S5-A32-AV1 | 8.4.3.6.5 | S5-R06 | aomenc <tag> | S5-AV1-SCC-01 | QP=[27,39,49,59,63] | S5-A32-AV1-<QP> |
| S5-A33-AV1 | 8.4.3.6.5 | S5-R07 | aomenc <tag> | S5-AV1-SCC-01 | QP=[27,39,49,59,63] | S5-A33-AV1-<QP> |
| S5-A34-AV1 | 8.4.3.6.5 | S5-R08 | aomenc <tag> | S5-AV1-SCC-01 | QP=[27,39,49,59,63] | S5-A34-AV1-<QP> |
| S5-A35-AV1 | 8.4.3.6.5 | S5-R09 | aomenc <tag> | S5-AV1-SCC-01 | QP=[27,39,49,59,63] | S5-A35-AV1-<QP> |
| S5-A36-AV1 | 8.4.3.6.5 | S5-R10 | aomenc <tag> | S5-AV1-SCC-01 | QP=[27,39,49,59,63] | S5-A36-AV1-<QP> |
| S5-A37-AV1 | 8.4.3.6.5 | S5-R11 | aomenc <tag> | S5-AV1-SCC-01 | QP=[27,39,49,59,63] | S5-A37-AV1-<QP> |
| S5-A38-AV1 | 8.4.3.6.5 | S5-R12 | aomenc <tag> | S5-AV1-SCC-01 | QP=[27,39,49,59,63] | S5-A38-AV1-<QP> |
| S5-A39-AV1 | 8.4.3.6.5 | S5-R13 | aomenc <tag> | S5-AV1-SCC-01 | QP=[27,39,49,59,63] | S5-A39-AV1-<QP> |
| S5-A40-AV1 | 8.4.3.6.6 | S5-R01 | aomenc <tag> | S5-AV1-SCC-02 | QP=[27,39,49,59,63] | S5-A40-AV1-<QP> |
| S5-A41-AV1 | 8.4.3.6.6 | S5-R02 | aomenc <tag> | S5-AV1-SCC-02 | QP=[27,39,49,59,63] | S5-A41-AV1-<QP> |
| S5-A42-AV1 | 8.4.3.6.6 | S5-R03 | aomenc <tag> | S5-AV1-SCC-02 | QP=[27,39,49,59,63] | S5-A42-AV1-<QP> |
| S5-A43-AV1 | 8.4.3.6.6 | S5-R04 | aomenc <tag> | S5-AV1-SCC-02 | QP=[27,39,49,59,63] | S5-A43-AV1-<QP> |
| S5-A44-AV1 | 8.4.3.6.6 | S5-R05 | aomenc <tag> | S5-AV1-SCC-02 | QP=[27,39,49,59,63] | S5-A44-AV1-<QP> |
| S5-A45-AV1 | 8.4.3.6.6 | S5-R06 | aomenc <tag> | S5-AV1-SCC-02 | QP=[27,39,49,59,63] | S5-A45-AV1-<QP> |
| S5-A46-AV1 | 8.4.3.6.6 | S5-R07 | aomenc <tag> | S5-AV1-SCC-02 | QP=[27,39,49,59,63] | S5-A46-AV1-<QP> |
| S5-A47-AV1 | 8.4.3.6.6 | S5-R08 | aomenc <tag> | S5-AV1-SCC-02 | QP=[27,39,49,59,63] | S5-A47-AV1-<QP> |
| S5-A48-AV1 | 8.4.3.6.6 | S5-R09 | aomenc <tag> | S5-AV1-SCC-02 | QP=[27,39,49,59,63] | S5-A48-AV1-<QP> |
| S5-A49-AV1 | 8.4.3.6.6 | S5-R10 | aomenc <tag> | S5-AV1-SCC-02 | QP=[27,39,49,59,63] | S5-A49-AV1-<QP> |
| S5-A50-AV1 | 8.4.3.6.6 | S5-R11 | aomenc <tag> | S5-AV1-SCC-02 | QP=[27,39,49,59,63] | S5-A50-AV1-<QP> |
| S5-A51-AV1 | 8.4.3.6.6 | S5-R12 | aomenc <tag> | S5-AV1-SCC-02 | QP=[27,39,49,59,63] | S5-A51-AV1-<QP> |
| S5-A52-AV1 | 8.4.3.6.6 | S5-R13 | aomenc <tag> | S5-AV1-SCC-02 | QP=[27,39,49,59,63] | S5-A52-AV1-<QP> |

##### 8.4.3.6.2	Common Parameters and Settings

The following common Scenario 5 command line options are used.

| --lag-in-frames=0 | No future frame is used as reference in low delay configuration. |
| --- | --- |
| --deltaq-mode=0 | disable block based adaptive quantization, so all blocks within a frame use the same quantization parameter |

##### 8.4.3.6.3	S5-AV1-01: Main 10 Profile with no fixed Intra

For these clips the following additional command line options are used.

| --kf-min-dist=9999 --kf-max-dist=9999 | Only the first frame is encoded as Intra (Key) frame. |
| --- | --- |
| --use-fixed-qp-offsets=1 | use fixed ratios to determine the QP for frames at different hierarchical layers. In av1 encoder, cq-level specified is only used for the frames at the highest temporal layer (non-reference frames), for Key frames and Inter frames at other layers, a fixed ratio is used to determine the quantization parameter. |

S5-AV1-01.sh <num_frame> <fps_norm> <fps_denom> <width> <height> <cq-level> <input> <output>

##### 8.4.3.6.4	S5-AV1-02: Main 10 Profile with fixed Intra every second

For these clips the following additional command line options are used.

| --min-q=<cq-level> --max-q=>cq-level> | Specifies min/max q equal to the cq-level to make sure all frames use the same QP |
| --- | --- |
| --disable-warning-prompt | This parameter is used to bypass the warning prompt that min-q and max-q are set to be the same value. |

IntraPeriod = power of 2 value that is greater than or equal to the frame rate (fps), such that near 1 second is achieved: 32 for 30fps sequences and 64 for 60fps sequences

S5-AV1-02.sh <num_frame> <fps_norm> <fps_denom> <width> <height> <cq-level> <intra_period> <input> <output>

##### 8.4.3.6.5	S5-SCC-01: Screen Content Profile with no fixed Intra

For these clips the following additional command line options are used.

| --kf-min-dist=9999 --kf-max-dist=9999 | Only the first frame is encoded as Intra (Key) frame. |
| --- | --- |
| --use-fixed-qp-offsets=1 | use fixed ratios to determine the QP for frames at different hierarchical layers. In av1 encoder, cq-level specified is only used for the frames at the highest temporal layer (non-reference frames), for Key frames and Inter frames at other layers, a fixed ratio is used to determine the quantization parameter. |
| --tune-content=screen | specify content type to be screen content. In AV1, screen content coding tools are enabled in all profiles. The encoder internally has an algorithm to detect if the input video is screen content or not. By default, if the encoder detects the input video is screen content, it will enable the screen content coding tool. When tune-content is set to screen, the encoder will bypass the screen content detection and assume the input video is screen content and enable screen content coding tools. |

S5-AV1-SCC-01.sh <num_frame> <fps_norm> <fps_denom> <width> <height> <cq-level> <input> <output>

##### 8.4.3.6.6	S5-SCC-02: Screen Content Profile with fixed Intra

For these clips the following additional command line options are used.

| --min-q=<cq-level> --max-q=>cq-level> | Specifies min/max q equal to the cq-level to make sure all frames use the same QP |
| --- | --- |
| --disable-warning-prompt | This parameter is used to bypass the warning prompt that min-q and max-q are set to be the same value. |
| --tune-content=screen | specify content type to be screen content. In AV1, screen content coding tools are enabled in all profiles. The encoder internally has an algorithm to detect if the input video is screen content or not. By default, if the encoder detects the input video is screen content, it will enable the screen content coding tool. When tune-content is set to screen, the encoder will bypass the screen content detection and assume the input video is screen content and enable screen content coding tools. |

IntraPeriod = power of 2 value that is greater than or equal to the frame rate (fps), such that near 1 second is achieved: 32 for 30fps sequences and 64 for 60fps sequences

S5-AV1-SCC-02.sh <num_frame> <fps_norm> <fps_denom> <width> <height> <cq-level> <intra_preiod> <input> <output>

##### 8.4.3.6.7	Test Results

Editor’s Note: to be completed.

End Change 1