3GPPSA4-e (AH) Video SWG post 116-e	S4aV210837

E-meeting, 14th November 2021

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PSEUDO CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.955 | CR | pseudo | rev |  | Current version: | 1.4.2 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Characterization Updates |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_5GVideo |  |  |  |  |  | Date: |  |  | 13/12/2021 |
| Category: | C |  |  |  |  |  | Release: |  |  | 17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  |  |  |  |  |  |  |  |  |  |
| Summary of change: |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

=====  CHANGE  =====

## 7.4	H.265/HEVC Characterization against H.264/AVC

7.4.1 Introduction

This clause provides characterization results for H.265/HEVC against H.264/AVC for different scenarios and metrics.

7.4.2 H.265/HEVC HM against H.264/AVC

#### 7.4.2.1	Overview

This clause provides a full characterization of H.265/HEVC HM against H.264/AVC according to clause 7.2.1. The results provided in clause 6 are used for the characterization.

#### 7.4.2.2	Scenario 1: Full HD

This clause provides characterization of H.265/HEVC HM mode configurations against H.264/AVC JM for Scenario 1 Full HD. In particular, Table 7.4.2.2-1 provides the BD rate gain of H.265/HEVC HM with S1-HM-01 against H.264/AVC JM with configuration S1-HM-01, i.e. with the Full HD SDR scenario reference sequences.

Table 7.4.2.2-1 BD rate gain of H.265/HEVC HM with S1-JM-01 against H.264/AVC HM with configuration S1-JM-01, i.e. with the Full HD SDR scenario reference sequences

| Reference sequence | Name | psnr | y_psnr | ms_ssim | vmaf |
| --- | --- | --- | --- | --- | --- |
| S1-R01 | Brest-Sedof-FHD | 36.045 | 35.396 | n/a | n/a |
| S1-R02 | Rain Fruits-FHD | 45.491 | 47.685 | n/a | n/a |
| S1-R03 | Park Joy-FHD | 26.546 | 26.725 | n/a | n/a |
| S1-R04 | Soccer-FHD | 55.701 | 56.041 | n/a | n/a |
| S1-R05 | Tunnel Flag-FHD | 46.424 | 47.388 | n/a | n/a |
| S1-R06 | Boat-FHD | 40.12 | 41.242 | n/a | n/a |
| S1-R07 | Fountain-FHD | 20.855 | 22.157 | n/a | n/a |
| S1-R08 | Riverbank-FHD | 42.188 | 41.579 | n/a | n/a |
| Minimum |  | 20.855 | 22.157 | n/a | n/a |
| Maximum |  | 55.701 | 56.041 | n/a | n/a |
| Average |  | 39.171 | 39.777 | n/a | n/a |

As an example, Figure 7.4.2.2-1 provides Rate-Quality curves and BD rate gain for psnr of H.265/HEVC HM with S1-JM-01 against H.264/AVC HM with configuration S1-JM-01 for reference sequence S1-R01.

Figure 7.4.2.2-1 Rate-Quality curves and BD rate gain for psnr of H.265/HEVC HM with S1-JM-01 against H.264/AVC HM with configuration S1-JM-01 for reference sequence S1-R01

All Rate-Quality curves and BD rate gain plots are provided in the attachment as well as online along with the test vectors.

#### 7.4.2.3	Scenario 3: Screen Content

This clause provides characterization of H.265/HEVC HM mode configurations against H.264/AVC JM for Scenario 3 Screen Content. In particular,

- Table 7.4.2.3-1 provides the BD rate gain of H.265/HEVC HM with S3-HM-01 against H.264/AVC JM with configuration S3-JM-01, i.e. with the screen content scenario reference sequences and no fixed intra.

- Table 7.4.2.3-2 provides the BD rate gain of H.265/HEVC HM with S3-HM-02 against H.264/AVC JM with configuration S3-JM-02, i.e. with the screen content scenario reference sequences with fixed Intra every second.

Table 7.4.2.3-1 BD rate gain of H.265/HEVC HM with S3-HM-01 against H.264/AVC JM with configuration S3-JM-01, i.e. with the screen content scenario reference sequences and no fixed intra

| Reference sequence | Name | psnr | y_psnr |
| --- | --- | --- | --- |
| S3-R01 | MovingText2-4K-10bit | Tbd |  | Tbd |  |
| S3-R02 | MovingText2-4K-8bit | Tbd |  | Tbd |  |
| S3-R03 | MovingText2-FullHD-10bit | Tbd |  | Tbd |  |
| S3-R04 | MovingText2-FullHD-8bit | Tbd |  | Tbd |  |
| S3-R05 | TextMixTransitions-4K-10bit | Tbd |  | Tbd |  |
| S3-R06 | TextMixTransitions-4K-8bit | Tbd |  | Tbd |  |
| S3-R07 | TextMixTransitions-FullHD-10bit | Tbd |  | Tbd |  |
| S3-R08 | TextMixTransitions-FullHD-8bit | Tbd |  | Tbd |  |
| S3-R09 | GraphicsMixSimple-4K-10bit | Tbd |  | Tbd |  |
| S3-R10 | GraphicsMixSimple-4K-8bit | Tbd |  | Tbd |  |
| S3-R11 | GraphicsMixSimple-FullHD-10bit | Tbd |  | Tbd |  |
| S3-R12 | GraphicsMixSimple-FullHD-8bit | Tbd |  | Tbd |  |
| S3-R13 | GraphicsMixTransitions-4K-10bit | Tbd |  | tbd |  |
| S3-R14 | GraphicsMixTransitions-4K-8bit | Tbd |  | Tbd |  |
| S3-R15 | GraphicsMixTransitions-FullHD-10bit | Tbd |  | Tbd |  |
| S3-R16 | GraphicsMixTransitions-FullHD-8bit | Tbd |  | Tbd |  |
| S3-R17 | Mission-Control | Tbd |  | Tbd |  |
| Minimum |  | Tbd |  | Tbd |  |
| Maximum |  | Tbd |  | Tbd |  |
| Average |  | Tbd |  | Tbd |  |

Table 7.4.2.3-2 BD rate gain of H.265/HEVC HM with S3-HM-02 against H.264/AVC JM with configuration S3-JM-02, i.e. with the screen content scenario reference sequences with fixed Intra every second

| Reference sequence | Name | psnr | y_psnr |
| --- | --- | --- | --- |
| S3-R01 | MovingText2-4K-10bit | Tbd |  | Tbd |  |
| S3-R02 | MovingText2-4K-8bit | Tbd |  | Tbd |  |
| S3-R03 | MovingText2-FullHD-10bit | Tbd |  | Tbd |  |
| S3-R04 | MovingText2-FullHD-8bit | Tbd |  | Tbd |  |
| S3-R05 | TextMixTransitions-4K-10bit | Tbd |  | Tbd |  |
| S3-R06 | TextMixTransitions-4K-8bit | Tbd |  | Tbd |  |
| S3-R07 | TextMixTransitions-FullHD-10bit | Tbd |  | Tbd |  |
| S3-R08 | TextMixTransitions-FullHD-8bit | Tbd |  | Tbd |  |
| S3-R09 | GraphicsMixSimple-4K-10bit | Tbd |  | Tbd |  |
| S3-R10 | GraphicsMixSimple-4K-8bit | Tbd |  | Tbd |  |
| S3-R11 | GraphicsMixSimple-FullHD-10bit | Tbd |  | Tbd |  |
| S3-R12 | GraphicsMixSimple-FullHD-8bit | Tbd |  | Tbd |  |
| S3-R13 | GraphicsMixTransitions-4K-10bit | Tbd |  | tbd |  |
| S3-R14 | GraphicsMixTransitions-4K-8bit | Tbd |  | Tbd |  |
| S3-R15 | GraphicsMixTransitions-FullHD-10bit | Tbd |  | Tbd |  |
| S3-R16 | GraphicsMixTransitions-FullHD-8bit | Tbd |  | Tbd |  |
| S3-R17 | Mission-Control | Tbd |  | Tbd |  |
| Minimum |  | Tbd |  | Tbd |  |
| Maximum |  | Tbd |  | Tbd |  |
| Average |  | Tbd |  | Tbd |  |

#### 7.4.2.4	Scenario 4: Messaging and Social Sharing

This clause provides characterization of H.265/HEVC HM mode configurations against H.264/AVC JM for Scenario 4 Messaging and Social Sharing. In particular,

- Table 7.4.2.4-1 provides the BD rate gain of H.265/HEVC HM with S4-HM-01 against H.264/AVC JM with configuration S4-JM-01, i.e. with the messaging and social sharing scenario reference sequences and no fixed intra.

- Table 7.4.2.4-2 provides the BD rate gain of H.265/HEVC HM with S4-HM-02 against H.264/AVC JM with configuration S4-JM-02, i.e. with the messaging and social sharing scenario reference sequences with fixed Intra every second.

Table 7.4.2.3-1 BD rate gain of H.265/HEVC HM with S4-HM-01 against H.264/AVC JM with configuration S4-JM-01, i.e. with the messaging and social sharing scenario reference sequences and no fixed intra

| Reference sequence | Name | psnr | y_psnr | ms_ssim |  |  | vmaf |
| --- | --- | --- | --- | --- | --- | --- | --- |
| S4-R01 | Vertical-Bees | Tbd | Tbd | Tbd | Tbd |
| S4-R02 | Vertical-Walking | Tbd | Tbd | Tbd | Tbd |
| S4-R03 | Neon-4K | Tbd | Tbd | Tbd | Tbd |
| S4-R04 | Skater-4K | Tbd | Tbd | Tbd | Tbd |
| Minimum |  | Tbd | Tbd | Tbd | Tbd |
| Maximum |  | Tbd | Tbd | Tbd | Tbd |
| Average |  | Tbd | tbd | Tbd | tbd |

Table 7.4.2.4-2 BD rate gain of H.265/HEVC HM with S3-HM-02 against H.264/AVC JM with configuration S3-JM-02, i.e. with the screen content scenario reference sequences with fixed Intra every second

| Reference sequence | Name | psnr | y_psnr | ms_ssim | vmaf |
| --- | --- | --- | --- | --- | --- |
| S4-R01 | Vertical-Bees | Tbd | Tbd | Tbd | Tbd |
| S4-R02 | Vertical-Walking | Tbd | Tbd | Tbd | Tbd |
| S4-R03 | Neon-4K | Tbd | Tbd | Tbd | Tbd |
| S4-R04 | Skater-4K | Tbd | Tbd | Tbd | Tbd |
| Minimum |  | Tbd | Tbd | Tbd | Tbd |
| Maximum |  | Tbd | Tbd | Tbd | Tbd |
| Average |  | Tbd | tbd | Tbd | tbd |

#### 7.4.2.5	Scenario 5: Online Gaming

This clause provides characterization of H.265/HEVC HM mode configurations against H.264/AVC JM for Scenario 5 Online Gaming. In particular,

- Table 7.4.2.5-1 provides the BD rate gain of H.265/HEVC HM with S5-HM-01 against H.264/AVC JM with configuration S5-JM-01, i.e. with the online gaming scenario reference sequences and no fixed intra.

- Table 7.4.2.5-2 provides the BD rate gain of H.265/HEVC HM with S5-HM-02 against H.264/AVC JM with configuration S5-JM-02, i.e. with the online gaming scenario reference sequences with fixed Intra every second.

Table 7.4.2.5-1 BD rate gain of H.265/HEVC HM with S5-HM-01 against H.264/AVC JM with configuration S5-JM-01, i.e. with the online gaming scenario reference sequences and no fixed intra

| Reference sequence | Name | psnr | y_psnr |
| --- | --- | --- | --- |
| S5-R01 | AOV | 45.131 | 44.661 |  |
| S5-R02 | Baolei-Man | 35.947 | 34.673 |  |
| S5-R03 | Baolei-Woman | 43.954 | 43.102 |  |
| S5-R04 | Baolei-Balloon | 52.179 | 52.212 |  |
| S5-R05 | Baolei-Yard | 48.097 | 47.978 |  |
| S5-R06 | Jianling-Temple | 23.311 | 22.879 |  |
| S5-R07 | Jianling-Beach | 36.726 | 34.483 |  |
| S5-R08 | Heroes-of-the-Storm | 31.683 | 31.535 |  |
| S5-R09 | Project-CARS | 40.397 | 37.746 |  |
| S5-R10 | World-of-WarCraft | 48.828 | 48.111 |  |
| S5-R11 | MineCraft | 28.148 | 25.702 |  |
| S5-R12 | CS-GO | 51.747 | 48.306 |  |
| S5-R13 | StarCraft | 44.375 | 42.703 |  |
| Minimum |  | 23.311 | 22.879 |  |
| Maximum |  | 52.179 | 52.212 |  |
| Average |  | 40.809 | 39.545 |  |

Table 7.4.2.5-2 BD rate gain of H.265/HEVC HM with S5-HM-02 against H.264/AVC JM with configuration S5-JM-02, i.e. with the online gaming scenario reference sequences and fixed intra every second

| Reference sequence | Name | psnr | y_psnr |
| --- | --- | --- | --- |
| S5-R01 | AOV | 34.64 | 34.777 |  |
| S5-R02 | Baolei-Man | 27.949 | 27.594 |  |
| S5-R03 | Baolei-Woman | 34.311 | 34.265 |  |
| S5-R04 | Baolei-Balloon | 43.508 | 44.334 |  |
| S5-R05 | Baolei-Yard | 41.937 | 42.689 |  |
| S5-R06 | Jianling-Temple | 21.483 | 21.544 |  |
| S5-R07 | Jianling-Beach | 25.154 | 24.539 |  |
| S5-R08 | Heroes-of-the-Storm | 22.909 | 22.959 |  |
| S5-R09 | Project-CARS | 31.016 | 31.605 |  |
| S5-R10 | World-of-WarCraft | 36.756 | 35.998 |  |
| S5-R11 | MineCraft | 26.716 | 25.681 |  |
| S5-R12 | CS-GO | 35.558 | 34.553 |  |
| S5-R13 | StarCraft | tbd | tbd |  |
| Minimum |  | 21.483 | 21.522 |  |
| Maximum |  | 43.508 | 44.334 |  |
| Average |  | tbd | tbd |  |

As an example,

- Figure 7.4.2.5-1 provides Rate-Quality curves and BD rate gain for psnr of H.265/HEVC HM with S5-HM-01 against H.264/AVC HM with configuration S5-JM-01 for reference sequence S5-R04

- Figure 7.4.2.5-2 provides Rate-Quality curves and BD rate gain for psnr of H.265/HEVC HM with S5-HM-02 against H.264/AVC HM with configuration S5-JM-02 for reference sequence S5-R04

Figure 7.4.2.5-1 Rate-Quality curves and BD rate gain for psnr of H.265/HEVC HM with S5-JM-01 against H.264/AVC HM with configuration S5-JM-01 for reference sequence S5-R04

Figure 7.4.2.5-2 Rate-Quality curves and BD rate gain for psnr of H.265/HEVC HM with S5-JM-01 against H.264/AVC HM with configuration S5-JM-02 for reference sequence S5-R04

#### All Rate-Quality curves and BD rate gain plots are provided in the attachment as well as online along with the test vectors.

#### 7.4.2.6	Summary

#### Table 7.4.2.6-1 provides a summary of BD rate gain in psnr and vmaf of H.265/HEVC HM against H.264/AVC JM for different scenarios and configurations.

#### Table 7.4.2.6-1 Summary of BD rate gain in psnr and vmaf of H.265/HEVC HM against H.264/AVC JM for different scenarios and configurations

| Scenario | average psnr | min psnr | max psnr | average vmaf | min vmaf | max vmaf |
| --- | --- | --- | --- | --- | --- | --- |
| S1 | 39.171 |  | 20.855 |  | 55.701 |  |  |  |  |  |  |  |
| S3 no intra |  |  |  |  |  |  | n/a |  | n/a |  | n/a |  |
| S3 intra |  |  |  | n/a | n/a | n/a |
| S4 no intra |  |  |  |  |  |  |  |  |  |  |  |  |
| S4 intra |  |  |  |  |  |  |
| S5 no intra | 40.809 | 23.311 | 52.179 | n/a | n/a | n/a |
| S5 intra |  |  | 21.483 |  | 43.508 |  | n/a |  | n/a |  | n/a |  |
| Minimum |  |  |  |  |  |  |  |  |  |  |  |  |
| Maximum |  |  |  |  |  |  |  |  |  |  |  |  |

7.4.3 H.265/HEVC SCC against H.264/AVC

#### 7.4.3.1	Overview

This clause provides a partial characterization of H.265/HEVC SCC against H.264/AVC according to clause 7.2.1. The results provided in clause 6 are used for the characterization.

#### 7.4.3.2	Scenario 3: Screen Content

This clause provides characterization of H.265/HEVC SCC mode configurations against H.264/AVC JM for Scenario 3 Screen Content. In particular,

- Table 7.4.3.2-1 provides the BD rate gain of H.265/HEVC SCC with S3-SCC-01 against H.264/AVC JM with configuration S3-JM-01, i.e. with the screen content scenario reference sequences and no fixed intra.

- Table 7.4.3.2-2 provides the BD rate gain of H.265/HEVC SCC with S3-SCC-02 against H.264/AVC JM with configuration S3-JM-02, i.e. with the screen content scenario reference sequences with fixed Intra every second.

Table 7.4.3.2-1 BD rate gain of H.265/HEVC SCC with S3-SCC-01 against H.264/AVC JM with configuration S3-JM-01, i.e. with the screen content scenario reference sequences and no fixed intra

| Reference sequence | Name | psnr | y_psnr |
| --- | --- | --- | --- |
| S3-R01 | MovingText2-4K-10bit | Tbd | Tbd |
| S3-R02 | MovingText2-4K-8bit | Tbd | Tbd |
| S3-R03 | MovingText2-FullHD-10bit | Tbd | Tbd |
| S3-R04 | MovingText2-FullHD-8bit | Tbd | Tbd |
| S3-R05 | TextMixTransitions-4K-10bit | Tbd | Tbd |
| S3-R06 | TextMixTransitions-4K-8bit | Tbd | Tbd |
| S3-R07 | TextMixTransitions-FullHD-10bit | Tbd | Tbd |
| S3-R08 | TextMixTransitions-FullHD-8bit | Tbd | Tbd |
| S3-R09 | GraphicsMixSimple-4K-10bit | Tbd | Tbd |
| S3-R10 | GraphicsMixSimple-4K-8bit | Tbd | Tbd |
| S3-R11 | GraphicsMixSimple-FullHD-10bit | Tbd | Tbd |
| S3-R12 | GraphicsMixSimple-FullHD-8bit | Tbd | Tbd |
| S3-R13 | GraphicsMixTransitions-4K-10bit | Tbd | tbd |
| S3-R14 | GraphicsMixTransitions-4K-8bit | Tbd | Tbd |
| S3-R15 | GraphicsMixTransitions-FullHD-10bit | Tbd | Tbd |
| S3-R16 | GraphicsMixTransitions-FullHD-8bit | Tbd | Tbd |
| S3-R17 | Mission-Control | Tbd | Tbd |
| Minimum |  | Tbd | Tbd |
| Maximum |  | Tbd | Tbd |
| Average |  | Tbd | Tbd |

Table 7.4.3.2-2 BD rate gain of H.265/HEVC SCC with S3-SCC-02 against H.264/AVC JM with configuration S3-JM-02, i.e. with the screen content scenario reference sequences with fixed Intra every second

| Reference sequence | Name | psnr | y_psnr |
| --- | --- | --- | --- |
| S3-R01 | MovingText2-4K-10bit | Tbd | Tbd |
| S3-R02 | MovingText2-4K-8bit | Tbd | Tbd |
| S3-R03 | MovingText2-FullHD-10bit | Tbd | Tbd |
| S3-R04 | MovingText2-FullHD-8bit | Tbd | Tbd |
| S3-R05 | TextMixTransitions-4K-10bit | Tbd | Tbd |
| S3-R06 | TextMixTransitions-4K-8bit | Tbd | Tbd |
| S3-R07 | TextMixTransitions-FullHD-10bit | Tbd | Tbd |
| S3-R08 | TextMixTransitions-FullHD-8bit | Tbd | Tbd |
| S3-R09 | GraphicsMixSimple-4K-10bit | Tbd | Tbd |
| S3-R10 | GraphicsMixSimple-4K-8bit | Tbd | Tbd |
| S3-R11 | GraphicsMixSimple-FullHD-10bit | Tbd | Tbd |
| S3-R12 | GraphicsMixSimple-FullHD-8bit | Tbd | Tbd |
| S3-R13 | GraphicsMixTransitions-4K-10bit | Tbd | tbd |
| S3-R14 | GraphicsMixTransitions-4K-8bit | Tbd | Tbd |
| S3-R15 | GraphicsMixTransitions-FullHD-10bit | Tbd | Tbd |
| S3-R16 | GraphicsMixTransitions-FullHD-8bit | Tbd | Tbd |
| S3-R17 | Mission-Control | Tbd | Tbd |
| Minimum |  | Tbd | Tbd |
| Maximum |  | Tbd | Tbd |
| Average |  | Tbd | Tbd |

#### 7.4.3.3	Scenario 5: Online Gaming

This clause provides characterization of H.265/HEVC SCC mode configurations against H.264/AVC JM for Scenario 5 Online Gaming. In particular,

- Table 7.4.3.3-1 provides the BD rate gain of H.265/HEVC SCC with S5-SCC-01 against H.264/AVC JM with configuration S5-JM-01, i.e. with the online gaming scenario reference sequences and no fixed intra.

- Table 7.4.3.3-2 provides the BD rate gain of H.265/HEVC SCC with S5-SCC-02 against H.264/AVC JM with configuration S5-JM-02, i.e. with the online gaming scenario reference sequences with fixed Intra every second.

Table 7.4.3.3-1 BD rate gain of H.265/HEVC SCC with S5-SCC-01 against H.264/AVC JM with configuration S5-JM-01, i.e. with the online gaming scenario reference sequences and no fixed intra

| Reference sequence | Name | psnr | y_psnr |
| --- | --- | --- | --- |
| S5-R01 | AOV |  |  |
| S5-R02 | Baolei-Man |  |  |
| S5-R03 | Baolei-Woman |  |  |
| S5-R04 | Baolei-Balloon |  |  |
| S5-R05 | Baolei-Yard |  |  |
| S5-R06 | Jianling-Temple |  |  |
| S5-R07 | Jianling-Beach |  |  |
| S5-R08 | Heroes-of-the-Storm |  |  |
| S5-R09 | Project-CARS |  |  |
| S5-R10 | World-of-WarCraft |  |  |
| S5-R11 | MineCraft |  |  |
| S5-R12 | CS-GO |  |  |
| S5-R13 | StarCraft |  |  |
| Minimum |  |  |  |
| Maximum |  |  |  |
| Average |  |  |  |

Table 7.4.3.3-2 BD rate gain of H.265/HEVC SCC with S5-SCC-02 against H.264/AVC JM with configuration S5-JM-02, i.e. with the online gaming scenario reference sequences and fixed intra every second

| Reference sequence | Name | psnr | y_psnr |
| --- | --- | --- | --- |
| S5-R01 | AOV |  |  |
| S5-R02 | Baolei-Man |  |  |
| S5-R03 | Baolei-Woman |  |  |
| S5-R04 | Baolei-Balloon |  |  |
| S5-R05 | Baolei-Yard |  |  |
| S5-R06 | Jianling-Temple |  |  |
| S5-R07 | Jianling-Beach |  |  |
| S5-R08 | Heroes-of-the-Storm |  |  |
| S5-R09 | Project-CARS |  |  |
| S5-R10 | World-of-WarCraft |  |  |
| S5-R11 | MineCraft |  |  |
| S5-R12 | CS-GO |  |  |
| S5-R13 | StarCraft |  |  |
| Minimum |  |  |  |
| Maximum |  |  |  |
| Average |  |  |  |

As an example,

- Figure 7.4.3.3-1 provides Rate-Quality curves and BD rate gain for psnr of H.265/HEVC HM with S5-SCC-01 against H.264/AVC HM with configuration S5-JM-01 for reference sequence S5-R04

- Figure 7.4.3.3-2 provides Rate-Quality curves and BD rate gain for psnr of H.265/HEVC SCC with S5-SCC-02 against H.264/AVC HM with configuration S5-JM-02 for reference sequence S5-R04

Figure 7.4.3.3-1 Rate-Quality curves and BD rate gain for psnr of H.265/HEVC SCC with S5-SCC-01 against H.264/AVC HM with configuration S5-JM-01 for reference sequence S5-R04

Figure 7.4.3.3-2 Rate-Quality curves and BD rate gain for psnr of H.265/HEVC SCC with S5-SCC-01 against H.264/AVC HM with configuration S5-JM-02 for reference sequence S5-R04

All Rate-Quality curves and BD rate gain plots are provided in the attachment as well as online along with the test vectors.

#### 7.4.3.4	Summary

Table 7.4.3.4-1 provides a summary of BD rate gain in psnr and vmaf of H.265/HEVC SCC against H.264/AVC JM for different scenarios and configurations.

Table 7.4.3.4-1 Summary of BD rate gain in psnr of H.265/HEVC SCC against H.264/AVC JM for different scenarios and configurations

| Scenario | average psnr | min psnr | max psnr |
| --- | --- | --- | --- |
| S3 no intra |  |  |  |
| S3 intra |  |  |  |
| S5 no intra |  |  |  |
| S5 intra |  |  |  |
| Minimum |  |  |  |
| Maximum |  |  |  |