Agenda item: 	4.12

Source: 	Google

Title: 	[FS_5GVideo] Proposed AV1-related updates to TR26.955

Document for	Discussion and Agreement

# Introduction

Attached proposed AV1-related updates to TR 26.955 are provided.

# Summary of changes

CHANGE 1

Clause 5.6

- Indicate that residual energy/distortion-based decisions that can adapt the coding type of the current frame are allowed.

CHANGE 2

Clause 8.4.2.1

- As all questions regarding AV1 configurations have been addressed, remove the editor’s note and opening square bracket to indicate that the AV1 configurations are agreed.

- Change the libaom build version to 3gpp-2021-10-15-3, capturing all changes to the technology required to address questions and concerns raised.

- Remove editor’s note regarding the document ‘aomenc - command line parameters.’ The publication of this document on the AOMedia website will precede completion of TR26.955.

- In Table 8.4.2.1-1 1 Change common command line options used by all of AV1 configuration file scripts to disable temporal key frame filtering.

CHANGE 3

Clause 8.4.2.2

- Change Table 8.4.2.2.1-1 ‘Test Tuple generation with AV1 for Full HD Scenario’ to only use QP: [27,39,49,59]

- Change Table 8.4.2.2.2-1 ‘Common Scenario 1 command line options’ to disable intra block copy prediction and palette prediction modes.

CHANGE 4

Clause 8.4.2.3

- Change Table 8.4.2.3.1-1 ‘Test Tuple generation with AV1 for 4K-TV Scenario’ to only use QP: [27,39,49,59]

- Change Table 8.4.2.3.2-1 ‘Common Scenario 2 command line options’ to disable intra block copy prediction and palette prediction modes.

CHANGE 5

Clause 8.4.2.4

- Change Table 8.4.2.4.1-1 ‘Test Tuple generation with AV1 for Screen Content Scenario’ to only use QP: [27,39,49,59]

CHANGE 6

Clause 8.4.2.4

- Change Table 8.4.2.4.3-1 ‘Additional Scenario 3 command line options for S3-AV1-01’ to disable intra block copy prediction and palette prediction modes.

- Change Table 8.4.2.4.4-1 ‘Additional Scenario 3 command line options for S3-AV1-02’ to disable intra block copy prediction and palette prediction modes.

CHANGE 7

Clause 8.4.2.5

- Change Table 8.4.2.5.1-1 ‘Test Tuple generation with AV1, Messaging and Social Sharing Scenario’ to only use QP: [27,39,49,59]

- Change Table 8.4.2.5.2-1 ‘Common Scenario 4 command line options’ to disable intra block copy prediction and palette prediction modes.

CHANGE 8

Clause 8.4.2.6

- Change Table 8.4.2.6.1-1 ‘Test Tuple generation with AV1, Online Gaming Scenario’ to only use QP: [27,39,49,59]

CHANGE 9

Clause 8.4.2.6

- Change Table 8.4.2.6.3-1 ‘Additional Scenario 5 command line options for S5-AV1-01’ to disable intra block copy prediction and palette prediction modes.

- Change Table 8.4.2.6.4-1 ‘Additional Scenario 5 command line options for S5-AV1-02’ to disable intra block copy prediction and palette prediction modes.

CHANGE 10

Clause 8.4.2

- As all questions regarding AV1 configurations have been addressed, remove the editor’s closing square bracket to indicate that the AV1 configurations are agreed.