3GPP TSG-SA WG4 Meeting post- #128	S4aV240042

Online, 9 July 2024

Agenda item: 	3.8

Source: 	China Mobile Com. Corporation

Title: 	[FS_Beyond2D] Available Datasets, tools, softwares for Stereoscopic Video Test Sequences

Document for	Discussion and Agreement

Introduction

During the last Telco meeting, a scenario on “UE-to-UE Beyond 2D Video Streaming” had been discussed and agreed (S4aV240022 [1]) to be documented in Permanent Document of FS_Beyond2D (S4-241321 [2]). This proposal provides some datasets, generation softwares, and capturing tools for stereoscopic video test sequences.

Proposed

- Public Datasets

| Dataset | Synthetic/Natural | Stereo | Depth | Resolution | Download Link |
| --- | --- | --- | --- | --- | --- |
| InStereo2K [3] | Natural | Y | Y | 1080*860 | https://github.com/YuhuaXu/StereoDataset |
| ETH3D [4] | Natural | Y | Y | ~0.3 MP | https://www.eth3d.net/datasets#low-res-two-view-test-data |
| Middlebury 2021 Mobile stereo datasets [5] | Natural | Y | Y | 1920*1080 | https://vision.middlebury.edu/stereo/data/scenes2021/ |
| Middlebury 2014 stereo datasets [5] | Natural | Y | Y | 2964*1988 | https://vision.middlebury.edu/stereo/data/scenes2014/ |

- Copyrights:

InStereo2K: This dataset is made freely available to academic and non-academic entities for non-commercial purposes such as academic research, teaching, scientific publications, or personal experimentation.

ETH3D: You are free to (https://creativecommons.org/licenses/by-nc-sa/4.0/)

- Share — copy and redistribute the material in any medium or format

- Adapt — remix, transform, and build upon the material

The licensor cannot revoke these freedoms as long as you follow the license terms.

Middlebury:We grant permission to use and publish all images and numerical results on this website. If you report performance results, we request that you cite our paper. Instructions on how to cite our datasets are listed on the datasets page. If you want to cite this website, please use the URL "vision.middlebury.edu/stereo/".

- Algorithm for 2D-to-Stereo3D Conversion

- Algorithm

Various open-source algorithms and platforms use deep neural networks to perform real-time end-to-end conversion of 2D videos and images to stereoscopic 3D format. The performance of these conversions may vary based on the algorithm's efficiency and the hardware capabilities.

The following are capabilities and requirements of the Deep3D [6] algorithm (https://github.com/HypoX64/Deep3D):

Prerequisites:

- Linux, Mac OS, Windows

- Python 3.7+

- ffmpeg 3.4.6+

- Pytorch 1.7.1

- CPU or NVIDIA GPU

Inference speed:

| Plan | 360p (FPS) | 720p (FPS) | 1080p (FPS) | 4k (FPS) |
| --- | --- | --- | --- | --- |
| GPU (2080ti) | 84 | 87 | 77 | 26 |
| CPU (Xeon Platinum 8260) | 27.7 | 14.1 | 7.2 | 2.0 |

- Related tools

As 2D-to-Stereo3D conversion algorithms usually take RGB video format, the Python codes below can be used to convert between YUV and RGB formats.

A Python code for converting YUV to RGB:

- import cv2

- import numpy as np

- def read_yuv_frame(file, width, height):

- # calculate size of YUV frame

- frame_size = width * height * 3 // 2

- # Read YUV frames

- yuv = np.fromfile(file, dtype=np.uint8, count=frame_size)

- if yuv.size != frame_size:

- return None

- # reconstruct YUV data to YUV420 format

- y = yuv[0:width*height].reshape((height, width))

- u = yuv[width*height:width*height + (width//2)*(height//2)].reshape((height//2, width//2))

- v = yuv[width*height + (width//2)*(height//2):].reshape((height//2, width//2))

- return y, u, v

- def yuv_to_rgb(y, u, v):

- # convert YUV420 to YUV444

- u = cv2.resize(u, (y.shape[1], y.shape[0]), interpolation=cv2.INTER_LINEAR)

- v = cv2.resize(v, (y.shape[1], y.shape[0]), interpolation=cv2.INTER_LINEAR)

- yuv = np.stack((y, u, v), axis=-1)

- # convert yuv to rgb

- rgb = cv2.cvtColor(yuv, cv2.COLOR_YUV2RGB)

- return rgb

- def main():

- # read YUV file path

- yuv_file_path = 'video.yuv'

- # set width and height

- width, height = 1920, 1080

- with open(yuv_file_path, 'rb') as file:

- while True:

- # red yuv frame

- yuv_frame = read_yuv_frame(file, width, height)

- if yuv_frame is None:

- break

- y, u, v = yuv_frame

- # convert yuv to rgb

- rgb_frame = yuv_to_rgb(y, u, v)

- # display rgb frame

- cv2.imshow('RGB Frame', rgb_frame)

- if cv2.waitKey(30) & 0xFF == ord('q'):

- break

- cv2.destroyAllWindows()

- if __name__ == "__main__":

- main()

- A Python code for converting RGB to YUV:

- import cv2

- import numpy as np

- import time

- def rgb_to_yuv420(frame):

- # Using OpenCV for converting RGB to YUV format

- yuv = cv2.cvtColor(frame, cv2.COLOR_RGB2YUV_I420)

- return yuv

- def main():

- # read RGB video file path

- rgb_video_path = 'video.mp4'

- # set output YUV file path

- yuv_file_path = 'output.yuv'

- # open video file

- cap = cv2.VideoCapture(rgb_video_path)

- # check if video file is open

- if not cap.isOpened():

- print("Error: Could not open video.")

- return

- # get width and height of video file

- width = int(cap.get(cv2.CAP_PROP_FRAME_WIDTH))

- height = int(cap.get(cv2.CAP_PROP_FRAME_HEIGHT))

- print(f'Video dimensions: {width}x{height}')

- # set framerate

- fps = 60

- frame_duration = 1.0 / fps

- with open(yuv_file_path, 'wb') as yuv_file:

- while cap.isOpened():

- start_time = time.time()

- # read RGB frame

- ret, frame = cap.read()

- if not ret:

- break

- # convert RGB to YUV

- yuv_frame = rgb_to_yuv420(frame)

- # write YUV frame to yuv file

- yuv_file.write(yuv_frame)

- elapsed_time = time.time() - start_time

- wait_time = max(1, int((frame_duration - elapsed_time) * 1000))

- if cv2.waitKey(wait_time) & 0xFF == ord('q'):

- break

- cap.release()

- cv2.destroyAllWindows()

- if __name__ == "__main__":

- main()

- Example

The following is the test sequence that auto-converting a 2D video frame (BQTerrance 1920x1080, 60fps) to a stereoscopic 3D video (1920x1080 for each eye, 60fps) using CMCC’s  2D-to-Stereo3D conversion algorithm.

- Stereo 3D Video Capturing

- Another option is to use a dual-lens camera to directly capture stereo 3D video. There are many mobile devices on the market with this capability. For example, SpatialLabs Eyes provided by Acer is a stereoscopic camera capable of capturing up to 8-MP (4K) per eye at 30 fps or 2K per eye at 60 fps. Or the ZTE Nubia Pad 3D II can capture stereo 3D video with the following specifications:

- The main camera setup is the dual-camera systems includes two identical 13 MP lenses. These cameras capture slightly different perspectives of the same scene, mimicking the way human eyes perceive depth. The AI then processes these images to produced a coherent 3D representation.

| Rear Camera | Specification |
| --- | --- |
| Number of Cameras | 2 (Dual) |
| Resolution | 13 MP (wide); 13 MP (wide) |
| Autofocus | AF, AF |
| Video Recording | 1200 @ 30 fps |
| Others | LED Flash, panorama, HDR, Stereoscopic AI-powered 3D capture |

The selfie camera setup features two lenses positioned near the center of the top bezel when the tablet is oriented horizontally (with the longer side on the top). Here are the relevant specs:

| Front Camera | Specification |
| --- | --- |
| Number of Cameras | 2 (Dual) |
| Resolution | 8 MP (ultra wide); 8 MP (ultra wide) |
| Aperture | f/2.2, f/2.2 |
| Field of View | 105°, 105° |
| Video Recording | 1200 @ 30 fps |
| Others | Stereoscopic AI-powered 3D capture |

- The captured videos (in mp4 file) need further processing (e.g., reading the right/left views and concatenate them into one video frame) to generate the proper test sequences.

- A Python code for doing these tasks:

- import numpy as np

- import json

- import subprocess

- def videoInfo(filename):

- proc = subprocess.run([

- *"ffprobe -v quiet -print_format json -show_format -show_streams".split(),

- filename

- ], capture_output=True)

- proc.check_returncode()

- return json.loads(proc.stdout)

- def readVideo(filename):

- cmd = ["ffmpeg", "-i", filename]

- streams = 0

- for stream in videoInfo(filename)["streams"]:

- index = stream["index"]

- if stream["codec_type"] == "video":

- width = stream["width"]

- height = stream["height"]

- cmd += "-map", f"0:{index}"

- streams = streams + 1

- cmd += "-f", "rawvideo", "-pix_fmt", "rgb24", "-"

- shape = np.array([streams, height, width, 3])

- with subprocess.Popen(cmd, stdout=subprocess.PIPE) as proc:

- while True:

- # One byte per each element

- data = proc.stdout.read(shape.prod())

- if not data:

- return

- yield np.frombuffer(data, dtype=np.uint8).reshape(shape)

- if __name__ == '__main__':

- import matplotlib.pyplot as plt

- from PIL import Image

- idx = 1

- for left, right in readVideo("./StereoCaptured.mp4"):

- # concatenate left and right

- img = np.concatenate((left[0:1080,:], right[0:1080,:]), axis=1)

- img = Image.fromarray(img)

- img.save(f"imgs/{idx}.jpg")

- idx += 1

A FFMPEG command for storage them into .yuv

- ffmpeg -start_number 1 -r 30 -i %d.jpg -pix_fmt yuv420p -s 1920x1200 out.yuv

Example

The following is the test sequence captured with ZTE Nubia Pad 3D II main camera (1920*1200, 30fps) and post-processed by the above tools.

Proposal

- Document clause 2 of this document to the Permanent Document of FS_Beyond2D under the "UE-to-UE Beyond 2D Video Streaming" scenario as candidate test sequences.

- For test sequences, consider the following options:

-		Public dataset with appropriate copyright authorization

-		5 Synthetic Sequences and 5 Captured Sequences

Reference

[1]	S4aV240022, “[FS_Beyond2D] Scenario UE-to-UE Beyond 2D Video Streaming”

[2]	S4-241321 “	[FS_Beyond2D] Permanent Document v0.0.2”

[3]	Bao, W., Wang, W., Xu, Y. et al. InStereo2K: a large real dataset for stereo matching in indoor scenes. Sci. China Inf. Sci. 63, 212101 (2020). https://doi.org/10.1007/s11432-019-2803-x

[4]	T. Schöps, J. L. Schönberger, S. Galliani, T. Sattler, K. Schindler, M. Pollefeys, A. Geiger, "A Multi-View Stereo Benchmark with High-Resolution Images and Multi-Camera Videos", Conference on Computer Vision and Pattern Recognition (CVPR), 2017.

[5]	D. Scharstein, H. Hirschmüller, Y. Kitajima, G. Krathwohl, N. Nesic, X. Wang, and P. Westling. High-resolution stereo datasets with subpixel-accurate ground truth. In German Conference on Pattern Recognition (GCPR 2014), Münster, Germany, September 2014.

[6]	Xie, J., Girshick, R., Farhadi, A. (2016). Deep3D: Fully Automatic 2D-to-3D Video Conversion with Deep Convolutional Neural Networks. In: Leibe, B., Matas, J., Sebe, N., Welling, M. (eds) Computer Vision – ECCV 2016. ECCV 2016. Lecture Notes in Computer Science(), vol 9908. Springer, Cham. https://doi.org/10.1007/978-3-319-46493-0_51