3GPP TSG-SA4 Meeting #(AH)Video SWG post 128 (2024-06-25 - Online)	S4aV240044

# online,  , 25 Jun - 24 Jul 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PSEUDO CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.956 | CR | pseudo | rev | - | Current version: | 0.0.3 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [FS_Beyond2D] Representation Format - Stereoscopic Video with extensions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_Beyond2D |  |  |  |  |  | Date: |  |  | 2024-07-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The study item description in SP-240479 addresses the following objectives - Identify and document beyond 2D formats, that are market-relevant within the next years, generated from established and emerging capturing systems (including cameras for spatial video capturing), contribution, and usable on display technologies (smartphones, VR HMDs, AR glasses, autostereoscopic and multiscopic displays). During SA4#128, several scenarios were defined, that are considered to address the distribution scenarios and evaluation frameworks. However, some of the scenarios already assume a specific Representation Format that seems to be of less relevance initially. The evaluation framework is important, once Representation formats are defined. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document focusses on stereoscopic video with extensions. It is a starting point. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | S4aV240027; [FS_Beyond2D] On Representation Formats; Qualcomm Germany; Thomas Stockhammer. Revisions: none Presenter: Thomas Stockhammer Online Discussion: - Ralf: Also at MWS, Volucap also looks at distribution. On VFA, also point clouds is added. - Thomas: on VFA, - Jiayi: We have been discussing representation formats as part of the scenario - Thomas: Would like to see that we want to - Emmanuel: It seems that distribution and consumption are typically disconnected. Only end-to-end scenarios for UE to UE seem to make sense. - Thomas: To some extent I agree although I don’t want to draw concrete conclusions on our scenario yet. - Ralf: we should not excluded any formats - Gilles: this is not the case - Alexis: Good to have the formats, we should also document the pluses and minuses. - Thomas: yes, this is exactly the case. Decision: - Agreed to work on the representation formats S4aV240027 is agreed. |

## =====  CHANGE ===== (add to References)

[X]	Apple HEVC Stereo Video - Interoperability Profile (Beta), Version 0.9, June 21, 2023, https://developer.apple.com/av-foundation/HEVC-Stereo-Video-Profile.pdf

[Y]	Mike Swanson, "Spatial Video", March 7 2024, https://blog.mikeswanson.com/spatial-video/

[Z]	Video Contour Map Payload, Version 0.9, June 21, 2023, https://developer.apple.com/av-foundation/Video-Contour-Map-Metadata.pdf

## [A]	ITU-T H.273 (09/23), Coding-independent code points for video signal type identification

## =====  CHANGE =====

### 4.3.X	Stereoscopic Video with Extensions

#### 4.3.X.1	Definition

Stereoscopic video presents one image to the user’s left eye and another image (typically correlated) to the user’s right eye to produce the stereopsis effect, defined as "the perception of depth produced by the reception in the brain of visual stimuli from both eyes in combination; binocular vision." [X].

Different type of Beyond 2D video using projections to left and right eye may be used [Y]:

-	rectangular, traditional 3D

-	spherically-projected 3D video as defined in TS 26.118.

-	either of the two may be extended with additional depth data, also referred to as video contour maps [Z].

In addition, the detailed signal properties of the video each eye needs to be defined:

-	Sample aspect ratio for each eye, defined according to the ITU-T H.273 [A], SampleAspectRatio. Typical parameters are 1:1 (value 1) or 4:3 (value 14).

-	Picture aspect ratio for each eye, defined according ???. Typical parameters are 1:1 or 16:9.

- 	Resolutions per eye of left eye and right eye are

-	for picture aspect ratio 1:1: 1080x1080, 1440x1440, 2160x2160, 4320x4320

-	for picture aspect ratio 16:9: 1280x720, 1440x1080 (with sample aspect ration 4:3), 3840x2160, 7680x4320

- 	Framerates for each eye are: 30 fps, 50fps, 60 fps, 90 fps, 120 fps, 144 fps and possibly fractional variants.

-	YUV, 4:2:0 chroma subsampling, 8 or 10 bits

-	Signal characteristics

-	Colour primaries, defined according to the ITU-T H.273 [A], ColourPrimaries. Typical parameters are BT-709 (value 1), and BT-2020/BT-2100 (value 9).

-	Transfer characteristics, defined according to the ITU-T H.273 [A], TransferCharacteristics. Typical parameters are BT-709 (value 1), BT-2020 (value 14), BT-2100 PQ (value 16) and BT-2100 PQ (value 18).

-	Matrix coefficients, defined according to the ITU-T H.273 [A], MatrixCoefficients. Typical parameters are BT-709 (value 1), and BT-2020/BT-2100 non-constant luminance (value 9).

-	Typical combined values are BT-709 SDR with (1,1,1), HDR PQ with (9,16,9) and HDR HLG with (9,18,9).

-	Projection parameters:

-	Projection: rectilinear, fisheye, equirectangular

-	Field-of-view and restricted coverage.

Additional metadata may be present as follows:

-	hero eye: A value that indicates which eye is the primary eye when rendering in 2D.

-	camera parameters:

-	distance between the camera lens centers

-	disparity adjustment:

-	horizontal disparity adjustment, a value that indicates a relative shift of the left and right images, which changes the zero parallax plane.

NOTE: The parameters may be aligned with TS 26.118 [B]

-	Camera intrinsics

-	Line time (per camera)

-	Examples: https://github.com/MPEGGroup/FileFormatConformance/tree/m62054_exintrinsics/data/file_features/under_consideration

-	Disparity/depth map: 10bit, same resolution as source content, monochrome

#### 4.3.X.2 	Production and Capturing Systems

The formats as defined in clause 4.3.X.1 may be captures at least in reduced parameters by mobile devices and Head Mounted Displays (HMD):

-	https://techcrunch.com/2023/12/11/apple-releases-spatial-video-recording-on-iphone-15-pro/

-	Spatial Video if 1080p at 30fps

-	https://9to5mac.com/2024/01/04/will-the-iphone-16-be-able-to-record-4k-spatial-video/

-	Spatial Video with 4K may come

-	https://appleinsider.com/articles/24/03/06/capturing-spatial-video-apple-vision-pro-vs-iphone-15-pro

-	The spatial video captured is in a square 1:1 format at 2200 pixels by 2200 pixels. It is a near-perfect recreation of the passthrough viewed by the user.

-	Once capture stereo on iPhone 15 and offline postprocess to acquire accompanying depth (using ZoeDepth or similar).

-	Meta Quest can record spatial video: https://360rumors.com/quest-3-3d-videos/

-	After recording, you’ll see that the video or photo will be captured in side by side format, with a square aspect ratio. Photos will also be side by side but they are stretched vertically, and need to be edited to fix that.

-	https://deovr.com/blog/84-record-vr-footage-on-the-meta-quest-3

-	The Quest 3 features two cameras that deliver full-color passthrough, allowing users to record content in 4K (2k per eye), using the Meta Quest Developer HUB.

-	The Quest 3's passthrough cameras record footage that is flat 120-100 (possibly 90) degrees.

Beyond User Generated Content, an ecosystem is developing around this format including movie production, documentaries and live sports. Examples are mentioned here:

-	https://www.apple.com/newsroom/2024/02/2024-mls-season-kicks-off-today-exclusively-on-mls-season-pass-on-apple-tv/

-	https://www.apple.com/newsroom/2024/01/apple-previews-new-entertainment-experiences-launching-with-apple-vision-pro/

-	https://www.macrumors.com/2024/01/08/vision-pro-movies-games/

#### 4.3.X.3 	Rendering and Display Systems

-	Backward-compatible to 2D (just view one eye)

-	Apple Vision Pro

-	Meta Quest: https://techcrunch.com/2024/02/01/meta-quest-adds-support-for-apples-spatial-video-ahead-of-vision-pro-launch/

-	OpenXR APIs exist

-	WebXR APIs exist

#### 4.3.X.4 	Supporting Information

Editor’s Note

-	Typical quality criteria for evaluating the format

-	Existing test and reference sequences

- 	Conversion from other formats (lossless, lossy)

-	Uncompressed data size

-	Known compression technologies: Side-by-side, MV-HEVC, Delivery with DASH/HLS/CMAF

-	Extensibility of the format

#### 4.3.X.5 	Benefits and Limitations

##### 4.3.X.5.1	Benefits

-	Simplicity

-	Device Support

-	In device decoding and rendering

-	Content Industry starts

-	Extensible

-	UCG production

-	Very good user experience: "It isn’t too hyperbolic to say that immersive video — when done right — makes you feel like you’ve been teleported to a new location. While you might “have seen” a place in a traditional flat video, with immersive video, you’ve “been there.” If you haven’t experienced video like this, I’m jealous, because there’s nothing like the first time. If you own a Vision Pro and haven’t watched these yet, stop reading and do it now!"

##### 4.3.X.5.2	Limitations

-	3DOF and 3DOF+ only

-