3GPP TSG-SA4 MBS Adhoc on EMM-EFEC	S4-AHI244

13 – 15 December 2011

Copenhagen, Denmark

Agenda item: 	4

Source: 	Qualcomm Incorporated (Rapporteur)

Title: 	Permanent Document on EMM-EFEC

Version:	0.1

Document for	Proposal and Approval

# Introduction

In SP-110555 (also available as S4-110792) a new work item on "Enhancement to FEC for MBMS" has been approved during SA#53.

This document is an SA4 internal document to capture agreements in progressing the Work Item. In addition to this document a time plan is maintained. The latest version is available in document S4-AHI234.

# Use Cases

## Services

### Streaming Delivery

#### Introduction

The purpose of the MBMS streaming delivery method is to deliver continuous multimedia data (i.e. speech, audio, video and DIMS) over an MBMS bearer. The streaming delivery method is particularly useful for multicast and broadcast of scheduled streaming content. RTP is the transport protocol for MBMS streaming delivery. RTP provides means for sending real-time or streaming data over UDP.

TS26.346 defines a generic mechanism for applying Forward Error Correction to streaming media. The mechanism consists of three components:

(i)	construction of an FEC source block from the source media packets belonging to one or several UDP packet flows related to a particular segment of the stream(s) (in time). The UDP flows include RTP, RTCP, SRTP and MIKEY packets.

(ii)modification of source packets to indicate the position of the source data from the source packet within the source block

(iii)definition of repair packets, sent over UDP, which can be used by the FEC decoder to reconstruct missing portions of the source block.

The details on transport for the streaming delivery service are provided in section 2.1.1.2.

An alternative way to deliver streaming services over MBMS is the use of DASH and FLUTE. This is use case is discussed in section 2.1.2.4.

#### Transport in streaming delivery service

The MBMS streaming framework operates on RTP packets or more precisely UDP payloads, incoming at same or different UDP ports. According to TS26.346, clause 8.2.2, the FEC layer for streaming delivery is based on top of the UDP layer. The legacy RTP packets and the UDP port information are used in order to generate FEC repair symbols. Original UDP payloads become FEC source packets by appending a 3 byte FEC source payload ID field at the end of each UDP payload. These packets are then UDP encapsulated and transported on the IP multicast bearer.

According to Figure 1 a copy of these packets is forwarded to the FEC encoder and is arranged in a source block with row width T bytes at the first empty row. The encoding symbol starts at the beginning of a new row, but it is preceded by a 3 byte field containing the UDP flow ID (1 byte) and the length field (2 bytes). In case the length of the packet is not an integer of the symbol the remaining bytes in the last row are filled up with zero bytes. The source block is filled up to k rows whereby k is flexible and can be changed dynamically for each source block. The selection of k depends on the desired delay, the available terminal memory and also might depend on aspects such as desired zapping time in mobile TV applications. Typically for a streaming service a protection period is defined and the value of the protection period dynamically determines the source block size.

Figure 1 MBMS Streaming Framework

After processing all packets to be protected within one source block, the FEC encoder generates n-k FEC repair symbols of size T by applying FEC. The generated FEC repair symbols can be transmitted individually or as blocks of symbols as payload of a single UDP packet. Each FEC source and repair packet contains sufficient information such that the receiver can correctly insert them in the receiver source and repair block.

#### Examples

Examples are audio streaming applications or video streaming applications with bitrates ranging from 32 kbit/s to one or several MBit/s.

### Download Delivery

#### Introduction

According to TR26.946, the MBMS Download Delivery Method allows the error-free transmission of files via the unidirectional MBMS Bearer Services. The files are "downloaded" and stored in the local files-system of the user equipment. Files may contain multimedia content or any other binary data. The MBMS Download Delivery Method allows the transmission of an arbitrary number of files within a single data transfer phase.

Figure 1: Definition of MBMS Download Sessions

Figure 1 is an example of an MBMS User Service based on the Download Delivery Method. The file transmission events are organized in MBMS Download Sessions. Each session is started with a File Delivery Table (FDT) instance, which describes in this example each file within the MBMS Download Session in terms of file name and file type (MIME Content Type). The service operator and the actual service determine the timing of MBMS Download Sessions. Depending on the service type, the MBMS Download session may require strict or more relaxed time-constraint delivery of content.

#### Transport in download delivery service

This clause explains briefly how files are constructed for and transported during a FLUTE session. The BM-SC takes a file, e.g. a video clip or a still image, which is used as the transport object for FLUTE (see figure 2). The BM-SC constructs source block by breaking the file into contiguous portions of approximately equal size. Each source block is broken into source symbols. One or more encoding symbols are carried as the payload of a FLUTE packet, thus the FLUTE packet size must be divisible by the encoding symbol size. The target FLUTE packet size is configured by the BM-SC and, together with the file size, is used to determine the encoding symbol length. When FEC is used it may be beneficial to include several symbols in each FLUTE packet. Based on the transport object size, the encoding symbol size and the maximum source block length, FLUTE calculates the source block structure (i.e., the number of source blocks and their length).

Figure 2: Constructing FLUTE packets

The BM-SC communicates the transport object size, the encoding symbol size and the file size to the receivers within the FLUTE session transmission such that the receiver can also calculate the source block structure in advance of receiving a file.

The FLUTE packet is constructed from FLUTE header and payload containing one or more encoding symbols.

The distinction between file and transport object is that the file is the object provided to the BM-SC and played-out or stored at the MBMS UE. Within the scope of FLUTE sessions, content encoding may be used, for instance to compress the file with gzip for delivery. In the presence of FLUTE session content encoding, the file and the transport object will be different binary objects, and in the absence of content encoding the transport object will be identical to the file. Any symbol calculations (including FEC) are performed on transport objects.

#### Download Examples

In a typical use case, multimedia files typically in 3GP or MP4 format are distributed through download delivery method. In this case the delivery rate and the media rate may be completely different as no real-time consumption is considered.

Table 1 shows some typical examples of file sizes for different types of multimedia content.

Table 1 Examples for Download delivery use cases

| Number | File Size | Example |
| --- | --- | --- |
| 1 | 1 MByte (1 048 576 bytes) | AAC encoded audio clip |
| 2 | 3 MByte (3 145 728 bytes) | MP3 audio clip |
| 3 | 128 MByte (134 217 728) bytes | 30 min SD movie coded at 500 kbit/s |
| 4 | 1.8 GByte (1 887 436 800) bytes | 2 hours HD movie coded at 2 MBit/s |

#### DASH over download delivery service

In another use case as indicated in TS26.346, section 5.6, the download delivery method may be used to distribute DASH formatted content over MBMS. MBMS is designed to serve large receive groups with same content. The MBMS Download Delivery Method is designed to deliver an arbitrary number of (binary) files via MBMS to a large receiver population. MBMS Download defines several methods to increase reliability such as file repair. The download delivery method supports the delivery of media segments and even media presentation descriptions. Media segment URIs are described using the FDT in FLUTE.

In this case the media bit-rate and the delivery bitrate are typically the same to maintain real-time delivery capabilities and therefore the delivery delay of a segment is typically lower bounded by the segment duration.

Table 2 shows some typical examples of DASH media segment files for live services. In these examples, only one representation with constant media rate is being delivered over download delivery service.

Table 2 Examples for DASH segments

| Number | Segment duration and media rate | FLUTE object (one segment) Size |
| --- | --- | --- |
| 1 | 1 sec DASH segment 250 kbit/s stream | 32 kByte (32 768 bytes) |
| 4 | 1 sec DASH segment for 1 Mbit/s stream | 128 kByte (131 072 bytes) |
| 2 | 4 sec DASH segment 250 kbit/s stream | 128 kByte (131 072 bytes) |
| 3 | 4 sec DASH segment for 1 Mbit/s stream | 512 kByte (524 288 bytes) |

## Radio Access

### UTRAN

The MBMS Bearer service reuses most of the legacy UMTS protocol stack in the packet-switched domain. Only minor modifications are introduced to support MBMS. The IP packets are processed in the Packet Data Convergence Protocol (PDCP) layer where for example header compression might be applied. In the Radio Link Control (RLC) the resulting PDCP- Protocol Data Units (PDUs), generally of arbitrary length, are mapped to fixed length RLC-PDUs. The RLC layer operates in unacknowledged mode as feedback links on the radio access network are not available for point-to-multipoint bearers. Functions provided at the RLC layer are for example segmentation and reassembly, concatenation, padding, sequence numbering, reordering and out-of-sequence and duplication detection. The Medium Access Control (MAC) layer maps and multiplexes the RLC-PDUs to the transport channel and selects the transport format depending on the instantaneous source rate. The MAC layer and physical layer appropriately adapt the RLC-PDU to the expected transmission conditions by applying, among others, channel coding, power and resource assignment, and modulation.

During the MBMS definition, appropriate for UTRAN bearer settings for the simulation of FEC parameters had been defined and are summarized in Table 3.

Table 3 Typical UTRAN bearer parameters

| UTRAN Bearer parameters |  |  |
| --- | --- | --- |
|  | Bearer rates | 64 kbit/s, 128 kbit/s, 256 kbit/s |
|  | RLC PDU size | 640 bytes, 1 280 bytes, 1 280 bytes respectively |
|  | RLC BLER | 1%, 5%, 10%, 15%, 20%, 30% |
|  | RLC block loss pattern | Independent random loss |

### LTE eMBMS

NOTE: This text is not yet agreed, but kept as a starting point until additional information is provided. Specifically communication with RAN has been initiated in S4-111014, but complete feedback has not yet been received.

To obtain some representative numbers for the performance of an FEC code in an LTE MBMS environment, some simple models are necessary for AL-FEC evaluation.

Figure 2 shows the mapping of RLC-SDUs to RLC-PDUs. RLC-SDUs in the context of MBMS are IP packets. The RLC header is 1 byte if the RLC SDU consists of 1 IP packet. The header is longer, if multiple IP packets are multiplexed in an RLC-SDU. A reasonable assumption is to use 3 byte header of the RLC-PDU assuming a 5 bit sequence number. The loss of one RLC-PDU results in the loss of all IP packets included in the RLC-PDU.

The MAC PDU consists of a number of MAC SDUs, where a MAC-SDUs is an RLC-PDU. The MAC multiplexer notifies the RLC layer of the available bits. The RLC layer would then create an RLC PDU that fits exactly into the available space in the MAC PDU. There is no need for fragmentation of MAC SDUs across subframes. Based on this, it can be assumed that the loss of one MAC-PDU results in the loss of one RLC-PDU.

Figure 2 Mapping of IP packets (RLC-SDUs) to RLC-PDUs (see TS 36.300, section 6.2.2)

LTE MBMS defines modulations and coding schemes with a MAC-PDU size ranging from 680 bit to 18336 bit for a 5 MHz bandwidth. Reasonable MCS configurations to cover different environments are between MCS 17 (for rural areas) and MCS 24 (for dense urban areas). For 5 MHz, this results in MAC-PDU payload sizes of 7992 bits and 14112 bits, respectively.

Each MAC-PDU is mapped to a subframe. At allocation level 1, LTE MBMS can use up to 6 out of the 10 subframes of a 10ms frame. Each subframe is 1ms.

The interleaving for MBMS in LTE is the same as for regular unicast LTE delivery of 1ms. As a first order approximation, it can be assumed that at most one MAC-PDU per 10ms frame is delivered and it is assumed that MAC-PDUs encounter iid random losses.

As a reasonable configuration proposal for the simulations in this document, we propose to use

- a typical value of MCS 21 with 1335 bytes for the MAC-PDU payload size. This results then in a maximum RLC-PDU payload size of 1332 bytes.

- This results in a maximum RLC layer net bitrate of 1065.6 kbit/s for each subframe within the 10ms frame.

Typical loss rates of MAC-PDUs are in the range of 1-2%, but higher loss rates may occur occasionally up 10% and very bad conditions up to 20%. The latter also permits to give some indication on the performance in case losses are more correlated.

Typical LTE MBMS bearer parameters are provided in Table 4.

Table 4 Typical LTE MBMS bearer parameters

| LTE eMBMS Download |  |  |
| --- | --- | --- |
|  | Bearer bitrates | 266.4 kbit/s, 532.8 kbit/s, 1.0656 Mbit/s |
|  | RLC-SDU size | 1332 byte |
|  | RLC-SDU frequency | 40ms, 20ms, 10ms |
|  | MAC PDU loss pattern | iid random |
|  | MAC-PDU loss probability | 1%, 2%, 5%, 10%, 20% |

## Simulation conditions and assumptions (UTRAN)

The simulation conditions for UTRAN-based MBMS are provided in Table 5.

NOTE: Additional details on the simulation methodology are provided in contribution S4-AHI236, but this document is not yet reviewed.

Table 5 Simulation Conditions for UTRAN-based MBMS

| UTRAN Download |  |  |
| --- | --- | --- |
|  | Bearer rates | 64 kbit/s, 128 kbit/s, 256 kbit/s |
|  | RLC-PDU size | 640 bytes, 1 280 bytes, 1 280 bytes respectively |
|  | RLC-PDU BLER | 1%, 5%, 10%, 15%, 20%, 30% |
|  | RLC-PDU block loss pattern | Independent random loss |
|  | Number of trials | At least 10,000 for files 512 KB, 3,000 for 3 072 KB |
|  | File sizes | 50 KB, 512 KB, 3 072 KB |
|  | FLUTE payload size | 456 bytes |
|  | ROHC | No |
|  | IPv4/UDP header | 28 bytes |
|  | FLUTE header | 16 bytes |
|  | FEC overhead | Varied in steps of X packets, where X=ceil(0.005N) and N is the number of packets containing source data |
| UTRAN Streaming |  |  |
|  | Bearer rates | 64 kbit/s, 128 kbit/s and 256 kbit/s |
|  | RLC PDU size | 640 bytes (for 64 kbit/s bearer) 1280 bytes (for 128 kbit/s bearer) 1280 bytes (for 256 kbit/s bearer) |
|  | RLC BLER | 1 %, 5 %, 10 %, 15 %, 20 %, 30 % |
|  | RLC block loss pattern | Independent random loss |
|  | Content length | 24 hours of media content |
|  | Media rates | Varied by steps of 1 % of bearer rate, assuming only a single media stream with constant bitrate (see note 1) |
|  | FEC overhead | Varied to sum FEC and Media to equal bearer rate |
|  | Source packet RTP payload size | 64 kbit/s: 456 bytes 128 kbit/s: 456 bytes 256 kbit/s: 768 bytes |
|  | Repair packet RTP payload size | Minimum value supported by the FEC code which is not less than 470 (for 64 kbit/s and 128 kbit/s) and 782 (for 256 kbit/s) - (see note 2) |
|  | Protection period | 5 s, 20 s |
|  | ROHC | No |
|  | IPv4/UDP/RTP header | 40 |
| NOTE 1: In practice, multiple media streams may be carried within a single MBMS bearer. However, only a single media stream is considered for FEC simulation purposes for simplicity. NOTE 2: The last repair packet of a block may be shorter if supported by the FEC code in order to fit within the protection period. |  |  |

## Simulation conditions and assumptions (LTE eMBMS)

The simulation conditions for LTE-based MBMS are provided in Table 6.

NOTE: Additional details on the simulation methodology are provided in contribution S4-AHI236, but this document is not yet reviewed.

Table 6 Simulation Conditions for LTE-based MBMS

| LTE eMBMS Download |  |  |
| --- | --- | --- |
|  | RLC-SDU | 266.4 kbit/s, 532.8 kbit/s, 1.0656Mbit/s |
|  | RLC-SDUsize | 1332 byte TBD |
|  | RLC-SDU period | 40ms, 20ms, 10ms |
|  | MAC PDU loss pattern | iid random or/and Markov model TBD |
|  | MAC-PDU lossprobability | 1%, 2%, 5%, 10%, 20% TBD |
|  | Number of trials | At least 10,000 for files 1 MB, at least 3,000 otherwise |
|  | File sizes | 50kB, 1MB, 3MB, 128MB, 1.8GB |
|  | FLUTE payload size | 1288 bytes TBD |
|  | ROHC | No |
|  | IPv4/UDP header | 28 bytes |
|  | FLUTE header | 16 bytes |
|  | FEC overhead | Varied in steps of X packets, where X=ceil(0.005N) and N is the number of packets containing source data |
| LTE eMBMS Streaming (based on DASH) |  |  |
|  | Bearer rates | 266.4 kbit/s, 1.0656 Mbit/s |
|  | RLC payload size | 1332 byte TBD |
|  | RLC-SDU period | 40ms, 10ms |
|  | MAC PDU loss pattern | iid random or/and Markov model TBD |
|  | MAC-PDU loss probability | 1%, 2%, 5%, 10%, 20% TBD |
|  | Content length | 24 hours of media content |
|  | Media rates | Varied by steps of FLUTE payload sizes, but constant |
|  | FEC overhead | Varied to sum FEC and Media to equal bearer rate |
|  | FLUTE payload size | 1288 bytes TBD |
|  | Media Segment duration | 1s, 4s |
|  | Segment to FLUTE object mapping | Each Segment is mapped to one FLUTE object |
|  | Maximum delivery delay of FLUTE object | media segment duration |
|  | ROHC | No |
|  | IPv4/UDP/FLUTE header | 44 |

# Evaluation Criteria

## Code Performance

tbd

NOTE: Two documents had been provided for this meeting on this subject, S4-AHI241 and S4-AHI242. Using both documents for evaluation had been considered but was not agreeable.

## Primary Performance Metrics

For download delivery, the FEC Overhead required to achieve 99% probability of recovery of the file provides a very good indication for the system level performance.

The Overhead required for 99 % probability of recovery is computed as <tbd More details are provided in S4-AHI236, but this document has not yet been reviewed>.

Results following the mode as provided in Annex A of TR26.946 are expected.

Table 7 provides a reporting format for UTRAN test cases.

Table 7FEC Overhead required for 99 % probability for UTRAN download test cases

| Test Case | Error conditions | File size | Fec Overhead for 128/256 kbit/s | Fec Overhead for 64 kbit/s | Report [G;T; Z; K;N] values (See TS 26.346) |
| --- | --- | --- | --- | --- | --- |
| UD1 | Low (1% BLER) | Small (50KB) |  |  |  |
| UD2 |  | Medium (512KB) |  |  |  |
| UD3 |  | Large (3072KB) |  |  |  |
| UD4 | Medium (5% BLER) | Small (50KB) |  |  |  |
| UD5 |  | Medium (512KB) |  |  |  |
| UD6 |  | Large (3072KB) |  |  |  |
| UD7 | High (10% BLER) | Small (50KB) |  |  |  |
| UD8 |  | Medium (512KB) |  |  |  |
| UD9 |  | Large (3072KB) |  |  |  |
| UD10 | 15% BLER | Small (50KB) |  |  |  |
| UD11 |  | Medium (512KB) |  |  |  |
| UD12 |  | Large (3072KB) |  |  |  |
| UD13 | 20% BLER | Small (50KB) |  |  |  |
| UD14 |  | Medium (512KB) |  |  |  |
| UD15 |  | Large (3072KB) |  |  |  |
| UD16 | 30% BLER | Small (50KB) |  |  |  |
| UD17 |  | Medium (512KB) |  |  |  |
| UD18 |  | Large (3072KB) |  |  |  |

Table 8 provides a reporting format for LTE test cases.

Table 8 FEC Overhead required for 99 % probability for LTE download delivery test cases

| Test Case | Error conditions (tbd) | File size | FEC Overhead | Report [G;T; Z; K;N] values (See TS 26.346) |
| --- | --- | --- | --- | --- |
| LD1 | Very Low (1% BLER) | 50 kB |  |  |
| LD2 |  | Audio (1 MB) |  |  |
| LD3 |  | Clip(3 MB) |  |  |
| LD4 |  | SD (128 MB) |  |  |
| LD5 |  | HD(1.8 GB) |  |  |
| LD6 | Low (2 % BLER) | 50 kB |  |  |
| LD7 |  | Audio (1 MB) |  |  |
| LD8 |  | Clip(3 MB) |  |  |
| LD9 |  | SD (128 MB) |  |  |
| LD10 |  | HD(1.8 GB) |  |  |
| LD11 | Medium (5 % BLER) | 50 kB |  |  |
| LD12 |  | Audio (1 MB) |  |  |
| LD13 |  | Clip(3 MB) |  |  |
| LD14 |  | SD (128 MB) |  |  |
| LD15 |  | HD(1.8 GB) |  |  |
| LD16 | High (10 % BLER) | 50 kB |  |  |
| LD17 |  | Audio (1 MB) |  |  |
| LD18 |  | Clip(3 MB) |  |  |
| LD19 |  | SD (128 MB) |  |  |
| LD20 |  | HD(1.8 GB) |  |  |
| LD21 | 20% BLER | 50 kB |  |  |
| LD22 |  | Audio (1 MB) |  |  |
| LD23 |  | Clip(3 MB) |  |  |
| LD24 |  | SD (128 MB) |  |  |
| LD25 |  | HD(1.8 GB) |  |  |

### RTP-based Streaming Delivery over UTRAN

For RTP-based streaming delivery, as a suitable measure it was considered to evaluate the maximum supported Media Rate (kbit/s) for Mean Time Between FEC Block Loss of 1 hour.

Results following the mode as provided in Annex A of TR26.946 are expected.

Table 9 provides a reporting format for UTRAN streaming test cases.

Table 9Maximum supported Media Rate (kbit/s)
for Mean Time Between FEC Block Loss of 1 hour for UTRAN streaming test cases

| Test Case | Error rates | Bearer rate | Protection Period | Performance | Report [T; K;N] values (See TS 26.346) |
| --- | --- | --- | --- | --- | --- |
| US1 | Low (1% BLER) | Low (64kbit/s) | 5 sec |  |  |
| US2 |  |  | 20 sec |  |  |
| US3 |  | Medium (128kbit/s) | 5 sec |  |  |
| US4 |  |  | 20 sec |  |  |
| US5 |  | High (256kbit/s) | 5 sec |  |  |
| US6 |  |  | 20 sec |  |  |
| US7 | Medium (5% BLER) | Low (64kbit/s) | 5 sec |  |  |
| US8 |  |  | 20 sec |  |  |
| US9 |  | Medium (128kbit/s) | 5 sec |  |  |
| US10 |  |  | 20 sec |  |  |
| US11 |  | High (256kbit/s) | 5 sec |  |  |
| US12 |  |  | 20 sec |  |  |
| US13 | High (10% BLER) | Low (64kbit/s) | 5 sec |  |  |
| US14 |  |  | 20 sec |  |  |
| US15 |  | Medium (128kbit/s) | 5 sec |  |  |
| US16 |  |  | 20 sec |  |  |
| US17 |  | High (256kbit/s) | 5 sec |  |  |
| US18 |  |  | 20 sec |  |  |

### DASH-based Streaming Delivery over LTE

For DASH-based streaming delivery, as a similarly suitable measure it is considered to evaluate the media rate to support a Mean Time Between FEC Block Loss of 1 hour.

Test cases are considered for

- Low Latency (LL) with 1 sec segment duration as well as

- Medium Latency (ML) of 4 sec segment duration.

- Low-Bitrate (LB) at around 260 kbit/s and

- High-Bitrate (HB) at around 1MBit/s are considered.

Table 9 provides a reporting format for LTE streaming test cases.

Table 10 Media Bitrate in kbit/s
for Mean Time Between FEC Block Loss of 1 hour for LTE use cases

| Test Case | Error conditions (tbd) | Scenario | Supported Media Bitrate | Report [T;K;N] values (See TS 26.346) |
| --- | --- | --- | --- | --- |
| LS1 | Very Low (1 % BLER) | LL-LB |  |  |
| LS2 |  | LL-HB |  |  |
| LS3 |  | ML-LB |  |  |
| LS4 |  | ML-HB |  |  |
| LS5 | Low (2 % BLER) | LL-LB |  |  |
| LS6 |  | LL-HB |  |  |
| LS7 |  | ML-LB |  |  |
| LS8 |  | ML-HB |  |  |
| LS9 | Medium (5 % BLER) | LL-LB |  |  |
| LS10 |  | LL-HB |  |  |
| LS11 |  | ML-LB |  |  |
| LS12 |  | ML-HB |  |  |
| LS13 | High (10 % BLER) | LL-LB |  |  |
| LS14 |  | LL-HB |  |  |
| LS15 |  | ML-LB |  |  |
| LS16 |  | ML-HB |  |  |
| LS17 | 20% BLER | LL-LB |  |  |
| LS18 |  | LL-HB |  |  |
| LS19 |  | ML-LB |  |  |
| LS20 |  | ML-HB |  |  |

## Implementation-specific Performance Metrics

Codes not only differ in terms of the code efficiency but also in other performance criteria. Two important aspects are the required memory for decoding in the MBMS client as well as the complexity of the considered decoding algorithm.

Therefore, to judge the complexity of a decoding algorithm, the decoding speed in terms of bit/s on top of a recognized mobile processor platform running a recognized mobile operating system can provide good insight into the feasibility of executing the code for mobile applications.

In terms of memory requirements, a reasonable measure is the required random access memory in the MBMS client to decode large files, such as considered in the video delivery use cases from above.

Another performance metric for successful integration into mobile platforms is the library footprint of the code and the footprint of hardware functions if any.

The complexity and memory requirements shall in particular be provided for the following use cases: <tbd, currently 1.8 GByte at highest error rate; 4sec @ 1MBit/s streaming; 20sec protection period for RTP based streaming at 384 kbit/s>.

## Summary

<<TO BE UPDATED ONCE AGREED>>

In summary, the following information is collected

- tbd test cases on code performance as documented in section XXX.

- 18 test cases on FEC Overhead for UTRAN download test cases as document in Table 7

- 25 test cases on FEC Overhead for LTE download test cases as document in Table 8

- 18 test cases on maximum supported media rate for UTRAN streaming test cases as documented in Table 9.

- 20 test cases on maximum supported media rate for LTE streaming test cases as documented in Table 10.

- Implementation-specific metrics.

This results in total in 81 + tbd  test cases.

# Benchmark Codes

## Ideal Code

### Description

A code is generally capable to handle one or a few or many of the following parameters

- T: source symbol size

- K: source block size and number of source symbols

- N: word length and number of encoding symbols

An ideal code with parameters (K, N, T)  can reconstruct the K source symbols from any set of K of the N encoding symbols. Ideal codes exists, but are usually very complex in encoding and decoding, especially if K is not small or if N needs to be large.

We use the property of ideal codes to determine the performance bound for any code considered in this context.

### Evaluation Criteria

#### Code Performance

tbd

#### Download Delivery UTRAN

| Test Case | Error rates | File size | Fec Overhead for 128/256 kbit/s | Fec Overhead for 64 kbit/s | Report [G;T; Z; K;N] values (See TS 26.346) |
| --- | --- | --- | --- | --- | --- |
| UD1 | Low (1% BLER) | Small (50KB) | 7.5 % | 7.0 % |  |
| UD2 |  | Medium (512KB) | 3.1 % | 3.3 % |  |
| UD3 |  | Large (3072KB) | 2.1 % | 2.4 % |  |
| UD4 | Medium (5% BLER) | Small (50KB) | 20.0 % | 21.8 % |  |
| UD5 |  | Medium (512KB) | 11.2 % | 13.0 % |  |
| UD6 |  | Large (3072KB) | 8.8 % | 11.0 % |  |
| UD7 | High (10% BLER) | Small (50KB) | 35.0 % | 39.0 % |  |
| UD8 |  | Medium (512KB) | 21.5 % | 25.8 % |  |
| UD9 |  | Large (3072KB) | 17.8 % | 22.6 % |  |
| UD10 | 15% BLER | Small (50KB) | 50.0 % | 56.0 % |  |
| UD11 |  | Medium (512KB) | 32.0 % | 40.5 % |  |
| UD12 |  | Large (3072KB) | 28.0 % | 36.0 % |  |
| UD13 | 20% BLER | Small (50KB) | 66.0 % | 76.0 % |  |
| UD14 |  | Medium (512KB) | 44.6 % | 57.0 % |  |
| UD15 |  | Large (3072KB) | 38.0 % | 52.0 % |  |
| UD16 | 30% BLER | Small (50KB) | 106.0 % | 130.0 % |  |
| UD17 |  | Medium (512KB) | 72.0 % | 100.0 % |  |
| UD18 |  | Large (3072KB) | 66.8 % | 92.0 % |  |

#### Download Delivery LTE

| Test Case | Error conditions (tbd) | File size | FEC Overhead | Report [G;T; Z; K;N] values (See TS 26.346) |
| --- | --- | --- | --- | --- |
| LD1 | Very Low (1% BLER) | 50 kB |  |  |
| LD2 |  | Audio (1 MB) |  |  |
| LD3 |  | Clip(3 MB) |  |  |
| LD4 |  | SD (128 MB) |  |  |
| LD5 |  | HD(1.8 GB) |  |  |
| LD6 | Low (2 % BLER) | 50 kB |  |  |
| LD7 |  | Audio (1 MB) |  |  |
| LD8 |  | Clip(3 MB) |  |  |
| LD9 |  | SD (128 MB) |  |  |
| LD10 |  | HD(1.8 GB) |  |  |
| LD11 | Medium (5 % BLER) | 50 kB |  |  |
| LD12 |  | Audio (1 MB) |  |  |
| LD13 |  | Clip(3 MB) |  |  |
| LD14 |  | SD (128 MB) |  |  |
| LD15 |  | HD(1.8 GB) |  |  |
| LD16 | High (10 % BLER) | 50 kB |  |  |
| LD17 |  | Audio (1 MB) |  |  |
| LD18 |  | Clip(3 MB) |  |  |
| LD19 |  | SD (128 MB) |  |  |
| LD20 |  | HD(1.8 GB) |  |  |
| LD21 | 20% BLER | 50 kB |  |  |
| LD22 |  | Audio (1 MB) |  |  |
| LD23 |  | Clip(3 MB) |  |  |
| LD24 |  | SD (128 MB) |  |  |
| LD25 |  | HD(1.8 GB) |  |  |

#### RTP-based Streaming over UTRAN

| Test Case | Error rates | Bearer rate | Protection Period | Performance | Report [T;K;N] values (See TS 26.346) |
| --- | --- | --- | --- | --- | --- |
| US1 | Low (1% BLER) | Low (64kbit/s) | 5 sec | 56.8 |  |
| US2 |  |  | 20 sec | 60.6 |  |
| US3 |  | Medium (128kbit/s) | 5 sec | 116.3 |  |
| US4 |  |  | 20 sec | 122.6 |  |
| US5 |  | High (256kbit/s) | 5 sec | 237.4 |  |
| US6 |  |  | 20 sec | 246.4 |  |
| US7 | Medium (5% BLER) | Low (64kbit/s) | 5 sec | 47.4 |  |
| US8 |  |  | 20 sec | 54.2 |  |
| US9 |  | Medium (128kbit/s) | 5 sec | 102.2 |  |
| US10 |  |  | 20 sec | 112.5 |  |
| US11 |  | High (256kbit/s) | 5 sec | 228.0 |  |
| US12 |  |  | 20 sec | 224.5 |  |
| US13 | High (10% BLER) | Low (64kbit/s) | 5 sec | 39.5 |  |
| US14 |  |  | 20 sec | 47.5 |  |
| US15 |  | Medium (128kbit/s) | 5 sec | 88.5 |  |
| US16 |  |  | 20 sec | 101.8 |  |
| US17 |  | High (256kbit/s) | 5 sec | 182.0 |  |
| US18 |  |  | 20 sec | 201.5 |  |

#### DASH-based streaming over LTE

| Test Case | Error conditions (tbd) | Scenario | Supported Media Bitrate | Report [T;K;N] values (See TS 26.346) |
| --- | --- | --- | --- | --- |
| LS1 | Very Low (1 % BLER) | LL-LB |  |  |
| LS2 |  | LL-HB |  |  |
| LS3 |  | ML-LB |  |  |
| LS4 |  | ML-HB |  |  |
| LS5 | Low (2 % BLER) | LL-LB |  |  |
| LS6 |  | LL-HB |  |  |
| LS7 |  | ML-LB |  |  |
| LS8 |  | ML-HB |  |  |
| LS9 | Medium (5 % BLER) | LL-LB |  |  |
| LS10 |  | LL-HB |  |  |
| LS11 |  | ML-LB |  |  |
| LS12 |  | ML-HB |  |  |
| LS13 | High (10 % BLER) | LL-LB |  |  |
| LS14 |  | LL-HB |  |  |
| LS15 |  | ML-LB |  |  |
| LS16 |  | ML-HB |  |  |
| LS17 | 20% BLER | LL-LB |  |  |
| LS18 |  | LL-HB |  |  |
| LS19 |  | ML-LB |  |  |
| LS20 |  | ML-HB |  |  |

#### Implementation-specific Performance Metrics

Obviously, as the ideal code is generally "non-existing", no implementation specific performance metrics are available.

## MBMS FEC RFC5053

### Description

The code is fully specified in IETF RFC 5053 and is also used in MBMS TS26.346.

Simulation results are mostly available in Annex A of TR26.946. Additional simulation results for non-documented cases are provided below.

### Evaluation Criteria

#### Code Performance

tbd

#### Download Delivery UTRAN

| Test Case | Error rates | File size | Fec Overhead for 128/256 kbit/s | Fec Overhead for 64 kbit/s | Report [G;T; Z; K;N] values (See TS 26.346) |
| --- | --- | --- | --- | --- | --- |
| UD1 | Low (1% BLER) | Small (50KB) | 8.0 % | 8.0 % |  |
| UD2 |  | Medium (512KB) | 3.4 % | 3.6 % |  |
| UD3 |  | Large (3072KB) | 2.2 % | 2.6 % |  |
| UD4 | Medium (5% BLER) | Small (50KB) | 21.0 % | 22 % |  |
| UD5 |  | Medium (512KB) | 11.4 % | 13.4 % |  |
| UD6 |  | Large (3072KB) | 9.0 % | 11.2 % |  |
| UD7 | High (10% BLER) | Small (50KB) | 35.0 % | 39.0 % |  |
| UD8 |  | Medium (512KB) | 21.5 % | 26.0 % |  |
| UD9 |  | Large (3072KB) | 18.1 % | 22.8 % |  |
| UD10 | 15% BLER | Small (50KB) | 50.0 % | 56.0 % |  |
| UD11 |  | Medium (512KB) | 32.1 % | 41.0 % |  |
| UD12 |  | Large (3072KB) | 28.1 % | 37.0 % |  |
| UD13 | 20% BLER | Small (50KB) | 66.0 % | 76.0 % |  |
| UD14 |  | Medium (512KB) | 45.0 % | 57.0 % |  |
| UD15 |  | Large (3072KB) | 38.2 % | 52.0 % |  |
| UD16 | 30% BLER | Small (50KB) | 106.0 % | 130.0 % |  |
| UD17 |  | Medium (512KB) | 72.0 % | 100.0 % |  |
| UD18 |  | Large (3072KB) | 67.0 % | 92.0 % |  |

#### Download Delivery LTE

| Test Case | Error rates (tbd) | File size | FEC Overhead | Report [G;T; Z; K;N] values (See TS 26.346) |
| --- | --- | --- | --- | --- |
| LD1 | Very Low (1 % BLER) | Audio (1 MB) |  |  |
| LD2 |  | Clip(3 MB) |  |  |
| LD3 |  | SD (128 MB) |  |  |
| LD4 |  | GD(4 GB) |  |  |
| LD5 | Low (2 % BLER) | Audio (1 MB) |  |  |
| LD6 |  | Clip(3 MB) |  |  |
| LD7 |  | SD (128 MB) |  |  |
| LD8 |  | GD(4 GB) |  |  |
| LD9 | Medium (5 % BLER) | Audio (1 MB) |  |  |
| LD10 |  | Clip(3 MB) |  |  |
| LD11 |  | SD (128 MB) |  |  |
| LD12 |  | GD(4 GB) |  |  |
| LD13 | High (10 % BLER) | Audio (1 MB) |  |  |
| LD14 |  | Clip(3 MB) |  |  |
| LD15 |  | SD (128 MB) |  |  |
| LD16 |  | GD(4 GB) |  |  |
| LD17 | 20% BLER | Audio (1 MB) |  |  |
| LD18 |  | Clip(3 MB) |  |  |
| LD19 |  | SD (128 MB) |  |  |
| LD20 |  | GD(4 GB) |  |  |

#### RTP-based Streaming over UTRAN

| Test Case | Error rates | Bearer rate | Protection Period | Performance | Report [T;K;N] values (See TS 26.346) |
| --- | --- | --- | --- | --- | --- |
| US1 | Low (1% BLER) | Low (64kbit/s) | 5 sec | 55.8 |  |
| US2 |  |  | 20 sec | 60.4 |  |
| US3 |  | Medium (128kbit/s) | 5 sec | 115.5 |  |
| US4 |  |  | 20 sec | 122.4 |  |
| US5 |  | High (256kbit/s) | 5 sec | 236.2 |  |
| US6 |  |  | 20 sec | 245.7 |  |
| US7 | Medium (5% BLER) | Low (64kbit/s) | 5 sec | 46.6 |  |
| US8 |  |  | 20 sec | 53.6 |  |
| US9 |  | Medium (128kbit/s) | 5 sec | 100.8 |  |
| US10 |  |  | 20 sec | 111.8 |  |
| US11 |  | High (256kbit/s) | 5 sec | 227.0 |  |
| US12 |  |  | 20 sec | 224.0 |  |
| US13 | High (10% BLER) | Low (64kbit/s) | 5 sec | 38.5 |  |
| US14 |  |  | 20 sec | 47.2 |  |
| US15 |  | Medium (128kbit/s) | 5 sec | 87.5 |  |
| US16 |  |  | 20 sec | 101.2 |  |
| US17 |  | High (256kbit/s) | 5 sec | 179.5 |  |
| US18 |  |  | 20 sec | 200.5 |  |

#### DASH-based streaming over LTE

| Test Case | Error rates (tbd) | Scenario | Supported Media Bitrate | Report [T;K;N] values (See TS 26.346) |
| --- | --- | --- | --- | --- |
| LS1 | Very Low (1 % BLER) | LL-LB |  |  |
| LS2 |  | LL-HB |  |  |
| LS3 |  | ML-HB |  |  |
| LS4 |  | HL-HB |  |  |
| LS5 | Low (2 % BLER) | LL-LB |  |  |
| LS6 |  | LL-HB |  |  |
| LS7 |  | ML-HB |  |  |
| LS8 |  | HL-HB |  |  |
| LS9 | Medium (5 % BLER) | LL-LB |  |  |
| LS10 |  | LL-HB |  |  |
| LS11 |  | ML-HB |  |  |
| LS12 |  | HL-HB |  |  |
| LS13 | High (10 % BLER) | LL-LB |  |  |
| LS14 |  | LL-HB |  |  |
| LS15 |  | ML-HB |  |  |
| LS16 |  | HL-HB |  |  |
| LS17 | 20% BLER | LL-LB |  |  |
| LS18 |  | LL-HB |  |  |
| LS19 |  | ML-HB |  |  |
| LS20 |  | HL-HB |  |  |

#### Implementation-specific Performance Metrics

RFC5053 Raptor is designed to perform software-based encoding and decoding. The different aspects

- complexity of the decoding algorithm

- memory requirements

- footprint of the code

have been carefully designed to meet the hardware and software requirements when selecting the code for Release-6 in 2005.

In S4-0500357, section 3 provides some overview on the complexity of the Raptor decoding process. "With very low overhead, decoding a Raptor code requires approximately 18.9 XOR operations per symbol. Each of these requires at least one of its operands to be read from memory. Approximately 8.2 of these 18.9 require both operands to be read from memory. Finally, approximately 9.6 require the result to be written back to memory. The cost in weighted operations of these is:

- XOR operations (including one memory read):		4 x 18.9k = 75.4 k

- Additional memory reads:									4 x 8.2k = 32.9 k

- Memory writes:												4 x 9.6k = 38.2 k

The total weighted operations per symbol word is therefore: 146.6 k"

Due to sub-blocking, Raptor can provide very efficient memory handling. The maximum subblock size in TS26.346 is restricted to 256 kByte.

The footprint for the Raptor library is reported to be in the range of 100-150 kByte where a major part of the memory is dedicated to specifying all 8188 systematic indices.

# Qualification and Selection Criteria

## Qualification

tbd

## Selection

tbd

Thomas Stockhammer, c_tstock@qualcomm.com