3GPP TSG-SA WG#4 Conference call on 5GMS	S4-AHI858

# October 9th, 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Draft CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.501 | CR | XXXX | rev | 1 | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Stage 2 for Media Session Establishment; Interactions with network assistance |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | KPN N.V |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GMSA |  |  |  |  |  | Date: |  |  | 2019-10-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | TS 26.501 states that “the Media Session Handling interface can be used for consumption, QoE reporting, requesting different policy an charging treatment or network assistance services”. This dCR addresses the session interactions with the network assistance service. The Media Session Handler can interact with the Media AF for the network assistance service, but the interactions are not fully specified. Note: S4-191075 (TS 26.501 CR0002 rev3), as agreed during SA4#105, was used as a baseline. Note: This dCR is an addition to S4-AHI855 which modifies Clause 5.2 to simplify the Media AF interactions and it moves optional media plane functions to a new section 5.7. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Procedures focussed on network assistance for progressive download and DASH streaming are moved to a (new) section 5.8. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The interactions/procedures with the network assistance service stay unclear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | (5.2), 5.8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*** First change ***

## 5.8	Establish a Unicast Downling Streaming Session with Media AF interactions for network assistance

### 5.8.1	General

## This clause describes the establishment of a unicast downlink streaming session with Media AF interactions for network assistance, as described in Steps 7 of Clause 5.1. The MediaSession Handler provides the needed information to the Media AF, so that the Media AF can provide bitrate recommendations and activate QoS policies using the PCF or NEF, for example an AF session with QoS (see TS 23.502 [3], Clause 4.15.6.6).

### 5.8.2	Progressive Download of On-Demand Content

This procedure describes the establishment of a unicast media downlink streaming session with Media AF interactions for network assistance. A streaming session may use DASH format, 3GP File Format (Progressive Download), 3GP Time Text or other (potentially non-3GPP defined) formats.

Figure 5.8-1: High Level Procedure for progressive download for on-demand media

Prerequisite:

-	The 5GMS Application Provider has provisioned the 5G Media Streaming system and has setup content ingest.

-	The 5GMS Aware Application has received the service announcement from the 5GMS Application Provider

-	The 5GMS Application Provider has agreed one or more policies with the network provider and the 5GMS Application Provider is authorized to apply QoS dynamically during a downlink streaming session.

Steps:

1: The App triggers the Service and Content Discovery procedure.

The Content Discovery procedure only involves the App and the external Application Server and hence is out of scope of the present document.

2: A Media Player Entry is selected.

3: The App triggers the Media Session handler to start the playback. The media player entry is provided to the Media Session handler.

4: When the 5GMS Aware Application has received a reference to the service access information, the Media Session Handler interacts with the Media AF to acquire the whole service access information.

5: The Media Session Handler triggers the 5GMS Player to start the session.

6: The 5GMS Player establishes the transport session, e.g. the TCP connection.

7: The 5GMS Player notifies the Media Session Handler about the description of the application flow, e.g. the 5-Tuple.

8: The Media Session Handler establishes a network assistance session with the Media AF. The Media Session Handler provides the description of the application flow.

9: The 5GMS Player sends the request for the progressive download content.

10: The 5GMS Player receives the initialization information of the progressive download content. The initialization information contains configuration parameters for reception of the media, and optionally also DRM information.

11: The 5GMS Player configures the rendering pipeline for media playback.

12: The 5GMS Player notifies the media session handler, providing the transport session information and some media content related information.

13: Optional: 5GMS Player acquires a DRM License from an External AS.

14: The 5GMS Player receives media content and puts it into the rendering pipeline.

15: While the 5GMS Player continuously receives and plays back the media content, the 5GMS Player notifies the Media Session Handler in case of a (critically) low buffer.

16: The Media Session Handler requests a boost at the Media AF

17: The Media AF interacts with the NEF to apply a temporary QoS policy using the Nnef_AFsessionWithQoS service (cf. TS 23.502 [3] Clause 5.2.6.9), which is derived from the application flow description obtained in step 7. The Media AF may also interact with other network function such as the PCF.

18: The Media AF notifies the Media Session Handler providing boost parameters.

19: The 5GMS Player continuously receives and plays back the media content, repeating step 15 to 18 when necesarry

### 5.8.3	DASH Streaming

It is assumed here that the key information to initialize the media decoding and rendering pipeline is present in the Media Player Entry (or referenced by the Media Player Entry). The intention is to provide the client with information to setup the media decoding and rendering pipeline in such a way that no pipeline reset is needed during the session. Implementations need to consider that parts of the information are provided with the initialization segments.

It is assumed that the client is enabled to use the same media decoding and rendering pipeline during the session.

Figure 5.8-2: High Level Procedure for DASH content

Prerequisite:

-	The 5GMS Application Provider has provisioned the 5G Media Streaming system and has setup content ingest.

-	The 5GMS Aware Application has received the service announcement from the 5GMS Application Provider

-	The 5GMS Application Provider has aggreed one or more policies with the network provider and the 5GMS Application Provider is authorized to apply the QoS policies dynamically during a downling streaming session.

Steps:

1: The App triggers the Service and Content Discovery procedure. The Content Discovery procedure only involves the App and the external Application Server.

2: A media content item is selected.

3: The App triggers the Media Session handler to start media playback. The Media Player Entry is provided to the Media Session handler.

4: When the 5GMS Aware Application has received a reference to the service access information, the Media Session Handler interacts with the Media AF to acquire the whole service access information.

5: The Media Session Handler triggers the 5GMS Player to start the session.

6: The 5GMS Player establishes the transport session for acquiring the manifest referenced by the Media Player Entry.

7: The 5GMS Player requests the manifest

8: The media access client receives the manifest

9: The 5GMS Player processes the manifest. It determines for example the number of needed transport sessions for media acquisition. The 5GMS Player should be able to use the manifest information to initialize the media pipelines for each media stream. The manifest should also contain information to start the DRM client initialization, when DRM is used.

10: The 5GMS Player notifies the media session handler about the manifest. The notification may contain parameters from the manifest

11: Optional: the 5GMS Player acquires the necessary DRM information, for example a DRM License.

12: The 5GMS Player configures the media rendering pipeline.

13: The 5GMS Player establishes the necessary transport sessions (e.g, TCP connections) for the content. For example, the media access client may establish one transport session for each media component (audio, video, etc) and possibly additional transport sessions for other media representations.

14: The 5GMS Player notifies the Media Session Handler about the description of application flows, e.g. the 5-Tuples.

15: The Media Session Handler establishes a network assistance session with the Media AF. The Media Session Handler provides the description of the application flows.

16: The Media Session Handler reports the possible bitrates.

17: The Media AF selects a bitrate and (optionally) interacts with the NEF to apply a QoS policiy using the Nnef_AFsessionWithQoS service (cf. TS 23.502 [3] Clause 5.2.6.9), which is derived from the application flow description obtained in step 14. The Media AF may also interact with other network functions such as the PCF.

18: The Media AF provides the Media Session Handler with a bitrate recommendation, which contains the bitrate selected in step 16.

19: The Media Session Handler notifies the 5GMS Player about the bitrate recommendation. The 5GMS Player uses the bitrate recommendation in its adaptation logic.

20: The 5GMS Player requests initialization information. The 5GMSP repeats this step for each required initialization segment.

21: The 5GMS Player receives the initialization information.

22: In case of a (critically) low buffer, the 5GMS player notifies the Media Session Handler about the low buffer level.

23: The Media Session Handler requests a boost at the Media AF.

24: The Media AF interacts with the NEF to temporarily modify the QoS policy using the Nnef_AFSessionWithQoS service (cf. TS 23.502 [3] Clause 5.2.6.9), which is derived from the application flow descriptions obtained in step 14. The Media AF may also interact with the other network functions such as the PCF.

25: The Media AF notifies the Media Session Handler providing boost parameters.

26: The Media Session Handler notifies the 5GMS Player providing boost parameters.

27: The 5GMS Player requests media segments according to the manifest.

28: the 5GMS Player receives media segments and puts the information into the according media rendering pipeline.

29: Previous steps are repeated according to the manifest information.

*** End of first change ***