3GPP TSG-SA4 Conf Call	S4-AHI967

# 30 April, 2020, Electronic Meeting												revision of S4-200574

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Pseudo CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | <Spec#> | CR | <CR#> | rev | <Rev#> | Current version: | <Version#> |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | M1d Procedure for Dynamic Policy Provisioning |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson LM |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | <Related_WIs> |  |  |  |  |  | Date: |  |  | <Res_date> |
| Category: | <Cat> |  |  |  |  |  | Release: |  |  | <Release> |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | As discussed is tdoc <>, the usage of N5 and N33 APIs require a certain set of parameters, which should not be discloded to a 5GMSd client. The 5GMSd AF stores parameters which identifies the 5GMSd Application Provider as authorized entity towards the 5G System. Additional parameters with MNO defined values are stored in the policy template, which are needed for the operations (like the QoS reference parameters). In addition, a set of bitrate parameters are needed when activating a QoS bearer via the N5 interface. Similar parameters for N33 are not yet completely identified. It is suggested to enable the 5GMSd client to determine the set of bitrate parameters. According Stage 2 alignment is needed. |  |  |  |  |  |  |  |  |
| Summary of change: |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

**** Discussion ****

The Policy Templates and the N5 / N33 APIs can be used for different dynamic PCC related use-cases, which are

- Dynamic QoS: The 5GMSd client desires to use a QoS bearer for the application flows.

- Background Data: The 5GMSd client desires to put a VoD asset download flow onto a background data policy

- Conditional Zero Rating (Dynamic Charging): The data of the traffic flow should not be charged (or differently charged), since the bitrate remains below a certain bitrate threshold. (also called Binge-on)

- Dynamic Charging: The data flow should be charged differently.

**** First Change ****

### 4.2.4	Dynamic Policy Configuration procedures

#### 4.2.4.1	General

These procedures are used by the 5GMSd Application Provider and the 5GMSd AF on M1d to configure different policy templates, which are used in combination of the dynamic policy invocation API. They are further elaborated in clause 5.X.

#### 4.2.3.2	Create a new Policy Template resource

This procedure is used by the 5GMSd Application Provider to create a new Policy Template resource. The 5GMSd Application Provider shall use the HTTP POST method for this purpose.

If the procedure is successful, the 5GMSd AF shall generate a resource id representing the new Policy Template Configuration. In this case, the 5GMSd AF shall respond with a 201 (Created) HTTP response message, and provide the URL to the newly created resource in the Location header field.

#### 4.2.3.3	Get all Policy Template ids

This procedure is used by the 5GMSd Application Provider to obtain the list of existing Policy Templates resource ids from the 5GMSd AF. The HTTP GET method shall be used for this purpose.

If the procedure is successful, the 5GMSd AF shall respond with a 200 (OK) response message that includes the Policy Template resource ids in the response message body.

#### 4.2.3.4	Read a Policy Template resource

This procedure is used by the 5GMSd Application Provider to obtain the properties of an existing Policy Template resource from the 5GMSd AF. The HTTP GET method shall be used for this purpose.

If the procedure is successful, the 5GMSd AF shall respond with a 200 (OK) response message that includes the Policy Templates in the response message body.

#### 4.2.3.5	Update properties of a Policy Template resource

The update operation is invoked by the 5GMSd Application Provider to modify / overwrite the properties of an existing Policy Templates resource. All available parameters may be updated at once using the HTTP PUT method (overwrite). Selected properties are modified using the HTTP PATCH.

If the procedure is successful, the 5GMSd AF shall respond with a 200 (OK) and provide the content of the resource in the response, reflecting the successful update operation.

#### 4.2.3.6	Delete a Policy Template resource

This operation is used by the 5GMSd Application Provider to delete a Policy Template resource. The HTTP DELETE method shall be used for this purpose. As a result, the 5GMSd AF will release any associated resources.

If the procedure is successful, the 5GMSd AF shall respond with a 200 (OK) response message.

Editor’s Note: Check the HTTP Status Codes.

**** Next Change ****

## 5.2a	Policy Templates Configuration API

### 5.2a.1	Overview

This clause specifies the API that a 5GMSd Application Provider uses at interface M1d to provision and manage 5GMSd AS policy template configurations by interacting with a 5GMSd AF. Each policy template configuration is represented by a PolicyTemplate, the data model for which is specified in clause 5.2a.2 below. The RESTful resources for managing policy templates are specified in clause 5.2a.3 and the operations on these resources are further elaborated in clause 5.2a.4.

### 5.2a.2	Data model

#### 5.2a.2.1 PolicyTemplate data type

#### A single PolicyTemplate data model is defined in Table 5.2a.2.2-1.

Table 5.2a.2.2-1: Definition of PolicyTemplate data type

| Property name | Type | Cardinality | Description |
| --- | --- | --- | --- |
| policyTemplateId | String | 1..1 | Unique identification (key) of the policy template. The Media Session Handler uses the policyTemplateId on M5d, when activating a dynamic policy. |
| api | enum | 1..1 | Identified, which N5 or N33 API should be called. N5: Npcf Policy Authorization Service, n33: AsSessionWithQoS or CHargableParty |
| afAppId | AfAppId | 1..1 | See Npcf Policy Authorzation Service |
| aspId | AspId | 1..1 | See Npcf Policy Authorzation Service |
| qosReference | String | 0..1 | See Npcf Policy Authorzation Service and AsSessionWithQoS |
| afChargId | String | 0..1 | See Npcf Policy Authorzation Service |
| bdtRefId | String | 0..1 | See Npcf Policy Authorzation Service amd ApplingBdtPolicy |
| maxBitrateDl | Bitrate | 0..1 | Defines a maximum authorized downlink bitrate for the policy template |
| maxBitrateUl | Bitrate | 0..1 | Defines a maximum authorized uplink bitrate for the policy template |

### 5.2a.3	Resource structure

The policy templates provisioning API is accessible through this URL paths:

{apiRoot}/3gpp-m1d/v1/application-service-providers/{application-service-provider-id}/{provisioning-subresource}

The following operations and the corresponding HTTP methods are supported. In each case, the sub-resource path specified in the second column shall be substituted into {provisioning-subresource} in the above URI template:

| Operation | {apiSpecificResourceUriPart}- | Allowed HTTP method(s) | Description |
| --- | --- | --- | --- |
| Create a new Policy Template resource | policyTemplates | POST | This is used to create a new Policy Template resource. |
| Fetch all Policy Template resource ids | policyTemplates | GET | This is used to retrieve all existing Policy Templates resource ids. |
| Fetch a Policy Template resource |  | policyTemplates/{policy-template-id} | GET | This is used to retrieve an existing Policy Template resource. |  |  |
| Update a Policy Template resource | policyTemplates/{policy-template-id} | PUT, PATCH | This operation is used to modify the properties of an existing Policy Template resource. |
| Delete a Policy Templates resource | policyTemplates/{policy-template-id} | DELETE | This operation is used to delete an existing Policy Template resource. |

### 5.2a.4	Operations

The 5GMSd Application Provider creates Policy Template resource using the HTTP POST method. The URL for the newly created policy template resource is returned within the LOCATION HTTP Header field.

The 5GMSd Application Provider reads a list of Policy Template resource ids using the HTTP GET method. The response contains a list of resource-ids.

The 5GMSd Application Provider sets / overwrites an Policy Template resource using the HTTP PUT method. The request body shall contain the PolicyTemplate object.

Application Provider modifies a PolicyTemplate objects using the HTTP PATCH method. The request body shall contain the PolicyTemplate object.

**** Next Change ****

#### 5.3.2.1	DynamicPolicy resource type

Table 5.3.2.1-1: Definition of Dynamic Policy resource

| Attribute name | Data type | Cardinality | Description |
| --- | --- | --- | --- |
| policyTemplateId | String | 1 | Identifies the Policy Template which should be applied to the application flow(s). |
| flowDescription | Object | 1 | Refer to subclause 5.3.8 of 3GPP TS 29.214 for encoding. |
| provisioningSessionId | String | 1 | Uniquely Identifies 5GMSd Application Service Configuration, which is linked to the Application Service Provider. |
| marBwDl | BitRate |  | Maximum requested bandwidth for the Downlink. |
| marBwUl | BitRate |  | Maximum requested bandwidth for the Uplink. |
| minDesBwDl | BitRate |  | Minimum desired bandwidth for the Downlink. |
| minDesBwUl | BitRate |  | Minimum desired bandwidth for the Uplink. |
| mirBwDl | BitRate |  | Minimum requested bandwidth for the Downlink. |
| mirBwUl | BitRate |  | Minimum requested bandwidth for the Uplink. |
| enforcementMethod | String | 1 | Description of the Policy Enforcement Method. The parameter is set by the 5GMSd AF. |