3GPP TSG-SA4 Ad hoc meeting on 5GMS3	S4-AHI865

# The Hague, The Netherlands, November 12-14, 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| draft CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.501 | CR | <CR#> | rev | 1 | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Stage 2 for interactions with network assistance |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | KPN N.V. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GMSA |  |  |  |  |  | Date: |  |  | 2019-11-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | TS 26.501 states that “the Media Session Handling interface can be used for consumption, QoE reporting, requesting different policy an charging treatment or network assistance services”. The Media Session Handler can interact with the 5GMSd AF for the network assistance service, but the interactions are not fully specified. This dCR addresses the session interactions with the network assistance service. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 5GMSd AF interactions for network assistance, and in particular for providing the 5GMS player with information about the available bandwidth, for downlink streaming are added in section 5.8. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The interactions with network assistance remain unclear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

**** First change ****

## 5.8	Establish a Unicast Media Downlink Streaming Session with 5GMSd AF interactions for NA bandwidth availability information.

### 5.8.1	General

This clause describes the establishment of a unicast downlink streaming session with 5GMSd AF interactions to obtain bandwidth availability information. The 5GMSd AF monitors 5GMS Players via the 5GMSd AS and provides the 5GMSd Players bandwidth information via the Media Session Handler, assisting the 5GMS Players in selecting a media content representation and sharing bandwidth among 5GMS players. The 5GMSd AF may also activate QoS policies to guarantee bandwidth availability using the PCF or NEF, for example an AF session with QoS (see TS 23.502 [3], Clause 4.15.6.6).

### 5.8.2	DASH Streaming

This procedure describes the establishment of a unicast media downlink streaming session with 5GMSd AF interactions for network assistance. A streaming session may use DASH format or other (potentially non-3GPP defined) formats.

It is assumed here that the key information to initialize the media decoding and rendering pipeline is present in the Media Player Entry (or referenced by the Media Player Entry). The intention is to provide the client with information to setup the media decoding and rendering pipeline in such a way that no pipeline reset is needed during the session. Implementations need to consider that parts of the information are provided with the initialization segments.

It is assumed that the client is enabled to use the same media decoding and rendering pipeline during the session.

Figure 5.8-1: High Level Procedure for DASH content

Prerequisite:

-	The 5GMS Application Provider has provisioned the 5G Media Streaming system and has setup content ingest.

-	The 5GMS Aware Application has received the service announcement from the 5GMS Application Provider.

-	The service announcement information contains the URL of the 5GMSd AF and information about the guarantees of the available bandwidth.

-	In case bandwidth availability is guaranteed, the 5GMS Application Provider has agreed one or more QoS policies with the network provider and is authorized to apply the policies dynamically.

-	The 5GMSd AS(s) are set up to provide logging information to the 5GMSd AF.

Steps:

1: The App triggers the Service and Content Discovery procedure. The Content Discovery procedure only involves the App and the external Application Server.

2: A media content item is selected.

3: The App triggers the Media Session handler to start media playback. The Media Player Entry is provided to the Media Session handler.

4: When the 5GMS Aware Application has received a reference to the service access information, the Media Session Handler interacts with the 5GMSd AF to acquire the whole service access information.

5: The Media Session Handler triggers the 5GMS Player to start the session.

6: The 5GMS Player establishes the transport session for acquiring the manifest referenced by the Media Player Entry.

7: The 5GMS Player requests the manifest

8: The media access client receives the manifest

9: The 5GMS Player processes the manifest. It determines for example the number of needed transport sessions for media acquisition. The 5GMS Player should be able to use the manifest information to initialize the media pipelines for each media stream. The manifest should also contain information to start the DRM client initialization, when DRM is used.

10: Optional: the 5GMS Player acquires the necessary DRM information, for example a DRM License.

11: The 5GMS Player configures the media rendering pipeline.

12: The 5GMS Player establishes the necessary transport sessions for the content (e.g. TCP connections). For example, the media access client may establish one transport session for each media component (audio, video, etc) and possibly additional transport sessions for other media representations.

13: The Media Session Handler is notified about the Flow description(s).

14: The Media Session Handler establishes a network assistance session with the 5GMSd AF. If the available bandwidth is guaranteed, the Media Session Handler provides the Flow description(s) to the 5GMSd AF obtained in step 13.

15: The 5GMS Player requests initialization information. The 5GMS Player repeats this step for each required initialization segment.

16: The 5GMS Player receives the initialization information.

17: The 5GMSd AF receives media segment request logs from the 5GMSd AS(s).

18: The 5GMSd AF estimates the bandwidth usage of 5GMSd Players by analysing recent media segment requests. The 5GMSd AF determines the total bandwidth usage and divides the total bandwidth among the 5GMS Players.

19: The 5GMSd AF The 5GMSd AF provides the Media Session Handler with bandwidth availability information as determined in step 18.

20: This step applies when bandwidth availability is guaranteed and the 5GMSd AF resides in the trusted data network: The 5GMSd AF interacts with the PCF or NEF to apply/modify a QoS policy when needed. When interacting with the PCF, the 5GMSd AF uses the Npcf_PolicyAuthorization service as defined in TS 23.502 (Clause 5.2.5.3). When interacting with the NEF, the 5GMSd AF uses the Nnef_AFSessionWithQoS service as defined in TS 23.502 (Clause 4.15.6.6).

21: This step applies when bandwidth availability is guaranteed and the 5GMSd AF resides in the untrusted data network: The 5GMSd AF interacts with the NEF to apply/modify a QoS policy when needed using the Nnef_AFSessionWithQoS service as defined in TS 23.502 (Clause 4.15.6.6).

22: The Media Session Handler notifies the 5GMS player about bandwidth availability, as obtained in step 19.

23: The 5GMS Player requests media segments according to the manifest and in a representation such that the bandwidth used to obtain the media segments does not exceed the bandwidth availability. Note that the 5GMS Player may request multiple media segments in parallel for different media components (audio, video, etc) and the combined bandwidth use for each media segment retrieval does not exceed the bandwidth availability.

24: the 5GMS Player receives media segments and puts the information into the according media rendering pipeline.

25: Previous steps are repeated. Steps 17-22 may (occasionally) also be repeated.

**** End of first change ****