3GPP TSG-SA MBS SWG Adhoc	S4-AHI870

# The Hague, The Netherlands, 12-14 November 2019			Revision of S4-191158

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| pseudo CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.512 | CR | <CR#> | rev | <Rev#> | Current version: | 0.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Stage 3 for Media Session Establishment; Interactions with policy control |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson LM |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GMS3 |  |  |  |  |  | Date: |  |  | 25 June 2019 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | TS 26.501 considers that the Media AF “may relay or initiate a request for different Policy or Charging Function (PCF) treatment or interact with other network functions”. This pCR addresses the according session interactions. The Media Session Handler can interact with the Media AF for different reasons (e.g. policy and charging function interaction, as identified in TR 26.891, Clause 6.4). A set of parameters is needed for the network assistance procedures. The number of parameters may depend on the use-case and the requested Network Assistance service. Note, the pCR starts with a general discussion, which should not be added to draft TS 26.512. |  |  |  |  |  |  |  |  |
| Summary of change: |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

**** First Change ****

### 4.4.2. Procedures for dynamic policy updates

This procedure is used by a Media Session Handler to perform the resource management of dynamic policies via the M5d interface. A dynamic policy consists of a Policy type indication, flow description(s), an 5GMS Application Provider identifier (ASP Id) and potentially other parameters.

Editor’s Note: Details on the parameter usage are for further study

A Policy type indication identifies the requested policy for an application flow such as specific QoS, background data or different charging treatments. Example values are defined in Clause [].

The flow description(s) allows the identification and classification of the media traffic, such as the packet filter sets [TS 23.501 Clause 5.7.6].

5GMS Application Provider identifier (ASP Id) allows the Media AF to setup an AF Session with e.g. the PCF or NEF. The Media AF derives the AF Identifier from the 5GMS Application Provider identifier.

In order to create a resource for an individual dynamic policy, the Media Session Handler shall first create a new application instance and then create the resource for the individual dynamic policy. When the Media Session Handler needs several individual dynamic policies, it repeats the second step as often as needed, i.e. reuses the application instance for additional dynamic policies.

The Media Session Handler creates a new application instance by sending an HTTP POST message to the Media AF. The body of the HTTP POST message shall include ASP Id to which the Media Session Handler is belonging. The Media AF creates a new Resource URL for the application instance. The Media AF shall respond to the Media Session Handler with a 201 Created message, including the URL for the created resource within the Location header field.

The Media Session Handler shall use the URL for the application instance in subsequent requests to the Media AF to create individual dynamic policies. The request body contains at least the requested policy type indication for the dynamic policy and a flow description. The Media AF shall respond to the Media Session Handler with a 201 Created message, including a Location header field containing the URL for the created individual dynamic policy resource and a message body.

The Media Session Handler can modify the data of an individual dynamic policy resource using either the HTTP PUT or PATCH methods.

The Media Session Handler can delete an individual dynamic policy resource using the HTTP DELETE methods. The Media AF shall trigger the appropriated actions towards other Network Functions like PCF or NEF.

**** Next Changes ****

## 5.x	DynamicPolicies API

### 5.x.1	Overview

The DynamicPolicies API is a RESTful API that allows the media session handler to request a specific policy and charging treatment. The API defines a set of data models, resources and the related procedures for the creation and management of the dynamic policy request. The corresponding JSON schema for the representation of the resources and operations defined by the API is provided in its complete form in Annex ZZ.

### 5.x.2	Data model

#### 5.x.2.1	Resource data types

##### 5.x.2.1.1	Type: ApplicationInstance

Table 5.x.2.1.1-1: Definition of type ApplicationInstance

| Attribute name | Data type | Cardinality | Description | Applicability (NOTE 1) |
| --- | --- | --- | --- | --- |
| aspid | String | 1 | 5GMSA Application Provider Id. |  |

##### 5.x.2.1.2	Type: DynamicPolicySessions

Table 5.x.2.1.2-1: Definition of type DynamicPolicySessions

| Attribute name | Data type | Cardinality | Description | Applicability (NOTE 1) |
| --- | --- | --- | --- | --- |

##### 5.x.2.1.3	Type: DynamicPolicySession

Table 5.x.2.1.3-1: Definition of type DynamicPolicySession

| Attribute name | Data type |  | Cardinality | Description | Applicability (NOTE 1) |
| --- | --- | --- | --- | --- | --- |
| policyTypeIndicator | String |  | 1 |  |  |
| flowDescription | Object | 1 | Refer to subclause 5.3.8 of 3GPP TS 29.214 for encoding |  |

##### 5.x.2.1.4	Type: DynamicPolicySessionReply

Table 5.x.2.1.3-1: Definition of type DynamicPolicySession

| Attribute name | Data type | Cardinality | Description | Applicability (NOTE 1) |
| --- | --- | --- | --- | --- |
| policyTypeIndicator | String | 1 |  |  |
| flowDescription | Object | 1 | Refer to subclause 5.3.8 of 3GPP TS 29.214 for encoding |  |
| enforcementMethod | String | 1 | Description of the Policy Enforcement Method |  |

### 5.x.3	Resource structure

#### 5.x.3.1	General

All resource URIs of this API should have the following root:

- {apiRoot}/3gpp-dynamicpolicies/v1/

All resource URIs in the subclauses below are defined relative to the above root URI.

The following resources and HTTP methods are supported for this API:

Table 5.4.3.1-1: Resources and methods overview

| Resource name | Resource URI | HTTP method | Meaning |
| --- | --- | --- | --- |
| New Application Instance | 3gpp-dynamicpolicies/v1/{aspId}/apps | GET | Forbidden |
|  |  | POST | Create a new “application instance” resource |
| Dynamic Policy Sessions | 3gpp-dynamicpolicies/v1/{aspId}/{appId} | GET | Read Dynamic Policy Sessions Resource |
|  |  | POST | Create a new “Dynamic Policy Session” |
| Dynamic Policy Session | 3gpp-dynamicpolicies/v1/{aspId}/{appId}/sessions/{sessionId} | GET | Read the Dynamic Policy Session Resource |
|  |  | PUT | Replace an existing Dynamic Policy Session Resource |
|  |  | PATCH | Modify an existing Dynamic Policy Session Resource |
|  |  | DELETE | Delete an existing Dynamic Policy Session Resource |

#### 5.x.3.2	Resource: New Application Instance

##### 5.x.3.2.1	Introduction

<some Intro>

##### 5.x.3.2.2	Resource definition

Resource URI: {apiRoot}/3gpp-dynamicpolicies/v1/{aspId}/apps

This resource shall support the resource URI variables defined in table 5.14.3.2.2-1.

Table 5.x.3.2.2-1: Resource URI variables for resource "New Application Instane"

| Name | Definition |
| --- | --- |
| apiRoot | See clause 5.2.4 of TS 29.122. |
| aspId | Identifier of the 5GMS Application Provider. |

##### 5.x.3.2.3	Resource Method GET

The GET method is not supported for this resource (forbidden). The Media AF shall respond with an error code.

##### 5.x.3.2.4	Resource Method POST

The POST method creates a new application instance resource for a given Media Session Handler. The Media Session Handler shall initiate the HTTP POST request message and the Media AF shall respond to the message. The Media AF shall construct the URI of the created resource using that URI.

This method shall support the URI query parameters, request and response data structures, and response codes, as specified in the Table 5.x.3.2.4-1 and Table 5.x.3.2.4-2.

Table 5.x.3.2.4-1: URI query parameters supported by the POST method on this resource

| Name | Data type | Cardinality | Remarks |
| --- | --- | --- | --- |

Table 5.x.3.2. 4-2: Data structures supported by the POST request/response by the resource

| Request body | Data type | Cardinality | Remarks |  |
| --- | --- | --- | --- | --- |
|  | ApplicationInstance | 1 | Parameters to create a application instance. |  |
| Response body | Data type | Cardinality | Response codes | Remarks |
|  |  |  | 201 Created | The application instance resource was created successfully. The URI of the created resource shall be returned in the "Location" HTTP header. |
| NOTE: The mandatory HTTP error status codes for the POST method listed in Table xxx also apply. |  |  |  |  |

#### 5.x.3.3	Resource: Dynamic Policy Sessions

##### 5.x.3.3.1	Introduction

<some Intro>

##### 5.x.3.3.2	Resource definition

Resource URI: {apiRoot}/3gpp-dynamicpolicies/v1/{aspId}/{appId}

This resource shall support the resource URI variables defined in Table 5.x.3.3.2-1.

Table 5.x.3.2.2-1: Resource URI variables for resource "Dynamic Policy Sessions"

| Name | Definition |
| --- | --- |
| apiRoot | See clause 5.2.4 of TS 29.122. |
| {aspId}/{appId} | Unique identifier, created by the Media AF during the create procedure (5.x.3.2.4). |

##### 5.x.3.3.3	Resource Method GET

The GET method allows a Media Session Handler to read own application instance resources. The Media Session Handler shall initiate the HTTP GET request message and the Media AF shall respond to the message.

This method shall support the URI query parameters, request and response data structures, and response codes, as specified in the table 5.x.3.3.3.1-1 and table 5.14.3.3.3.1-2.

Table 5.x.3.3.3-1: URI query parameters supported by the GET method on this resource

| Name | Data type | Cardinality | Remarks |
| --- | --- | --- | --- |

Table 5.x.3.3.3-2: Data structures supported by the GET request/response by the resource

| Request body | Data type | Cardinality | Remarks |  |
| --- | --- | --- | --- | --- |
|  | none |  |  |  |
| Response body | Data type | Cardinality | Response codes | Remarks |
| NOTE: The mandatory HTTP error status codes for the GET method listed in Table xxxx also apply. |  |  |  |  |

##### 5.x.3.2.4	Resource Method POST

The POST method creates a new Dynamic Policy resource for a given Media Session Handler. The Media Session Handler shall initiate the HTTP POST request message and the Media AF shall respond to the message. The Media AF shall construct the URI of the created resource using that URI.

This method shall support the URI query parameters, request and response data structures, and response codes, as specified in the Table 5.x.3.2.4-1 and Table 5.x.3.2.4-2.

Table 5.x.3.2.4-1: URI query parameters supported by the POST method on this resource

| Name | Data type | Cardinality | Remarks |
| --- | --- | --- | --- |

Table 5.x.3.2. 4-2: Data structures supported by the POST request/response by the resource

| Request body | Data type | Cardinality | Remarks |  |
| --- | --- | --- | --- | --- |
|  | ApplicationInstance | 1 | Parameters to create a application instance. |  |
| Response body | Data type | Cardinality | Response codes | Remarks |
|  |  |  | 201 Created | The application instance resource was created successfully. The URI of the created resource shall be returned in the "Location" HTTP header. |
| NOTE: The mandatory HTTP error status codes for the POST method listed in table xxx also apply. |  |  |  |  |

#### 5.x.3.4	Resource: Dynamic Policy Session

##### 5.x.3.4.1	Introduction

<some Intro>

##### 5.x.3.4.2	Resource definition

Resource URI: {apiRoot}/3gpp-dynamicpolicies/v1/{aspId}/{appId}/sessions/{sessionId}

This resource shall support the resource URI variables defined in Table 5.x.3.3.2-1.

Table 5.x.3.2.2-1: Resource URI variables for resource "Dynamic Policy Sessions"

| Name | Definition |
| --- | --- |
| apiRoot | See clause 5.2.4 of TS 29.122. |
| {aspId}/{appId} | Unique identifier, created by the Media AF during the create procedure (5.x.3.2.4). |
| {sessionId} | Unique identifier, created by the Media AF during the create procedure (5.x.3.3.4). |

##### 5.x.3.4.3	Resource Method GET

##### 5.x.3.4.4	Resource Method PUT

##### 5.x.3.4.5	Resource Method PATCH

##### 5.x.3.4.6	Resource Method DELET